3GPP TSG-RAN WG2 Meeting #128                                    R2-2410754

Orlando, USA, Nov.18~22, 2024

Agenda item:		8.2.3

Source:		Samsung

Title:			Discussion on A-IoT paging

Document for:	Discussion & Decision

- 1 Introduction

The agreements that have been made related to A-IoT paging in the last RAN2 meeting is as follows.

Agreements on AIoT paging

- The A-IoT paging message can include information to avoid duplicate response from the device to a reader.  This information should be short and simple.  FFS how to indicate.  Wait for further information and requirements from other WGs and make this decision in normative phase.

- Based on this information the device determines whether to skip sending the response to paging.

- From RAN2 perspective it is feasible to support paging multiple device IDs, however depending on TB size it may not be possible to include multiple device IDs.    RAN2 will determine the need for this after considering aspects related to RAN2 multiplexing and other WGs.

- If multiple device IDs in single paging is supported, if A-IoT paging message contains multiple device IDs, reader can configure either contention free RA or contention-based RA

Based on the agreements, in this contribution, we will further discuss remaining issues on A-IoT paging.

- 2 Discussion

Issue 1: Paging message transfer

Among the AIoT paging messages associated with the same CN request, the configuration infomration in each message may be changed. Fig. 1 shows one example on the resource configuration change for different AIoT paging messages. Specifically, the CN request is to inventory 100 devices. At the beginning, the AIoT reader may need assign a long period (i.e., period 1) to collect responses since large number of devices cause more collisions. After Period 1, 80 devices have been inventoried. Thus, the AIoT devices can assign a short period (i.e., period 2) for inventory considering the small number of remaining devices.

Fig. 1 Configuration information change among AIoT paging messages

Proposal 1-1: For the same request from CN, RAN2 is kindly asked to discuss whether the AIoT Paging messages can contain different configuration information (e.g., resource configuration) or not.

With multiple transmissions of AIoT paging message, the Reader should be able to determine when to stop its transmission. There are various reasons to stop the transmisison of AIoT paging message, e.g., the required number of devices are successfully inventoried, the time period used for inventory expires, CN requests to stop the inventory, etc. Thus, when determing to stop the transmission, the AIoT reader can send an indication to devices.

Proposal 1-2: RAN2 is kindly asked to discuss how to stop the transmission of AIoT paging message.

Issue 2: information contained in the AIoT paging message

- ID information

According to the current discusison, the paging may contain the following ID information:

- ID of a specific device

- ID of a group of device

- ID list of several devices

The above ID information comes from the CN. The issue is whether those ID information is visible to AIoT reader or not. If it is invisible to reader, the ID information can be included in the AIoT paging message as a “container”; if it is visible to reader, the CN should provide those ID information to AIoT reader explicitly.

Proposal 2-1: RAN2 is kindly asked to discuss whether the ID information for the paged device(s) is visible to AIoT reader or not.

- Resource allocation information

We have agreed to support the contention-free procedure and contention-based RA procedures:

- Contention-free RA procedure

The AIoT device can perform D2R data transmission during dedicated time domain/access occasion. The AIoT paging message can indicate such dedicated time domain/access occasion via, e.g., the sequence number of time domain/access occasion (as Fig. 1 (a)), the number of time domain/access occasion since the AIoT paging message is received by AIoT device (as Fig. 1 (b)), can be indicated via AIoT paging message.

Fig. 2 Indication of dedicated access occasion

- Contention-based RA procedure

The slotted-ALOHA-like random access can be considered as a baseline, where an A-IoT device will randomly select a time domain resource for very first D2R message transmission. The A-IoT reader can provide information assisting the determination of the time resource/random access occasion via AIoT paging message. One possible information is a parameter Q, which indicates 2Q time resource/random access occassions in total so that the A-IoT device can randomly select one resource/occasion among them for D2R transmission.

Proposal 2-2: Depending on the RA type, the AIoT paging message can include the following resource allocation information:

  - Contention-free RA: information indicating a dedicated time resource/random access occasion

  - Contention-based RA: a parameter Q indicating 2Q time resource/random access occassions in total

In addition, the frequency-domain random access resource information may be included if the FDM is applied for resource allocation. However, this depends on RAN1 progress.

Proposal 2-3: RAN2 is kindly asked to discuss whether the frequency-domain resource information can be included in the AIoT paging message to support the FDM based resource allocation.

- Reader ID

The intention of including reader ID is to support the scenarios with multiple readers in the same area. This case complexes the whole system design, and whether this is a valid case needs other WGs’ input. Considering this is the last meeting for the SI stage, RAN2 can focus on single reader case first, and the reader ID is not included in the Paging message.

Proposal 2-4: RAN2 is kindly asked to agree that the reader ID is not needed in the Paging message for SI stage.

- 3 Conclusion

Based on the discussion above, we have the following observations and proposals:

Paging message transfer

Proposal 1-1: For the same request from CN, RAN2 is kindly asked to discuss whether the AIoT Paging messages can contain different configuration information (e.g., resource configuration) or not.

Proposal 1-2: RAN2 is kindly asked to discuss how to stop the transmission of AIoT paging message.

Paging message content

Proposal 2-1: RAN2 is kindly asked to discuss whether the ID information for the paged device(s) is visible to AIoT reader or not.

Proposal 2-2: Depending on the RA type, the AIoT paging message can include the following resource allocation information:

  - Contention-free RA: information indicating a dedicated time resource/random access occasion

  - Contention-based RA: a parameter Q indicating 2Q time resource/random access occassions in total

Proposal 2-3: RAN2 is kindly asked to discuss whether the frequency-domain resource information can be included in the AIoT paging message to support the FDM based resource allocation.

Proposal 2-4: RAN2 is kindly asked to agree that the reader ID is not needed in the Paging message for SI stage.