3GPP TSG-RAN WG2 Meeting #128	R2-2410756

Orlando, USA, Nov. 18th~22nd , 2024

Agenda item:	8.13.3

Source:	Samsung

Title:	Consideration on CP and UP issues for multi-hop SL relay

WID/SID:	NR_SL_relay_enh2

Document for:	Discussion and Decision

Introduction

In August meeting, Rel-19 SL relay was firstly discussed and the following agreements on CP part were reached:

The following connections are assumed as a baseline to be needed:

-	From last Relay UE perspective:

A direct (non-relayed) PC5 connection with the first or an intermediate Relay UE, and

A direct (non-relayed) Uu connection with serving gNB, if in RRC_CONNECTED.

-	From intermediate relay UE perspective (including first relay UE):

A direct (non-relayed) PC5 connection with each of two adjacent (remote or relay) UEs, and

An end-to-end Uu connection with serving gNB, if in RRC_CONNECTED.

FFS what RRC states are supported for the intermediate relay UE.

-	From U2N Remote UE perspective:

A direct (non-relayed) PC5 connection with Intermediate Relay UE, and

An end-to-end Uu RRC connection with serving gNB, if in RRC_CONNECTED.

Agreements:

The multi-hop CP protocol stack is end-to-end for Uu-PDCP and above and hop-by-hop for SRAP and below (as in Rel-17/18).

The multi-hop UP protocol stack is end-to-end for Uu-PDCP and above and hop-by-hop for SRAP and below (as in Rel-17/18).

In this contribution, we will share our view on CP and UP aspects.

Discussion

Approach 1 vs Approach 2

During the post-e-mail discussion, the following two approaches were extensively discussed:

- Approach 1: The network needs to directly control each of the intermediate relay UEs via Uu RRC.

- Approach 2: Only the last relay UE requires control by the network via Uu RRC.

In general, compared with Approach 2, Approach 1 is more aligned with the legacy design, which means that the legacy design can be reused as much as possible. Thus, it is better to first analyse if any show-stoppers exist for Approach 1. In the following table, some analyses w.r.t. the major concern to Approach 1 are given.

|  | Comments |
| --- | --- |
| Supported scenario | Both Approach 1 and Approach 2 support the same valid scenarios. The scenario where two remote UEs under a single relay UE can connect to two different cells is not valid. How do two different remote UEs detect two different cells under the same relay UE? |
| Signaling overhead and latency | Approach 1: the signalling overhead and latency can be controllable considering the practical deployment. The signalling overhead and the latency are highly depends on the number of hops along the transmission path. In a practical deployment, multi-hop SL relay cannot be tens of hops, and the number of hops should be very limited. Then, the signalling overhead and latency for Approach 1 is not a concern. |
| E2E QoS guarantee | Approach 1: it can achieve more flexible E2E QoS guarantee. With multi-hop, the E2E QoS guarantee becomes more difficult than single hop SL relay. For IDLE/INACTIVE UE, the RLC channel configuration for QoS guarantee is pre-configured or provided by system information. Thus, it cannot be adaptive to the PC5 link quality. |
| Security issue | Approach 1: no security issue. In multi-hop SL relay, the SRAP header is handled by each intermediate node. The IDLE/INACTIVE UE is not controllable by the NW. So, whether Approach 2 has security issue or not need other WGs involvement for evaluation. |

According to the above analysis, there is no showstopper to support Approach 1. However, considering current situation between two camps, it may be difficult to conclude on one approach in the coming meeting. In our understanding, the intention of Approach 2 is to reduce the signalling overhead and latency. However, for a reasonable deployment, signalling overhead and latency are not a real problem. Thus, Approach 2 can be considered as an optimization to Approach 1. Thus, RAN2 can take Approach 1 as a baseline for the RRC connection establishment/resume/re-establishment of the remote UE.

Proposal 1-1: RAN2 is kindly asked to take Approach 1 as the baseline for the RRC connection establishment/resume/re-establishment of the remote UE.

The spirit of Approach 1 is that each node over the multi-hop SL relay should be under the control of network, which is same as single hop SL relay. Following this spirit, RAN2 can further discuss whether any optimization from Approach 2 can be considered. Technically, if the intermediate relay UE is in IDLE/INACTIVE state, the signalling interaction between intermediate relay UE and the gNB can be reduced. Especially, the INACTIVE intermediate relay UE still has the configuration from the NW side, which is regarded as a kind of controlling from the NW side. Thus, RAN2 can discuss whether to allow releasing the intermediate relay UE to INACTIVE state after the connection establishment of remote UE.

Proposal 1-2: As an optimization inspired by Approach 2, RAN2 is kindly asked to discuss whether to allow releasing the intermediate relay UE to INACTIVE state after the connection establishment of remote UE.

Failure case handling

- Remote UE

The legacy failure case handling can be reused. For example, the remote UE can suspend RLM if it is connected to gNB via relay UE; it triggers the RRC re-establishment procedure when detecting PC5 RLF.

Proposal 2-1: RAN2 is kindly asked to agree that the legacy failure handling can be reused by remote UE.

- Last relay UE

In R17, several cases will trigger the notification message transfer. In Rel-19, the last relay UE has the same functionalities as the relay UE in the single hop SL relay. Thus, the handling on the failure case at the last relay UE can reuse the legacy design (i.e., the initiation of notification message).

Proposal 2-2: RAN2 is kindly asked to agree that the legacy procedure for Notification message initiation can be reused for the last relay UE.

- Intermediate relay UE

In R17, when receiving a notification message from the relay UE, the remote UE will trigger the RRC re-establishment procedure. However, in Rel-19, multiple intermediate relay UEs are along the transmission path. Whenever there is a PC5 link problem, the whole transmission path will be interrupted. Thus, to reduce the interruption time, one of important topics for Rel-19 is how to recover the transmission path quickly when there is PC5 link problem. In general, the notification message is useful to link problem notification among intermediate relay UEs. With such message, two options can be considered:

- Option 1: E2E recovery

Whenever an intermediate relay UE has a PC5 link problem, it will send the Notification message to its child node, and such message will be forwarded to the remote UE hop-by-hop. Then, after receiving it, the remote UE can follow the legacy design, i.e., initiate RRC re-establishment procedure. This option may not be a problem for single hop relay. For multi-hop relay, it also cause large recovering time since the remote UE needs carry out re-establishment procedure.

- Option 2: Hop-by-hop recovery

To resolve the issue of Option 1, another method is that the recovering is performed hop-by-hop. Specifically, when an intermediate relay UE has PC5 link problem or receives the notification message from the parent node, it can try to recovery by itself. If a new relay UE is reselected, the transmission path from the remote UE to the gNB can be established quickly.

Proposal 2-3: RAN2 is kindly asked to discuss the following two options for failure recovery at the intermediate relay UE:

  - Option 1: E2E recovery

  - Option 2: Hop-by-hop recovery

System information

For remote UE, the SI can be transferred via the relay UEs. Moreover, the remote UE can request the SI via the relay UEs, where the relay UE sends the request SI message to the remote UE via PC5-RRC message. Such scheme can be reused for remote UE in R19 by conveying the SI message via intermediate relay UEs.

Proposal 3-1: RAN2 is kindly asked to agree that the legacy SI acquisition scheme can be reused for the remote UE in Rel-19

For the intermediate relay UE, it can reuse the SI acquisition scheme of remote UE to derive the system information as long as considering the intermediate relay UE as “remote UE” for its upstream node.

Proposal 3-2: RAN2 is kindly asked to agree that the intermediate relay UE can reuse the legacy SI acquisition scheme to obtain/request SI via the last relay UE.

Paging

In Rel-17, the Paging delivery can be realized by either the relay UE monitoring or RRC message transfer of relay UE. In Rel-19, the similar method can be applied. However, the paging delivery should be performed by last relay UE since the last relay UE is close to gNB.

Proposal 4-1: RAN2 is kindly asked to agree that the Paging delivery of remote UE is realized by the last relay UE by the ways of either Paging monitoring or RRC message transfer.

Similarly, the paging delivery for the intermediate relay UEs should be also relies on the paging monitoring or RRC message transfer at the last relay UE side.

Proposal 4-2: RAN2 is kindly asked to agree that the Paging delivery of the intermediate relay UE is realized by the last relay UE by the ways of either Paging monitoring or RRC message transfer.

QoS

In R17, the QoS split over sidelink relay is determined by gNB. Similarly, gNB can determine the QoS split for multi-hop sidelink relay since gNB has well knowledge on the PC5 link quality and traffic load of each relay UE. After determining such QoS split, the gNB can determine the configuration of PC5 RLC channel or Uu RLC channel. Thus, the QoS split is gNB implementation issue without specification impact.

Proposal 5: RAN2 is kindly asked to agree that the QoS split is determined by gNB.

SRAP

As legacy, SRAP is responsible for the operation of bearer mapping. In legacy, the relay UE realizes the mapping between Uu RLC channel and PC5 RLC channel.  Since the SRAP contains the remote UE ID and the bearer ID information, the mapping configuration in SRAP contains either egress PC5 RLC channel ID or egress Uu RLC channel ID. Moreover, with one relay UE only, the mapping configuration needn’t contain the next-node ID.

In Rel-19, the last relay UE performs the mapping between the Uu RLC channel and PC5 RLC channel. With the Remote UE ID and bearer ID in SRAP header, as legacy, the mapping configuration can indicate the egress PC5 RLC channel ID for the downlink traffic, and egress PC5 Uu RLC channel ID for the uplink traffic. Meanwhile, the downlink mapping also needs indicate the next-hop relay UE ID.

Proposal 6-1: RAN2 is kindly asked to agree that, for the last relay UE, the existing SRAP-configuration can be enhanced by adding next-hop relay UE ID for the downlink traffic.

For the intermediate relay UE, the mapping is between two PC5 RLC channels. For the uplink/downlink, the mapping should indicate egress PC5 RLC channel ID and next-hop relay UE ID. To differentiate the DL and UL mapping, the SRAP configuration should contain the mapping for DL and UL separately.

Proposal 6-2: RAN2 is kindly asked to agree that, for the intermediate relay UE, the existing SRAP-configuration can be enhanced by adding next-hop relay UE ID and configuring the mapping for DL and UL separately.

Conclusion

Proposal 1-1: RAN2 is kindly asked to take Approach 1 as the baseline for the RRC connection establishment/resume/re-establishment of the remote UE.

Proposal 1-2: As an optimization inspired by Approach 2, RAN2 is kindly asked to discuss whether to allow releasing the intermediate relay UE to INACTIVE state after the connection establishment of remote UE.

Proposal 2-1: RAN2 is kindly asked to agree that the legacy failure handling can be reused by remote UE.

Proposal 2-2: RAN2 is kindly asked to agree that the legacy procedure for Notification message initiation can be reused for the last relay UE.

Proposal 2-3: RAN2 is kindly asked to discuss the following two options for failure recovery at the intermediate relay UE:

  - Option 1: E2E recovery

  - Option 2: Hop-by-hop recovery

Proposal 3-1: RAN2 is kindly asked to agree that the legacy SI acquisition scheme can be reused for the remote UE in Rel-19

Proposal 3-2: RAN2 is kindly asked to agree that the intermediate relay UE can reuse the legacy SI acquisition scheme to obtain/request SI via the last relay UE.

Proposal 4-1: RAN2 is kindly asked to agree that the Paging delivery of remote UE is realized by the last relay UE by the ways of either Paging monitoring or RRC message transfer.

Proposal 4-2: RAN2 is kindly asked to agree that the Paging delivery of the intermediate relay UE is realized by the last relay UE by the ways of either Paging monitoring or RRC message transfer.

Proposal 5: RAN2 is kindly asked to agree that the QoS split is determined by gNB.

Proposal 6-1: RAN2 is kindly asked to agree that, for the last relay UE, the existing SRAP-configuration can be enhanced by adding next-hop relay UE ID for the downlink traffic.

Proposal 6-2: RAN2 is kindly asked to agree that, for the intermediate relay UE, the existing SRAP-configuration can be enhanced by adding next-hop relay UE ID and configuring the mapping for DL and UL separately.

References