# 3GPP TSG RAN WG2 #128									 R2-2410761

# Orlando, USA, November 18th – 22nd, 2024

# Source:	Hanbat National University

# Title:		Discussion on RRM measurement for XR enhancements

# Agenda item:	8.7.3

# Document for:	Discussion/Decision

# Introduction

The Rel-19 work item on XR Phase 3 contains the RRM measurement gaps/restrictions related enhancements in its scope [1]. as follows:

Specify enhancements to enable transmission/reception in gaps/restrictions that are caused by RRM measurements (from inter-frequency RRM measurement gaps, or intra-frequency measurements, or other scheduling restrictions etc). [RAN1, RAN2, RAN4] - Specify the corresponding measurement gap and scheduling restriction to enable the identified enhancements with RRM performance impact taken into consideration, work being triggered by LS. [RAN4]

In this regard, in order to skip a particular gap/restrictions, RAN1 made the following agreements on the explicit indication by DCI in the 3GPP RAN1#116 meeting [2]:

Agreement At least for self-scheduling case, for explicit indication by DCI to skip a particular gap/restriction, one bit indication is included as part of DCI formats 0_1/1_1 and 0_2/1_2. - FFS: Cross carrier scheduling. - FFS: DCI formats 0_3/1_3. - Explicit indication can be configured for each DCI format individually by higher layer. Agreement In case of cross carrier scheduling, for explicit indication by DCI to skip a particular gap/restriction, one bit indication included as a part of DCI formats 0_1/1_1 and 0_2/1_2 corresponds to a scheduled cell.

RAN2 made the following agreements on RRM measurement gap skipping in 3GPP RAN2#127-bis meeting [3]:

Agreements for RRM measurement gap skipping - RAN2 assumes that at least some impact on DSR from MG skipping can be avoided by NW implementation. FFS whether there is an impact which would require some specification changes/enhancements. - No need to have delay-aware LCP enhancements specific for MG skipping, i.e. MG skipping and delay-aware LCP are designed as independent features - RAN2 can further evaluate whether there is any impact on DRX from MG skipping. For the moment, the issue is unclear. - RAN2 will focus its work on supporting the solution chosen by RAN1/RAN4 RAN2 can discuss whether there is a need to additionally have other solution (e.g. RRC-based) which can be decided after RAN1/RAN4 evaluation and knowing more details of DCI-based solution

In this contribution, we discuss some considerations for RRM measurement gap/restrictions related enhancement while also considering the related RAN1 progress.

# Discussions

RAN1 made some progress on determining how to skip a particular gap/restriction for RRM measurements. It was agreed to employ explicit indication by DCI to skip a particular gap/restriction both for self-scheduling and cross-scheduling cases. In addition, including one bit indication in the DCI formats 0_1/1_1 and 0_2/1_2 was also agreed. Explicit indication by DCI can provide some flexibility needed for the XR traffic support such as packet arrival time, packet size, scheduling, etc. uncertainty handling.

Although the use of DCI is decided by RAN1 for indicating whether to skip a measurement gap, the details are not decided, e.g., whether DCI bit one indicates to skip or not to skip, etc. In addition, it is still unclear which part of L2 is impacted, e.g., DSR, DRX, etc.

Proposal 1: Continue to discuss potential RAN2 impacts by the introduction of DCI-based solution to skip a particular gap/restriction for RRM measurements.

Another topic which may have potential impact on RAN2 is the user assistance information (UAI), regarding this RAN1 yet reached consensus on supporting the UAI, as seen in the conclusion made in RAN1#117 meeting:

Conclusion RAN1 does not further discuss new UE assistance information related to channel conditions, traffic, UE mobility. Conclusion There is no consensus in RAN1 to support UE assistance information related to measurements occasions. The reason for this situation is lack of consensus on the need/feasibility for UAI and lack of technical understanding on issues outside of RAN1 expertise (e.g. impact of RRM measurement performance). It is up to other working groups to trigger further work in RAN1 on UE assistance information.

There may be some benefit of using UAI, e.g., direct use of information available at UE. However, the triggering of UAI discussion can be done after RAN1 and RAN4 progress on the support of UAI.

Proposal 2: RAN2 wait discussion on UAI until other working groups (i.e., RAN1 and RAN4) make progress.

# Conclusion

In this contribution, we discussed RRM measurement gap/restrictions related enhancement. Our observations and proposal are reproduced below:

Proposal 1: Continue to discuss potential RAN2 impacts by the introduction of DCI-based solution to skip a particular gap/restriction for RRM measurements.

Proposal 2: RAN2 wait discussion on UAI until other working groups (i.e., RAN1 and RAN4) make progress.

# References

- RP-240791, “Revised WID on XR (eXtended Reality) for NR Phase 3,” Mar. 2024.

- 3GPP RAN1 Chair’s notes RAN1#118-bis, Oct. 2024.

- 3GPP RAN2 Chair’s notes RAN1#127-bis, Oct. 2024.