3GPP TSG-RAN WG2 Meeting #128											        R2-2410764

Orlando, 18-22 Nov. 2024

Agenda item:			8.8.4

Title:						Further considerations on intended broadcast service area provision

Source:					Continental Automotive

Document for:		Discussion and Decision

# Introduction

The Non-Terrestrial Networks (NTN) for NR Phase 3 work item aims at specifying support for broadcast services via NR NTN in Rel-19 [1]:

- Specify signaling of the intended service area of a broadcast service (e.g. MBS broadcast) via NR NTN [RAN2, RAN3] - Specify SIB signaling to indicate the intended service area in case the satellite footprint covers a larger area. [RAN2] - Specify the necessary signaling between CN and NG-RAN. [RAN3]

During RAN2#125bis [2] the following agreements were made:

Agreements:

- For MBS broadcast service we don’t restrict the work to any satellite constellation type

- We prioritize working on a solution for MBS broadcast but we don’t preclude other broadcast services, namely ETWS

- We will cover at least the case where the indicated intended service area covers a portion of a NTN cell

- The intended service area can cover the area of more than one NTN cells (or portions thereof)

- Can discuss next time whether the broadcast transmission can be limited to the intended service area only (i.e. no transmission happens outside of the intended serive area)

- At least the following geographical area formats to model service area can be further considered (the signalling of other information than the geographical information can be considered):

- Circles (like for TN coverage)

- Geographical area information, e.g. via polygons, to better approximate the intended shape of service area

Further, during RAN#126 [3] and RAN2#127bis [4] the following agreements were made:

Agreements:

- For MBS broadcast service, both EFC and EMC are supported.

- RAN2 will not define means for the NW to prevent the reception of the content of the service outside of the intended service area.

- MBS broadcast intended service area is provided via system information

- For MBS broadcast RAN2 considers the following possibilities for including the service area information: SIB20/ SIB21/ MBSBroadcastConfiguration. FFS for ETWS

- When intended service area (e.g., geographical area/TN coverage) is provided for MBS broadcast service, it needs to be associated with MBS session (FFS on the details)

Agreements:

1.	For each MBS service we include one or more intended service area IDs into MCCH. FFS whether the list of the intended service areas (and related IDs) is also included in MCCH or if it is provided in a new or existing SIB. We will consider possible enhancements (including enhancements left up to UE implementation) to allow UE skipping MCCH re-acquisition when UE is not within intended service area of any interested broadcast service.

In this paper, we share further considerations on intended broadcast service area provision as well as service continuity in NTN.

# 2. Discussion

NR NTN deployments promise to provide ubiquitous coverage as well as efficient access to broadcast and multimedia services. However, regional regulations or other circumstances may constrain the reception of MBS. Thus, the concept of “intended MBS service areas” was introduced to meet these requirements.

In past meetings [2], different scenarios were discussed in which MBS content reception is restricted to a portion of an NTN cell or where intended MBS service areas overlap multiple NTN cells. In our understanding MBS may be provided by the serving as well as neighbouring cell(s) in NGSO deployments, as depicted in Fig. 1.

Fig. 1 - Intended MBS service area covered by serving and neighbour cells

Moreover, satellite movements in NGSO deployments could cause the NTN cell(s) serving an intended service area to change continuously. Thus, in scenarios with earth-moving cells, the UE that is interested in receiving a particular MBS service or already established an MBS session may face service degradation, resulting in poor user experience.

Observation 1: in scenarios with earth-moving cells, which are under the scope of RAN2 agreements, NTN cell(s) serving an intended MBS service area may change constantly over time. These frequent changes in NTN cell(s) serving an intended MBS service area may result in degraded user experience.

Thus, the network should provide further guidance to the UE for MBS service selection in an intended MBS service area. For example, in addition to broadcasting the intended MBS service area, the network provides MBS estimated service times for intended MBS service areas where serving NTN cell(s) change continuously. Based on those estimated service times, the UE decides whether and when to initiate MBS service.

Proposal 1: RAN2 should further discuss MBS service continuity between neighbouring intended MBS service areas in scenarios with earth-moving cells.

Proposal 2: In addition to broadcasting the intended MBS service area, MBS service time estimates for intended MBS service areas where serving NTN cell(s) continuously change are provided by the network.

Of course, some network coordination of neighbouring NTN cells may be needed. For example, MBS resource allocation needs to be aligned between the serving and neighbour cell(s). Hence, RAN3 should be involved to clarify any network signalling impacts.

Proposal 3: RAN2 to send an LS to RAN3 regarding necessary changes to coordinate MBS provisioning for intended MBS service areas where serving NTN cell(s) continuously change.

# Conclusions

The following proposals have been made in this contribution:

Observation 1: in scenarios with earth-moving cells, which are under the scope of RAN2 agreements, NTN cell(s) serving an intended MBS service area may change constantly over time. These frequent changes in NTN cell(s) serving an intended MBS service area may result in degraded user experience.

Proposal 1: RAN2 should further discuss MBS service continuity between neighbouring intended MBS service areas in scenarios with earth-moving cells.

Proposal 2: In addition to broadcasting the intended MBS service area, MBS service time estimates for intended MBS service areas where serving NTN cell(s) continuously change are provided by the network.

Proposal 3: RAN2 to send an LS to RAN3 regarding necessary changes to coordinate MBS provisioning for intended MBS service areas where serving NTN cell(s) continuously change.

# References

- RP-234076, New WID: Non-Terrestrial Networks (NTN) for NR Phase 3, RAN1 Vice-Chair, RAN#102, December 11-15, 2023, Edinburgh, UK.

- R2-2403733, Report from Break-out session on NR-NTN and IoT-NTN, Session Chair (ZTE Corporation), RAN2#125bis, Changsha, China, April 15th – 19th, 2024.

- R2-2405703, Report from Break-out session on NR-NTN and IoT-NTN, Session Chair (ZTE Corporation), RAN2#126, Fukuoka, Japan, May 20th-24th, 2024.

- R2-2409213, Report from Break-out session on NR-NTN and IoT-NTN, Session Chair (ZTE Corporation), RAN2#127bis, Hefei, China, Oct. 14th-18th, 2024.