3GPP TSG RAN WG2 #128		R2-2410779

Orlando, US, Nov 18th  – 22th , 2024

Source:	CEWiT

Title:	Discussion on A-IOT paging procedure

Agenda Item:	8.2.3

Document for:	Discussion and Decision

- Introduction

In RAN#102 meeting, a new SI for Ambient IoT in NR has been approved [1]. The following RAN2-led objectives have been identified from the SID.

- RAN2-led: - Study and decide which functions are needed for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission, and study those functions. For example: - Paging - Random access - Data transmission, including necessary radio resource control aspects, respecting the limitation in the General Scope - Interactions with upper layers For functionalities not listed above, they are studied only if found essential.

In RAN2#125bis [2] meeting, RAN2 made the following agreements on paging.

Agreements

- Legacy paging message for device will not be supported.

- Legacy paging occasion and legacy DRX for the device is not supported.  This doesn’t preclude solutions that address device monitoring (taking into account discussions from RAN1 as well).

- RAN2 assumes that the device will not support tracking/RAN area update procedure.

- For the case of reaching single or group of devices, an identifier may be required to identify the device/group of devices in the trigger message.    FFS pending the details from SA2

In RAN2#126 [3] meeting, RAN2 made the following agreements on paging.

Agreements 1 RAN2 will study the following cases for AIoT paging message: - a message containing an ID of a single A-IoT device. - a message containing a group ID that maps to multiple A-IoT devices. - a message that does not contain an ID, i.e., addressed for all devices that can receive the AIoT message. - a message containing multiple IDs of A-IoT devices. Need to confirm the need for this use case based on SA2 discussion. What device ID and group ID and scenarios is depending on SA2 discussion. 2 AIoT paging message indicate information from which the device can determine resources to be used for response (D2R message). FFS how (e.g. implicit/explicit/configured/preconfigured) and what resources (dedicated and/or shared) are provided to the device taking into account RAN1 discussion. 3 From RAN2 perspective, we assume the device can receive as long as there is enough energy. We will wait for RAN1 further progress on device monitoring details.

In RAN2#127 [4] meeting, RAN2 made the following agreements on paging.

Agreements 1 Support mechanism for reader to be able to trigger multiple/subsequent AIoT paging messages that are associated with the same request from the CN. Study how to avoid duplicated response from devices - wait for further RAN1 progress on indication of the start of access occasion.

In RAN2#127bis [5] meeting, RAN2 reached following agreements on paging.

Agreements 1 The A-IoT paging message can include information to avoid duplicate response from the device to a reader. This information should be short and simple. FFS how to indicate. Wait for further information and requirements from other WGs and make this decision in normative phase. - Based on this information the device determines whether to skip sending the response to paging. 3 From RAN2 perspective it is feasible to support paging multiple device IDs, however depending on TB size it may not be possible to include multiple device IDs. RAN2 will determine the need for this after considering aspects related to RAN2 multiplexing and other WGs. - If multiple device IDs in single paging is supported, if A-IoT paging message contains multiple device IDs, reader can configure either contention free RA or contention-based RA - For all use cases, device determines the random access type from the paging message. FFS whether it is explicitly or implicitly. FFS whether we will down selection for all RA types (2step, 3step, CFRA) or try to unify the design for 2step, 3step

In this contribution, we discuss the contents, monitoring and other aspect for paging message for A-IoT.

- Discussion

2.1 Contents of A-IoT paging message

RAN2 agreed for the following agreement for AIOT paging contents.

1 RAN2 will study the following cases for AIoT paging message: - a message containing an ID of a single A-IoT device. - a message containing a group ID that maps to multiple A-IoT devices. - a message that does not contain an ID, i.e., addressed for all devices that can receive the AIoT message. - a message containing multiple IDs of A-IoT devices. Need to confirm the need for this use case based on SA2 discussion. What device ID and group ID and scenarios is depending on SA2 discussion.

Upon receiving a service request, the reader initiates communication by sending an A-IoT paging message that signals the device(s) required to respond. This paging message includes the identifiers of the targeted devices, which are crucial for ensuring that only the intended A-IoT devices are alerted and can respond accordingly. In addition to the device identifiers, the paging message can carry supplementary information to enhance the communication process. This may include the type of RACH to be used, the type of service requested, resource configurations for transmission of Msg1 and/or Msg3, Furthermore, the message may offer guidance on generating a random number (Q) or a random device identifier, which is essential for avoiding collisions during the RACH process and establishing unique identifiers for communication. Depending on the specific type of service being requested, additional context is provided in the paging message. For instance, if the service involves simple inventory management, the related information will support inventory tasks such as identifying or locating devices. If the service involves both inventory and command execution, the paging message will provide further instructions that enable the device to perform read/write operations as needed.

This structured approach ensures that the A-IoT device can efficiently handle service requests while minimizing communication delays and optimizing resource usage.

Proposal 1: Support the additional following contents for paging message :

- Explicit indication for type of RACH i.e. 2-step or 3-step

- Type of service i.e. inventory only or inventory and command

- Resource configuration (s) for transmission of Msg1 and/or Msg3

- Information related to random Number (Q) or random device ID Generation

- Additional indication for re-access the random access by the device

When the paging message includes the IDs of multiple devices, it must also specify the transmission resources for MSG1 for each device. Once a device decodes the paging message, it needs to identify which specific resource to use among those provided. Therefore, a clear association between each device ID and its corresponding resource must be included in the message.

Proposal 2: RAN 2 to support including mapping association indication between D2R resources and device IDs in the paging message.

2.2 Repeated paging response

In scenarios where a device receives multiple paging messages from various readers, it may end up responding to all of them. This situation can lead to unnecessary energy consumption and resource depletion. Given that A-IoT devices are typically power-constrained, it is advantageous for these devices to conserve energy and resources. One effective way to address this issue is to include the reader ID along in the paging message. The information related to the network can also be given in the network. By doing so, the device can identify and distinguish between messages from different readers. This allows the device to selectively respond to only one paging message, thus minimizing energy use and optimizing resource management.

Proposal 3: RAN 2 to support including reader ID in A-IoT paging message to avoid repeated paging response from the device.

2.3 Monitoring behaviour

In RAN2#126, it has been agreed that the device can receive as long as it has enough energy.

3 From RAN2 perspective, we assume the device can receive as long as there is enough energy. We will wait for RAN1 further progress on device monitoring details.

The monitoring of the paging message by the device is depend on the energy harvesting cycle. The paging monitoring cycle can be design based on the ON/OFF harvesting cycle. The energy harvesting related issues still not yet fully discussed in the RAN . Therefore, it is better to wait until the RAN1 has progress on this.

The ability of a device to monitor paging messages is closely linked to its energy harvesting cycle, which dictates how it conserves and utilise energy. A well-structured paging monitoring cycle can be designed around an ON/OFF harvesting pattern. In this approach, the device alternates between periods of active energy harvesting (ON) and periods of inactivity or sleep mode (OFF). During the ON phase, the device collects energy from its environment, while in the OFF phase, it conserves energy to maximise its operational lifespan.

Implementing this ON/OFF cycle allows the device to optimise its energy usage while still being responsive to incoming paging messages. However, it is essential to note that the specific issues related to energy harvesting have not yet been fully explored within the RAN 1.

Given the current state of discussions surrounding energy harvesting in the RAN 1, it would be better to wait for further advancements and recommendations from the RAN1 before finalising any design strategies.

Proposal 4: Postpone the discussion on monitoring A-IoT paging messages until RAN1 provides an update on its progress.

- Conclusion

In this contribution, we provide our views on aspects of paging message for Ambient IoT. The proposals are summarized as follows.

Proposal 1: Support the additional following contents for paging message :

- Explicit indication for type of RACH i.e. 2-step or 3-step

- Type of service i.e. inventory only or inventory and command

- Resource configuration (s) for transmission of Msg1 and/or Msg3

- Information related to random Number (Q) or random device ID Generation

- Additional indication for re-access the random access by the device

Proposal 2: RAN 2 to support including mapping association indication between D2R resources and device IDs in the paging message.

Proposal 3: RAN 2 to support including reader ID in A-IoT paging message to avoid repeated paging response from the device.

Proposal 4: Postpone the discussion on monitoring A-IoT paging messages until RAN1 provides an update on its progress.

# References

- RP-234058, New SID: Study on solutions for Ambient IoT (Internet of Things) in NR, Huawei (moderator, RAN1 Vice-Chair).

[2]  RAN2#125-bis Chair Notes.

[3]  RAN2#126 Chair Notes.

[4]  RAN2#127 Chair Notes.

[5]  RAN2#127-bis Chair Notes.