3GPP TSG RAN WG2#128	R2-2410785

Orlando, USA, 18th - 22nd November 2024

Agenda Item:	8.7.4.2

Source: 	Meta

Title:  	Discussion on DSR Enhancements

Document for:	Discussion and Decision

# Introduction

In the Revised WID on XR (eXtended Reality) for NR Phase 3, one of the objectives is to specify enhancements for scheduling [1]:

- Specify Enhancements for Scheduling, as follows: - For the UL, Study and if justified, Specify enhancements using delay/deadline information, for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs. [RAN2]. - Note: LCP implementation complexity need to be taken into account when evaluating solutions. - Note: Check in RAN#105

In RAN2 #126-bis meeting, the following agreements were made on DSR enhancements:

Agreements on DSR enhancements - We do not change the definition of delay-critical data. - For the sake of RAN2 discussions, we use the following terms: triggering threshold, reporting threshold(s). - Companies should analyse the impact of setting the triggering threshold to value lower than largest reporting threshold on DSR procedure, e.g. triggering, cancellation etc. - For Rel-19 DSR, the buffered data is divided into multiple portions based on the multiple reporting time threshold levels configured for an LCG. The Rel-19 DSR indicates the following information for each portion for which BS>0: - Buffer size of data volume in each portion - Shortest remaining time among PDCP SDUs buffered in each portion. - There is no need to include PSI in the enhanced DSR MAC CE. - A one-bit indication may indicate whether a certain/further pair of remaining time information and buffer size information is present in the new DSR MAC CE for the associated LCG. - FFS whether old and new DSR can be configured/used at the same time or we always use a new DSR in case there is at least one LCG configured with multiple reporting thresholds.

In this paper, we discuss our views on the DSR enhancements.

# Discussion

Release 18 introduced the Delay Status Report (DSR) to provide feedback on buffered data remaining time when a UE requests uplink resources, allowing gNB to perform delay-aware uplink scheduling. However, a limitation is that when multiple PDU sets in a LCG have different remaining times, only the shortest remaining time below the threshold is reported. To address this, consider enhancing DSR to report remaining time with finer granularity and potentially additional information such as multiple pairs of remaining time and buffer details. Meanwhile, it is important for DSR to avoid introducing excessive signal overhead.

In the RAN2 meeting, the inclusion of non-delay critical data that are in front of the delay critical data in the DSR data volume calculation is discussed. We think this can be addressed by UE implementation and there is no need to include non-delay critical data in the DSR data volume calculation.

Proposal 1:  No need to include non-delay critical data in the DSR data volume calculation.

# Conclusion

In this paper, we discuss our views on the DSR enhancements.

Proposal 1:  No need to include non-delay critical data in the DSR data volume calculation.

# References

[1] RP-241771, “Revised WID on XR (eXtended Reality) for NR Phase 3” Melbourne, Australia, September 9-12, 2024