3GPP TSG-RAN WG2 Meeting #128		R2-2410790

Orlando, US, 18th Nov. – 22nd Nov. 2024

Agenda Item:	8.5.4

Source:	vivo

Title:	Discussion on adaptation of common signal transmissions

Document for:	Discussion and Decision

Introduction

In RAN2#127bis meeting, adaptation of paging occasion for network energy saving was further discussed and the agreements below were achieved [1]:

Agreements on paging adaptation 1. Select option-b as baseline for R19 NES paging enhancement. 2. R2 should aim at signaling overhead minimization (e.g., Ns=8 and FFS for other larger values) 3. Allowing legacy and R19 UEs to co-ex in the same PF/PO is possible, based on NW configuration. 4. Legacy UE is not barred. 5. Rel-19 UEs only monitor the PO(s) according to Rel-19 paging configuration.

Moreover, RAN1 and RAN4 had made some progress on the PRACH adaptation and SSB adaptation in time domain. In this contribution, we will further provide our consideration on the adaptation of paging occasions in time domain, solutions for PRACH adaptation and answer to the LS of RAN4 for SSB adaptation.

Discussion

## 2.1 Adaptation of paging occasions in the time domain

- The value N in one paging cycle of option b

Agreement in RAN2#125bis 1. For adaptation of paging occasions in time domain, RAN2 to study a) bundle paging frames and b) extend the values of N to have increased interval between PFs (e.g. T/64, T/128 ...) and compensating decrease in number of PFs by increasing POs per PF. Agreement in RAN2#127bis 1. Select option-b as baseline for R19 NES paging enhancement.

In RAN2#127bis meeting, RAN2 agreed on option b as a baseline for Rel-19 paging enhancement. For option b, the first issue is how to extend the value N (number of total PF(s) in T). In the table below, the legacy minimum N is 1/16T. For N = 1/16T, the network can configure at least {2, 4, 8, 16} legacy PFs respectively in each DRX cycle for {T=32 rf, T=64 rf, T=128 rf, T=256 rf} with the legacy paging configuration.

| N T | oneT | 1/2T | 1/4T | 1/8T | 1/16T | 1/32T | 1/64T | 1/128T | 1/256T |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| T = 320 ms | 32 | 16 | 8 | 4 | 2 | 1 |  |  |  |
| T = 640 ms | 64 | 32 | 16 | 8 | 4 | 2 | 1 |  |  |
| T = 1280 ms | 128 | 64 | 32 | 16 | 8 | 4 | 2 | 1 |  |
| T = 2560 ms | 256 | 128 | 64 | 32 | 16 | 8 | 4 | 2 | 1 |

Table 1: the number of total PF(s) in T.

For Rel-19 paging adaptation, we think that the network should support configuring at least one PF for different T that the specification supports (e.g. T= 32 rf, 64 rf, 128 rf, 256 rf) to achieve greater NES gain. Therefore, new N values {1/32T, 1/64T, 1/128T, 1/256T} should be supported (highlight in table 1). However, for N value {1/64T, 1/128T, 1/256T}, if configured, the network should ensure all the UEs in the NES cell have at least one PF per DRX cycle. For example, if N = 1/64T is configured, the minimum T among all UEs in cell should not be less than 64 rf, which can rely on RAN and CN coordination.

Proposal 1: Support new N value {1/32T, 1/64T, 1/128T, 1/256T} for option b.

- Signalling overhead of option b

Agreement in RAN2#127bis 2. R2 should aim at signaling overhead minimization (e.g., Ns=8 and FFS for other larger values)

In RAN2#127bis meeting, RAN2 has agreed to study the signalling overhead of option b. In the table below, we analyze the signaling overhead of option b when SCS is 30kHz, {T=32 rf, N=1/32T} and {T=64 rf, N=1/64T} for instance.

| SCS=30kHz T=320ms N=onethirtysecondT | firstPDCCH-MonitoringOccasionOfPO=32 radio frames =320 sub frames =640 slots =8960 symbols The number of bits used for a PO offset: 14 bits |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| The number of POs of one PF | Ns =1 | Ns =2 | Ns = 4 | Ns = 8 | Ns =16 | Ns = 32 | Ns = 64 | Ns = 128 |
| The signaling of firstPDCCH-MonitoringOccasionOfPO | 14 bits | 28 bits | 56 bits | 112 bits | 224 bits | 448 bits | 896 bits | 1792 bits |
| SCS=30kHz T=640ms N=onesixtyfouthT | firstPDCCH-MonitoringOccasionOfPO=64 radio frames =640 sub frames =1280 slots =17920 symbols The number of bits used for a PO offset: 15 bits |  |  |  |  |  |  |  |
| The number of POs of one PF | Ns=1 | Ns =2 | Ns =4 | Ns =8 | Ns =16 | Ns =32 | Ns =64 | Ns =128 |
| The signaling of firstPDCCH-MonitoringOccasionOfPO | 15 bits | 30 bits | 60 bits | 120 bits | 240 bits | 480 bits | 960 bits | 1920 bits |

Table 2: signaling overhead of option b.

As the number of POs increases, the bits of firstPDCCH-MonitoringOccasionOfPO will be increased to 14*x or 15*x from the examples we listed, where x is the number of POs in one PF. When the number of POs in one PF is greater than or equal to 8, the signaling overhead of firstPDCCH-MonitoringOccasionOfPO has exceeded hundreds of bits. However, the maximum SIB1 size is only 2976 bits. We think that the hundreds of signaling overhead caused by firstPDCCH-MonitoringOccasionOfPO is unacceptable.

TS 38.331 NOTE 2: The physical layer imposes a limit to the maximum size a SIB can take. The maximum SIB1 or SI message size is 2976 bits.

Observation 1: When the number of POs in one PF is greater than or equal to 8, the signaling overhead of firstPDCCH-MonitoringOccasionOfPO has exceeded hundreds of bits.

Regarding the above issue, we proposed during last meeting that it could be solved by reducing the maximum number of POs because paging adaptation is assumed to be applied for low load scenario. In last meeting, RAN2 agreed that the number of POs includes at least 8, and FFS for other larger values. For legacy case, the minimum total number of POs (N*Ns in one T) that can be supported are {2, 4, 8, 16} when T = {32 rf, 64 rf, 128 rf, 256 rf}, N = 1/16T and Ns = 1. For NES scenario, we think that it should at least support the legacy minimum total number of POs in one PF for different T. Thus, considering the signaling overhead issue, we propose the Ns values can be extended to 8 and 16.

Proposal 2: Support new Ns value = {8, 16} for option b.

However, as we have observed above, when the number of POs in one PF is greater than or equal to 8, the signaling overhead of firstPDCCH-MonitoringOccasionOfPO has exceeded hundreds of bits. We think that the hundreds of signaling overhead caused by firstPDCCH-MonitoringOccasionOfPO is unacceptable. Therefore, the signaling overhead of firstPDCCH-MonitoringOccasionOfPO can also be optimized. From our understanding, the easiest way is to restrict the maximum value range of firstPDCCH-MonitoringOccasionOfPO. For different SCS and N, the new value range can be set to (0…x) for which x can be predefined, i.e.,

- firstPDCCH-MonitoringOccasionOfPO-v19xy   CHOICE {

- SCS15KHZonethirtysecondT    SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..X1),

- SCS30KHZonethirtysecondT                   SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..X2),

- SCS60KHZonethirtysecondT                                    SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0.. X3)

- SCS120KHZonethirtysecondT        SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0.. X4),

- SCS480KHZonethirtysecondT     SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0.. X5)

- }      OPTIONAL,           -- Need R

Proposal 3: RAN2 to discuss whether to restrict the maximum value range of firstPDCCH-MonitoringOccasionOfPO for different SCS and N to a smaller value range compared to the theoretical maximum value range at stage-3 specification, so that the bits used for indicating firstPDCCH-MonitoringOccasionOfPO can be reduced.

- UE capability reporting

For paging adaptation, the UE can report the capability of supporting paging adaptation to the gNB after registration with AMF, and the gNB can report the capability to the AMF. After receiving this capability from the UE, AMF can forward this capability to the anchor gNB for RAN paging. The AMF forwards this capability information to all gNBs in the latest TA of the UE for CN paging. Similarly, the anchor gNB can also forward this capability information to other gNBs for RAN paging. Then, the serving gNB can identify whether the paging is for a UE supporting paging adaptation or not based on the exchanged UE capability, which assists the network to determine the paging location. For example, when the network wants to page a UE that supports paging adaptation, if the network is not aware of whether paging adaptation is supported by this UE, the NW will not know whether to apply legacy configuration or the NES configuration. Thus, we think the capability of supporting paging adaptation should be introduced.

Proposal 4: A new UE capability is defined for the NES-capable UEs to indicate the support of paging adaptation.

## 2.2 Adaptation of SSB in time domain

Agreements: - RAN4 to discuss whether there are any impacts on at least the following RRM requirements due to SSB adaption: - L1-RSRP for serving cell - L1-SINR - BFD - CBD - SCell activation - TCI state switching - NR intra-frequency L3 measurement - NR inter-frequency L3 measurement However, L3 measurement requirement in RAN4 is defined based on SMTC configuration rather than SSB configuration in legacy. RAN4 would like to ask RAN2 that whether there is any SMTC impact due to SSB adaptation.

From the above question in RAN4’s LS [2], RAN4 asked RAN2 whether there is any SMTC impact due to SSB adaptation. Currently, UE performs L3 measurement based on the SMTC configured to the UE. Considering the adaptation of SSB, there are two options to be considered:

- Option 1: The network ensures that the SMTC periodicity is larger than the periodicity of adapted SSB. There is no SMTC impact due to SSB adaptation.

- Option 2: SMTC periodicity can adapt with the SSB periodicity.

For option 1, the network can ensure that the UEs always has SSB during measurement. If the periodicity of adapted SSB is larger than SMTC periodicity, the network can reconfigure the SMTC periodicity . For example, if there is a cell, its SSB periodicity adapts from 20ms to 80ms, the network can configure SMTC to 80ms directly. There is no SMTC impact due to SSB adaptation. For option 2, multiple SMTC configurations need to be introduced based on the SSB periodicity, which will introduce extra signalling overhead. In addition, if the SMTC is for neighbour cell measurements and the neighbour cell supports SSB adaptation, it needs target gNB dynamically informs serving gNB the SSB periodicity, which will also have extra delay and signalling overhead. Thus, option 1 is the easiest way that we prefer.

Proposal 5: Send LS reply to RAN4 to inform there is no SMTC impact due to SSB adaptation, e.g. the network ensures that the SMTC periodicity is larger than the periodicity of adapted SSB.

## 2.3 Adaptation of PRACH resources in time domain

- Legacy UE handling

To achieve the NES purpose in PRACH adaptation, the network could configure legacy PRACH resources sparsely to reduce the monitoring time between legacy ROs when the system load is low. When the number of camping UEs increases a lot, additional NES-related PRACH resources with shorter period can be activated for RACH offloading. Meanwhile, the legacy UEs can still perform random access procedure with the legacy PRACH resources. The following note in the WID [3] for adaptation of common signal/channel transmissions also points out that there shall be no negative impact on legacy UE. Since we do not see significant benefits to bar the legacy UEs, we propose RAN2 to not introduce cell barring mechanism for PRACH adaptation.

- Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown.

Proposal 6: The NW does not bar legacy UEs for the purpose of PRACH adaptation.

- RACH triggering events

Currently, a number of triggering events for random access are specified in TS38.300 as following:

The random access procedure is triggered by a number of events: - Initial access from RRC_IDLE; - RRC Connection Re-establishment procedure; - DL or UL data arrival, during RRC_CONNECTED or during RRC_INACTIVE while SDT procedure (see clause 18.0) is ongoing, when UL synchronisation status is "non-synchronised"; - UL data arrival, during RRC_CONNECTED or during RRC_INACTIVE while SDT procedure is ongoing, when there are no PUCCH resources for SR available; - Handover, except for when RACH-less HO is configured; - SR failure; - Explicit request by RRC upon synchronous reconfiguration; - RRC Connection Resume procedure from RRC_INACTIVE; - To establish time alignment for a primary or a secondary TAG; - Request for Other SI (see clause 7.3); - Beam failure recovery; - Consistent UL LBT failure on SpCell; - SDT in RRC_INACTIVE (see clause 18); - Positioning purpose during RRC_CONNECTED requiring random access procedure, e.g., when timing advance is needed for UE positioning; - Early UL synchronization with an LTM candidate cell; - RACH-based LTM cell switch.

RAN2 needs to discuss whether all the above triggering events can be supported for additional PRACH resources. In RAN1#116bis [4], RAN1 had agreed to support additional PRACH resources for RRC_IDLE/INACTIVE and RRC_CONNECTED UEs, so the corresponding triggering events of RRC_IDLE/INACTIVE and RRC_CONNECTED should be considered. For the events like “handover”, “Early UL synchronization with an LTM candidate cell” and “RACH-based LTM cell switch”, even if the PRACH resources are configured for target cells, the function of the additional PRACH resources is no different from that of the legacy PRACH resources, since the random access in additional PRACH resources is only to reduce the PRACH congestion when the legacy PRACH resources may be configured sparsely for NES. Therefore, the PRACH procedure for all triggering events of legacy PRACH resources should also be supported for additional PRACH resources.

RAN1#116bis Agreement Support adaptation mechanisms of PRACH in time-domain for following: - UE in idle/inactive mode - UE in connected mode

Proposal 7: The PRACH procedure for all triggering events of legacy PRACH resources should also be supported for additional PRACH resources.

Conclusion

Based on the discussion, we have the following proposals:

Adaptation of paging occasions in the time domain

Proposal 1: Support new N value {1/32T, 1/64T, 1/128T, 1/256T} for option b.

Observation 1: When the number of POs in one PF is greater than or equal to 8, the signaling overhead of firstPDCCH-MonitoringOccasionOfPO has exceeded hundreds of bits.

Proposal 2: Support new Ns value = {8, 16} for option b.

Proposal 3: RAN2 to discuss whether to restrict the maximum value range of firstPDCCH-MonitoringOccasionOfPO for different SCS and N to a smaller value range compared to the theoretical maximum value range at stage-3 specification, so that the bits used for indicating firstPDCCH-MonitoringOccasionOfPO can be reduced.

Proposal 4: A new UE capability is defined for the NES-capable UEs to indicate the support of paging adaptation.

Adaptation of SSB adaptation in the time domain

Proposal 5: Send LS reply to RAN4 to inform there is no SMTC impact due to SSB adaptation, e.g. the network ensures that the SMTC periodicity is larger than the periodicity of adapted SSB.

Adaptation of PRACH resources in time domain

Proposal 6: The NW does not bar legacy UEs for the purpose of PRACH adaptation.

Proposal 7: The PRACH procedure for all triggering events of legacy PRACH resources should also be supported for additional PRACH resources.

# 4. Reference

- RAN2#127bis meeting minutes

- R4-2416911, LS on SSB adaptation, Apple.

- RP-242354, Revised WID: Enhancements of network energy savings for NR, Ericsson.

- RAN1#116bis meeting minutes