3GPP TSG RAN WG2 #128		R2-2410794

Orlando, USA, Nov 18th – 22nd, 2024

Source: 	Fujitsu

Agena item:	8.11.2

Title:	Discussion on random access in SBFD

Document for:	Discussion and decision

Background

In RAN2#127-bis meeting, RAN2 made some agreements as follows [1].

- Agreements on random access in SBFD RAN2 understand that if additional RO is selected by SBFD-aware UE, early identification via Msg1 is possible from NW point of view for this UE without specification impact. From R2 point of view, there is no need to introduce SBFD as a new feature combination in the current PRACH preamble partitioning framework. Upon initiation of CBRA RACH procedure for a SBFD-aware UE, UE selects one type of ROs between legacy-ROs and additional-ROs based on certain specified/configured conditions/prioritizations, if no additional indication (FFS if there needs to be any) is from network. For the PRACH transmission re-attempt in one RACH procedure, after certain (configured) number of times of RACH attempt in SBFD RACH occasions, UE is allowed to switch to legacy RACH occasions. FFS about the case when UE select legacy ROs first. The following two RACH configuration options are considered for SBFD based random access: - Option 1: Use one single RACH configuration based on the existing parameters of the single RACH configuration. Can extend the existing parameters if needed. - Option 2: Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration For RACH configuration Option 2, RAN2 needs to specify RRC signalling for the new SBFD based RACH configuration with a new set of parameters. The RACH configuration for SBFD is transmitted via SIB1. FFS dedicated RRC signalling detail. FFS whether NW can provide both configurations.

In this contribution, we would like to discuss and provide our views on PRACH re-transmission in SBFD.

Discussion

For PRACH re-transmission (PRACH transmission re-attempt in above agreement), there is an ambiguity on the condition of “certain (configured) number of times of RACH attempt”. In our understanding, there are two counters to count the number of PRACH (re-)transmission for determining condition of switching from the SBFD RACH occasion to the legacy RACH occasion. The first one is PREAMBLE_TRANSMISSION_COUNTER which is used for counting preamble transmission and the second one is PREAMBLE_POWER_RAMPING_COUNTER which is used for calculating PRACH transmission power when power ramping is enabled (clause 5.1.3 in TS 38.321) [2].

- set PREAMBLE_RECEIVED_TARGET_POWER to preambleReceivedTargetPower + DELTA_PREAMBLE + (PREAMBLE_POWER_RAMPING_COUNTER – 1) × PREAMBLE_POWER_RAMPING_STEP + POWER_OFFSET_2STEP_RA;

Basically, both counters increment corresponding to preamble transmissions. The PREAMBLE_TRANSMISSION_COUNTER increments whenever the preamble is transmitted while the PREAMBLE_POWER_RAMPING_COUNTER does not increment when there is no notification from Layer 1. Based on this fact, the selection of the appropriate counter is important for avoiding CLI in SBFD symbols, which is the rationale for switching from SBFD RACH occasion to legacy RACH occasion. Precisely, for the case that the PREAMBLE_TRANSMISSION_COUNTER is used in counting the certain (configured) number of times of RACH attempt, switching from SBFD RACH occasion to legacy RACH occasion may be occurred even if PRACH transmission power is not changed in RACH procedure (i.e., CLI is not exactly reflected in RACH procedure). On the other hand, as aforementioned, since the power ramping procedure is performed based on the PREAMBLE_POWER_RAMPING_COUNTER, the optimized switching from SBFD RACH occasion to legacy RACH occasion can be achieved by consideration of CLI, as a result, the performance of PRACH reception at the gNB side is able to improve. Therefore, we propose to support PREAMBLE_POWER_RAMPING_COUNTER to count the certain (configured) number of times of RACH attempt for switching RACH occasion.

Proposal 1: RAN2 to support PREAMBLE_POWER_RAMPING_COUNTER to count the certain (configured) number of times of RACH attempt for switching condition from SBFD RACH occasion to legacy RACH occasion.

In addition to proposal1, the subsequent issue is whether the value of counter is reset or not after switching to legacy RACH occasion because RAN1 agreed that the PRACH power control parameter between SBFD symbols and non-SBFD symbols may be different regardless of SBFD PRACH configuration types, say Option 1 and 2 that Option 1 is sharing RACH configuration with the legacy one and the related RAN1 agreement is excerpted in below[3], and Option 2 is independent RACH configuration from legacy one, which is able to configure different PRACH power control parameter from legacy one.

Agreement

For RACH configuration Option 1, support separate power control parameters for PRACH transmission in SBFD symbols and non-SBFD symbols

-	FFS: Details of the separate power control parameters

If the RACH occasion switching is performed with the same value of the counter, it may significantly increase PRACH transmission power because PRACH power control parameters in SBFD symbols is configured to relatively lower value compared with PRACH power control parameters in legacy symbols considering CLI. Besides, SSB-to-RO mapping rule and RO grouping for PRACH repetition may be different between SBFD RACH occasions and legacy RACH occasions. Hence, we propose to restart RACH procedure from the RACH initialization after switching from SBFD RACH occasion to legacy RACH occasion for initializing counters and adopting different RACH configuration.

Proposal 2: After switching SBFD RACH occasion to legacy RACH occasion, RACH procedure should be initialized.

Conclusion

In this contribution, the following proposals were made:

Proposal 1: RAN2 to support PREAMBLE_POWER_RAMPING_COUNTER to count the certain (configured) number of times of RACH attempt for switching condition from SBFD RACH occasion to legacy RACH occasion.

Proposal 2: After switching SBFD RACH occasion to legacy RACH occasion, RACH procedure should be initialized.

References

- R2_127b R18 MIMO MUSIM R19 MIMO LPWUS SBFD (Erlin) 2024-10-17 1710

- TS 38.321

- Chair’s note RAN1#118bis