3GPP TSG-RAN2 Meeting #128	R2-2410800

Orlando, USA, Nov18th –22nd, 2024

Source: 			ZTE Corporation

Title: 	Discussion on generalization aspects and simulation assumption

Agenda item:	8.3.5

Document for: 	Discussion and Decision

- Introduction

In the last meeting, we have agreed the following agreement:

- Discussion on what (type of information)/how generalization study can take place in Nov. meeting

In this document, we’d like to share our views on the generalization issue. Besides, in the [POST127bis][022][AI mobility], companies discussed the methodology and simulation assumptions for measurement event prediction, RLF prediction and system level simulation. However, there are some controversial questions. We’d like also provide our views on these questions in this document.

- Discussion

## 2.1 Generalization

Generalization reflects whether one model is generalizable to different scenarios/configurations, and is one of the important aspects to verify the performance of AI/ML. In the AI-PHY study, generalization performance is considered as one KPI to evaluate AI model performance, as follows:

Common KPIs (if applicable): - Performance - Intermediate KPIs - Link and system level performance - Generalization performance

In our understanding, in AI mobility study, we also need to study generalization capability of AI model. Regarding how to study generalization, the methodology in AI-PHY study can be reused. In the AI-PHY, the following cases are agreed to verify the generalization performance of AI model:

# TR 38.843 clause 6.3.1 The following cases are considered for verifying the generalization performance of an AI/ML model over various scenarios/configurations as a starting point: - Case 1: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a dataset from the same Scenario#A/Configuration#A - Case 2: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B - Case 3: The AI/ML model is trained based on training dataset constructed by mixing datasets from multiple scenarios/configurations including Scenario#A/Configuration#A and a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B, and then the AI/ML model performs inference/test on a dataset from a single Scenario/Configuration from the multiple scenarios/configurations, e.g., Scenario#A/Configuration#A, Scenario#B/Configuration#B, Scenario#A/Configuration#B. - Notes: Companies to report the ratio for dataset mixing. Number of the multiple scenarios/configurations can be larger than two. - The following case for generalization verification, can be optionally considered by companies: - Case 2A: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model is updated based on a fine-tuning dataset different than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B. After that, the AI/ML model is tested on a different dataset than Scenario#A/Configuration#A, e.g., subject to Scenario#B/Configuration#B, Scenario#A/Configuration#B. - Companies to report the fine-tuning dataset setting (e.g., size of dataset) and the improvement of performance.

As we can see, four cases are considered in the AI-PHY study. However, Case 2A is a bit complex, since it is related to model update. Up to now, we have not discussed how to update an AI model yet. Besides, Case 2A is optionally considered by companies in AI-PHY study, so in order to reduce our workload, we suggest to consider Case 1-3 in the AI mobility generalization study. In our view, Case 1 can be considered as benchmark. By comparing Case 1 and Case 2/3, we can see how much performance has degraded and evaluate whether it is acceptable. So, we suggest companies report the simulation results with Case 1 and at least one of Case 2 and Case 3.

Proposal 1: Regarding how to study generalization issue, Case 1-3 in the AI-PHY study can be reused, including:

-	Case 1: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a dataset from the same Scenario#A/Configuration#A

-	Case 2: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B

-	Case 3: The AI/ML model is trained based on training dataset constructed by mixing datasets from multiple scenarios/configurations including Scenario#A/Configuration#A and a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B, and then the AI/ML model performs inference/test on a dataset from a single Scenario/Configuration from the multiple scenarios/configurations, e.g.,  Scenario#A/Configuration#A, Scenario#B/Configuration#B, Scenario#A/Configuration#B.

Companies to report the ratio for dataset mixing. Number of the multiple scenarios/configurations can be larger than two.

Proposal 2: For generalization study, companies need to report simulation results with Case 1 and at least one of Case 2 and Case 3.

For which generalization aspects can be considered, the following are considered in the AI-PHY study:

# TR 38.843 clause 6.3.1 - Scenarios: - Various deployment scenarios, e.g., UMa, UMi and others; e.g., 200m ISD or 500m ISD and others; e.g., same deployment, different cells with different configuration/assumption; e.g., gNB height and UE height; - Various outdoor/indoor UE distributions, e.g., 100%/0%, 20%/80%, and others - Various UE mobility, e.g., 3km/h, 30km/h, 60km/h and others - Configurations (parameters and settings): - Various UE parameters, e.g., number of UE Rx beams (including number of panels and UE antenna array dimensions) - Various gNB settings, e.g., DL Tx beam codebook (including various Set A of beam(pairs) and gNB antenna array dimensions) - Various Set B of beam (pairs) - T1 for measurement /T2 for prediction for BM-Case2 - Other scenarios/configurations (parameters and settings) are not precluded and can be reported

However, some of them are not applicable for AI-mobility study, e.g. outdoor/indoor UE distributions. Because we focus on the mobility study, in our view, the UE shall be outdoor. Besides, for some aspects, we only agreed one value. For example, we only agreed that UE height is 1.5m, I’m wondering whether we need to discuss and decide other candidate values. Finally, other generalization aspects also can be considered, e.g. measurement period. Because in reality, the measurement interval between two adjacent measurements are not fixed. In our understanding, the following scenario/configuration can be considered in the generalization study:

-	Scenarios:

-	Various deployment scenarios, e.g., UMa, UMi and others; e.g., 200m ISD or 500m ISD and others; e.g., same deployment, different cells with different configuration/assumption; e.g., gNB height and UE height.

-	Various UE mobility, e.g., 30km/h, 60km/h, 90km/h and 120km/h.

-	Configurations (parameters and settings):

-	Various UE parameters, e.g., number of UE Rx beams, UE antenna configuration, measurement period, L3 filtering parameter, system bandwidth;

-	Various gNB settings, e.g., the number of BS Tx beams, BS Antenna configuration.

Proposal 3: For generalization performance verification, consider the following scenario/configuration:

-	Scenarios:

-	Various deployment scenarios, e.g., UMa, UMi and others; e.g., 200m ISD or 500m ISD and others; e.g., same deployment, different cells with different configuration/assumption; e.g., gNB height and UE height;

-	Various UE mobility, e.g., 30km/h, 60km/h, 90km/h, and 120km/h.

-	Configurations (parameters and settings):

-	Various UE parameters, e.g., number of UE Rx beams, UE antenna configuration, measurement period, L3 filtering parameter, system bandwidth

-	Various gNB settings, e.g., the number of BS Tx beams, BS Antenna configuration.

Although many generalization aspects can be considered, in our understanding, it’s better that only one or two generalization aspect(s) is considered in one simulation. In this way, we can know how much the generalization aspect(s) impacts prediction performance. Furthermore, in the RRM measurement prediction, we have agreed the six scenarios with different prioritization. To reduce our simulation workload and better compare simulation results among companies, we suggest to prioritize generalization simulation in the scenario with high priority (e.g. FR2 to FR2 intra-frequency temporal domain case A).

Proposal 4: For generalization simulation for RRM measurement prediction, prioritize to consider the scenario with high priority (e.g. FR2 to FR2 intra-frequency temporal domain case A).

## 2.2 Methodology and simulation assumption for RLF measurement prediction

In the post email discussion [POST127bis][022][AI mobility] Simulation Assumptions (OPPO), companies discussed about how to perform radio link monitoring (RLM) in the simulation. In this way, companies can collect dataset for RLF event (e.g. SINR values at different time instances, RLF occurrence). The generated datasets can be used for at least baseline scenario to evaluate KPI for both direct and indirect RLF prediction. Based on the TR 36.839, the way to simulate RLM is: The SINR value(s) of radio link monitoring reference signal (RLM-RS) is measured every indication period, and the linear filtering is performed over a sliding window (evaluation period), as shown in Figure 1. Based on the filtered SINR value and Qin/Qout value, the UE can determine whether out-of-sync and in-sync indication is triggered or not.

Figure 1: Radio link monitoring model

As mentioned in post email discussion, the RLM-RS are configured by the network and can come from beam(s) that are not serving beam. Rapp proposed two options to determine the serving signal of RLM RS, we observed that almost all of companies prefer to use serving beam as RLM-RS. Generally, we are fine with serving beam, but we need to clarify how to determine the serving beam. In reality, the UE performs L1 measurement and reports the results to the network, then the network may modify the serving beam based on the reported beam results. The same logic can be applied in the simulation, i.e. L1 measurement is performed periodically, the serving beam is the best beam based on the latest measurement results, and time offset between best beam change and serving beam change can be optionally considered. However, this approach will increase simulation complexity. Another simplified way is that the serving beam is the beam with best measurement results (e.g. RSRP). However, in this way, the RLF caused by too late beam switching will not occur, which cause less RLF events are collected. For example, if interference occurs suddenly, with the first approach, the UE may not be able to switch serving beam in time, which may cause an RLF occurs; while with the simplified approach, the serving beam always can be changed in time, so the RLF may be avoided. In our understanding, each approach has its advantage and disadvantage, companies can discuss and determine which approach is used.

Proposal 5: RAN2 to discuss and decide RLM-RS (or serving beam, if agreed) is based on:

- Option 1: the latest measurement results and optionally the time offset between best beam change and serving beam change;

- Option 2: the best beam of the serving cell.

In the previous meeting, we have agreed that FR2 study will be prioritized for RLF prediction. While, in the FR2 scenario, the UE has multiple Rx beams, we are wondering which UE Rx beam is used to detect RLM-RS. In our understanding, the following options can be considered:

- Option 1: Use different UE Rx beams in order. For example, at 1st sample, UE Rx beam 1 is used; at 2nd sample, UE Rx beam 2 is used, and so on;

- Option 2: The UE use the best Rx beam to detect RLM-RS;

- Option 3: The UE performs L1 measurement periodically (similar to beam management), and selects best beam to detect Rx beam based on the latest measurement results.

In our view, option 3 is most practical, but increases simulation complexity and difficulty, since sample rate in RLF and measurement may be different. Option 2 is the simplest, but with option 2, the RLF caused by too-late beam switching will not occur, so less RLF event will be collected. The selection of used Rx beam impacts the RLF prediction accuracy. In order to better compare the simulation results among companies, we suggest to clarify which Rx beam is used to detect RLM-RS.

Proposal 6: RAN2 to discuss which Rx beam is used to detect RLM-RS in in FR2 scenario:

- Option 1: Use the different UE Rx beams in order. For example, at 1st sample, UE Rx beam 1 is used; at 2nd sample, UE Rx beam 2 is used, and so on;

- Option 2: Use the best Rx beam to detect RLM-RS;

- Option 3: UE performs L1 measurement periodically, and selects best beam to detect Rx beam based on the latest measurement results.

## 2.3 Methodology and simulation assumption for system level simulation

In the post email discussion, companies discussed handover model for FR2 temporal domain prediction case A. However, we observed that companies have different understandings. Some companies suggested it can be up to company’s implementation. However, in our understanding, it’s hard to compare the simulation results among companies without a common understanding of handover model, especially when companies’ understandings are pretty dispersed. In this section, we’d like to share our view on how to model handover procedure for temporal domain prediction case A.

Assuming a predicted A3 event at future t1 is reported at t0 and one potential real A3 event is triggered at t2, companies’ understandings of handover model mainly include:

- Understanding 1: Not consider actual measurement results:

  - If handover preparation time >= t1– t0, handover command is sent at t0 + handover preparation time;

  - Else (if handover preparation time < t1-t0), handover command is sent at t1.

- Understanding 2: Consider actual measurement results, and handover command is initiated by either actual measurement results or predicted measurement results:

  - If handover preparation time >= t1– t0, handover commend is sent at t0 + handover preparation time;

  - Else (if handover preparation time < t1-t0), the handover command is sent at t1 or t2 (depends on which one comes first).

- Understanding 3: Consider actual measurement results, and handover command is initiated by only actual measurement results

  - If handover preparation time >= t2-t0, handover command is sent at t0 + handover preparation time;

  - If handover preparation time < t2-t0, handover command is sent at t2.

- Understanding 4: Consider actual measurement results, and handover command is initiated by only actual measurement results (not consider event occurrence, but consider the time that entering condition is met):

  - If handover preparation time >= t2-TTT, handover command is sent at t0 + handover preparation time;

  - If handover preparation time < t2-t0, handover command is sent at t2-TTT.

For the understandings above, the first controversial point is whether actual measurement results can be used to trigger handover command. Our understanding is yes. When performing temporal domain prediction Case A, no measurement is reduced, the UE can get actual measurement results. Besides taking them as AI model input, of course, these results also can be used to evaluate whether the measurement event is triggered or not. Some companies have concern that this will increase evaluation complexity. However, in our view, the simulation needs to reflect reality. In our view, even if with RRM measurement prediction, there is no reason for the UE not to trigger measurement event based on the actual measurement results which have been already obtained. The second controversial point is whether only actual measurement results can trigger handover command. Our understanding is no. If only actual measurement results can trigger handover command (i.e. Understanding 3 and understanding 4), for understanding 3, the only benefit that AI can get is to skip the HO preparation time. If we assume HO preparation time is 50ms, we are wondering how much difference it can show between AI and non-AI. For understanding 4, we have not agreed to trigger measurement results reporting when the entering condition of measurement event is met in current specification. Based on the analysis above, understanding 2 is preferred.

Proposal 7: Suggest to adopt the following handover model in the system level simulation for temporal domain case A: assuming a predicted A3 event at future t1 is reported at t0 and one potential real A3 event is triggered at t2:

  - If handover preparation time >= t1– t0, handover commend is sent at t0 + handover preparation time;

  - Else (if handover preparation time < t1-t0), the handover command is sent at t1 or t2 (depends on which one comes first).

- Conclusion and proposals

In this contribution, we have the following observations and proposals:

Proposal 1: Regarding how to study generalization issue, Case 1-3 in the AI-PHY study can be reused, including:

-	Case 1: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a dataset from the same Scenario#A/Configuration#A

-	Case 2: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B

-	Case 3: The AI/ML model is trained based on training dataset constructed by mixing datasets from multiple scenarios/configurations including Scenario#A/Configuration#A and a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B, and then the AI/ML model performs inference/test on a dataset from a single Scenario/Configuration from the multiple scenarios/configurations, e.g.,  Scenario#A/Configuration#A, Scenario#B/Configuration#B, Scenario#A/Configuration#B.

Companies to report the ratio for dataset mixing. Number of the multiple scenarios/configurations can be larger than two.

Proposal 2: For generalization study, companies need to report simulation results with Case 1 and at least one of Case 2 and Case 3.

Proposal 3: For generalization performance verification, consider the following scenario/configuration:

-	Scenarios:

-	Various deployment scenarios, e.g., UMa, UMi and others; e.g., 200m ISD or 500m ISD and others; e.g., same deployment, different cells with different configuration/assumption; e.g., gNB height and UE height;

-	Various UE mobility, e.g., 30km/h, 60km/h, 90km/h, and 120km/h.

-	Configurations (parameters and settings):

-	Various UE parameters, e.g., number of UE Rx beams, UE antenna configuration, measurement period, L3 filtering parameter, system bandwidth

-	Various gNB settings, e.g., the number of BS Tx beams, BS Antenna configuration.

Proposal 4: For generalization simulation for RRM measurement prediction, prioritize to consider the scenario with high priority (e.g. FR2 to FR2 intra-frequency temporal domain case A).

Proposal 5: RAN2 to discuss and decide RLM-RS (or serving beam, if agreed) is based on:

- Option 1: the latest measurement results and optionally the time offset between best beam change and serving beam change;

- Option 2: the best beam of the serving cell.

Proposal 6: RAN2 to discuss which Rx beam is used to detect RLM-RS in in FR2 scenario:

- Option 1: Use the different UE Rx beams in order. For example, at 1st sample, UE Rx beam 1 is used; at 2nd sample, UE Rx beam 2 is used, and so on;

- Option 2: Use the best Rx beam to detect RLM-RS;

- Option 3: UE performs L1 measurement periodically, and selects best beam to detect Rx beam based on the latest measurement results.

Proposal 7: Suggest to adopt the following handover model in the system level simulation for temporal domain case A: assuming a predicted A3 event at future t1 is reported at t0 and one potential real A3 event is triggered at t2:

  - If handover preparation time >= t1– t0, handover commend is sent at t0 + handover preparation time;

  - Else (if handover preparation time < t1-t0), the handover command is sent at t1 or t2 (depends on which one comes first).

- Reference

- Report of RAN2 127bis meeting.