3GPP TSG-RAN WG2 #128	R2-2410846

Orlando, USA, 18th – 22nd November, 2024

Agenda Item:	8.1.3

Source:	Ericsson

Title:	NW-side data collection for beam management and positioning

Document for:	Discussion

# 1	Introduction

The discussion on the topic of NW-side data collection has progressed during the last RAN2 meetings. The discussion has focused on different technical aspects, such as when the UE should start/stop the NW-side data collection, how the UE should report the logged data, etc.

In this paper, we addressed the FFSs captured in the meeting notes, and further details to be discussed in RAN2.

# 2	Discussion on NW-side data collection for beam management

Related to network side data collection, the following agreements were reached during the last RAN2#12 meeting.

Agreements on NW side data collection

- Periodic logging is supported for training data collection procedure in R19

- Event-triggered data logging will be supported.  At least radio condition based event triggered logging will be supported.  FFS the details of radio condition based event.  FFS if other events are supported.

- Periodic reporting of logged data is not supported.

- On-demand reporting of the logged measurements will be specified

- UEInformationRequest/UEInformationResponse is used for on-demand reporting of AI/ML training data collection.   FFS of details of the message

- The UE can indicates the availability of logged data to the network to assist network to trigger UEInformationRequest.  FFS trigger/definition of availability indication.   and FFS how data availability indication is sent to the network.

- Low priority SRB will be used.    FFS new SRB or use of SRB4

- For data collection for both NW-sided/UE sided BM model training, at least L1-RSRPs and/or beam-IDs needs to be collected by UE.  FFS if other data needs to be collected based on RAN1 progress.

In the following, we further develop the expected technical impacts related to this topic, taking into account the above agreements and the online discussion during the last meeting.

## 2.1	Configuration aspects and content of the data collection report

In RAN2#127, it was agreed that when the network wants to perform NW-side data collection, it will transmit to the UE a measurement configuration for AI/ML-enabled features/FGs for data collection and logging of measurements. For the case of beam management, the NW may configure the UE with a specific set of CSI-RS/SSB resources specifically configured for training purposes. On such resources, the UE will then start performing layer-1 measurements and log the measurement results and any possible useful information associated to it. 
Going into more details on the necessary configuration, we know that from the point of view of the gNB, this set of CSI-RS/SSB resources used for training purposes may correspond to the set A and set B. However, unlike the case of UE-side data collection, the UE does not need to know that a certain resource set is a Set A or B, because that will be left to the NW implementation, and the UE just has to perform the same type of measurements in those sets of resources. The UE only needs to know that the purpose of this set of resources is to perform measurements for the purpose of NW-side data collection, rather than for conventional operations.

RAN2 assumes that the radio resource configuration for the purpose of NW-side data collection consists of a set of CSI-RS/SSB resources on which the UE shall perform radio measurements and the related logging.

Another aspect to be discussed is how to report the L1 measurements related to NW-side data collection purposes in case the UE does not support data logging. In this case in our view, there is no motivation to support L3 reporting. The L3 reporting framework is very different from the L1 reporting framework in UCI, and if data logging cannot be supported by the UE, it does not make sense from a technical point of view to replicate the UCI framework over L3, also considering the high specification/design complexity.

RAN2 assumes that if data logging is not supported by the UE, the L1 measurements results associated to the NW-side data collection are reported via UCI. Up to RAN1 to define the need of any enhancement.

Related to the content of the data collection report, in RAN2#127-bis, it was agreed that “at least L1-RSRPs and/or beam-IDs needs to be collected by UE”. In our view, along with such information, also the timestamp of the measurement might be beneficial, especially for the case of event-based measurements. The timestamp of a logged measurement is already included in the logged MDT report, as a tool to assist the network in retrieving NW operating conditions at the moment in which the measurement was taken by the UE. In the same way, in the context of AIML, a timestamp would allow the network to correlate a given UE radio measurement with the NW conditions/configurations experienced at the network, which is critical to ensure a proper AIML model setup.

Along with the L1-RSRP and beams IDs, the UE includes the timestamp related to the point in time in which a reported measurement was taken.

However, all the above assumptions captured in Proposal 1, Proposal 2, Proposal 3 should be confirmed by RAN1.

Ask RAN1 confirmation on the following topics, including any RAN2 related agreement:

  - Radio resource configuration via CSI measurement framework for the purpose of NW-side data collection

  - Reporting via UCI of the L1 measurements in case the data logging is not supported by the UE

  - Content of the data collection report, including at least the L1-RSRPs, the associated measured beam IDs, the timestamp of the performed measurement

A further topic discussed in RAN2#127 was on whether multiple configurations for NW-side data collection should be provided to the UE. 
On this regard, we first note that unlike the inference case, the data collection for NW-side model training does not have any requirement on latency. Hence, whenever the network wants to configure the UE to perform radio measurements for the purpose of data collection in a different set of resources, the network can simply reconfigure the UE. Even if this reconfiguration causes a delay gap in the radio measurement, this is not a problem for the case of NW-side data collection. The NW can simply de-configure the UE when data collection is no longer needed, and reconfigure it when needed, without the need for any activation/deactivation mechanism which would just cost extra standardization effort.

No need to introduce multiple configurations for NW-side data collection, and hence no need to introduce dynamic activation/deactivation mechanisms.

## 2.2	When the UE should perform/stop the radio measurements for NW-side data collection

In RAN2#127-bis, it was agreed that both periodic logging and event-triggered data logging will be supported.

For the case of periodic logging, in our view, this means that the UE will start the data logging upon receiving the data collection configuration from the network.

The UE starts the data collection upon receiving a L3 configuration for NW-side data collection.

The logging periodicity as well as the logging duration can be configurable. In particular, related to the logging periodicity, that depends on the CSI-RS/SSB configuration which should be included as part of the CSI measurement configuration. Logging duration and logging periodicity can be considered both for the case of periodic data logging and event-triggered data logging.

The logging periodicity/interval (for both periodic and event-triggered data collection) is given by the specific CSI measurement configuration.

For the stopping of data collection, it was agreed in RAN2#127 that the data collection can be stopped once the UE faces issues such as becoming power limited. However, in general, the data collection, once started, can continue for a certain configurable time (as in logged MDT), for both the case of periodic and event triggered data collection; or it can continue until certain events are fulfilled, for the case of event-triggered data collection.

Besides the case of memory limitation issues (already agreed in RAN2#127), UE stops the NW-side data collection procedure when:

  - data collection has been done for a certain configurable logging duration

  - the event that triggered the data collection is no longer fulfilled.

Related to the event-triggered data collection, the triggering conditions were discussed in RAN2#127-bis. It was eventually agreed that “at least radio condition based event triggered logging will be supported”, and it is “FFS the details of radio condition based event”. 
In our view, it is kind of straightforward to assume that the measured level of L1-RSRP can be used as triggering condition, given the BM use case at hand. However, during the last meeting, it was argued that the L1-RSRP are not filtered (unlike the L3 measurements) and their values can fluctuate, so this may lead to the UE continuously starting and stopping the data collection. Given such considerations, RAN2 can discuss whether to consider alternative logging conditions. For example, the data logging can be anchored to the L3 measurements, i.e. when the L3 measurements drop above/below certain thresholds, the UE starts/stops logging the L1-RSRP, or to other L1-driven events such as the UE starting T310 timers, or the UE identifying beam failures.

Considering the L1-RSRP level fluctuations, RAN2 to consider even-triggered conditions, based on the L3 events/measurements such as L3 filtered RSRP level, the starting of T310 timer, or on beam failures etc.

## 2.3	Design of the UEInformationResponse/Request framework for AIML

In RAN2#127-bis, it was agreed that the UEInformationRequest/UEInformationResponse is used for on-demand reporting of AI/ML training data collection, and that an availability indication will be used by the UE to signal the availability of collected data. 
The first issue to discuss is which SRB to use for the transmission of the UEInformationRequest/UEInformationResponse. The UEInformationRequest is already mapped to SRB1, so in our view SRB1 can be reused. The UEInformationResponse is instead mapped to SRB1 or SRB2 (when logged measurement information is included).

In legacy, the UEInformationRequest is mapped to SRB1, and the UEInformationResponse is mapped to SRB1 or SRB2 (when logged measurement information is included).

In last meeting, it was proposed that for the UEInformationResponse the SRB4 could be used. However, in our view this proposal is not very well motivated given that SRB4 is today used for QoE which however uses a completely different data transmission mechanisms based on RRC segmentation, and not based on the “on-demand” framework, that instead RAN2 has decided to adopt for the reporting of AIML collected data. On the other hand, SRB2 is used for the logged MDT that, as said, resembles very much the AIML use cases, both on the signalling mechanism (both based on “on-demand”) and on the prioritization (both logged MDT and AIML-related collected data can be assumed to have the same priority). 
Hence, our preference is to reuse the SRB2, or a new SRB that can be assumed to be used for all the AIML possible use cases from now on.

The UEInformationRequest requesting AIML data is transmitted via SRB1 (as any other data request contained in such message), and the UEInformationResponse is transmitted via SRB2, or via a new defined SRB (to be used for all AIML use cases).

Another FFS captured during the last meeting is the content of the UEInformationRequest/UEInformationResponse messages and the details of the availability indication. 
We first note that the UEInformationRequest/UEInformationResponse framework based on availability indication, is used also for retrieving SON and MDT-related reports. Hence, it is important to distinguish the signalling of the AIML-related use cases from the legacy use cases. This means that the availability indication should at least contain a reference to the type of data that are available for transmission, e.g. “NW-side collected data”, and in turn, the existing UEInformationRequest should be extended to include at least an indicator indicating that the gNB is requesting “NW-side collected data”.

The availability indication transmitted by the UE should contain at least a reference to the type of data that are available for transmission (e.g. “AIML NW-side collected data”), and the UEInformationRequest should contain at least a reference to the type of data that are requested (e.g. “AIML NW-side collected data”).

How the availability indication is sent to the network is also FFS. In our view, the network cannot always know when there are data available for transmission. This is particularly true for the event-triggered data collection. Additionally, the network cannot know if the UE has stopped the data collection, e.g. due to event-triggering conditions not being any longer fulfilled, or due to UE internal events such as UE memory becoming full. These are definitely information that the network should know as soon as possible, so that the network can start retrieving the data, in order to free up the UE memory and avoid that the UE needs to skip the logging of some radio measurements. In our view, it is sufficient if the UE just signals to the network when it has stopped the data collection, so that the network can start retrieving the data.

When the UE stops the data collection (e.g. due to fulfilling/unfulfilling of the event-triggered data collection, or UE internal conditions such as memory limitations), the UE informs the gNB about the availability of collected data.

In legacy, the data availability indication is transmitted in any RRC complete message, such as RRCResumeComplete, RRCReconfigurationComplete, RRCReestablishmentComplete message. However, the classical SON/MDT reports are designed to be transmitted in gNBs different than the gNB in which the measurement was configured (for the case of logged MDT), or different from the gNB in which a certain event (such as RLF) occurred (for the case of SON reports, such RLF-Reports, RA-Reports, etc). For the BM use cases however we have not agreed yet on whether inter-gNB signalling for the transmission of AIML collected data is supported. Hence, as baseline, RAN2 needs to assume that the collected data should be at least transmitted to the gNB that configured the AIML NW-side data collection. This means that we cannot limit the transmission of the availability indication exclusively to the RRC complete messages that may be transmitted just sporadically when connected to a certain gNB. For this reason, we consider that as baseline the availability indication should be transmitted via UEAssistanceInformation.

As baseline, the data availability indication is transmitted at least in the UEAssistanceInformation. Whether it can be transmitted also in RRC complete messages (as in SON/MDT) depends on the handling of collected data during mobility.

### 2.3.1	How to handle the case of collected data exceeding single RRC message size

In RAN2#127, it was agreed that no standardized RRC segmentation procedure is needed, as for the logged MDT measurements. The UE implementation can determine how many entries to include in the list radio measurements information, such that the maximum PDCP SDU size is not exceeded.
Then, with this agreement in place, RAN2 just need to discuss how to transmit the collected AIML data that could not fit into a single RRC message. Again, we can get some inspiration from the logged MDT and foresee the following options:

- In each transmitted RRC message, the UE indicates to the gNB that there are still collected data available for transmission, and then the gNB further requests the UE to transmit the next RRC message (via UEInformationRequest/Response); or

- Once the gNB requests the UE to transmit the AIML collected data, the UE continues the transmission of a certain amount of samples (as indicated by the gNB in the UEInformationRequest) in the following RRC UEInformationResponse messages. In such case, it would be useful for the network to get to know how many data are stored/left for transmission so that the gNB can allocate the necessary resources and decide when to fetch those data and how many data (e.g. taking into account cell load).

- Once the gNB requests the UE to transmit the AIML collected data, the UE continues the transmission of all the remaining samples in following RRC UEInformationResponse messages without any further request from the gNB. Also in such case, it would be useful for the network to get to know how many data are stored/left for transmission so that the gNB can allocate the necessary resources and decide when/whether to fetch those data taking into account cell load.

In case all the collected data do not fit into a single RRC message, RAN2 to discuss the following options:

  - In each transmitted RRC message, the UE indicates to the gNB that there are still collected data available for transmission, and then the gNB further requests the UE to transmit the next RRC message (via UEInformationRequest/Response) (similar to logged MDT).

  - The gNB indicates (i.e. in the UEInformationRequest) the amount of collected data the UE should transmit in the following RRC UEInformationResponse messages, or the amount of RRC UEInformationResponse messages the UE should transmit, without any further request from the gNB.

  - The gNB indicates (i.e. in the UEInformationRequest) that all the remaining collected data should be transmitted by the UE in the following RRC UEInformationResponse messages without any further request from the gNB.

As said above, in order to facilitate the gNB decisions of whether to fetch or postpone the fetching of the data, it would be beneficial if the UE could indicate as part of the availability indication, how many data the UE has collected or how many RRC messages are expected to be transmitted.

The UE indicates in the availability indication how many data or RRC samples are available for transmission.

## 2.4	UE memory capabilities

In RAN2#127, we have agreed that the UE stores the logged training data at AS layer with a minimum AS layer memory size supported by the UE. FFS on the memory size. Before discussing the minimum value of the memory size, we propose waiting for more RAN1 inputs related to the data that the UE could log and report to the gNB, so that RAN2 can do a more thorough analysis of such minimum requirement. Additionally, in our view if the UE supports more memory than this minimum memory size, that should be indicated by the UE.

RAN2 to wait RAN1 inputs on the type of data to be logged by the UE, before discussing the minimum memory size for storing collected data.

Define in a UE capability some different minimum memory values that a UE can support for the logging of collected data.

## 2.5 Handling of collected data during mobility

Several contributions submitted to the last RAN2#127-bis meeting addressed the handling of AIML collected data during mobility. The discussion so far as mainly focused on aspects related to how to configure/start/stop the data collection, and how to report the collected data. It is the natural next step to start discussing what happens at the data collected but not yet transmitted, when certain mobility-related events occur, e.g. a handover is executed, or an RLF/HOF is detected, or an RRC state transition is performed by the UE. In our view, there are mainly two alternatives:

- The UE discards/clears the collected data when performing a mobility procedure

- The UE retains the collected data, and possibly transmit them to a different node than the node configuring the data collection (e.g. the target gNB of the HO, or the gNB to which the UE reconnects/reestablishes after an RLF)

Needless to say, that the option 2 above is certainly preferrable from a technical point of view, because it allows the source gNB to retrieve all the collected data, even when mobility events take place before the UE manages to transmit all the collected data. However, the standardization effort of the option 2 should be also assessed in relationship to the time left for this Rel.19 WI. 
For example, if option 2 is pursued, RAN2 should first discuss whether the data retaining should be supported in all mobility cases or only in some cases, e.g. the retaining of the data may be supported only in case of HO, whereas in case of RLF the UE may discard all the data. Then, it should be discussed how the UE is informed on whether it is allowed or not allowed to transmit the AIML collected data to another gNB, e.g the target gNB of the HO. 
Finally, since the node retrieving the data should transmit the collected data back to the source gNB, some work on the Xn interface might be needed, and as such RAN3 involvement would be needed.

Given the above considerations, we propose for simplicity to limit the scope of this discussion for Rel.19.

For Rel.19, as baseline, RAN2 assumes that the UE discards/clears the AIML collected data at mobility events (e.g. handover to a different gNB than the source gNB, RLF/HOF, RRC state transitions).

Nevertheless, if there is a strong preference to pursue optimizations in this area, we propose addressing the following aspects.

If it is agreed to enable the UE to retain the AIML collected data at mobility events, RAN2 to discuss the following:

  - Whether the retaining of the data should be supported for all mobility-related events, i.e. HO, RLF, RRC state transitions, or only for some of them.

  - How the UE is informed on whether it should retain the data or not when executing the mobility-related event.

  - Inter-gNB signalling and RAN3 involvement

## 2.6 OAM-centric data collection

For the OAM-centric data collection, the RAN2 impact is quite limited. This is because the agreements reached so far are also suitable for the OAM-centric use cases. However, there might be more specific OAM-related aspects that should be addressed by RAN3 and SA5. For example, SA5 should consider whether to introduce other Mx measurement for the specific BM use cases. Additionally, RAN3/SA5 could consider whether to enhance the management MDT for the selection of the UEs to perform the data collection.
RAN2 can start by indicating the agreements reached so far by RAN2 on this topic, and ask RAN3/SA5 to consider these agreements for their work.

RAN2 to send an LS to RAN3/SA5 to indicate the agreements reached so far by RAN2 on the topic of NW-side data collection, and ask them to consider such agreements in their work.

# 3	Discussion on NW-side data collection for positioning

Network-side data collection for positioning occurs in the following network-based positioning methods:

- Case 3a: NG-RAN node assisted/LMF-based positioning with gNB side model (direct positioning)

- Case 3b: NG-RAN node assisted/LMF-based positioning with LMF-side model (assisted positioning)

Case 3a and Case 3b both are based on UL measurements performed at the gNB and share similar flows as the legacy UL-TDOA positioning method. As such the impact is primarily in RAN3 domain NRPPa.

The main impact in terms of RAN2 is the RRC configuration for SRS which can follow the legacy mechanism. However, one additional aspect to consider is that for AI/ML model both measurements and label are needed. For the training, the necessary information is the gNB measurements (RTOA) and the label/ground truth is UE location or true RTOA. The LMF would have to configure the reporting of UE location coinciding the SRS transmission periodicity. The gNB for the case 3a can also obtain the UE location using RRC; that is there would be no requirement of having an LPP session. This is already possible in legacy as below for MDT.

#### –	CommonLocationInfo

The IE CommonLocationInfo is used to transfer detailed location information available at the UE to correlate measurements and UE position information.

CommonLocationInfo information element

-- ASN1START

-- TAG-COMMONLOCATIONINFO-START

CommonLocationInfo-r16 ::= SEQUENCE {

gnss-TOD-msec-r16          OCTET STRING     OPTIONAL,

locationTimestamp-r16      OCTET STRING     OPTIONAL,

locationCoordinate-r16     OCTET STRING     OPTIONAL,

locationError-r16          OCTET STRING     OPTIONAL,

locationSource-r16         OCTET STRING     OPTIONAL,

velocityEstimate-r16       OCTET STRING     OPTIONAL

}

-- TAG-COMMONLOCATIONINFO-STOP

-- ASN1STOP

CommonLocationInfo field descriptions

gnss-TOD-msec Parameter type gnss-TOD-msec defined in TS 37.355 [49]. The first/leftmost bit of the first octet contains the most significant bit.

locationTimeStamp Parameter type DisplacementTimeStamp defined in TS 37.355 [49]. The first/leftmost bit of the first octet contains the most significant bit.

locationCoordinate Parameter type LocationCoordinates defined in TS 37.355 [49]. The first/leftmost bit of the first octet contains the most significant bit.

locationError Parameter LocationError defined in TS 37.355 [49]. The first/leftmost bit of the first octet contains the most significant bit.

locationSource Parameter LocationSource defined in TS 37.355 [49]. The first/leftmost bit of the first octet contains the most significant bit.

velocityEstimate Parameter type Velocity defined in TS 37.355 [49]. The first/leftmost bit of the first octet contains the most significant bit.

UE sending ground truth UE location coordinate to gNB via RRC is more efficient compared to sending it first to LMF which further forwarding to gNB, especially when UE is requested to send updated location coordinate when moving.

Case 3a/3b NW side data collection should be driven by RAN3 as it has primarily NRPPa impact. The main impact on RAN2 can be to align the gNB measurements and UE reporting of ground truth during training.

UE sending ground truth UE location coordinate directly to gNB via RRC is an efficient way to support gNB-side data collection.

A window to report UE ground truth coinciding with SRS transmission periodicity is configured.

UE sends ground truth UE location coordinate to gNB via RRC for gNB-side data   collection.

# 4	Conclusion

In the previous sections we made the following observations:

Observation 1	In legacy, the UEInformationRequest is mapped to SRB1, and the UEInformationResponse is mapped to SRB1 or SRB2 (when logged measurement information is included).

Observation 2	Case 3a/3b NW side data collection should be driven by RAN3 as it has primarily NRPPa impact. The main impact on RAN2 can be to align the gNB measurements and UE reporting of ground truth during training.

Observation 3	UE sending ground truth UE location coordinate directly to gNB via RRC is an efficient way to support gNB-side data collection.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 assumes that the radio resource configuration for the purpose of NW-side data collection consists of a set of CSI-RS/SSB resources on which the UE shall perform radio measurements and the related logging.

Proposal 2	RAN2 assumes that if data logging is not supported by the UE, the L1 measurements results associated to the NW-side data collection are reported via UCI. Up to RAN1 to define the need of any enhancement.

Proposal 3	Along with the L1-RSRP and beams IDs, the UE includes the timestamp related to the point in time in which a reported measurement was taken.

Proposal 4	Ask RAN1 confirmation on the following topics, including any RAN2 related agreement:

a.	Radio resource configuration via CSI measurement framework for the purpose of NW-side data collection

b.	Reporting via UCI of the L1 measurements in case the data logging is not supported by the UE

c.	Content of the data collection report, including at least the L1-RSRPs, the associated measured beam IDs, the timestamp of the performed measurement

Proposal 5	No need to introduce multiple configurations for NW-side data collection, and hence no need to introduce dynamic activation/deactivation mechanisms.

Proposal 6	The UE starts the data collection upon receiving a L3 configuration for NW-side data collection.

Proposal 7	The logging periodicity/interval (for both periodic and event-triggered data collection) is given by the specific CSI measurement configuration.

Proposal 8	Besides the case of memory limitation issues (already agreed in RAN2#127), UE stops the NW-side data collection procedure when:

a.	data collection has been done for a certain configurable logging duration

b.	the event that triggered the data collection is no longer fulfilled.

Proposal 9	Considering the L1-RSRP level fluctuations, RAN2 to consider even-triggered conditions, based on the L3 events/measurements such as L3 filtered RSRP level, the starting of T310 timer, or on beam failures etc.

Proposal 10	The UEInformationRequest requesting AIML data is transmitted via SRB1 (as any other data request contained in such message), and the UEInformationResponse is transmitted via SRB2, or via a new defined SRB (to be used for all AIML use cases).

Proposal 11	The availability indication transmitted by the UE should contain at least a reference to the type of data that are available for transmission (e.g. “AIML NW-side collected data”), and the UEInformationRequest should contain at least a reference to the type of data that are requested (e.g. “AIML NW-side collected data”).

Proposal 12	When the UE stops the data collection (e.g. due to fulfilling/unfulfilling of the event-triggered data collection, or UE internal conditions such as memory limitations), the UE informs the gNB about the availability of collected data.

Proposal 13	As baseline, the data availability indication is transmitted at least in the UEAssistanceInformation. Whether it can be transmitted also in RRC complete messages (as in SON/MDT) depends on the handling of collected data during mobility.

Proposal 14	In case all the collected data do not fit into a single RRC message, RAN2 to discuss the following options:

a.	In each transmitted RRC message, the UE indicates to the gNB that there are still collected data available for transmission, and then the gNB further requests the UE to transmit the next RRC message (via UEInformationRequest/Response) (similar to logged MDT).

b.	The gNB indicates (i.e. in the UEInformationRequest) the amount of collected data the UE should transmit in the following RRC UEInformationResponse messages, or the amount of RRC UEInformationResponse messages the UE should transmit, without any further request from the gNB.

c.	The gNB indicates (i.e. in the UEInformationRequest) that all the remaining collected data should be transmitted by the UE in the following RRC UEInformationResponse messages without any further request from the gNB.

Proposal 15	The UE indicates in the availability indication how many data or RRC samples are available for transmission.

Proposal 16	RAN2 to wait RAN1 inputs on the type of data to be logged by the UE, before discussing the minimum memory size for storing collected data.

Proposal 17	Define in a UE capability some different minimum memory values that a UE can support for the logging of collected data.

Proposal 18	For Rel.19, as baseline, RAN2 assumes that the UE discards/clears the AIML collected data at mobility events (e.g. handover to a different gNB than the source gNB, RLF/HOF, RRC state transitions).

Proposal 19	If it is agreed to enable the UE to retain the AIML collected data at mobility events, RAN2 to discuss the following:

a.	Whether the retaining of the data should be supported for all mobility-related events, i.e. HO, RLF, RRC state transitions, or only for some of them.

b.	How the UE is informed on whether it should retain the data or not when executing the mobility-related event.

c.	Inter-gNB signalling and RAN3 involvement

Proposal 20	RAN2 to send an LS to RAN3/SA5 to indicate the agreements reached so far by RAN2 on the topic of NW-side data collection, and ask them to consider such agreements in their work.

Proposal 21	A window to report UE ground truth coinciding with SRS transmission periodicity is configured.

Proposal 22	UE sends ground truth UE location coordinate to gNB via RRC for gNB-side data   collection.