3GPP TSG-RAN WG2 #128	R2-2410874

# Orlando, US, 18th – 22nd November 2024

Agenda Item:	6.1

Source:	Ericsson

Title:	Discussion on RAN1 LS on waveform determination for PUSCH scheduled by fallbackRAR UL grant and MsgA PUSCH

Document for:	Discussion

# 1	Introduction

An LS from RAN1 was received, [1] which discusses different interpretations of how the parameters msg3-transformPrecoder and msgA-transformPrecoder should apply since they are present in the RACH-ConfigCommon which have several instances.

This contribution proposes our view on the interpretation of these cases and the action in the LS.

# 2	Discussion

RAN1 states the following action to RAN2:

To RAN2 ACTION: RAN1 respectfully asks RAN2 to answer the following questions. Q1: From RAN2 point of view, which msg3-transformPrecoder is used to determine waveform for PUSCH scheduled by fallbackRAR UL grant and for MsgA PUSCH when msgA-TransformPrecoder is not configured respectively? Q2: Is the related UE behavior already defined in RAN2 spec?

RAN1 also highlights the following text from 3GPP TS 38.214:

For a MsgA PUSCH, the UE shall consider the transform precoding either 'enabled' or 'disabled' according to the higher layer configured parameter msgA-TransformPrecoder. If higher layer parameter msgA-TransformPrecoder is not configured, the UE shall consider the transform precoding either 'enabled' or 'disabled' according to the higher layer configured parameter msg3-transformPrecoder.

Firstly, let’s see the definition of msg3-transformPrecoder, msgA-transformPrecoder and transformPrecoder in 3GPP TS 38.331:

msg3-transformPrecoder Enables the transform precoder for Msg3 transmission according to clause 6.1.3 of TS 38.214 [19]. If the field is absent, the UE disables the transformer precoder (see TS 38.213 [13], clause 8.3). msgA-TransformPrecoder Enables or disables the transform precoder for MsgA transmission (see clause 6.1.3 of TS 38.214 [19]). transformPrecoder Enables or disables transform precoding for type1 and type2. If the field is absent, the UE enables or disables transform precoding in accordance with the field msg3-transformPrecoder in RACH-ConfigCommon from rach-ConfigCommon included directly within BWP configuration (i.e., not included in additionalRACH-ConfigList), see TS 38.214 [19], clause 6.1.3.

Based on this, we can try to answer the questions stated:

Q1: From RAN2 point of view, which msg3-transformPrecoder is used to determine waveform for PUSCH scheduled by fallbackRAR UL grant and for MsgA PUSCH when msgA-TransformPrecoder is not configured respectively?

It is not stated within the field description in 38.331 where the definition of the parameter exists, but as a general guideline in MAC, the RRC parameters will be applied in chapter 5.1.1 based on the selection procedure in 5.1.1b) (which also consider featurecombination preambles). Based on this, if nothing further is stated it could be interpreted as the “closest” RACH-ConfigCommon, i.e. not the top one.

Q2: Is the related UE behavior already defined in RAN2 spec?

It seems like the behavior could be concluded by existing specifications according to the reasoning in the answer to Q1. However, it should be emphasized that RAN2 signaling and procedures in this case is there to fulfill the functionality of the RAN1 feature and could be adapted to how RAN1 wants to apply the parameters. This is also highlighted in the field description of a similar parameter (transformPrecoder), where it has been added which specific rach-ConfigCommon that should be referenced. Similar clarifications could be made to msg3-transformPrecoder if necessary.

In our view, RAN2 specifications currently points to the rach-ConfigCommon specific to the configuration. However, it would be possible to tailor this towards intended RAN1 behavior.

RAN2 send a reply-LS with information stating that current RAN2 specifications is according to interpretation in Alt2 in the RAN1 LS, but for similar parameters this has previously been adapted to a different behavior, and if RAN1 prefers a different interpretation, clarifications in the RAN2 specs might be necessary similar to what has been done for parameter transformPrecoder.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	In our view, RAN2 specifications currently points to the rach-ConfigCommon specific to the configuration. However, it would be possible to tailor this towards intended RAN1 behavior.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 send a reply-LS with information stating that current RAN2 specifications is according to interpretation in Alt2 in the RAN1 LS, but for similar parameters this has previously been adapted to a different behavior, and if RAN1 prefers a different interpretation, clarifications in the RAN2 specs might be necessary similar to what has been done for parameter transformPrecoder.

# 4	References

- R1-2409301, LS on waveform determination for PUSCH scheduled by fallbackRAR UL grant and MsgA PUSCH, Hefei, China, October 14th – 18th, 2024.