3GPP TSG-RAN WG2 Meeting #128												R2-2410879

Orlando, USA, November 18th – 22nd, 2024

Agenda Item:	6.1.2.1

Source: 	Sharp

Title:  	NTN DRX Timer Handling and URLLC One-Shot HARQ Feedback

Document for: 	Discussion

# Introduction

In RAN2#127bis meeting, RAN2 discussed whether the current DRX timer handling should be revised to support one-shot HAFQ feedback in NTN, as follows:

R2-2409123 Correction on drx HARQ RTT timer in NTN LG Electronics Inc. CR Rel-17 38.321 17.10.0 1967 - F NR_NTN_solutions-Core - Vivo, Qualcomm explains that we agree to not further clarify. LG thinks that there is a difference between NTN and TN. - Qualcomm and Huawei think that these changes were for URLLC. - Sharp asks if one shot feedback is supported for NTN, as that would be the only reason to align. LG understands that there were no restrictions but if there is we should say that this is only for TN. - Ericsson explains that this was for NR-U and there was an agreement that this is applicable to outside of NR-U. Ericsson supports fixing this. - Qualcomm thinks that to support one shot we would need to support a number of other things that RAN1 should discuss and we shouldn’t make such decisions in RAN2. Check if one shot feedback is supported for NTN The CR is postponed

This document discusses the issue from the CR [1].

# Discussion

## Background: Rel-17 IIOT/URLLC and Rel-17 NTN

As a feature of Rel-17 IIOT/URLLC WI, DRX operation was modified to start multiple DRX HARQ RTT Timers for multiple HARQ processes whose HARQ feedback is reported in Rel-16/17 one-shot HARQ feedback. If HARQ processes to report the feedback are configured by RRC and actual feedback is triggered by a DCI, it is possible that HARQ RTT Timer is still running when one-shot feedback is transmitted. For this case, restarting timer was added by Rel-17 IIOT/URLLC MAC CR, R2-2204406 [2], implemented in TS 38.321 v17.0.0, as follows:

R2-2204406 (RAN2#117-e) 1> if a DRX group is in Active Time: 2> monitor the PDCCH on the Serving Cells in this DRX group as specified in TS 38.213 [6]; 2> if the PDCCH indicates a DL transmission; or 2> if the PDCCH indicates a one-shot HARQ feedback as specified in clause 9.1.4 of TS 38.213 [6]; or 2> if the PDCCH indicates a retransmission of HARQ feedback as specified in clause 9.1.5 of TS 38.213 [6]: 3> start or restart the drx-HARQ-RTT-TimerDL for the corresponding HARQ process(es) whose HARQ feedback is reported in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback; NOTE 3: When HARQ feedback is postponed by PDSCH-to-HARQ_feedback timing indicating a non-numerical k1 value, as specified in TS 38.213 [6], the corresponding transmission opportunity to send the DL HARQ feedback is indicated in a later PDCCH requesting the HARQ-ACK feedback. 3> stop the drx-RetransmissionTimerDL for the corresponding HARQ process(es) whose HARQ feedback is reported. 3> if the PDSCH-to-HARQ_feedback timing indicate a non-numerical k1 value as specified in TS 38.213 [6]: 4> start the drx-RetransmissionTimerDL in the first symbol after the (end of the last) PDSCH transmission (within a bundle) for the corresponding HARQ process.

In Rel-17 NTN WI, a DRX timer handling to compensate a large propagation delay and disabled HARQ feedback were introduced. When a very large propagation delay is expected, some HARQ processes can be configured with disabled HARQ feedback. HARQ processes with enabled HARQ feedback are considered to deliver non-time-sensitive data whose latency requirement is not so stringent. Instead, DRX HARQ RTT Timer duration compensates the latest UE-gNB RTT. These features were added by Rel-17 NTN MAC Correction CR, R2-2206686 [3], implemented in TS 38.321 v17.1.0, as follows:

R2-2206686 (RAN2#118-e) 1> if a DRX group is in Active Time: 2> monitor the PDCCH on the Serving Cells in this DRX group as specified in TS 38.213 [6]; 2> if the PDCCH indicates a DL transmission; or 2> if the PDCCH indicates a one-shot HARQ feedback as specified in clause 9.1.4 of TS 38.213 [6]; or 2> if the PDCCH indicates a retransmission of HARQ feedback as specified in clause 9.1.5 of TS 38.213 [6]: 3> if this Serving Cell is configured with downlinkHARQ-FeedbackDisabled: 4> if the corresponding HARQ process is configured with HARQ feedback enabled: 5> set HARQ-RTT-TimerDL-NTN for the corresponding HARQ process equal to drx-HARQ-RTT-TimerDL plus the latest available UE-gNB RTT value; 5> start the HARQ-RTT-TimerDL-NTN for the corresponding HARQ process in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback; 3> else: 4> start or restart the drx-HARQ-RTT-TimerDL for the corresponding HARQ process(es) whose HARQ feedback is reported in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback;

The later agreed NTN CR did not consider aspects of Rel-17 one-shot HARQ feedback, i.e., restarting DRX HARQ RTT timer and involvement of multiple HARQ processes. If one-shot HARQ feedback for multiple HARQ processes is used for a cell configured with downlinkHARQ-FeedbackDisabled in NTN, the current DRX operation does not specify which DRX HARQ RTT timer should be started. It means that the current specification is broken if one-shot HARQ feedback for multiple HARQ processes is supported. If it is supported, some specification changes are necessary.

The current specification is broken if one-shot HARQ feedback for multiple HARQ processes is supported in NTN with downlinkHARQ-FeedbackDisabled.

## Necessity of One-Shot HARQ Feedback in NTN

Enhanced Type-3 HARQ ACK codebook (a.k.a. Rel-17 one-shot HARQ feedback) was introduced as a Rel-17 IIOT/URLLC feature to avoid the HARQ retransmission delay due to LBT or other conflict in unlicensed spectrum. The latency requirement of IIOT/URLLC applications usually has a range between 0.5ms to tens of milliseconds, depending on target applications. This means that one-shot HARQ feedback is helpful mainly for IIOT/URLLC applications with stringent latency requirements.

In contrast, downlinkHARQ-FeedbackDisabled was introduced for NTN scenario with long propagation delay which makes HARQ retransmission useless for time-sensitive applications. Since IIOT/URLLC applications are considered time-sensitive, HARQ retransmission may not be feasible. A sensible implementation will be that IIOT/URLLC traffic is served by retransmission-less HARQ process.

Only time-insensitive applications will be served by HARQ process with enabled HARQ feedback. The proposed change in R2-2409123 [1] can be only for a scenario that one-shot HARQ feedback is used for HARQ processes which is supposed to serve time-insensitive applications, i.e., non-IIOT/URLLC applications.

The proposed change in R2-2409123 is targeted to support IIOT/URLLC feature in non-IIOT/URLLC scenario. It may not be so essential.

If RAN2 does not change the DRX operation, the current behavior is still broken. Hence, we need to specify a restriction that one-shot HARQ feedback is not supported when downlinkHARQ-FeedbackDisabled is configured.

## On Text Proposal

One could argue that there is no such restriction. Then, RAN2 should fix the problem. If RAN2 agrees to fix the problem, a text proposal from R2-2409123 [1] can be used as a baseline as follows:

R2-2409123 (RAN2#127bis) 1> if a DRX group is in Active Time: 2> monitor the PDCCH on the Serving Cells in this DRX group as specified in TS 38.213 [6]; 2> if the PDCCH indicates a DL transmission; or 2> if the PDCCH indicates a one-shot HARQ feedback as specified in clause 9.1.4 of TS 38.213 [6]; or 2> if the PDCCH indicates a retransmission of HARQ feedback as specified in clause 9.1.5 of TS 38.213 [6]: 3> if this Serving Cell is configured with downlinkHARQ-FeedbackDisabled: 4> if the corresponding HARQ process(es) is configured with HARQ feedback enabled: 5> set HARQ-RTT-TimerDL-NTN for the corresponding HARQ process(es) equal to drx-HARQ-RTT-TimerDL plus the latest available UE-gNB RTT value; 5> start or restart the HARQ-RTT-TimerDL-NTN for the corresponding HARQ process(es) whose HARQ feedback is reported in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback. 3> else: 4> start or restart the drx-HARQ-RTT-TimerDL for the corresponding HARQ process(es) whose HARQ feedback is reported in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback.

The text proposal has an ambiguity on the corresponding HARQ process. This expression was introduced since Rel-15 for the case that DL transmission is indicated with the corresponding HARQ process. When Rel-17 IIOT/URLLC WI updated the DRX operation, it was modified to “the corresponding HARQ process(es) whose HARQ feedback is reported” is used. Hence, the same expression is necessary. The following text proposal can be considered:

1> if a DRX group is in Active Time: 2> monitor the PDCCH on the Serving Cells in this DRX group as specified in TS 38.213 [6]; 2> if the PDCCH indicates a DL transmission; or 2> if the PDCCH indicates a one-shot HARQ feedback as specified in clause 9.1.4 of TS 38.213 [6]; or 2> if the PDCCH indicates a retransmission of HARQ feedback as specified in clause 9.1.5 of TS 38.213 [6]: 3> if this Serving Cell is configured with downlinkHARQ-FeedbackDisabled: 4> if the corresponding HARQ process(es) whose HARQ feedback is reported is configured with HARQ feedback enabled: 5> set HARQ-RTT-TimerDL-NTN for the corresponding HARQ process(es) equal to drx-HARQ-RTT-TimerDL plus the latest available UE-gNB RTT value; 5> start or restart the HARQ-RTT-TimerDL-NTN for the corresponding HARQ process(es) whose HARQ feedback is reported in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback. 3> else: 4> start or restart the drx-HARQ-RTT-TimerDL for the corresponding HARQ process(es) whose HARQ feedback is reported in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback.

Since the current behaviour is not working, RAN2 should not leave the specification as it is. We see that RAN2 should choose one option among configuration restriction and DRX behavior change.

RAN2 to down-select one option for this issue:

  - Specify that one-shot HARQ feedback is not supported when downlinkHARQ-FeedbackDisabled is configured.

  - Fix the problem with TP in Section 5.

# Conclusions

Based on the discussion given above, we have the following proposal:

- RAN2 to down-select one option for this issue:

  - Specify that one-shot HARQ feedback is not supported when downlinkHARQ-FeedbackDisabled is configured.

  - Fix the problem with TP in Section 5.

# Reference

[1] R2-2409123	Correction on drx HARQ RTT timer in NTN	LG Electronics Inc.

[2] R2-2204016	Introduction of enhanced IIoT&URLLC support for NR	Samsung

[3] R2-2206866	Corrections to Release-17 NR Non-Terrestrial Networks (NTN)	InterDigital

# Text Proposal

…

1>	if a DRX group is in Active Time:

2>	monitor the PDCCH on the Serving Cells in this DRX group as specified in TS 38.213 [6];

2>	if the PDCCH indicates a DL transmission; or

2>	if the PDCCH indicates a one-shot HARQ feedback as specified in clause 9.1.4 of TS 38.213 [6]; or

2>	if the PDCCH indicates a retransmission of HARQ feedback as specified in clause 9.1.5 of TS 38.213 [6]:

3>	if this Serving Cell is configured with downlinkHARQ-FeedbackDisabled:

4>	if the corresponding HARQ process(es) whose HARQ feedback is reported is configured with HARQ feedback enabled:

5>	set HARQ-RTT-TimerDL-NTN for the corresponding HARQ process(es) equal to drx-HARQ-RTT-TimerDL plus the latest available UE-gNB RTT value;

5>	start or restart the HARQ-RTT-TimerDL-NTN for the corresponding HARQ process(es) whose HARQ feedback is reported in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback.

3>	else:

4>	start or restart the drx-HARQ-RTT-TimerDL for the corresponding HARQ process(es) whose HARQ feedback is reported in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback.