3GPP TSG-RAN WG2 Meeting#128                           	R2 -2410892

Orlando, USA, Nov 18th – 22nd, 2024

Agenda item:	7.3.2 Corrections

Source: 	Nordic Semiconductor ASA, Samsung

Title:	Scope and uniqueness of SatelliteID

Document for:	Discussion and Decision

# Introduction

The scope and uniqueness/connection/relation of the satelliteId is not fully clear across various SIBs (SIB31/32/33/3/5) and 3GPP releases. One example was discussed in RAN2#127bis [1] in the context of satelliteId-r18 between SIB3/5, SIB31 and SIB33. Another example was contributed to RAN2#127bis [2] in the context of S&F operation and the serving cell assistance information. The connection/relation between satelliteId-R18 in SIB31 for the serving satellite reference location and satelliteId-r17s in SIB32 for satellite assistance information is not clear.

SA2#165 contribution [3] in the context of S&F discussed that an MME could include a list of satellites a UE can attempt to retry the related NAS procedure. From RAN2 point of view the satellites are identified with satelliteId, however, currently it would not be clear from the existing specifications whether the satelliteIDs found in RAN2 specifications are unique across different uses across 3GPP releases.

This paper discusses the use of the satelliteID and its uniqueness/connection/relation to different SIBs and uses across 3GPP releases.

# Discussion

## Connections and relations between satelliteIds

3GPP Release-17 introduced satelliteId-r17 as part of the SIB32(-NB) and discontinuous coverage identifying the per satellite assistance information. The satelliteId-r17 is not used in any other context up to 3GPP Release-19.

3GPP Release-18 introduced the satelliteId-r18 for multiple uses:

- Connecting satellite assistance information for neighbouring cells in SIB33(-NB) to corresponding frequencies in SIB(-NB)3/5.

- Connecting satellite reference location in SIB31 and configured measurement objects for distance between UE and serving cell moving reference location.

However, it is not clear whether the number spaces overlap entirely or are mutually exclusive. From the current specification language (or the lack of it) they seem to be mutually exclusive, which would prohibit cross referencing a satellite identified by the satelliteId-r18 in SIB(-NB)31 and a satellite identified by any satelliteId-r17 found in SIB(-NB)32. This would prohibit identifying which of those satellites in a SIB(-NB)32 would be the serving satellite in S&F context.

The satelliteId-r17 and the satelliteId-r18 seem to be two different number spaces.

Clarify in the specifications that the satelliteId-r17 and the satelliteId-r18 share the same number space.

## Uniqueness of the satelliteId

Both the satelliteid-r17 and the satelliteId-r18 are 8-bit values with maximum value of 255. Known LEO/MEO deployments/constellations and future planned LEO/MEO deployments/constellations barely fit into the current satelliteId number space for one satellite operator deployment.

Both the satelliteId-r17 and the satelliteId-r18 can identify maximum 256 satellites, which mean they are not likely capable of serving more than one bigger constellation.

It is clear that a satelliteId cannot uniquely identify all deployed satellites among satellite operators and in some cases not even satellites of a one operator. Current specifications do not also state what is the scope of the satelliteId. Most of the satellite operators are already using International Network / Worldwide MCC 901 and the associated MNC, which would provide a natural scope for a satelliteId within a PLMN, since PLMNs are already in use within satellite deployments.

satelliteIds are already used within the context of PLMNs.

A RAN can be shared among multiple PLMNs and that should not be any different technically within satellite deployments. However, assuming the satelliteIds were unique within one PLMN then in the case of multiple supported PLMNs the same satelliteIds shall be unique to the same satellite within all PLMNs advertised by the satellite.

satelliteIds are unique among all PLMNs shared by the RAN.

A satelliteId is unique within a PLMN, in other words satelliteID+PLMN code may be considered a global identifier for a satellite.

Multiple satelliteId+PLMN codes can identify the same satellite as long as the satelliteId is the same.

# Conclusion

In this paper, we discussed connections/relations between different uses of satellisteIds and the uniqueness of the satelliteId in different deployments. RAN2 is kindly asked to consider the following proposals and should there be a need for clarification reflect them in specifications.

- Clarify in the specifications that the satelliteId-r17 and the satelliteId-r18 share the same number space .

A satelliteId is unique within a PLMN, in other words satelliteID+PLMN code may be considered a global identifier for a satellite.

Multiple satelliteId+PLMN codes can identify the same satellite as long as the satelliteId is the same.

# References

- R2-2408589, “Correction on satelliteId in SIB3/SIB5”, Apple, RAN2#127bis.

- R2-2408905, “Considerations on multi-satellite for S&F Satellite operation”, Novamint, Sateliot, RAN2#127bis.

- S2-2409843, “Introduction of Store and Forward feature in EPC”, Nokia, Novamint, NEC, Tencent, Sateliot, CATT, Intel, Vivo, CATT, SA2#165.