3GPP TSG RAN WG2#128	R2-2409509

Orlando, USA, 18th - 22nd November 2024

3GPP TSG-RAN WG1 Meeting #118bis	R1-2409174

Hefei, China, October 14th – 18th, 2024

Title:	Reply LS on IUC Scheme-2 and Random Selection

Response to:	R2-2407591

Release:	Rel-17

Work Item:	NR_SL_enh-Core

Source:	RAN1

To:	RAN2

Cc:

Contact Person:

Name:	Shichang Zhang

E-mail Address:	shichangzhang@oppo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments: None

1. Overall Description:

RAN1 thanks RAN2’s LS in R2-2407591 (R1-2407589) and would like to provide RAN1’s responses to the following question from RAN2.

In TS 38.321, it is specified that #### 5.22.1.2b Re-selection for using a received resource conflict indication If the MAC entity has been configured with Sidelink resource allocation mode 2 to transmit using pool(s) of resources in a carrier as indicated in TS 38.331 [5] based on full sensing, or partial sensing or random selection or any combination(s), the MAC entity shall for each Sidelink process: 1> if sl-interUECoordinationScheme2 enabling reception/transmission of a resource conflict indication is configured by RRC; and <Text Removed>. But it is not clear to RAN2 whether the configuration of both Inter-UE coordination scheme-2 and random-selection is supported or not. Q1: Is configuration of both Inter-UE coordination scheme-2 and random-selection supported or not, for normal resource pool and for exceptional resource pool? 2. Actions: To RAN WG1 ACTION: RAN2 respectfully asks RAN1 to provide answer to Q1 above.

RAN1 would like to provide the following answer:

- RAN1 has not made any explicit agreement regarding the (pre-)configuration of both Inter-UE coordination scheme-2 and random-selection. No additional RAN1 specification work is required to support (pre-)configuration of both Inter-UE coordination scheme-2 and random-selection.

- In other words, both Inter-UE coordination scheme-2 and random-selection can be (pre-)configured, for normal resource pool and for exceptional resource pool, without any update of RAN1 specifications.

- Note: RAN1 has NOT explicitly considered support for the combination of Inter-UE coordination scheme-2 and random-selection in a single resource pool when writing the RAN1 specification. It is up to RAN2 how to proceed.

2. Actions:

To RAN2: RAN1 respectfully asks RAN2 to take the above response into account in the future work.

3. Date of Next TSG-RAN1 Meetings:

RAN1#119	November 18th - 22th 2024	Orlando, US

RAN1#120	February 17th – 21th 2025	Athens , GR