3GPP TSG-RAN WG3 #125bis	R3-245442

Hefei, China, 14th – 18th Oct 2024

Agenda Item:	11.4

Source:	Lenovo

Title: 		(TP to BLCR 38.423) Discussion on UE performance feedback in AIML for NR-DC

Document for:	Discussion and Approval

- Introduction

In this paper, we discuss issues related to the UE performance collection in DC scenario.

- Discussion

5.1 Mobility optimization for NR-DC 5.1.1 Use case description Mobility in NR-DC can be optimized by means of AI/ML. Mobility Optimization for NR-DC is studied by assuming inference at the MN only. The main use case is limited to Dual Connectivity only and Conditional Dual Connectivity procedures are out of scope. 5.1.2 Potential Standard impacts The Dual Connectivity procedures (e.g., SN Addition, MN-initiated SN Change) are enhanced to trigger the collection of measured UE performance.

As concluded in the study item phase, for the mobility optimization for NR-DC, the normative phase will focus on the collection of UE performance for MN initiated SN addition and SN change scenario.

For the mobility optimization for NR-DC, the normative phase will focus on the collection of UE performance for MN initiated SN addition and SN change scenario.

In our view, the procedure will be very similar as how UE performance is collected by the source NG-RAN node from the target NG-RAN node in the regular handover scenario. An example stage 2 procedure is provided as below.

Before the triggering of SN addition or SN change, MN will first initiate a Data Collection Reporting Initiation procedure towards the target SN, which includes the configuration related to the UE performance collection that the target SN shall conduct once triggered. The procedure will be associated with a pair of measurement IDs.

Then, when MN initiates the SN addition or SN change, the MN will send a SN ADDITION REQUEST message towards the target SN, which includes the pair of measurement IDs (represented by Data Collection ID IE) which can refer to the previously initiated Data Collection Reporting Initiation procedure in the first step.

After target SN accepts the SN ADDTION REQUEST, and upon the complete of the SN addition procedure, e.g., target SN receives the SN RRC Reconfiguration Complete message from UE, the target SN will start measuring the UE performance as configured. And report to MN via DATA COLLECTION UPDATE message.

In addition, it’s worth noticing that in this discussion, from MN perspective, the only difference between MN initiated SN addition and SN change is that in the latter change MN needs to release the source SN.

For UE performance collection, the procedure signaling between MN and target SN is the same for SN addition and SN change.

For MN to collect UE performance from target SN, MN shall configure the UE performance collection via Data Collection Reporting Initiation procedure before triggering SN addition or SN change.

SN ADDTION REQUEST message is enhanced to include Data Collection ID IE that links to a previously configured UE performance collection.

Target SN will start UE performance collection after the complete of SN addition or SN change, e.g., upon receiving SN RRC Reconfiguration Complete.

Target SN sends the measured UE performance back to MN via the DATA COLLECTION UPDATE message.

Another aspect to be discussed is if the existing list of UE performance metrics can be reused for DC scenario or not. Note that after SN addition or SN change, the following types of radio bearers could exist depending on where it is anchored.

- MN terminated MCG/SCG/split bearer with QoS flow anchored at MN

- SN terminated MCG/SCG/split bearer with QoS flow anchored at SN

In the meanwhile, the existing UE performance IE consists of average UE throughput DL, average UE throughput UL, average packet delay, and average packet loss DL as below:

9.2.3.179 UE Performance This IE indicates the UE performance measurements metrics. IE/Group Name; Presence; Range; IE Type and Reference; Semantics Description. Average UE Throughput DL; O; Bit Rate 9.2.3.4. Average UE Throughput UL; O; Bit Rate 9.2.3.4. Average Packet Delay; O; 9.2.3.187. Average Packet Loss DL; O; Packet Loss Rate 9.2.3.11.

Average UE throughput DL/UL is defined as Bit Rate delivered by NG-RAN in UL or to NG-RAN in DL, which represents the average of QoS flow level throughput between NG-RAN and UPF. In the context of DC, Average UE Throughput DL/UL can be measured by target SN which represents the average bit rate for SN terminated MCG/SCG/Split bearers with QoS flow anchored at SN.

9.2.3.4 Bit Rate This IE indicates the number of bits delivered by NG-RAN in UL or to NG-RAN in DL or by the UE in sidelink within a period of time, divided by the duration of the period. It is used, for example, to indicate the maximum or guaranteed bit rate for a GBR QoS flow, or an aggregate maximum bit rate. IE/Group Name; Presence; Range; IE type and reference; Semantics description. Bit Rate; M; INTEGER (0..4,000,000,000,000,…); The unit is: bit/s.

Average UE Throughput DL/UL metrics can be reused for SN to measure and report the average bit rate for SN terminated MCG/SCG/Split bearers with QoS flow anchored at SN.

For average packet loss DL metric, it is defined as packet loss rate per QoS flow level as below. It can also be reused to represent the average packet loss rate for SN terminated MCG/SCG/Split bearers with QoS flow anchored at SN with QoS flow anchored at SN.

9.2.3.11 Packet Loss Rate This IE indicates the Packet Loss Rate for a QoS flow. IE/Group Name; Presence; Range; IE type and reference; Semantics description. Packet Loss Rate; M; INTEGER (0..1000, ...); Ratio of lost packets per number of packets sent, expressed in tenth of percent..

Average Packet Loss DL metric can be reused for SN to measure and report the average packet loss for SN terminated MCG/SCG/Split bearers with QoS flow anchored at SN.

The average packet delay metric is defined in 38.314 as following.

4.2.1.2 Packet delay 4.2.1.2.1 General Packet delay includes RAN part of delay and CN part of delay. The RAN part of DL packet delay measurement comprises: - D1 (DL delay in over-the-air interface), referring to Average delay DL air-interface in TS 28.552 [2] 5.1.1.1.1. - D2 (DL delay on gNB-DU), referring to Average delay in RLC sublayer of gNB-DU in TS 28.552 [2] 5.1.3.3.3. - D3 (DL delay on F1-U), referring to Average delay on F1-U in TS 28.552 [2] 5.1.3.3.2. - D4 (DL delay in CU-UP), referring to Average delay DL in CU-UP in TS 28.552 [2] 5.1.3.3.1. The DL packet delay measurements, i.e. D1 (the DL delay in over-the-air interface ), D2 (the DL delay in gNB-DU), D3 (the DL delay on F1-U) and D4 (the DL delay in CU-UP), should be measured per DRB per UE. NOTE: The delay measurements D1, D2 and D4 are also applicable for EUTRA in case of EN-DC related DL delay measurements on the MN side. The RAN part (including UE) of UL packet delay measurement comprises: - D1 (UL PDCP packet average delay, as defined in clause 4.3.1.1). - D2.1 (average over-the-air interface packet delay, as defined in 4.2.1.2.2). - D2.2 (average RLC packet delay, as defined in 4.2.1.2.3). - D2.3 (average delay UL on F1-U, it is measured using the same metric as the average delay DL on F1-U defined in TS 28.552 [2] clause 5.1.3.3.2). - D2.4 (average PDCP re-ordering delay, as defined in 4.2.1.2.4). The UL packet delay measurements, i.e. D1(UL PDCP packet average delay), D2.1(average over-the-air interface packet delay), D2.2(average RLC packet delay), D2.3(average delay UL on F1-U) and D2.4(average PDCP re-ordering delay), should be measured per DRB per UE. The unit of D1, D2.1, D2.2, D2.3 and D2.4 is 0.1ms. NOTE: The delay measurements D1, D2.1, D2.2 and D2.4 are also applicable for EUTRA in case of EN-DC related UL delay measurements on the MN side.

From the definition, the RAN part of DL or UL packet delay consists of lower layer delay measured by DU and upper layer delay measured by CU-UP in split architecture. For DC, depending on the radio bearer type, that would mean:

| DC radio bearer type | Lower layer delay measured by | Upper layer delay measured by |
| --- | --- | --- |
| MN terminated MCG bearer | MN | MN |
| MN terminated SCG bearer | SN | MN |
| MN terminated split bearer | MN, SN | MN |
| SN terminated MCG bearer | MN | SN |
| SN terminated SCG bearer | SN | SN |
| SN terminated split bearer | MN, SN | SN |

In order to accurately reflect the delay caused by SN, RAN3 discusses the following options:

- Support SN to measure packet delay per DRB level for all types of bearers. In this case, according to the configuration MN can understand if the measured packet delay includes lower layer delay or upper layer delay or both.

- Support SN to measure average packet delay ONLY for SN terminated SCG bearer and SN terminated split bearer by SN. For which, the average packet delay considers both lower layer and upper layer delay.

To reflect the packet delay and be understood by MN correctly, RAN3 discusses the following two options:

  - Support SN to measure packet delay per DRB level for all types of bearers, which may consider lower layer delay or upper layer delay or both depending on the bearer type.

  - Support SN to measure average packet delay ONLY for SN terminated SCG bearer and SN terminated split bearer by SN, which considers both lower layer and upper layer delay.

3	Conclusion

Based on the discussion above, we observe:

Observation 1	For the mobility optimization for NR-DC, the normative phase will focus on the collection of UE performance for MN initiated SN addition and SN change scenario.

Observation 2	For UE performance collection, the procedure signaling between MN and target SN is the same for SN addition and SN change.

Based on the discussion above, we propose:

Proposal 1	For MN to collect UE performance from target SN, MN shall configure the UE performance collection via Data Collection Reporting Initiation procedure before triggering SN addition or SN change.

Proposal 2	SN ADDTION REQUEST message is enhanced to include Data Collection ID IE that links to a previously configured UE performance collection.

Proposal 3	Target SN will start UE performance collection after the complete of SN addition or SN change, e.g., upon receiving SN RRC Reconfiguration Complete.

Proposal 4	Target SN sends the measured UE performance back to MN via the DATA COLLECTION UPDATE message.

Proposal 5	Average UE Throughput DL/UL metrics can be reused for SN to measure and report the average bit rate for SN terminated MCG/SCG/Split bearers with QoS flow anchored at SN.

Proposal 6	Average Packet Loss DL metric can be reused for SN to measure and report the average packet loss for SN terminated MCG/SCG/Split bearers with QoS flow anchored at SN.

Proposal 7	To reflect the packet delay and be understood by MN correctly, RAN3 discusses the following two options:

a.	Support SN to measure packet delay per DRB level for all types of bearers, which may consider lower layer delay or upper layer delay or both depending on the bearer type.

b.	Support SN to measure average packet delay ONLY for SN terminated SCG bearer and SN terminated split bearer by SN, which considers both lower layer and upper layer delay.

- TP to 38.423

<<<<<<<<<<<<<<<<<<< START OF CHANGE >>>>>>>>>>>>>>>

8.3.1	S-NG-RAN node Addition Preparation

8.3.1.1	General

The purpose of the S-NG-RAN node Addition Preparation procedure is to request the S-NG-RAN node to allocate resources for dual connectivity operation for a specific UE. Possible parallel requests are identified by the PCell ID when the initiating NG-RAN node UE AP IDs are the same.

The procedure uses UE-associated signalling.

8.3.1.2	Successful Operation

Figure 8.3.1.2-1: S-NG-RAN node Addition Preparation, successful operation

The M-NG-RAN node initiates the procedure by sending the S-NODE ADDITION REQUEST message to the S-NG-RAN node.

When the M-NG-RAN node sends the S-NODE ADDITION REQUEST message, it shall start the timer TXnDCprep.

The allocation of resources according to the values of the Allocation and Retention Priority IE included in the QoS Flow Level QoS Parameters IE for each QoS flow shall follow the principles specified for the PDU Session Resource Setup procedure in TS 38.413 [5].

The S-NG-RAN node shall choose the ciphering algorithm based on the information in the UE Security Capabilities IE and locally configured priority list of AS encryption algorithms and apply the key indicated in the S-NG-RAN node Security Key IE as specified in TS 33.501 [28].

If the TSC Traffic Characteristics IE is included for a QoS flow in the S-NODE ADDITION REQUEST message, the S-NG-RAN node shall behave the same as the NG-RAN node in the PDU Session Resource Setup procedure, specified in TS 38.413 [5].

***** Irrelevant Text Skipped ****

Interactions with the S-NG-RAN node Reconfiguration Completion procedure:

If the S-NG-RAN node admits at least one PDU session resource, the S-NG-RAN node shall start the timer TXnDCoverall when sending the S-NODE ADDITION REQUEST ACKNOWLEDGE message to the M-NG-RAN node except for a request for conditional configuration. The reception of the S-NODE RECONFIGURATION COMPLETE message shall stop the timer TXnDCoverall if TXnDCoverall is running.

Interaction with the Activity Notification procedure

Upon receiving an S-NODE ADDITION REQUEST message containing the Desired Activity Notification Level IE, the S-NG-RAN node shall, if supported, use this information to decide whether to trigger subsequent Activation Notification procedures according to the requested notification level.

Interaction with the Data Collection Reporting and the Data Collection Reporting Initiation procedures:

If the Data Collection ID IE is contained in the S-NODE ADDITION REQUEST message, the target S-NG-RAN node shall, if supported, report to the M-NG-RAN node after successful S-NG-RAN node addition, via the Data Collection Reporting procedure, the requested information configured via the previous Data Collection Reporting Initiation procedure corresponding to the NG-RAN node1 Measurement ID IE, allocated by the M-NG-RAN node, and the NG-RAN node2 Measurement ID IE, allocated by the target S-NG-RAN node,.

<<<<<<<<<<<<<<<<<<< NEXT CHANGE >>>>>>>>>>>>>>>

9.1.2.1	S-NODE ADDITION REQUEST

This message is sent by the M-NG-RAN node to the S-NG-RAN node to request the preparation of resources for dual connectivity operation for a specific UE.

Direction: M-NG-RAN node  S-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |
| UE Security Capabilities | M |  | 9.2.3.49 |  | YES | reject |
| S-NG-RAN node Security Key | M |  | 9.2.3.51 | This IE is ignored if the S-CPAC Request Information IE is present in the Conditional PSCell Addition Information Request IE. | YES | reject |
| S-NG-RAN node UE Aggregate Maximum Bit Rate | M |  | UE Aggregate Maximum Bit Rate 9.2.3.17 | The UE Aggregate Maximum Bit Rate is split into M-NG-RAN node UE Aggregate Maximum Bit Rate and S-NG-RAN node UE Aggregate Maximum Bit Rate which are enforced by M-NG-RAN node and S-NG-RAN node respectively. | YES | reject |
| Selected PLMN | O |  | PLMN Identity 9.2.2.4 | The selected PLMN of the SCG in the S-NG-RAN node. | YES | ignore |
| Mobility Restriction List | O |  | 9.2.3.53 |  | YES | ignore |
| Index to RAT/Frequency Selection Priority | O |  | 9.2.3.23 |  | YES | reject |
| PDU Session Resources To Be Added List |  | 1 |  |  | YES | reject |
| >PDU Session Resources To Be Added Item |  | 1 .. <maxnoofPDUSessions> |  | NOTE: If neither the PDU Session Resource Setup Info – SN terminated IE nor the PDU Session Resource Setup Info – MN terminated IE is present in a PDU Session Resources To Be Added Item IE, abnormal conditions as specified in clause 8.3.1.4 apply. | – |  |
| >>PDU Session ID | M |  | 9.2.3.18 |  | – |  |
| >>S-NSSAI | M |  | 9.2.3.21 |  | – |  |
| >>S-NG-RAN node PDU Session Aggregate Maximum Bit Rate | O |  | PDU Session Aggregate Maximum Bit Rate 9.2.3.69 |  | – |  |
| >>PDU Session Resource Setup Info – SN terminated | O |  | 9.2.1.5 |  | – |  |
| >>PDU Session Resource Setup Info – MN terminated | O |  | 9.2.1.7 |  | – |  |
| M-NG-RAN node to S-NG-RAN node Container | M |  | OCTET STRING | Includes the CG-ConfigInfo message as defined in subclause 11.2.2 of TS 38.331 [10] | YES | reject |
| S-NG-RAN node UE XnAP ID | O |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | reject |
| Expected UE Behaviour | O |  | 9.2.3.81 |  | YES | ignore |
| Requested Split SRBs | O |  | ENUMERATED (srb1, srb2, srb1&2, ...) | Indicates that resources for Split SRBs are requested. | YES | reject |
| PCell ID | O |  | Global NG-RAN Cell Identity 9.2.2.27 |  | YES | reject |
| Desired Activity Notification Level | O |  | 9.2.3.77 |  | YES | ignore |
| Available DRB IDs | C-ifSNterminated |  | DRB List 9.2.1.29 | Indicates the list of DRB IDs that the S-NG-RAN node may use for SN-terminated bearers. | YES | reject |
| S-NG-RAN node Maximum Integrity Protected Data Rate Uplink | O |  | Bit Rate 9.2.3.4 | The S-NG-RAN node Maximum Integrity Protected Data Rate Uplink is a portion of the UE’s Maximum Integrity Protected Data Rate in the Uplink, which is enforced by the S-NG-RAN node for the UE’s SN terminated PDU sessions. If the S-NG-RAN node Maximum Integrity Protected Data Rate Downlink IE is not present, this IE applies to both UL and DL. | YES | reject |
| S-NG-RAN node Maximum Integrity Protected Data Rate Downlink | O |  | Bit Rate 9.2.3.4 | The S-NG-RAN node Maximum Integrity Protected Data Rate Downlink is a portion of the UE’s Maximum Integrity Protected Data Rate in the Downlink, which is enforced by the S-NG-RAN node for the UE’s SN terminated PDU sessions. | YES | reject |
| Location Information at S-NODE reporting | O |  | ENUMERATED (pscell, ...) | Indicates that the user’s Location Information at S-NODE is to be provided. | YES | ignore |
| MR-DC Resource Coordination Information | O |  | 9.2.2.33 | Information used to coordinate resource utilisation between M-NG-RAN node and S-NG-RAN node. | YES | ignore |
| Masked IMEISV | O |  | 9.2.3.32 |  | YES | ignore |
| NE-DC TDM Pattern | O |  | 9.2.2.38 |  | YES | ignore |
| SN Addition Trigger Indication | O |  | ENUMERATED (SN change, inter-MN HO, intra-MN HO, ...) | This IE indicates the trigger for S-NG-RAN node Addition Preparation procedure | YES | reject |
| Trace Activation | O |  | 9.2.3.55 |  | YES | ignore |
| Requested Fast MCG recovery via SRB3 | O |  | ENUMERATED (true, ...) | Indicates that the resources for fast MCG recovery via SRB3 are requested. | YES | ignore |
| UE Radio Capability ID | O |  | 9.2.3.138 |  | YES | reject |
| Source NG-RAN Node ID | O |  | Global NG-RAN Node ID 9.2.2.3 | The NG-RAN Node ID of the source NG-RAN node, or the source SN in e.g. NR-DC to NR-DC (conditional) handover. | YES | ignore |
| Management Based MDT PLMN List | O |  | MDT PLMN List 9.2.3.133 |  | YES | ignore |
| UE History Information | O |  | 9.2.3.64 |  | YES | ignore |
| UE History Information from the UE | O |  | 9.2.3.110 |  | YES | ignore |
| PSCell Change History | O |  | ENUMERATED (reporting full history, ...) |  | YES | ignore |
| IAB Node Indication | O |  | ENUMERATED (true, ...) |  | YES | reject |
| No PDU Session Indication | O |  | ENUMERATED (true, ...) | This IE applies only if the UE is an IAB-MT. | YES | ignore |
| CHO Information SN Addition | O |  |  |  | YES | reject |
| >Source M-NG-RAN node ID | M |  | Global NG-RAN Node ID 9.2.2.3 |  | – |  |
| >Source M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the source M-NG-RAN node | – |  |
| >Estimated Arrival Probability | O |  | INTEGER (1..100) |  | – |  |
| SCG Activation Request | O |  | 9.2.3.154 |  | YES | ignore |
| Conditional PSCell Addition Information Request | O |  |  |  | YES | reject |
| >Maximum Number of PSCells To Prepare | M |  | INTEGER (1..8, ...) | Indicates the maximum number of PSCells that the target SN may prepare. | – |  |
| >Estimated Arrival Probability | O |  | INTEGER (1..100) | Indicates the arrival probability for the UE towards the candidate target SN. | – |  |
| >S-CPAC Request Information | O |  | 9.2.3.192 | Indicates that SN addition is for S-CPAC preparation. | YES | reject |
| >S-CPAC Reference Configuration Request | O |  | ENUMERATED (request, …) | Indicates that the reference configuration for S-CPAC is requested. | YES | ignore |
| S-NG-RAN node UE Slice Maximum Bit Rate | O |  | UE Slice Maximum Bit Rate List 9.2.3.167 | This IE indicates the S-NG-RAN node portion of the UE Slice Aggregate Maximum Bit Rate as specified in TS 23.501 [7] | YES | reject |
| F1-terminating IAB-donor Indicator | O |  | ENUMERATED (true, ...) | This IE applies only if the UE is an IAB-MT. | YES | reject |
| Selected NID | O |  | NID 9.2.2.65 | This IE, together with the Selected PLMN IE, indicates the SNPN proposed for the SCG to the S-NG-RAN node. | YES | ignore |
| QMC Coordination Request | O |  | 9.2.3.197 | This IE contains information for managing configuration and reporting of one or more QoE and/or RAN visible QoE measurements at the S-NG-RAN node subject to addition. | YES | ignore |
| Source SN to Target SN QMC Information | O |  | QMC Configuration Information 9.2.3.156 | This IE contains S-NG-RAN node-related QMC Configuration Information to be forwarded to the target S-NG-RAN node. | YES | ignore |
| Source M-NG-RAN node ID | O |  | Global NG-RAN Node ID 9.2.2.3 | The NG-RAN Node ID of the source M-NG-RAN node in e.g. NR-DC to NR-DC handover. | YES | ignore |
| IAB Authorization Status | O |  | ENUMERATED (authorized, not authorized, …) | Indicates the IAB node´s authorization status. | YES | ignore |
| Data Collection ID | O |  | 9.2.3.184 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions. Value is 256 |

| Condition | Explanation |
| --- | --- |
| ifSNterminated | This IE shall be present if there is at least one PDU Session Resource Setup Info – SN terminated in the PDU Session Resources To Be Added List IE. |

<<<<<<<<<<<<<<<<<<<<<< END OF CHANGE >>>>>>>>>>>>>>>>>>>>