3GPP TSG-RAN WG3 Meeting #126	R3-247029

Orlando, FL, USA, November 18th – 22nd 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.401 | CR | 0439 | rev | 3 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support for Wireless Access Backhaul |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, ZTE, Nokia, Nokia Shanghai Bell, Huawei, Samsung, Lenovo, Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_WAB_5GFemto-Core |  |  |  |  |  | Date: |  |  | 2024-10-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To support Wireless Access Backhaul. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added the following: -The definitions and the abbreviations. -The description of WAB architecture. -The description of WAB-node integration procedure. -The description of WAB-node configuration aspects. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Wireless Access Backhaul not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 3.2, 6.1.x (new), X (new), X.1 (new), X.2 (new), X.2.1 (new), X.2.2 (new), X.2.3 (new). |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.300, TS 38.305, TS 38.455 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR … CR … |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR … CR … |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: Revised based on additional RAN3#125-bis discussions. Rev2: Revised based on further additional RAN3#125-bis discussions. Rev3: Submitted to RAN3#126. |  |  |  |  |  |  |  |  |

-------------------------------------------Start of changes-------------------------------------------

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purpose of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. 
A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

AI/ML Model Inference: follows the definition of “AI/ML inference” as specified in clause 3.1 of TS 28.105 [34].

AI/ML Model Training: follows the definition of “ML model training” as specified in clause 3.1 of TS 28.105 [34].

Associated QoS Flow: as defined in TS 23.247 [27].

Associated QoS flow information: Information encompassing: QoS flow QoS parameters for associated QoS flows and mapping information between mapped (unicast) QoS flows and associated QoS flows. The respective information is included in a way that non-supporting RAN nodes would not establish respective RAN resources irrespective the multicast session state.

BH-5GC: The 5GC serving the WAB-MT.

BH-AMF: The AMF serving the WAB-MT.

BH-gNB: The gNB serving the WAB-MT.

BH-RAN-node: The NG-RAN node serving the WAB-MT.

BH-UPF: The UPF serving the WAB-MT for backhauling.

Boundary IAB-node: an IAB-node with one RRC interface terminating at a different IAB-donor-CU than the F1 interface. This definition applies to partial migration, inter-donor redundancy and inter-donor RLF recovery.

>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<

U2N Relay UE: as defined in TS 38.300 [2].

U2N Remote UE: as defined in TS 38.300 [2].

WAB-gNB: The gNB functionality, as defined in TS 38.300 [2], that provides NR access interface towards the UE.

WAB-MT: The WAB-node’s function that terminates the Uu interface to the BH-RAN-node using the procedures and behaviours specified for UEs. Corresponds to the MWAB-UE function defined in TS 23.501 [3].

WAB-node: An NG-RAN node comprising the WAB-MT and the WAB-gNB functionality.

## 3.2	Abbreviations

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. 
A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

5GC	5G Core Network

>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<

UL	Uplink

WAB	Wireless Access Backhaul

-------------------------------------------Next change-------------------------------------------

# 6	NG-RAN architecture

## 6.1	Overview

>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<

### 6.1.x	Wireless Access Backhaul architecture

A WAB-node consists of a WAB-gNB and a WAB-MT. The WAB-gNB is based on the gNB functionality specified in TS 38.300 [2] and serves UEs by means of a terrestrial NR Uu radio link.

The WAB-MT is served by a BH-RAN-node. The WAB-gNB traffic, including NG, Xn and OAM traffic is transported via backhaul PDU session(s) of the WAB-MT.

NOTE: The use of other types of backhaul, e.g. non-3GPP backhaul, is up to implementation.

The WAB-gNB and the WAB-MT may connect to the same PLMN or to different PLMNs.

Figure 6.1.x-1 shows the WAB architecture for 5GS.

Figure 6.1.x-1: The WAB architecture

In in-band scenarios, backhaul and access of the WAB-node use terrestrial radio links. In out-of-band scenarios, the backhaul can use a terrestrial or a non-terrestrial radio link, while the access uses terrestrial radio link.

The WAB-MT may connect to a public PLMN or an SNPN.

The WAB-gNB may connect to a public PLMN or an SNPN.

Figure 6.1.x-2 shows protocol stacks for NG Control plane and NG User plane transport via the wireless backhaul.

Figure 6.1.x-2: Protocol stacks for NG Control plane and NG User plane transport

Figure 6.1.x-3 shows protocol stacks for Xn Control plane and Xn User plane transport for WAB-node.

Figure 6.1.x-3: Protocol stacks for Xn Control plane and Xn User plane transport

-------------------------------------------Next change-------------------------------------------

# X	Wireless Access Backhaul

## X.1	WAB-node integration procedure

Figure X.1-1 WAB-node integration procedure

Phase 1: WAB-MT setup. The WAB-MT of a WAB-node connects to the network in the same way as a UE by performing RRC connection setup procedure with the BH-RAN-node. The WAB-MT then performs, authorization and authentication with the BH-5GC. After the WAB-MT is authorized, the WAB-MT can establish one or more PDU sessions for backhauling.

Phase 2: WAB-gNB setup. This phase includes the following 3 sub-phases:

Sub-phase 2-1: WAB-gNB initialization. In this phase, the WAB-gNB is configured by the OAM (e.g., with the information of AMF(s) to serve the UE) and the WAB-gNB is service-authorized by the SeGW or by the OAM.

Sub-phase 2-2: NG connection setup. The WAB-gNB establishes NG connection(s) toward the AMF(s). This step follows legacy procedures. After the NG is set up, the WAB-gNB can start serving UE(s).

Sub-phase 2-3: Xn connection setup. If needed, the WAB-gNB can establish Xn connection(s) towards the BH-RAN-node and/or other NG-RAN node(s).

## X.2	Configuration of WAB-node

### X.2.1	General

The following configurations of the WAB-node may need to be updated as the node moves:

-	The parameters that enable the WAB-gNB to select and connect to the AMF(s) that serve the UE(s).

-	The parameters that enable the WAB-gNB to connect to, and communicate with, the OAM system.

-	The parameters that the WAB-gNB should broadcast, e.g., the TAC(s), the cell ID(s), the RANAC(s).

A WAB-node may be provisioned with the parameters pertinent to different potential locations of the WAB-node.

Alternatively, the OAM can provision configuration parameters to the WAB-node based on the location of the node. In that case, the continuity of OAM connectivity needs to be ensured as the WAB-node moves.

### X.2.2	IP address configuration for the WAB-gNB

A WAB-MT obtains IP address(es) for the PDU sessions in the same manner as a legacy UE.

The IP address(es) of the WAB-MT can be used for the backhauling of WAB-gNB’s NG, Xn and OAM traffic.

In case the WAB-gNB uses the IPsec tunnel mode to protect the OAM, NG and/or Xn traffic, the IP address(es) of the WAB-MT is used as outer IP address(es), and the allocation of the inner tunnel IP address(es) is outside of 3GPP scope.

### X.2.3	TAC/RANAC (re-)configuration for a WAB-gNB’s cell

The TAC/RANAC of WAB-gNB’s cell is configured by the OAM, and it can be reconfigured by the OAM during the mobility of WAB-node. The TAC/RANAC of the WAB-gNB’s cell may be the same as, or different than, the TAC/RANAC of the co-located WAB-MT’s serving cell. The TAC/RANAC broadcast by the WAB-gNB’s cell can be changed in order to reflect the WAB-node’s physical location.

-------------------------------------------End of changes-------------------------------------------