3GPP TSG-RAN3 Meeting #126	R3-247147

Orlando, USA, Nov 18th – 22nd, 2024

Agenda Item:	16.2

Source: 	Qualcomm Incorporated

Title: 	(TP to 38.769) Conclusion for Topology 1 and updates to terminologies

Document for:	Discussion/Decision

# Background

In this paper, we continue to provide our views on aspects related to Topology 1 for Ambient IoT (A-IoT) based on the agreements and open issues identified in the previous RAN3 meetings.

# Discussion

## Terminologies

Interfaces are always drawn between two logical nodes and not two logical domains (RAN domain or CN domain). RAN3 have used logical domains (e.g., A-IoT RAN, A-IoT CN) to discuss RAN-CN functional split so far.

But moving forward, when discussing interfaces (e.g. XX interface) and its corresponding procedures, we propose RAN3 should refer to logical nodes and not consider logical boxes e.g., “A-IoT RAN node” instead of “A-IoT RAN” when referring to the RAN node in Topology 1. We therefore propose the following:

When referring to the RAN node in Topology 1, refer it as "A-IoT RAN node" instead of "A-IoT RAN"

Further, in the previous meetings, the details of the AIoT CN were left to SA2 as captured in the TR below and it was captured that the AIoT CN may include AMF and A-IoT related functions for both T1 and T2, pending SA2 decision as seen from the below text in the TR.

A-IoT CN:	Hosts certain functions for A-IoT as of the functional split between RAN and CN.

NOTE: the details of A-IoT CN are subject to SA2.

Editor’s Note 3: The protocol stack does not detail how A-IoT upper layer information is transported over XXAP, details are pending on SA2 agreements. And the A-IoT CN may include AMF and A-IoT related functions which is also up to SA2 decision.

Editor’s Note 3:	The A-IoT CN could include AMF and A-IoT related functions. This is up to SA2 decision.

In the last SA2 meeting, SA2 agreed on the following interim conclusion i.e., to support a new CN function to support AIoT as seen below from TR 23.700-13:

A new core network function is introduced to support Ambient IoT (e.g. AIoTF) service for both the topology 1 and topology 2.

To align with SA2 conclusion, we therefore propose to use the SA2 defined terminology “AIoTF” to refer to this new core network function.

SA2 concluded to support a new CN function “AIoTF” to support A-IoT for topology 1 and topology 2

RAN3 should use the terminology “AIoTF” to refer to the new CN function agreed by SA2 to support A-IoT, instead of referring to multiple core network functions as “A-IoT CN”.

Further the following FFS was captured in the Chair’s Notes.

For Topology 2, whether AIoT RAN needs to be revised as AIoT enabled gNB needs to be further checked in next meeting

In our view, it is still early to decide whether the A-IoT RAN node in T1 can be considered as an A-IoT enabled gNB (i.e., an NG-RAN node) because the legacy NG-RAN node as defined in 38.300 has a specific definition.

AS per TS 38.300, NG-RAN node is either a gNB (or ng-eNB), providing NR (or E-UTRA) user plane and control plane protocol terminations towards the UE, hosts host several functions (e.g., connection setup, paging, system information broadcast, header compression, ciphering, integrity protection), interconnected with each other via Xn interface, connects to AMF/UPF via NG-C/NG-U interface.

However, the A-IoT RAN node is an entity which interacts with an A-IoT device (not a UE) via a new A-IoT radio interface and several of the above functionalities e.g., header compression, ciphering, integrity protection, connection setup, “legacy” paging doesn’t apply to A-IoT RAN node.

A-IoT RAN node in Topology 1 interacts with a A-IoT device (not a UE) via a new A-IoT radio interface (not Uu) and doesn’t host several of the legacy NG-RAN node functionalities. It is also not clear whether there is a need for A-IoT RAN nodes to be interconnected via an Xn-like interface, whether A-IoT RAN node can connect to AMF etc.

We therefore propose the following:

Continue to use "A-IoT RAN node" for T1 and "A-IoT-enabled gNB" in the TR. Decide in normative phase whether the A-IoT RAN node can be specified as an NG-RAN node or needs to be considered as a new entity.

## Indirect path vs. Direct path

For Topology 1, the interface between the A-IoT RAN node and AIoTF (“XX interface”) is still not clear (e.g., whether it is a direct interface or are there any intermediate nodes (e.g., AMF) in between) as seen from the below text in the TR.

In Topology 1, the XX interface could be based on NG or a new interface carried over NG or a new interface.

Editor’s Note 7: The functions represented by the XX interfaces are FFS. It is also FFS whether this interface represents a new logical interface or is equal to NG. E.g. for topology 1 it may only represent a single interface instance, e.g. a new interface between A-IoT RAN node and A-IoT CN, for topology 2 it might represent either 2 interface instances, one instance for NG and one instance “XX” for a new interface between A-IoT CN and A-IoT RAN, or one instance for NG alone.

SA2 is still discussing whether existing 5GC entities (e.g., AMF) is involved in procedures related to A-IoT and have captured direct path vs. indirect path (via AMF) as two potential solutions in the SA2 TR.

To align with SA2, we propose RAN3 to also capture the two solutions in the TR 38.769:

- Indirect path solution: A-IoT RAN node indirectly (via the AMF) communicates with an AIoTF.

- Direct path solution: A-IoT RAN node directly communicates with an AIoTF.

Table 1 further compares the two options by showing its protocol stack, discussing the interfaces involved and discussing its pros/cons.

Table 1: Evaluation of transport solutions for Topology 1

| Indirect path solution | T1-Option 2 (Direct path solution) |
| --- | --- |
| (Potentially) Two new protocols need to be defined. Protocol b/w A-IoT RAN node and AMF (i.e., YYAP), if NGAP is not reused Protocol b/w A-IoT RAN node and AIoTF (i.e., XXAP) | Only 1 new protocol needs to be defined. Protocol between A-IoT RAN node and AIoTF (i.e., XXAP). |
| (Potentially) Two new interfaces need to be defined. Interface between A-IoT RAN node and AMF (YY) if N2 is not reused Interface between AMF and AIoTF (Naf) | Only 1 interface needs to be defined. Interface between the A-IoT RAN node and AIoTF (i.e., XX) |
| Different protocol stack at A-IoT RAN node and AIoTF SCTP-based stack between A-IoT RAN node and AMF. HTTP-based stack between AMF and AIoTF. | Common protocol stack at A-IoT RAN node and AIoTF |
| Need upgrades to legacy AMF. | No impact to legacy AMF. |
| More backhaul latency due to involvement of AMF. AMF is primarily used as a forwarding entity (“router”) for forwarding packets between A-IoT RAN node and AIoTF. Potentially unnecessary involvement of AMF. | Less backhaul latency (no involvement of AMF) |

Based on the above analysis, we have the following proposals:

To align with SA2, RAN3 should capture in TR that the following 2 options are possible for the AIoT RAN node to communicate with AIoTF in T1:

  - Indirect path solution: AIoT RAN node indirectly (via the AMF) communicates with the AIoTF

  - Direct path solution: AIoT RAN node directly communicates with the AIoTF

Update the logical system architecture and protocol stack of Topology 1 to show the two different options (Indirect path vs. indirect path).

Indirect path solution has the following drawbacks compared to Direct path solution:

  - Need upgrades at legacy AMF to support two new interfaces (AIoT RAN node-AMF, AMF-AIoTF)

  - More backhaul latency (due to involvement of AMF)

  - Potentially unnecessary involvement of AMF (just as a “router”) for AIoT

  - Potentially additional protocol b/w AIoT RAN node and AMF (YYAP) needs to be defined to carry end-to-end packets between AIoT RAN node and AIoTF (XXAP)

RAN3 to capture the following conclusions for T1:

- From RAN3 perspective, it is feasible to support Indirect path solution or Direct path solution.

- However, the Direct path solution is RAN3’s preference i.e., the AIoT RAN node directly communicates with the AIoTF (without going via AMF)

In Topology 1, irrespective of direct/indirect path solution, there is an end-to-end “XXAP” protocol between the AIoT RAN node and AIoTF

## Coordination of A-IoT radio resources among readers

In the TR, the following is captured:

The A-IoT RAN node should enable the coordination of the usage of the A-IoT radio resources among readers.

However, it is not clear from the above sentence whether this refers to coordination of the usage of A-IoT radio resource among readers within the A-IoT RAN node or across A-IoT RAN nodes or both. Also, it is not clear how this is to be achieved e.g., whether we need a Xn-like interface between the A-IoT RAN nodes to support this coordination or whether this is done via implementation.

In our view, there is no need to support Xn-like signaling between A-IoT RAN nodes to coordinate the usage of A-IoT radio resources and this can be done via implementation. Therefore, the following is proposed.

In Topology 1, there is no need to support Xn-like signaling between A-IoT RAN nodes to coordinate usage of A-IoT radio resources.

In Topology 1, the A-IoT RAN node can enable the coordination of the usage of the A-IoT radio resources among readers (within the A-IoT RAN node or across A-IoT RAN nodes) via implementation

## Reader awareness at AIoTF

In the last meeting, there was a brief discussion on Reader selection in T1 and how/whether BS Reader awareness is there at the AIoTF. There were some views that the Readers within an AIoT RAN node are “internal” and not known to the AIoTF and the AIoT RAN node should select the BS Readers in T1.

However, in our view, Readers within an AIoT RAN node should be “discoverable” by the AIoTF. There is no need to introduce such a restriction to keep the Readers “internal” to the AIoT RAN node and not expose it to the AIoTF. Even for LCS, currently TRPs hosted by a NG-RAN node are made aware at the CN via TRP Information Exchange procedures.

We therefore think that the AIoTF should be able to discover the “Readers” and can obtain information about the Readers supported by an AIoT RAN node via XX signalling procedure.

In Topology 1, there are two possibilities regarding Reader “awareness” at AIoTF

  - Option 1: Readers within an AIoT RAN node are “internal” and not known to the AIoTF

  - Option 2: Readers within an AIoT RAN node are “discoverable” by the AIoTF

In NR positioning, TRPs supported by a NG-RAN node and the TRP information can be made aware at the LMF via TRP Information Exchange procedure and there is no limitation to keep the TRPs “hidden” at the CN.

In Topology 1, Readers within an AIoT RAN node should be “discoverable” by the AIoTF

AIoTF obtains the information of Readers supported by an AIoT RAN node via XXAP signaling procedure.

## Reader selection

The following is captured in the interim conclusions of the SA2 TR:

The AIoTF receives an AIoT service request from the AF and triggers the BS/UE readers to perform AIoT service operations towards the AIoT Devices (s).

Upon receiving an A-IoT service request from AF, AIoTF should be able to discover/select the suitable A-IoT RAN nodes and Reader(s) within the selected AIoT RAN node(s) e.g., based on criteria provided by AF (e.g., a certain area of interest, certain targeted devices, list of Readers supported within the AIoT RAN node).

Therefore, the following is proposed:

AIoTF should select suitable AIoT RAN node(s) and Reader(s) within the selected AIoT RAN node(s) for performing A-IoT operations (e.g., Inventory/Command)

# Summary

Based on the discussion above, we have following observations and proposals:

2.1 Terminologies

Proposal 1:	When referring to the RAN node in Topology 1, refer it as "A-IoT RAN node" instead of "A-IoT RAN"

Observation 1:	SA2 concluded to support a new CN function “AIoTF” to support A-IoT for topology 1 and topology 2

Proposal 2:	RAN3 should use the terminology “AIoTF” to refer to the new CN function agreed by SA2 to support A-IoT, instead of referring to multiple core network functions as “A-IoT CN”.

Observation 2:	A-IoT RAN node in Topology 1 interacts with a A-IoT device (not a UE) via a new A-IoT radio interface (not Uu) and doesn’t host several of the legacy NG-RAN node functionalities. It is also not clear whether there is a need for A-IoT RAN nodes to be interconnected via an Xn-like interface, whether A-IoT RAN node can connect to AMF etc.

Proposal 3:	Continue to use "A-IoT RAN node" for T1 and "A-IoT-enabled gNB" in the TR. Decide in normative phase whether the A-IoT RAN node can be specified as an NG-RAN node or needs to be considered as a new entity.

2.2 Indirect path vs. Direct path

Proposal 4:	To align with SA2, RAN3 should capture in TR that the following 2 options are possible for the AIoT RAN node to communicate with AIoTF in T1:

a. Indirect path solution: AIoT RAN node indirectly (via the AMF) communicates with the AIoTF

b. Direct path solution: AIoT RAN node directly communicates with the AIoTF

Proposal 5:	Update the logical system architecture and protocol stack of Topology 1 to show the two different options (Indirect path vs. indirect path).

Observation 3:	Indirect path solution has the following drawbacks compared to Direct path solution:

- Need upgrades at legacy AMF to support two new interfaces (AIoT RAN node-AMF, AMF-AIoTF)

- More backhaul latency (due to involvement of AMF)

- Potentially unnecessary involvement of AMF (just as a “router”) for AIoT

- Potentially additional protocol b/w AIoT RAN node and AMF (YYAP) needs to be defined to carry end-to-end packets between AIoT RAN node and AIoTF (XXAP)

Proposal 6:	RAN3 to capture the following conclusions for T1:

 From RAN3 perspective, it is feasible to support Indirect path solution or Direct path solution.

 However, the Direct path solution is RAN3’s preference i.e., the AIoT RAN node directly communicates with the AIoTF (without going via AMF)

Proposal 7:	In Topology 1, irrespective of direct/indirect path solution, there is an end-to-end “XXAP” protocol between the AIoT RAN node and AIoTF

2.3 Coordination of A-IoT radio resources among readers

Proposal 8:	In Topology 1, there is no need to support Xn-like signaling between A-IoT RAN nodes to coordinate usage of A-IoT radio resources.

Proposal 9:	In Topology 1, the A-IoT RAN node can enable the coordination of the usage of the A-IoT radio resources among readers (within the A-IoT RAN node or across A-IoT RAN nodes) via implementation

2.4 Reader awareness at AIoTF

Observation 4:	In Topology 1, there are two possibilities regarding Reader “awareness” at AIoTF

- Option 1: Readers within an AIoT RAN node are “internal” and not known to the AIoTF

- Option 2: Readers within an AIoT RAN node are “discoverable” by the AIoTF

Observation 5:	In NR positioning, TRPs supported by a NG-RAN node and the TRP information can be made aware at the LMF via TRP Information Exchange procedure and there is no limitation to keep the TRPs “hidden” at the CN.

Proposal 10:	In Topology 1, Readers within an AIoT RAN node should be “discoverable” by the AIoTF

Proposal 11:	AIoTF obtains the information of Readers supported by an AIoT RAN node via XXAP signaling procedure.

2.5 Reader selection

Proposal 12:	AIoTF should select suitable AIoT RAN node(s) and Reader(s) within the selected AIoT RAN node(s) for performing A-IoT operations (e.g., Inventory/Command)

# Annex 1 : TP to 38.769

## 6.4	RAN architecture aspects

This clause attempts to identify and describe architectural elements necessary to define a RAN architecture for support of Ambient IoT embedded in the overall 5G system architecture in support of topology 1 and topology 2 (as defined in TR 38.848 [2]).

This chapter also attempts to identify a functional split between RAN and CN.

The logical system architecture for A-IoT consists of the following architectural elements:

A-IoT device:	Equipment with characteristics outlined e.g., in TS 22.369 [10] and TR 38.848 [2].

A-IoT RAN node:	Hosts certain functions for A-IoT in topology 1 as part of the functional split between RAN and CN.

Editor’s Note 4: Further details regarding A-IoT functions hosted in the A-IoT RAN node and the respective functional split to be decided by RAN2, RAN3 and SA2.

A-IoT radio:	Radio interface between A-IoT device and A-IoT RAN node in topology 1 and between A-IoT device and A-IoT-enabled UE in topology 2.

Editor’s Note 5: Further details on A-IoT radio to be discussed by RAN1 and RAN2.

A-IoT CN:	Hosts certain functions for A-IoT as of the functional split between RAN and CN and may include AMF, AIoTF, UPF etc. This is up to SA2 decision

AIoTF:               Core network function introduced to support Ambient IoT service for both the topology 1 and topology 2. Details up to SA2.

NOTE: the details of A-IoTF are subject to SA2.

Editor’s Note 6: Further details regarding A-IoT functions hosted in the A-IoTF and the respective functional split to be decided by RAN2, RAN3 and SA2.

XX interface:	Interface between the A-IoT RAN node and the A-IoTF in topology 1 or the interface between the A-IoT enabled gNB and the A-IoTF on which certain A-IoT specific functions are performed.

Editor’s Note 7: The functions represented by the XX interfaces are FFS. It is also FFS whether this interface represents a new logical interface or is equal to NG. E.g., for topology 1 it may only represent a single interface instance, e.g., a new interface between A-IoT RAN node and A-IoTF, for topology 2 it might represent either 2 interface instances, one instance for NG and one instance “XX” for a new interface between A-IoTF and A-IoT enabled gNB, or one instance for NG alone.

Common reader function:	A function that communicates with the A-IoT device by means of A-IoT radio.

Editor’s Note 8: Further details on Common reader function is to be discussed by RAN1 and RAN2.

A-IoT RAN node function:	A function that contains e.g., the control of the A-IoT radio resources used towards the A-IoT device.

Editor’s Note 9: further details are FFS. Note that “control of A-IoT radio resources” does not necessarily imply dynamic configuration of resources but could also rely on static assignment of resources by means of OAM. Aspects concerning coordination of the Upper Layer functions (e.g., Inventory, Command) e.g., in case these functions have to be performed over a multitude of instances of the Common Reader Function are FFS.

Reader: Used to refer to an entity which hosts the Common reader function (i.e., A-IoT RAN node in case of Topology 1 and A-IoT enabled UE in case of Topology 2)

### 6.4.1	Support of Topology 1

Figure 6.4.1-1 and Figure 6.4.1-2 depicts two variants of a logical system architecture for topology 1, where the Common reader function and A-IoT RAN node function are deployed within an A-IoT RAN node.

Figure 6.4.1-1 Logical system architecture for topology 1

Figure 6.4.1-1 Logical system architecture for topology 1 (direct path)

Figure 6.4.1-2 Logical system architecture for topology 1 (indirect path via AMF)

For Topology 1, architecture and protocol aspects of split RAN architecture are not studied.

In Topology 1, the XX interface could be based on NG or a new interface carried over NG or a new interface.

Figure 6.4.1-3 and 6.4.1-4 shows the Protocol stack for Topology 1 for direct path and indirect path respectively, assuming a SCTP-based transport:

Figure 6.4.1-3. Protocol Stack for Topology 1 (direct path)

Figure 6.4.1-4. Protocol Stack for Topology 1 (indirect path via AMF)

NOTE:	The protocol stack in Figure 6.4.1-3 and 6.4.1-4 does not illustrate how A-IoT upper layer information, if any, is transported over XXAP. Details are subject to SA2 agreements.

For topology 1, the XXAP is terminated at an A-IoT RAN node.

The signalling transport for XXAP at the A-IoT RAN node is SCTP/IP. Other options of signalling transport for XXAP at the A-IoT RAN node (e.g., HTTP/2/TLS/TCP) were discussed, but will not be pursued.

In Topology 1, an A-IoT RAN node may serve one or more readers.

The A-IoT RAN node should enable the coordination of the usage of the A-IoT radio resources among readers within the A-IoT RAN node or across A-IoT RAN nodes via implementation. There is no need to support Xn-like signalling between the A-IoT RAN nodes to coordinate usage of A-IoT radio resources among readers.

Reader selection may need coordination between A-IoT RAN node and A-IoTF.

In Topology 1, Readers within an AIoT RAN node are “discoverable” by the AIoTF. AIoTF obtains the information of Readers supported by an AIoT RAN node via XX signaling procedure. With this knowledge, AIoTF can select the AIoT RAN node(s) and readers within the selected A-IoT RAN node(s) for Inventory/Command.

### 6.4.1	Conclusion for Topology 1

From RAN3 perspective, indirect path solution has the following drawbacks compared to Direct path solution:

- Need upgrades at legacy AMF to support two new interfaces (AIoT RAN node-AMF, AMF-AIoTF)

- More backhaul latency (due to involvement of AMF)

- Potentially unnecessary involvement of AMF (just as a “router”) for AIoT

- Potentially additional protocol b/w AIoT RAN node and AMF (YYAP) needs to be defined to carry end-to-end packets between AIoT RAN node and AIoTF (XXAP)

From RAN3 perspective, it is feasible to support Indirect path solution or Direct path solution.

However, the Direct path solution is RAN3’s preference i.e., the AIoT RAN node directly communicates with the AIoTF (without going via AMF)

In Topology 1, irrespective of direct/indirect solution, there is an end-to-end “XXAP” protocol between the AIoT RAN node and AIoTF.