3GPP TSG-RAN3 Meeting #126	R3-247151

Orlando, USA, Nov 18th – 22nd, 2024

Agenda Item:	10.2

Source: 	Qualcomm Incorporated

Title: 	MRO enhancements for R18 mobility mechanisms

Document for:	Discussion/Decision

# Background

In this paper, we discuss potential MRO enhancements for R18 mobility mechanisms for the following objective in Rel-19 SON/MDT WI based on the open issues and agreements last meeting.

MRO enhancement for R18 mobility mechanisms, including, Lower layer triggered mobility (LTM), CHO with candidate SCGs, subsequent CPAC [RAN3, RAN2]:

- Specification of the inter-node information exchange, including possible enhancements to interfaces [RAN3]

- Identify and specify necessary UE reporting to enhance the mobility parameter tuning [RAN2]

# Discussion

## MRO for LTM

### RACH-less LTM failure scenarios

In the previous meetings, some companies proposed to discuss RACH-less LTM to RACH-based LTM “fall back” scenarios. In our understanding and as per TS 38.321, there is no “fall back” from RACH-less LTM to RACH-based LTM.

As per TS 38.321, UE considers RACH-less LTM as ongoing only if it receives a valid TA from LTM CSC or if the UE-based TA measurement is successful.

In other words, if the UE doesn’t receive a valid TA from LTM CSC or if the UE-based TA measurement is not successful, UE doesn’t even start the RACH-less LTM cell switch procedure and directly attempts a RACH-based LTM procedure.

If the UE receives a TA value “FFF” in the CSC or if the UE-based TA measurement is not successful, UE doesn’t even start the RACH-less LTM cell switch procedure and directly attempts a RACH-based LTM procedure. In other words, there is no “fall back” from RACH-less LTM to RACH-based LTM and there is only RACH-less LTM “initiation” failure.

However, there could be a scenario where the RACH-less LTM is initiated (because of UE having a valid TA either received from LTM CSC or via UE-based measurement) but the RACH-less LTM cell switch execution is not successful e.g., because UE couldn’t determine that the network has successfully received its first UL data. In such a case, T304 could expire.

RACH-less LTM cell switch execution may fail (i.e., T304 may expire) because UE couldn’t determine that the network has successfully received its first UL data after RACH-less LTM cell switch execution

Upon T304 expiry, UE performs cell selection and if the selected cell is an LTM candidate cell and if network configured the UE to try LTM after LTM execution failure (i.e., LTM recovery), then the UE attempts RACH-based LTM execution once, otherwise re-establishment is performed. We propose RAN3 to consider these the following scenarios as part of MRO for LTM:

RAN3 to consider the following scenarios:

  - RACH-less LTM “initiation” failure (e.g., due to TA = FFF received in CSC or unsuccessful UE-based TA measurement) resulting in UE triggering RACH-based LTM procedure

  - RACH-less LTM execution failure

    - followed by successful RACH-based LTM recovery

    - followed by RACH-based LTM recovery failure

### Distinguishing UE-based TA vs. NW provided TA

RAN2 agreed the following:

Unless RAN3 defines a NW-based solution: The UE logs and reports whether and how the UE got the TA value used for a failed LTM switch (gNB indicated or UE determined).

During LTM preparation, source gNB provides the UE with Early UL Sync configuration (obtained from candidate cells) and whether UE-based TA measurement should be performed for each LTM candidate cell. Also, the source gNB-DU knows that it has sent a PDCCH order to trigger Early UL sync procedure. The network can store this Early UL sync configuration and whether it has provided a PDCCH order and there is no need for UE to report it in SON reports.

For each LTM candidate cell, source gNB knows whether it has provided the UE with Early UL Sync configuration, whether it has configured the UE to perform UE-based TA measurement and whether it has sent a PDCCH order to trigger Early UL sync procedure with that candidate cell.

Source gNB can store the Early UL sync configuration and UE-based TA configuration and can determine whether the UE had determined the TA via UE-based TA measurement or gNB indicated TA value in CSC by “retrieving” the UE context upon receiving the RLF Report

Send an LS to RAN2 that RAN3 has agreed on NW-based solution and that there is no need for UE to report to the gNB in RLF Report how the UE got the TA value (gNB indicated or UE determined)

### Optimizing LTM candidate cells at gNB-CU

For failures due to wrong selection of candidate LTM cell, CU is in charge of root cause analysis and performs optimization.

Aim to log some info to deduce the ltmCandidate (similar like choCandidate) in SHR to indicate whether a neighbour cell is an LTM candidate cell or not, TBD if explicit/implicit.

During LTM preparation, gNB-CU determines the LTM candidate cells (e.g., based on L3 measurements from UE). It was also agreed in RAN3 meeting that gNB-CU is responsible for optimizing LTM candidate cells. But it is not yet clear how the gNB-CU optimizes the LTM candidate cell list (e.g., based on UE assistance in RLF Report or based on a network-based solution).

RAN2 further agreed the following:

Unless RAN3 defines a NW-based solution: Introduce an explicit indication in RLF-Report to indicate whether a neighbour cell is an LTM candidate cell.

In Rel-17, we similarly discussed how to optimize the CHO candidate cell list and there were 2 options discussed - a network-based solution and a UE based solution. After a contentious discussion, both solutions were agreed, albeit for different sub cases:

- UE based solution (for the case of CHO execution failure): UE reports the CHO candidate cell list in RLF Report, in case of CHO execution failure.

- Network based solution (for the case of RLF shortly after a successful CHO execution): Source gNB provides the CHO configuration (which includes CHO candidate cell list and CHO execution conditions) to target gNB in SN STATUS TRANSFER. Target gNB stores it and provides it back to source gNB in HANDOVER REPORT

In Rel-17, the UE based solution was agreed first, but then we realized that the UE releases the CHO configuration after a successful CHO execution and therefore UE based solution can’t work in the case that there is a RLF shortly after the successful CHO execution. Therefore, the network-based solution was also agreed.

To optimize CHO candidate cell list, Rel-17 specified two solutions - a UE based solution (UE reports the CHO candidate cell list in RLF Report) and a network-based solution (source gNB provides the CHO configuration to target gNB in SN STATUS TRANSFER which stores it and provides it back to source gNB in HANDOVER REPORT).

LTM is different from CHO in that we support subsequent LTM and therefore UE stores the LTM configuration even after a successful LTM execution. Also, Rel-18 only supports intra-CU LTM and therefore the gNB-CU can have the LTM candidate cells in its UE context even if there is inter-DU mobility within the gNB-CU.

In Rel-19 MRO for LTM, we should avoid having two solutions (i.e., both UE based and network-based solution) for optimizing LTM candidate cell list. Also, existing information e.g., the timeConnFailure and C-RNTI reported in RLF Report can be used to retrieve the UE’s LTM candidate cells at the time of RLF.

We propose to use a network-based solution as follows:

There is no need for UE to report the LTM candidate cell list (or an indication whether a cell was LTM candidate or not) in RLF Report. Existing information e.g., the timeConnFailure and C-RNTI reported in RLF Report can be used to retrieve the UE’s LTM candidate cells at the time of RLF. Send a LS to RAN2 informing this NW-based solution.

### UHI/MHR enhancements for LTM

A UE supporting LTM might undergo both LTM and normal L3 HO scenarios depending on the gNB implementation. A gNB might do a back-to-back L3 HO and LTM due to L3 execution conditions being met in gNB-CU and LTM execution condition met in gNB-DU.

Therefore, it useful for the gNB to know whether the UE’s visited cells were due to LTM or L3 HO. Without such knowledge, the gNB might unnecessarily be trying to optimize ping-pong handovers even though they happened due to different reasons (i.e., L3 HO and LTM)

We therefore propose RAN3 to discuss how to distinguish genuine ping-pong handovers vs. back-to-back L3 HO and LTM. One solution could be to optimize the UHI to include a HO-type (i.e., LTM or L3 HO) similar to how it has been agreed by RAN2 for RLF Report.

RAN3 should discuss how gNB can distinguish genuine ping-pong handovers vs. back-to-back L3 HO and LTM (e.g., via MHR/UHI enhancements)

Enhance UHI and/or MHR to include a “HO-type” (i.e., LTM or L3 HO).

### Optimizing beam information indicated in LTM CSC

From TS 38.300,

The gNB-DU decides to execute cell switch to a target cell and transmits an LTM cell switch command MAC CE triggering cell switch by including a target configuration ID which indicates the index of the candidate configuration of the target cell, a beam indicated with a TCI state, or beams indicated with DL and UL TCI states, and a timing advance command for the target cell, if available.

As mentioned above, the gNB-DU can indicate beam information (TCI state ID, UL TCI state ID) in LTM cell switch command MAC CE to the UE.

The UE can use this information during LTM execution e.g., considers the SSB associated to the TCI state indicated by TCI state ID field as the one used for configured uplink grant selection for the initial uplink transmission towards the candidate cell during RACH-less LTM cell switch.

It is possible that the gNB-DU might not have indicated the right TCI state ID and therefore the UE might have used the wrong one thereby resulting in LTM cell switch failure. We therefore propose the following:

RAN3 should discuss solutions on how to optimize the target beam(s) (indicated via TCI state(s)) in LTM cell switch command MAC CE to UE for LTM execution.

### LTM-CHO coexistence

Suppose CHO and LTM are both configured and there is a LTM failure (or a RLF shortly after a successful LTM). The following recovery mechanisms are possible, if configured by the network:

- UE will perform CHO recovery if the selected cell after RLF is a CHO candidate cell

- UE will perform LTM recovery if the selected cell after RLF is an LTM candidate cell

- It is up to UE on what recovery UE should perform if the selected cell after RLF is both a CHO candidate and LTM candidate cell

If no recovery is configured by the network, the UE simply performs cell selection and performs reestablishment on the selected cell

Further there could be scenarios where a UE is configured with both CHO candidate cells and LTM candidate cell. The gNB might want to avoid configuring CHO in case there is always an LTM that gets executed resulting in a “wasteful” CHO configuration.

Therefore, the following is proposed.

RAN3 should discuss scenarios when a UE is configured with both LTM and CHO and whether any enhancements are needed to enable joint MRO for L3 HO and LTM e.g., indicate to gNB that CHO was configured before LTM was executed (or vice-versa)

## 2.2 MRO for CHO with candidate SCG

### 2.2.1 Failure Scenarios

Agreements and open issues so far:

MRO for Case 1a, 1b, 2a, 2b, 3a, 3b, 7a, 7b, 8a, 8b, 9 will be addressed

Do not distinguish between fast MCG recovery/no-fast MCG recovery (after the CHO with candidate SCG execution failure)

FFS whether there is any specification impact for case 4/5/6.

Case 1c, 2c, 3c, 7c, 8c are FFS

FFS whether to support concurrent failure cases (RLF + SCG failure). The definition of these cases needs to be further clarified.

Concurrent MCG and SCG failures can be deprioritized as this is not specific to CHO with candidate SCG. This was also considered in R17/R18 (e.g., MCG failure before UE sends SCGFailureInformation) but no enhancements were specified.

The other scenario where fast MCG recovery via SCG could not be initiated because SCG fails immediately after MCG fails is already deprioritized as RAN3 agreed to not distinguish between fast MCG recovery/no-fast MCG recovery (after the CHO with candidate SCG execution failure). Therefore, the following is proposed:

There is no need to support concurrent failure cases (RLF + SCG failure) i.e., there is no need to support case 1c, 2c, 3c, 7c, 8c

### 2.2.2 Optimizing the association of PCell and PSCell

RLF Report is enhanced to include:

- Identifier of candidate PCell(s) which met the configured CHO execution conditions when the RLF is encountered

- Identifier of candidate PSCell(s) which met the configured CPAC execution conditions when the RLF is encountered

- Identifier of candidate PCell(s) or PSCell(s) that fulfilled execution conditions before the RLF is encountered.

FFS whether to enhance RLF Report to include the fulfilled events before the RLF is encountered

Consider a scenario where the UE encounters an RLF or SCG failure before the CHO with candidate SCG execution and non-associated PCells and PSCells met the execution condition

In the example below, Execution condition of candidate PCell A1 and candidate PSCell B3 is met but the CHO with candidate SCG execution didn’t happen because there is no (A1, B3) associated configuration.

It would be beneficial if the network can know what execution conditions were met (i.e., A1 and B3) so that the network can fine-tune the CHO with candidate SCG configuration eg., by including a (A1, B3) configuration or (A2, B3) confguration to avoid RLF or SCG failures in the future.

RAN3 considers “case 10:UE meets the execution conditions of non-associated candidate PCells and PSCells before encountering an RLF or SCG Failure” in the list of scenarios for MRO for CHO with candidate SCG

“Identifier of candidate PCell(s) or PSCell(s) that fulfilled execution conditions before the RLF is encountered” (already agreed) is sufficient to optimize case 10.

### 2.2.4 CHO Recovery after CHO with candidate SCG execution failure

When initial CHO execution attempt fails or HO fails, the UE performs cell selection, and if the selected cell is a CHO candidate and if network configured the UE to try CHO after handover/CHO failure, then the UE attempts CHO execution once, otherwise re-establishment is performed.

Suppose a UE receives a CHO with candidate SCG config, both the execution conditions are met but the CHO execution fails (e.g., RACH in target cell fails). UE performs cell selection, and the selected cell is also a CHO candidate cell.

As per the below text from 38.331, UE sets the choCellId (CHO recovery cell ID) in RLF Report only in case CHO recovery is attempted after a legacy CHO failure and not in case of CHO recovery is attempted after a CHO with candidate SCG execution failure.

1>  if the cell selection is triggered by detecting radio link failure of the MCG or re-configuration with sync failure of the MCG, except for an LTM cell switch procedure following cell selection performed while timer T311 was running, as specified in 5.3.7.3, or mobility from NR failure, and

1>  if attemptCondReconfig is configured; and

1>  if the selected cell is not configured with CondEventT1, or the selected cell is configured with CondEventT1 and leaving condition has not been fulfilled; and

1>  if the selected cell is one of the candidate cells for which the reconfigurationWithSync is included in the masterCellGroup in the MCG VarConditionalReconfig and the condExecutionCondPSCell is not configured for the corresponding condReconfigId in the MCG VarConditionalReconfig:

2>  if the UE supports RLF-Report for conditional handover, set the choCellId in the VarRLF-Report to the global cell identity, if available, otherwise to the physical cell identity and carrier frequency of the selected cell;

2>  apply the stored condRRCReconfig associated to the selected cell and perform actions as specified in 5.3.5.3;

NOTE 1:  It is left to network implementation to how to avoid keystream reuse in case of CHO based recovery after a failed handover without key change.

Currently, UE sets the choCellId (CHO recovery cell ID) in RLF Report only in case CHO recovery is attempted after a legacy CHO failure and not in case of CHO recovery is attempted after a CHO with candidate SCG execution failure.

We therefore have the following proposal:

RAN3 should consider scenarios where UE attempts CHO recovery after a CHO with candidate SCG execution failure.

UE should log the CHO recovery cell ID in RLF Report even when UE attempts CHO recovery after a CHO with candidate SCG execution failure.

Consider Scenario 2 again where the UE encounters an RLF before the CHO with candidate SCG execution because candidate PSCell (B3 in the example below) satisfies the execution condition but the associated candidate PCell (i.e., A2) doesn’t satisfy the corresponding execution condition.

Suppose the UE attempts CHO recovery and observed that both PCell A2 and PCell A3 are suitable cells, the UE should be able to prioritize selecting PCell A2 for CHO recovery because its associated PSCell’s (B3) execution condition was met before rather than selecting PCell A3 for CHO recovery because its associated PCell’s (B4) execution condition was not met before.

We therefore have the following proposal:

In case the UE attempts CHO recovery after a CHO with candidate SCG execution failure and observes that there are 2 or more suitable cells, UE may prioritize selecting a cell for CHO recovery based on whether its associated PSCell’s execution condition was met before or not

## 2.3 MRO for Subsequent CPAC (S-CPAC)

RAN3 agreements:

Work on the scenarios of failure in S-CPAC.

The optimization of non-failure scenarios (e.g., near failure and ping-pong) is not excluded.

R19 SON/MDT solution discussion is based on R18 work.

Reuse SCG FAILURE INFORMATION REPORT message over Xn for MN to report SCG failure of SCPAC to the concerned SN.

FFS whether to capture in stage-2 the following text: “For the CPAC failures which occurred during an S-CPAC procedure, CPC Execution above refers to the initial CPC execution or the subsequent CPC execution”

FFS whether the differentiation between initial and following CPC is needed

A subsequent CPAC is defined as a conditional PSCell addition or change procedure that is executed after a PSCell addition, a PSCell change, a PCell change or an SCG release based on pre-configured subsequent CPAC configuration of candidate PSCell(s) without reconfiguration and re-initiation of CPC/CPA.

The UE keeps the configured subsequent CPAC configuration (unless the network indicates to release it) and evaluates the execution conditions of candidate PSCells after completion of a PSCell addition, a PSCell change, a PCell change or an SCG release.

UE keeps the configured S-CPAC configuration unless the network indicates to release it

The S-CPAC configuration contains

- candidate SCG configuration(s) of candidate PSCell(s)

- execution conditions

- MCG configuration (to be applied when subsequent CPAC execution is triggered)

- reference configuration

- security update configuration

In case a UE configured with S-CPAC configuration encounters an SCG failure, it sends a SCGFailureInformation to MN (including the measurement results) and waits for MN’s subsequent action e.g., whether it wants UE to add a new PSCell or keep it in standalone mode

In case the MN wants to add a new PSCell for the UE and the new PSCell is an already prepared candidate PSCell whose configuration is provided to the UE as part of S-CPAC configuration, MN should be able to simply indicate the PSCell to add without providing the whole configuration. UE can simply activate the indicated PSCell’s configuration and start evaluating the S-CPAC configuration.

For example, UE encounters an SCG failure in source PSCell A0 and reports in SCGFailureInformation that B4 (an already prepared candidate, but not the immediate candidate) was the strongest cell at the time of SCG failure

We therefore have the following proposals:

RAN3 should discuss solutions how to perform an optimized PSCell addition post an SCG failure for a UE configured with S-CPAC configuration.

In case the MN wants to add a new PSCell for the UE and the new PSCell is an already prepared candidate PSCell whose configuration is provided to the UE as part of S-CPAC configuration, MN should be able to simply indicate the PSCell to add without providing the whole configuration.

Upon receiving such an indication to activate a PSCell, UE can simply activate the indicated PSCell’s configuration and start evaluating the S-CPAC configuration.

# Summary

2.1	MRO for LTM

Observation 1.	If the UE receives a TA value “FFF” in the CSC or if the UE-based TA measurement is not successful, UE doesn’t even start the RACH-less LTM cell switch procedure and directly attempts a RACH-based LTM procedure. In other words, there is no “fall back” from RACH-less LTM to RACH-based LTM.

Observation 2.	RACH-less LTM cell switch execution may fail (i.e., T304 may expire) because UE couldn’t determine that the network has successfully received its first UL data after RACH-less LTM cell switch execution

Proposal 1:	RAN3 to consider the following scenarios:

a.	RACH-less LTM “initiation” failure (e.g., due to TA = FFF received in CSC or unsuccessful UE-based TA measurement) resulting in UE triggering RACH-based LTM procedure

b.	RACH-less LTM execution failure

i.	followed by successful RACH-based LTM recovery

ii.	followed by RACH-based LTM recovery failure

Observation 3.	For each LTM candidate cell, source gNB knows whether it has provided the UE with Early UL Sync configuration, whether it has configured the UE to perform UE-based TA measurement and whether it has sent a PDCCH order to trigger Early UL sync procedure with that candidate cell.

Proposal 2:	Source gNB can store the Early UL sync configuration and UE-based TA configuration and can determine whether the UE had determined the TA via UE-based TA measurement or gNB indicated TA value in CSC by “retrieving” the UE context upon receiving the RLF Report

Proposal 3:	Send an LS to RAN2 that RAN3 has agreed on NW-based solution and that there is no need for UE to report to the gNB in RLF Report how the UE got the TA value (gNB indicated or UE determined)

Observation 4.	To optimize CHO candidate cell list, Rel-17 specified two solutions - a UE based solution (UE reports the CHO candidate cell list in RLF Report) and a network-based solution (source gNB provides the CHO configuration to target gNB in SN STATUS TRANSFER which stores it and provides it back to source gNB in HANDOVER REPORT).

Proposal 4:	There is no need for UE to report the LTM candidate cell list (or an indication whether a cell was LTM candidate or not) in RLF Report. Existing information e.g., the timeConnFailure and C-RNTI reported in RLF Report can be used to retrieve the UE’s LTM candidate cells at the time of RLF. Send a LS to RAN2 informing this NW-based solution.

Proposal 5:	RAN3 should discuss how gNB can distinguish genuine ping-pong handovers vs. back-to-back L3 HO and LTM (e.g., via MHR/UHI enhancements)

Proposal 6:	Enhance UHI and/or MHR to include a “HO-type” (i.e., LTM or L3 HO).

Proposal 7:	RAN3 should discuss solutions on how to optimize the target beam(s) (indicated via TCI state(s) in LTM cell switch command MAC CE to UE for LTM execution.

Proposal 8:	RAN3 should discuss scenarios when a UE is configured with both LTM and CHO and whether any enhancements are needed to enable joint MRO for L3 HO and LTM e.g., indicate to gNB that CHO was configured before LTM was executed (or vice-versa)

2.2 MRO for CHO with candidate SCG

Proposal 9:	There is no need to support concurrent failure cases (RLF + SCG failure) i.e., there is no need to support case 1c, 2c, 3c, 7c, 8c

Proposal 10:	RAN3 considers “case 10:UE meets the execution conditions of non-associated candidate PCells and PSCells before encountering an RLF or SCG Failure” in the list of scenarios for MRO for CHO with candidate SCG

Proposal 11:	“Identifier of candidate PCell(s) or PSCell(s) that fulfilled execution conditions before the RLF is encountered” (already agreed) is sufficient to optimize case 10.

Observation 5.	Currently, UE sets the choCellId (CHO recovery cell ID) in RLF Report only in case CHO recovery is attempted after a legacy CHO failure and not in case of CHO recovery is attempted after a CHO with candidate SCG execution failure.

Proposal 12:	RAN3 should consider scenarios where UE attempts CHO recovery after a CHO with candidate SCG execution failure.

Proposal 13:	UE should log the CHO recovery cell ID in RLF Report even when UE attempts CHO recovery after a CHO with candidate SCG execution failure.

Proposal 14:	In case the UE attempts CHO recovery after a CHO with candidate SCG execution failure and observes that there are 2 or more suitable cells, UE may prioritize selecting a cell for CHO recovery based on whether its associated PSCell’s execution condition was met before or not

2.3 MRO for Subsequent CPAC (S-CPAC)

Observation 6.	UE keeps the configured S-CPAC configuration unless the network indicates to release it

Proposal 15:	RAN3 should discuss solutions how to perform an optimized PSCell addition post an SCG failure for a UE configured with S-CPAC configuration.

Proposal 16:	In case the MN wants to add a new PSCell for the UE and the new PSCell is an already prepared candidate PSCell whose configuration is provided to the UE as part of S-CPAC configuration, MN should be able to simply indicate the PSCell to add without proviing the whole configuration.

Proposal 17:	Upon receiving such an indication to activate a PSCell, UE can simply activate the indicated PSCell’s configuration and start evaluating the S-CPAC configuration.

# 4 Annex 1: MRO for CHO with candidate SCG - Scenarios

| Cases | Figure: CHO with candidate SCG(s) is configured | Brief Description |
| --- | --- | --- |
| Case 1 |  | Failure happens before CHO and/or CPAC execution. 1a) RLF@srcPCell 1b) SCG failure 1c) MCG failure+SCG failure |
| Case 2 |  | CPC/CPA execution condition is met. Failure happens before CHO execution condition is met. 2a) RLF@srcPCell 2b) SCG failure 2c) MCG failure + SCG failure |
| Case 3 |  | CHO execution condition is met. No CHO only or CHO with SCG configuration. Failure happens before CPAC execution condition is met. 3a) RLF@srcPCell 3b) SCG failure 3c) MCG failure + SCG failure |
| Case 4 |  | CHO execution condition is met. There is CHO only or CHO with SCG configuration. CHO execution to a candidate PCell fails. |
| Case 5 |  | CHO execution condition is met. There is CHO only or CHO with SCG configuration. RLF shortly after CHO execution success |
| Case 6 |  | CHO execution condition is met. There is CHO only or CHO with SCG configuration. SCG failure shortly after CHO execution success |
| Case 7 |  | Both CHO and CPAC execution conditions are met. 7a) CHO execution to a candidate PCell failure. 7b) CPAC to a candidate PSCell failure. 7c) Execution to SCG failure and toMCG failure. |
| Case 8 |  | Failure happens shortly after CHO and CPAC execution success 8a) RLF@targetPCell 8b) SCG failure 8c) MCG failure + SCG failure |
| Case 9 |  | Failure due to the execution conditions for candidate PCell and the associated candidate PSCell cannot be met simultaneously. 9a) RLF@srcPCell 9b) SCG failure 9c) MCG failure + SCG failure |