3GPP TSG-RAN WG3 Meeting #126	R3-247189

Orlando, US, 18 - 22 Nov, 2024

Title:	(TP to TR 38.769) A-IoT RAN Architecture aspects

Agenda Item:	16.2

Source:	Huawei

Document for:	other

# Introduction

This contribution provides our analyses on someof the leftover aspects on A-IoT Architecture aspects, including:

- Resource Control/Allocation Function

- Reader Selection Function

- Protocol Stack for T2 UP/NAS based solution

- XX interface

- Comparision of the Topology 2 solutions

And the corresponding TP to RAN3 baseline pCR is provided in section 5.

# Discussion

## Resource Control/Allocation Function

The following can be found in current RAN3 pCR:

A-IoT RAN node function: A function that contains e.g., the control of the A-IoT radio resources used towards the A-IoT device. 6.4 Editor’s Note 9: further details are FFS. Note that “control of A-IoT radio resources” does not necessarily imply dynamic configuration of resources but could also rely on static assignment of resources by means of OAM. Aspects concerning coordination of the Upper Layer functions (e.g., Inventory, Command) e.g., in case these functions have to be performed over a multitude of instances of the Common Reader Function are FFS. For Topology 1: - The A-IoT RAN node should enable the coordination of the usage of the A-IoT radio resources among readers. - 6.5.2.1 step 1b. The A-IoT RAN node allocates and coordinates the usage of A-IoT radio resources. For Topology 2: - 6.4.2 Editor’s Note 4: The A-IoT enabled gNB performs radio resource management for A-IoT related radio resources, details are pending on RAN1 and RAN2 mechanisms. - In Topology 2, the RAN architecture should enable the coordination of the usage of the A-IoT radio resources among readers. - 6.5.3.1.1 RRC based solution step 1b. The A-IoT enabled gNB allocates and coordinates usage of A-IoT radio resources. 6.5.1.3 A-IoT radio resource allocation in case of NAS/UP based solutions In NAS/UP based solutions, A-IoT radio resources can be requested in advance to the NAS/UP based communication with the A-IoT device or can be requested along with the NAS/UP based communication. There are different ways to trigger A-IoT radio resource allocation, e.g., upon CN request or upon UE request, etc. Some aspects of A-IoT radio allocation may be preconfigured by OAM. 6.5.3.1.2 Editor’s Note 2: how and where to depict signalling suitable for triggering A-IoT RAN node functions for A-IoT radio resource management needs further discussions for direct communication between A-IoT CN and A-IoT-enabled UE.

It is noted that RAN2 also have the following agreemetns:

- RAN2 is responsible for the interface between intermediate node (i.e. Reader) and RAN for topology 2. - Dedicated resource configuration is only given to the UE reader via dedicated signalling. Mechanisms for shared resource pool amongst readers are not considered in this release. - The AIoT radio resource can be (re)configured during RRC reconfiguration (including during HO procedure, after re-establishment, etc).

For Topology 1, this resource control/allocation function has already well captured in the pCR.

For Topology 2, as agreed by RAN2, the details of the Radio Resource control/allocation over NR Uu are subject to RAN2, we can refer to section 6.3.6 of the TR. And for T2 RRC based solution, Radio Resource control/allocation step has already been captured in the call flow. For T2 NAS/UP based solution, section 6.5.1.3 has already captured how to achieve these aspects, and based on offline email discussion, there will be steps related to these aspects to be added for NAS/UP based solutions.

Proposal 1: On the Resource Control/Allocation Function, remove the 6.4 Editor’s Note 9 and 6.5.3.1.2 Editor’s Note 2, complete the discussion on this topic in the study.

## Reader Selection Function

The following agreements were made in last meeting on Reader Selection Function:

- in T1, reader selection may need the coordination between AIoT RAN and AIoT CN;

- in T2, all options are still open, different alternatives are possible as per call flows.

- In Topology 1, an A-IoT RAN node may serve one or more readers.

And the following can be found in current pCR:

For Topology 1: - Reader selection may need coordination between A-IoT RAN node and A-IoT CN. - 6.5.2 Editor’s Note 1: Reader selection is FFS. For Topology 2: - 6.4.2 Editor’s Note 5: In Topology 2, it is FFS on reader selection, may need coordination between A-IoT RAN node and A-IoT CN. - 6.5.3 Editor’s Note: Reader selection is FFS.

For Topology 1, the A-IoT CN will first select the A-IoT RAN and trigger the Inventory Request towards these A-IoT RAN, and within the Inventory Request, the A-IoT CN may further provide TRP/Reader ID list, e.g., based on OAM configuration, or stored information from previous Inventory Reports, and these are stage 3 details.

For Topology 2, in NAS/UP based solution, A-IoT CN selects the A-IoT-enabled UE. And in RRC based solution, similar to Topology 1, Reader selection may also need coordination between A-IoT-enabled gNB and A-IoT CN.

Proposal 2: On the Reader Selection Function, for Topology 1, current description in the pCR is sufficient. For Topology 2, capture in the pCR that: In NAS/UP based solution, A-IoT CN selects the A-IoT-enabled UE. In RRC based solution, Reader selection may need coordination between A-IoT-enabled gNB and A-IoT CN.

## Protocol Stack for T2 UP/NAS based solution

In last meeting, for T2, only the protocol stack of RRC based solution was captured, the protocol stacks for NAS/UP based solutions were removed from the agreed TP.

There were few questions asked, including:

Question 1: Whether to show A-IoT CN in the protocol stacks?

Based on further offline email discussion before the meeting, the protocol stacks are futher updated to align with SA2 TR, with the AIoTF behind AMF/UPF.

Question 2: Whether to capture other protocol stacks to show radio resource control for NAS/UP based solutions?

In the newly added section 6.5.1.3, several ways for the A-IoT radio resource allocation have already been listed, and there is one issue found but not answered for the additional protocol stacks dedicated for resource control, which is related to the necessity of XXAP interaction for the UE request case.

There is no need to add additional protocol stacks dedicated for resource allocation for NAS/UP based solutions.

Proposal 3: On the Protocol Stack for T2 UP/NAS based solution, agree the TP provided in R3-247190 [5].

## XX interface

The following can be found in current RAN3 pCR:

- XX interface: Interface between the A-IoT RAN and the A-IoT CN on which certain A-IoT specific functions are performed. - 6.4 Editor’s Note 7: The functions represented by the XX interfaces are FFS. It is also FFS whether this interface represents a new logical interface or is equal to NG. E.g., for topology 1 it may only represent a single interface instance, e.g., a new interface between A-IoT RAN and A-IoT CN, for topology 2 it might represent either 2 interface instances, one instance for NG and one instance “XX” for a new interface between A-IoT CN and A-IoT RAN, or one instance for NG alone. - In Topology 1, the XX interface could be based on NG or a new interface carried over NG or a new interface. - For topology 1, the XXAP is terminated at an A-IoT RAN node. - The signalling transport for XXAP at the A-IoT RAN node is SCTP/IP. - 6.4.2 Editor’s Note 2: In Topology 2, the XX interface could be based on NG or a new interface carried over NG or a new interface. XX signaling could be transported via XX-C or XX-U, which is FFS.

For this aspect, currently, based on SA2 interm conclusion, for Topology 1 and Topology 2, they probably decide to support multiple options, e.g., for T1, direct connection between RAN and AIoTF, and indirect connection via AMF, for T2, support both UP based solution and RRC based solution, but anyway they are not final conclusions yet.

From RAN3 perspective, the current Editor’s Notes are quite ok to be updated as NOTEs, and RAN3 can further discuss the details of XX interface at normative phase.

Proposal 4: On the XX interface, update existing 6.4 Editor’s Note 7 and 6.4.2 Editor’s Note 2 to NOTEs, and further discuss the details of XX interface at normative phase.

## Comparision of the Topology 2 solutions

In table 1, we try to compare the three solutions of Topology 2 from different perspectives:

Table 1: Comparison of the Topology 2 solutions

|  |  | RRC based solution | NAS based solution | UP based solution |
| --- | --- | --- | --- | --- |
| UE Reader authorization |  | Performed by A-IoT CN and inform to the A-IoT-enabled gNB. | Performed by A-IoT CN and inform to the A-IoT-enabled gNB. | Performed by A-IoT CN and inform to the A-IoT-enabled gNB. |
| A-IoT radio resource control at A-IoT-enabled gNB |  | Support | Support | Support |
| Awareness of A-IoT services by the A-IoT-enabled gNB |  | Yes | No | No |
| A-IoT procedures exchanging |  | XXAP + A-IoT-enabled UE’s RRC | A-IoT-enabled UE’s NAS | A-IoT-enabled UE’s PDU session |
| WGs impact | RAN2 | High | Low | Unclear which group to define the new layer above PDU layer. |
|  | RAN3 | Low (Resue XXAP Topo1) | Low |  |
|  | CT1 | Low | High |  |
|  | SA2 | Medium | Medium | Medium |

Considering that the down selection among these solutions is under SA2 discussion, there seems no strong need for RAN3 to capture a comparison of them in our TP.

# 3	Conclusion

In this contribution, we further discuss the architecture aspects for A-IoT, and get the following proposals:

Proposal 1: On the Resource Control/Allocation Function, remove the 6.4 Editor’s Note 9 and 6.5.3.1.2 Editor’s Note 2, complete the discussion on this topic in the study.

Proposal 2: On the Reader Selection Function, for Topology 1, current description in the pCR is sufficient. For Topology 2, capture in the pCR that: In NAS/UP based solution, A-IoT CN selects the A-IoT-enabled UE. In RRC based solution, Reader selection may need coordination between A-IoT-enabled gNB and A-IoT CN.

Proposal 3: On the Protocol Stack for T2 UP/NAS based solution, agree the TP provided in R3-247190 [5].

Proposal 4: On the XX interface, update existing 6.4 Editor’s Note 7 and 6.4.2 Editor’s Note 2 to NOTEs, and further discuss the details of XX interface at normative phase.

The corresponding TP is provided in section 5.

# 4	Reference

[1] R3-247036 (BL pCR to TR 38.769) Study on solutions for Ambient IoT in NR, Huawei, CMCC

[2] 3GPP TR 38.769 Study on solutions for ambient IoT (Internet of Things) (Release 19) V1.0.0 (2024-09)

[3] RP-240826 Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR, CMCC, Huawei, T-Mobile USA

[4] 3GPP TR 23.700-13 Study on Architecture support of Ambient power-enabled Internet of Things, (Release 19) V1.1.0 (2024-10)

[5] R3-247190 (TP to TR 38.769) Protocol Stacks for Topology 2, Huawei, CMCC, ZTE, CATT, NEC, Xiaomi, Lenovo, China Telecom, Samsung, Qualcomm

# 5	Text Proposal

--------------------Start of the Change--------------------

## 6.4	RAN architecture aspects

This clause attempts to identify and describe architectural elements necessary to define a RAN architecture for support of Ambient IoT embedded in the overall 5G system architecture in support of topology 1 and topology 2 (as defined in TR 38.848 [2]).

This chapter also attempts to identify a functional split between RAN and CN.

The logical system architecture for A-IoT consists of the following architectural elements:

A-IoT device:	Equipment with characteristics outlined e.g., in TS 22.369 [10] and TR 38.848 [2].

A-IoT RAN:	Hosts certain functions for A-IoT as part of the functional split between RAN and CN.

Editor’s Note 4: Further details regarding A-IoT functions hosted in the A-IoT RAN and the respective functional split to be decided by RAN2, RAN3 and SA2.

A-IoT radio:	Radio interface between A-IoT device and A-IoT RAN node in topology 1 and between A-IoT device and A-IoT-enabled UE in topology 2.

Editor’s Note 5: Further details on A-IoT radio to be discussed by RAN1 and RAN2.

A-IoT CN:	Hosts certain functions for A-IoT as of the functional split between RAN and CN.

NOTE 1: the details of A-IoT CN are subject to SA2.

Editor’s Note 6: Further details regarding A-IoT functions hosted in the A-IoT CN and the respective functional split to be decided by RAN2, RAN3 and SA2.

XX interface:	Interface between the A-IoT RAN and the A-IoT CN on which certain A-IoT specific functions are performed.

NOTE x: The functions represented by the XX interfaces and whether this interface represents a new logical interface or is equal to NG needs further discussion in normative phase. E.g., for topology 1 it may only represent a single interface instance, e.g., a new interface between A-IoT RAN and A-IoT CN, for topology 2 it might represent either 2 interface instances, one instance for NG and one instance “XX” for a new interface between A-IoT CN and A-IoT RAN, or one instance for NG alone.

Common reader function:	A function that communicates with the A-IoT device by means of A-IoT radio.

Editor’s Note 8: Further details on Common reader function is to be discussed by RAN1 and RAN2.

A-IoT RAN node function:	A function that contains e.g., the control of the A-IoT radio resources used towards the A-IoT device.

NOTE y:  “control of A-IoT radio resources” does not necessarily imply dynamic configuration of resources but could also rely on static assignment of resources by means of OAM.

### 6.4.1	Support of Topology 1

Figure 6.4.1-1 depicts a logical system architecture for topology 1, where the Common reader function and A-IoT RAN node function are deployed within an A-IoT RAN.

Figure 6.4.1-1 Logical system architecture for topology 1

For Toplogy 1, architecture and protocol aspects of split RAN architecture are not studied.

In Topology 1, the XX interface could be based on NG or a new interface carried over NG or a new interface.

Figure 6.4.1-2 shows the Protocol stack for Topology 1, assuming a SCTP-based transport:

Figure 6.4.1-2. Protocol Stack for Topology 1

NOTE:	The protocol stack in Figure 6.4.1-2 does not illustrate how A-IoT upper layer information, if any, is transported over XXAP. Details are subject to SA2 agreements.

For topology 1, the XXAP is terminated at an A-IoT RAN node.

The signalling transport for XXAP at the A-IoT RAN node is SCTP/IP. Other options of signalling transport for XXAP at the A-IoT RAN node (e.g., HTTP/2/TLS/TCP) were discussed, but will not be pursued.

Editor’s Note 3: The A-IoT CN may include AMF and A-IoT related functions which is up to SA2 decision.

In Topology 1, an A-IoT RAN node may serve one or more readers.

The A-IoT RAN node should enable the coordination of the usage of the A-IoT radio resources among readers.

Reader selection may need coordination between A-IoT RAN node and A-IoT CN.

### 6.4.2	Support of Topology 2

Figure 6.4.2-1 depicts a logical system architecture for topology 2, where the Common reader function is located at an A-IoT-enabled UE, and the A-IoT RAN node function is located at an A-IoT-enabled gNB.

The following definitions apply:

A-IoT-enabled gNB:	A gNB supporting A-IoT RAN node function in topology 2, which is able to communicate with the A-IoT-enabled UE via NR Uu interface.

A-IoT-enabled UE:	A UE supporting Common reader function, which is able to communicate with the A-IoTdevice via the A-IoT radio interface.

Figure 6.4.2-1 Logical system architecture for topology 2

Editor’s Note 1:	Figure 6.4.2-1 doesn’t illustrate the protocol between A-IoT enabled UE and A-IoT CN, if needed, the figure needs to be revised in case such is defined by SA2.

NOTE x: In Topology 2, the XX interface could be based on NG or a new interface carried over NG or a new interface. XX signaling could be transported via XX-C or XX-U.

Editor’s Note 3:	The A-IoT CN could include AMF and A-IoT related functions, which is up to SA2 decision.

The A-IoT enabled gNB performs radio resource management for A-IoT related radio resources, details are subject to RAN1 and RAN2 mechanisms.

In Topology 2, the RAN architecture should enable the coordination of the usage of the A-IoT radio resources among readers.

An A-IoT-enabled gNB could support both topology 1 and topology 2, this is an implementation matter.

In NAS/UP based solution, A-IoT-enabled UE is selected by the A-IoT CN.

In RRC based solution, A-IoT-enabled UE selection may need coordination between A-IoT RAN node and A-IoT CN.

#### 6.4.2.1	Solutions for Topology 2

##### 6.4.2.1.0	General

To support Topology 2, the following solutions are to be studied for conveying A-IoT upper layer information:

-	RRC based solution. With this solution, A-IoT CN applies A-IoT upper layer information explicitly over XXAP signaling. A-IoT upper layer information is then relayed explicitly to/from the A-IoT-enabled UE via NR Uu RRC.

-	NAS based solution. With this solution, there is no explicit termination of A-IoT upper layer information at A-IoT-enabled gNB. A-IoT upper layer information is transmitted over A-IoT enabled UE's NAS.

-	UP based solution. With this solution, there is no explicit termination of A-IoT upper layer information at A-IoT-enabled gNB. A-IoT upper layer information is transmitted as A-IoT-enabled UE's user plane data.

NOTE:	The protocol stack for each solution option does not illustrate A-IoT CN internal architecture and how A-IoT upper layer information is transported, if any. Details are subject to SA2 agreements.

##### 6.4.2.1.1	Solution1: RRC based solution

Upon receiving XXAP: A-IoT related message from A-IoT CN, the A-IoT-enabled gNB transmits the related information towards the A-IoT-enabled UE via NR Uu RRC, and vice versa.

Figure 6.4.2.1.1-1: RRC based solution of Topology 2

--------------------End of the Change--------------------