3GPP TSG-RAN3 Meeting #126	R3-247199

# Orlando, FL, US, 18th – 22nd November, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR | 1416 | rev | - | Current version: | 17.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on IAB RESOURCE COORDINATION |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Lenovo, Nokia, Nokia Shanghai Bell, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IAB_enh-Core |  |  |  |  |  | Date: |  |  | 2024-11-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - In R17, IAB RESOURCE COORDINATION REQUEST/RESPONSE messages are introduced to exchange information of boundary node and parent node for the purpose of IAB resource coordination. According to TS 38.423, the Parent Node Cells List IE contains list of cells served by the parent node IAB-DU. However the parent node of an IAB-node could be an IAB node or an IAB-donor DU. So the description of the Parent Node Cells List IE is incorrect and needs to be updated to cover the scenario where the parent node of an IAB-node is an IAB-donor DU as well. - The range “maxnoofServedCellsIAB” is not used for IAB Cell Information IE. So the definition of the range “maxnoofServedCellsIAB” needs to be removed in clause 9.2.2.94. - The IE name of “Parent-Node Cells List” and “Parent-Node Cells List Item” needs to be corrected since it’s not alined with the procedure text. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update the description of the Parent Node Cells List IE to cover the scenario where the parent node of an IAB-node is an IAB-donor DU as well. - Remove the definition of the range “maxnoofServedCellsIAB” in clause 9.2.2.94. - Replace “Parent-Node Cells List” with “Parent Node Cells List”, replace “Parent-Node Cells List Item” with “Parent Node Cells List Item”. Impact analysis Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release). This CR has no impact on the functional point of view. The impact can be considered isolated because the change affects only the IAB resource coordination procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - The scenario where the parent node of an IAB-node is an IAB-donor DU is not supported for the parent node configuration exchange via IAB RESOURCE COORDINATION REQUEST/RESPONSE messages. - The definition of the range “maxnoofServedCellsIAB” in clause 9.2.2.94 is redundant. - IE name of “Parent-Node Cells List” and “Parent-Node Cells List Item” is not alined with the procedure text. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.4.6, 9.1.4.7, 9.2.2.94 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | - |  |  |  |  |  |  |  |  |

| This CR's revision history: |  |
| --- | --- |

-------------------------------------------Start of changes-------------------------------------------

9.1.4.6 	IAB RESOURCE COORDINATION REQUEST

This message is sent by an F1-terminating/non-F1-terminating IAB-donor to a non-F1-terminating/F1-terminating IAB-donor of a boundary IAB-node, for the purpose of coordination of the semi-static resources of a single or dual-connected boundary IAB-node.

Direction: F1-terminating IAB-donor  non-F1-terminating IAB-donor, non-F1-terminating IAB-donorF1-terminating IAB-donor.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| F1-terminating IAB-Donor UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | This IE refers to the Source NG-RAN node UE XnAP ID or to the M-NG-RAN node UE XnAP ID or to the S-NG-RAN node UE XnAP ID. | YES | reject |
| Non F1-terminating IAB-Donor UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | This IE refers to the Target NG-RAN node UE XnAP ID or to the S-NG-RAN node UE XnAP ID or to the M-NG-RAN node UE XnAP ID. | YES | reject |
| Boundary Node Cells List |  | 0..1 |  | List of cells served by the boundary IAB-node IAB-DU. | YES | reject |
| >Boundary Node Cells List Item |  | 1 .. <maxnoofServedCellsIAB > |  |  | EACH | reject |
| >>Boundary Node Cell Information | M |  | IAB Cell Information 9.2.2.94 |  | – |  |
| Parent Node Cells List |  | 0..1 |  | List of cells served by the parent node IAB-DU or parent IAB-donor-DU. | YES | reject |
| >Parent Node Cells List Item |  | 1 .. < maxnoofServingCells > |  |  | EACH | reject |
| >>Parent Node Cell Information | M |  | IAB Cell Information 9.2.2.94 |  | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofServedCellsIAB | Maximum number of cells served by an IAB-DU. Value is 512. |
| maxnoofServingCells | Maximum no. of serving cells for an IAB-MT. Value is 32, as defined by the maxNrofServingCells in TS 38.331 [10]. |

9.1.4.7	IAB RESOURCE COORDINATION RESPONSE

This message is sent by a non-F1-terminating/F1-terminating IAB-donor to an F1-terminating/non-F1-terminating IAB-donor of a boundary IAB-node, in response to an IAB RESOURCE COORDINATION REQUEST message.

Direction: non-F1-terminating IAB-donor  F1-terminating IAB-donor, F1-terminating IAB-donor  non-F1-terminating IAB-donor.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| F1-terminating IAB-Donor node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | This IE refers to the Source NG-RAN node UE XnAP ID or to the M-NG-RAN node UE XnAP ID or to the S-NG-RAN node UE XnAP ID. | YES | reject |
| Non F1-terminating IAB-Donor node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | This IE refers to the Target NG-RAN node UE XnAP ID or to the S-NG-RAN node UE XnAP ID or to the M-NG-RAN node UE XnAP ID. | YES | reject |
| Boundary Node Cells List |  | 0..1 |  | List of cells served by the boundary IAB-node IAB-DU. | YES | reject |
| >Boundary Node Cells List Item |  | 1 .. <maxnoofServedCellsIAB > |  |  | EACH | reject |
| >>Boundary Node cell Information | M |  | IAB Cell Information 9.2.2.94 |  | – |  |
| Parent Node Cells List |  | 0..1 |  | List of cells served by the parent node IAB-DU or parent IAB-donor-DU. | YES | reject |
| >Parent Node Cells List Item |  | 1 .. < maxnoofServingCells > |  |  | EACH | reject |
| >>Parent Node Cell Information | M |  | IAB Cell Information 9.2.2.94 |  | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofServedCellsIAB | Maximum number of cells served by an IAB-DU. Value is 512. |
| maxnoofServingCells | Maximum no. of serving cells for an IAB-MT. Value is 32, as defined by the maxNrofServingCells in TS 38.331 [10]. |

-------------------------------------------Next change-------------------------------------------

9.2.2.94	IAB Cell Information

This IE contains IAB-DU cell resource configuration, cell specific signal/channel configuration and multiplexing info of the cell of an IAB-node or IAB-donor-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| NR CGI | M |  | 9.2.2.7 |  |
| CHOICE IAB-DU Cell Resource Configuration-Mode-Info | O |  |  |  |
| >TDD |  |  |  |  |
| >>TDD Info |  | 1 |  |  |
| >>>gNB-DU Cell Resource Configuration-TDD | M |  | gNB-DU Cell Resource Configuration 9.2.2.95 | Contains TDD resource configuration of gNB-DU’s cell. |
| >>>Frequency Info | O |  | NR Frequency Info 9.2.2.19 |  |
| >>>Transmission Bandwidth | O |  | NR Transmission Bandwidth 9.2.2.20 |  |
| >>>Carrier List | O |  | NR Carrier List 9.2.2.63 | If included, the Transmission Bandwidth IE shall be ignored. |
| >FDD |  |  |  |  |
| >>FDD Info |  | 1 |  |  |
| >>>gNB-DU Cell Resource Configuration-FDD-UL | M |  | gNB-DU Cell Resource Configuration 9.2.2.95 | Contains FDD UL resource configuration of gNB-DU’s cell. |
| >>>gNB-DU Cell Resource Configuration-FDD-DL | M |  | gNB-DU Cell Resource Configuration 9.2.2.95 | Contains FDD DL resource configuration of gNB-DU’s cell. |
| >>>UL Frequency Info | O |  | NR Frequency Info 9.2.2.19 |  |
| >>>DL Frequency Info | O |  | NR Frequency Info 9.2.2.19 |  |
| >>>UL Transmission Bandwidth | O |  | NR Transmission Bandwidth 9.2.2.20 |  |
| >>>DL Transmission Bandwidth | O |  | NR Transmission Bandwidth 9.2.2.20 |  |
| >>>UL Carrier List | O |  | NR Carrier List 9.2.2.63 | If included, the UL Transmission Bandwidth IE shall be ignored. |
| >>>DL Carrier List | O |  | NR Carrier List 9.2.2.63 | If included, the DL Transmission Bandwidth IE shall be ignored. |
| IAB STC Info | O |  | 9.2.2.96 | STC configuration of this gNB-DU’s cell. |
| RACH Config Common | O |  | OCTET STRING | Includes the rach-ConfigCommon as defined in subclause 6.3.2 of TS 38.331 [10]. |
| RACH Config Common IAB | O |  | OCTET STRING | Includes the IAB-specific rach-ConfigCommonIAB-r16 as defined in subclause 6.3.2 of TS 38.331 [10]. |
| CSI-RS Configuration | O |  | OCTET STRING | Includes the NZP-CSI-RS-Resource IE as defined in subclause 6.3.2 of TS 38.331 [10]. |
| SR Configuration | O |  | OCTET STRING | Includes the SchedulingRequestResourceConfig IE as defined in subclause 6.3.2 of TS 38.331 [10]. |
| PDCCH Configuration SIB1 | O |  | OCTET STRING | Includes the PDCCH-ConfigSIB1 IE as defined in subclause 6.3.2 of TS 38.331 [10]. |
| SCS Common | O |  | OCTET STRING | Includes the subCarrierSpacingCommon as defined in subclause 6.2.2 of TS 38.331 [10]. |
| Multiplexing Info | O |  | 9.2.2.79 | Contains information on multiplexing with cells configured for collocated IAB-MT, if applicable. |

-------------------------------------------End of changes-------------------------------------------