3GPP TSG-RAN WG3 #126	R3-247223

Orlando, USA, Nov 18th – 22nd, 2024

Source:				NEC

Agenda item: 	10.2. MRO Enhancements

Title: 	(TP to BLCR of 38.473) Discussion on MRO for LTM with RLF-report unavailabile

Document for:				Discussion and decision

- Introduction

During previous RAN3 meetings, MRO enhancements for LTM were discussed, and the following agreements were made:

RAN3#123 bis: Work on scenarios of near failure LTM. Work on scenarios for the differentiation of too early LTM, too late LTM and LTM to wrong cell. RAN3#124: MRO for LTM: RAN3 to prioritize MCG LTM over SCG LTM. RAN3 takes RAN2#125-bis agreement on MRO for LTM scenarios as baseline for further study. For failures due to wrong selection of candidate LTM cell, CU is in charge of root cause analysis and performs optimization. For failures due to inappropriate cell switch triggering (e.g. wrong cell selection at cell switch, wrong cell switch timing, …) source DU performs optimization. RACH information in LTM: RAN3 to discuss enhancements related to the information of RACH-less access and RACH-based access from UE related to MRO for LTM. RAN3#125: MRO for LTM: Reuse the existing connection failure definition of too late handover, too early handover, handover to wrong cell for LTM failure case. Add a sentence for LTM handover. FFS to enhance failure scenario. Take the existing detection mechanism descriptions as baseline. For failures due to inappropriate cell switch triggering, CU initiates analysis and may forwards the RLF report to the DU responsible for the failure. CU needs to forward the RLF related information to DU using existing Access and Mobility Indication procedure. FFS for additional information needed. RAN3#125 bis: For TP to BL CR to TS 38.300: - Remove editor’s note 1 and 2. No need to capture LTM recovery in the failure definitions. - For detection mechanism, use same approach as for CHO i.e. reuse existing sub-bullets to add LTM. - Change“successful handover" definition, as proposed in R3-245230. For TP to BL CR to TS 38.401: - Capture wrong LTM prepared cell selection. Add failure type to F1AP ACCESS AND MOBILITY INDICATION message. CU does not forward the RLF Report to DU if failure is due to wrong selection of prepared LTM cell.

In this contribution, we would like to provide our further consideration on MRO for LTM.

- Discussion

The current RAN3 discussion on MRO for LTM only focuses on the case that RLF-report is fetched by the gNB-CU from the UE, or obtained from a neighbouring gNB. And as it was agreed last meeting, gNB-CU transmits to the gNB-DU the message including RLF-report and LTM failure type information. Additionally, source C-RNTI is also sent together in the message, as the RLF-report only includes the target C-RNTI of the UE in case of mobility failure. That is, it is assumed that the RLF-Report is always available at the CU when sending to the DU the information related with MRO for LTM.

Observation 1: The current discusssion on MRO for LTM focuses on the case that the RLF-report is available at the CU.

However, there may be cases that the RLF-report is not available at the CU. For example, it is not availabe because the UE does not have the RLF-report, i.e., the content of the RLF-report associated with the LTM failure is replaced by a following connection failure before being reported to the network. For another example, the RLF-report cannot be obtained by the gNB-CU due to network congestion.

Observation 2: There are some cases that RLF-report is not available at the CU.

For the case that RLF-report is not available for the gNB-CU, the gNB-CU can still identify the happening of LTM failure event. Below explains how gNB-CU identify the LTM failure events:

- Too late LTM:  after the configuring of LTM configuration to the UE, if the gNB-CU finds that LTM cell switch is not triggered by the gNB-DU, and the UE re-establishes/re-connectes to a cell of the network, it can identify the happening of too late LTM cell switch.

- Too early LTM: if the gNB-CU finds that a LTM cell switch is triggered by the gNB-DU, the UE does not access to the target LTM cell or soon after the UE accesses to the target LTM cell, the UE re-establishes/re-connects to the source cell of the LTM cell switch procedure, the gNB-DU can identify the happening of too early LTM cell switch.

- LTM to wrong cell:  if the gNB-CU finds that a LTM cell switch is triggered by the gNB-DU, the UE does not access to the target LTM cell or soon after the UE accesses to the target LTM cell, the UE re-establishes/re-connects to a cell which is neither the source cell nor the target cell of the LTM cell switch procedure, the gNB-DU can identify the happening of LTM cell switch to wrong cell.

Observation 3: CU can identify LTM failure even if RLF-report is not available for the CU.

For the cases with RLF-report absent, it is beneficial that the gNB-CU can indicate the LTM failure information to the gNB-DU, such that the gNB-DU can do some optimization e.g., to optimize the timing for the LTM cell switch execution as soon as possible.

Proposal 1: CU can send to DU the LTM failure information even when RLF-report is absent.

And in case of the absent of RLF-report, in addition to the failure type (i.e., too late LTM, too early LTM and LTM to wrong cell) and UE ID (gNB-DU F1AP ID and source C-RNTI), some basic information can also be provided to the gNB-DU by the gNB-CU, for example the ID of the source cell/failed cell of the gNB-DU, target cell of the LTM cell switch procedure, and ID of the re-established/re-connected cell.

Proposal 2: The following LTM failure information can be sent from CU to DU even when RLF-report is absent:

- Failure type (too late LTM, too early LTM and LTM to wrong cell)

- UE ID (gNB-DU F1AP ID, source C-RNTI)

- ID of the source/failed cell of the gNB-DU, and ID of target LTM cell

- ID of re-established/re-connected cell.

# 3. Conclusion

In this paper, we provide our further consideration on MRO for LTM, and have the following proposals:

Observation 1: The current discusssion on MRO for LTM focuses on the case that the RLF-report is available at the CU.

Observation 2: There are some cases that RLF-report is not available at the CU.

Observation 3: CU can identify LTM failure even if RLF-report is not available for the CU.

Proposal 1: CU can send to DU the LTM failure information even when RLF-report is absent.

Proposal 2: The following LTM failure information can be sent from CU to DU even when RLF-report is absent:

- Failure type (too late LTM, too early LTM and LTM to wrong cell)

- UE ID (gNB-DU F1AP ID, source C-RNTI)

- ID of the source/failed cell of the gNB-DU, and ID of target LTM cell

- ID of re-established/re-connected cell.

Annex	TP for LTM for 38.473

8.11.1	Access and Mobility Indication

8.11.1.1	General

This procedure is initiated by gNB-CU to send the Access and Mobility related Information to gNB-DU.

The procedure uses non-UE-associated signalling.

8.11.1.2	Successful Operation

Figure 8.11.1.2-1: Access and Mobility Indication procedure. Successful operation

The Access and Mobility Indication procedure is initiated by ACCESS AND MOBILITY INDICATION message sent from gNB-CU to gNB-DU.

If the ACCESS AND MOBILITY INDICATION message contains the RA Report List IE the gNB-DU shall take it into account for optimisation of RACH access procedures.

If the ACCESS AND MOBILITY INDICATION message contains the RLF Report Information List IE the gNB-DU shall take it into account for optimisation of mobility parameters.

If the ACCESS AND MOBILITY INDICATION message contains the Successful HO Report Information List IE the gNB-DU may take it into account for optimisation of mobility parameters.

If the ACCESS AND MOBILITY INDICATION message contains the Successful PSCell Change Report Information List IE, the gNB-DU may take it into account for optimisation of PSCell change/addition related parameters.

If the ACCESS AND MOBILITY INDICATION message contains the LTM Failure information IE, the gNB-DU may take it into account for optimization of LTM cell switch related paramenters.

8.11.1.3	Abnormal Conditions

Not applicable.

[snip]

### 9.2.10	Self Optimisation Support Messages

#### 9.2.10.1	ACCESS AND MOBILITY INDICATION

This message is sent by gNB-CU to gNB-DU to provide access and mobility information to the gNB-DU.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| RA Report List |  | 0..1 |  |  | YES | ignore |
| >RA Report Item |  | 1 .. <maxnoofRAReports> |  |  | - |  |
| >>RA Report Container | M |  | OCTET STRING | Includes the RA-ReportList-r16 IE as defined in subclause 6.2.2 in TS 38.331 [8]. | - |  |
| >>UE Assistant Identifier | O |  | gNB-DU UE F1AP ID 9.3.1.5 |  | - |  |
| RLF Report Information List |  | 0..1 |  |  | YES | ignore |
| >RLF Report Information Item |  | 1 .. <maxnoofRLFReports> |  |  | - |  |
| >>NR UE RLF Report Container | M |  | OCTET STRING | Includes the nr-RLF-Report-r16 IE contained in the UEInformationResponse message defined in TS 38.331 [8]. | - |  |
| >>UE Assistant Identifier | O |  | gNB-DU UE F1AP ID 9.3.1.5 |  | - |  |
| >>C-RNTI | O |  | 9.3.1.32 | C-RNTI allocated at the source gNB-DU | YES | ignore |
| >>RLF Report Failure Type | O |  | ENUMERATED (too late LTM, too early LTM, LTM to wrong cell,...) |  | YES | ignore |
| Successful HO Report Information List |  | 0..1 |  |  | YES | ignore |
| >Successful HO Report Information Item |  | 1 .. <maxnoofSuccessfulHOReports> |  |  | - |  |
| >>Successful HO Report Container | M |  | OCTET STRING | Includes the SuccessHO-Report IE as defined in subclause 6.2.2 in TS 38.331 [8]. | - |  |
| Successful PSCell Change Report Information List |  | 0..1 |  |  | YES | ignore |
| >Successful PSCell Change Report information Item |  | 1..<maxnoofSuccessfulPSCellChangeReports> |  |  | - |  |
| >>Successful PSCell Change Report Container | M |  | OCTET STRING | Includes the SuccessPSCell-Report IE as defined in TS 38.331 [8]. | - |  |
| LTM Failure information | O |  | x.x.x.x | Includes information of a LTM cell switch failure event. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofRAReports | Maximum no. of RA Reports, the maximum value is 64. |
| maxnoofRLFReports | Maximum no. of RLF Reports, the maximum value is 64. |
| maxnoofSuccessfulHOReports | Maximum no. of Successful HO Reports, the maximum value is 64. |
| maxnoofSuccessfulPSCellChangeReports | Maximum no. of Successful PSCell Change Reports. Value is 64. |

#### x.x.x.x	LTM failure Information

The LTM failure Information IE contains information about a LTM failure event.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| UE Assistant Identifier | M |  | gNB-DU UE F1AP ID 9.3.1.5 |  |
| C-RNTI | M |  | 9.3.1.32 | C-RNTI allocated at the source gNB-DU |
| LTM failure type | M |  | ENUMERATED (too late LTM, too early LTM, LTM to wrong cell,...) |  |
| Source cell CGI | M |  | NR CGI 9.3.1.12 | NR CGI of source cell of the LTM cell switch procedure or failed cell of the gNB-DU. |
| Target cell CGI | O |  | NR CGI 9.3.1.12 | NR CGI of target cell for LTM cell switch procedure |
| Re-establishment cell CGI | O |  | NR CGI 9.3.1.12 | NR CGI of cell where UE attempted re-establishment or where UE successfully re- connected after the LTM failure |