3GPP TSG-RAN WG3 #126	 R3-247225

Orlando, USA, 18th – 22nd November 2024

Agenda Item:	10.3.1

Source:	NEC

Title: 	Discussion on SON/MDT enhancements for NTN

Document for:	Discussion

1	Introduction

A new WID on Data collection for SON/MDT in NR standalone and MR-DC Phase 4 has been approved at the RAN#102 meeting [1]. In the WID, “Support of SON/MDT enhancements for Intra-NTN mobility” is included.

The objective of this work item is to specify data collection enhancement in NR standalone and MR-DC for SON/MDT purpose. The specific objectives of this work item are: - MRO enhancement for R18 mobility mechanisms, including, Lower layer triggered mobility (LTM), CHO with candidate SCGs, subsequent CPAC [RAN3, RAN2]: - Specification of the inter-node information exchange, including possible enhancements to interfaces [RAN3] - Identify and specify necessary UE reporting to enhance the mobility parameter tuning [RAN2] - Support of SON/MDT enhancements for [RAN3, RAN2]: - Intra-NTN mobility - Network Slicing - Support of the leftovers in Rel-18 SON/MDT [RAN3, RAN2]: - RACH optimization for SDT - MHI Enhancement for SCG Deactivation/Activation - MRO for MR-DC SCG failure If needed, co-operate with RAN1, SA2, SA5, CT4.

In this contribution, we describe our view for SON/MDT for NTN.

2	Discussion

2.1 MRO for time based CHO

In last meeting, the RLF report for single time-based trigger condition was discussed and following WA was made.[3]

WA: down-select among below three alternatives. Alt 1: the RLF report includes for single time-based trigger condition: - Absolute time (measured UTC time) when RLF occurred Alt 2: a flag to indicate whether the RLF happens before T1 or after T1 Alt 3: based on existing timers in RLF report. Further check the granularity of the current timers.

Now let’s look at current time-based condition. CondeventT1 is specified in TS38.331[4] as time-based CHO.

condEventT1-r17 SEQUENCE { t1-Threshold-r17 INTEGER (0..549755813887), duration-r17 INTEGER (1..6000) }, t1-Threshold The field counts the number of UTC seconds in 10 ms units since 00:00:00 on Gregorian calendar date 1 January, 1900 (midnight between Sunday, December 31, 1899 and Monday, January 1, 1900). duration This field is used for defining the leaving condition T1-2 for conditional HO event condEventT1. Each step represents 100ms.

The T1-Threshold is determined by absolute time (UTC) and is still set with a high granularity (i.e., 10 ms). Therefore, T1 is not a timer dependent on other events in the UE and has the flexibility to be set independently of the time at which T1 is configured. Duration, on the other hand, is defined as the time elapsed from T1.

Observation 1: The T1-Threshold, which is the trigger for time-based CHO, is determined by absolute time and has a high granularity (i.e., 10 ms), making it independent of the time at which the CHO is configured and providing high operational flexibility.

Next, in current spec [4], following time information has been included. These timers can be candidates to reuse for this purpose.

timeConnFailure This field is used to indicate the time elapsed since the last HO execution until connection failure. Actual value = field value * 100ms. The maximum value 1023 means 102.3s or longer. timeSinceCHO-Reconfig This field is used to indicate the time elapsed between the initiation of the last conditional reconfiguration execution towards the target cell and the reception of the latest conditional reconfiguration for this target cell. Actual value = field value * 100ms. The maximum value 1023 means 102.3s or longer. timeSinceFailure This field is used to indicate the time that elapsed since the connection (radio link or handover) failure. Value in seconds. The maximum value 172800 means 172800s or longer. In the case of failure(s) (either at source or at target or at both) associated to DAPS handover, this field indicates the time elapsed since the latest connection (radio link or handover) failure.

Here, we explain our view on how the following two cases agreed in RAN3#125 would be calculated if existing timers were reused.

Case 1 Too Late CHO Execution: RLF happens in the source cell before UE executes CHO (before or after T1 threshold starts), and the UE re-connects the cell other than the source cell. - Distinguish if the RLF happens before or after T1 Case 2 Too Early CHO Execution/CHO Execution to Wrong Cell: HOF happens or RLF happens shortly after successful CHO, and the UE re-connects in the source cell or a third cell.

For Case1: Too Late CHO Execution

Case 1-1 refers to the case where the RLF occurs before the T1-Threshold, while Case 1-2 refers to the case where the RLF occurs after the T1-Threshold.

For Case2: Too Early CHO Execution

Case 2-1 refers to cases where a hand-over failure occurs due to CHO execution, Case 2-2 refers to the case where the RLF occurs before the T1-Threshold + Duration, and Case 2-3 refers to the case where the RLF occurs after the T1-Threshold + Duration.

In both case1 and 2, the time when the RLF occurred can be determined by matching the timeSinceCHO-Reconfig or timeSinceFailure with the time of the respective network side event. However, it should be noted that the timeSinceCHO-Reconfig has a limited value range, so the period between the reconfiguration and T1-Threshold must be limited to 102.3 s to be able to determine the time. In addition, timeSinceFailure can only be calculated every 1 sec due to the rough granularity of the timer. Therefore, these are not sufficient compared to the value range and granularity of T1-Threshold in condEventT1, and the reuse of existing timers limits the flexibility of the SON function.

Observation 2: Existing timers do not have a sufficient duration or granularity compared to T1-threshold which is the trigger for time-based CHO.

Another option is ‘Alt 2: a flag to indicate whether the RLF happens before T1 or after T1’, but this only provides information on whether the T1-threshold value should be earlier or later in the network, and does not contribute to how much the value should be changed.

Therefore, ‘Alt 1: the RLF report includes for a single time-based trigger condition’, which is the most straightforward and has the same granularity and duration as the T1-threshold, is appropriate for the SON function in a Time-based CHO.

Proposal 1: The RLF report should include Absolute time (based on UTC) for time-based CHO.

2.2 MRO for Location based CHO

In last meeting, the RLF report for location-based trigger condition was discussed and following agreement was made.[3]

[Agreement] For earth fixed case, there is no need to report the measured distance from the UE. FFS on the earth moving case.

Now let’s look at current location-based condition for the earth moving case. CondeventD2 is specified in TS38.331[4] as location-based CHO with moving reference location.

condEventD2-r18 SEQUENCE { distanceThreshFromReference1-r18 INTEGER(0.. 65535), distanceThreshFromReference2-r18 INTEGER(0.. 65535), hysteresisLocation-r18 HysteresisLocation-r17, timeToTrigger-r18 TimeToTrigger } distanceThreshFromReference1, distanceThreshFromReference2 Distance from a fixed reference location configured with referenceLocation1 or referenceLocation2 for condEventD1. Distance from a moving reference location determined by the UE based on the serving cell movingReferenceLocation broadcast in SIB19 or referenceLocation and the corresponding epoch time and satellite ephemeris configured within the MeasObjectNR associated to the event for condEventD2. Each step represents 50m. timeToTrigger Time during which specific criteria for the event needs to be met in order to execute the conditional reconfiguration evaluation. HysteresisLocation-r17 ::= INTEGER (0..32768) The IE HysteresisLocation is a parameter used within entry and leave condition of a location based event triggered reporting condition. The actual value is field value * 10 meters.

In also the earth moving case, we think that it is sufficient to combine the existing location information reported in the RLF report with the information held by the network, and there is no need to report relative distances to optimize location-based CHOs.

Observation 3: Current locationInfo is sufficient for Location based CHO.

Proposal 2: 	For location-based CHO, locationInfo in RLF report is sufficient, and there is no need to introduce further location information.

3	Summary

In this discussion paper, SON for NTN enhancements were discussed and the following observations and proposals were made:

Observation 1: The T1-Threshold, which is the trigger for time based CHOs, is determined by absolute time and has a high granularity of 10 ms, making it independent of the time at which the CHO is configured and providing high operational flexibility.

Observation 2: Existing timers do not have a sufficient duration or granularity compared to T1-threshold which is the trigger for time-based CHO.

Proposal 1: 	 RLF report should include Absolute time (based on UTC).

Observation 3: Current LocationInfo is sufficient for Location based CHO.

Proposal 2:	For location-based CHO, locationInfo in RLF report is sufficient, and there is no need to introduce further location information.

4	References

- RP-234038, New WID: Data collection for SON (Self-Organising Networks)/MDT (Minimization of Drive Tests) in NR standalone and MR-DC (Multi-Radio Dual Connectivity) Phase 4, RAN#102, CMCC

- R3-245757, SoD of SON for intra-NTN mobility, RAN3#125-bis, Huawei(moderator)

- Draft meeting report RAN3#125-bis, vOct25

- TS38.331, Radio Resource Control (RRC) protocol specification, v18.3.0