3GPP TSG RAN WG3 126	R3-247233

Orlando, USA, Nov 18th –22nd, 2024

Title: 	Discussions on enhancements for AIML support for CCO

Source: 	Jio

Agenda item:	11.3

Document Type:	Discussion and decision

# 1. Introduction

During the last plenary RAN#105 meeting in Melbourne Australia, the SI AI/ML was concluded as for the 3GPP TR 38.743 V2.0.0 (2024-08) and a New WID on Enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN (RP-242385, Acronym: NR_AIML_NGRAN_enh) was announced with the aim of specifying new AI/ML-based use cases and introduce further enhancements to finalize the Rel-18 leftovers based on the conclusions captured in TR 38.743.

During last RAN3#125-bis meeting discussions on timing information follow:

Introducing separate time information for future coverage state and predicted CCO issue.

Time for future coverage state: The point in time when the future coverage state will be applied.

Time for predicted CCO issue: The point in time when the CCO issue is predicted to happen.

The following question arises in the content:

How CU can deal with different future CCO  issues, generated in multiple DUs, for different predicted CCO issues?

In this Discussion contribution, we provide further discussion related the aforementioned open issue to AI/ML based Coverage and Capacity Optimization (CCO).

# 2. Discussion

## gNB CU Future CCO state Interpretation

Following 3GPP TS 38.300, sub-section 15.5.5.3:

“An NG-RAN node may autonomously adjust within and switch between coverage configurations. When a change is executed, a NG-RAN node may notify its neighbour NG-RAN nodes using the NG-RAN NODE CONFIGURATION UPDATE message with the list of cells and SSBs with modified coverage included. The list contains the CGI of each modified cell with its coverage state indicator and optionally the SSB index of each modified SSB with its coverage state indicator.

The coverage state indicator may be used at the receiving NG-RAN node to adjust the functions of the Mobility Robustness Optimisation, e.g. by using the coverage state indicator to retrieve a previously stored Mobility Robustness Optimisation state. The coverage state indicator may also be used at the receiving NG-RAN node to adopt coverage configurations matching with neighbouring cells coverage configurations”

It is logically concluded that based on a reactively detected CCO issue or a proactively predicted CCO issue in NG-RAN node 1 (gNB1), NG-RAN node 1 adjusts its configuration as a first step and then in a second step it notifies to a neighbor NG-RAN node 2 the future CCO (i.e. a list of cells and SSB beams) with the respective CCO coverage state indicators. This facilitates the receiving NG-RAN node 2 to react on the future CCO and adopt matching coverage configurations.

Imagine the following scenario of three gNBs covering the same area with different SSB beams, as per figure 1. Between gNB1 and gNB2 there is a coverage hole detected which proactively needs some rectifications on the SSB beams. Hence gNB1 CU will predict the CCO issue, inform via F1 interface the gNB1 DU about the condition, thus forcing the gNB1 DU to generate the future CCO state to resolve the problem. As a consequence gNB1 DU will send the future CCO state to the gNB1 CU which, on its turn, will forward via Xn interface the predicted and future CCO issue to the gNB2 CU.

Now according to the agreed AI/ML CCO split architecture the model inference it is located in the gNB2-CU, thus it is for the gNB2 CU to decide on the next proposal via F1 interface to gNB2 DU to match the gNB1 CCO modifications. Hence since the intelligence is on the gNB2-CU there is no obvious reason for the gNB2 CU to send the received future CCO state from gNB1 CU towards the gNB2 DU. In the opposite way it would rather make the solution more complicated in general, increasing the processing load, degrading the overall RAN performance and increasing the F1 signaling overhead.

Observation 1: Since the legacy model inference resides in the gNB2 CU functionality, any received future CCO issue via Xn interface from a neighbour gNB1 CU is recommended or better to be kept in the gNB2 CU and not sent to the gNB2 DU, in order to avoid complications in the functionality and make the performance, processing load and F1 signalling overhead optimum

According to figure 1 and the aforementioned approach the gNB1 CU would forward the future CCO state, as concluded from gNB1 DU, to the gNB2 CU. So it is then more optimum for MRO and for the benefit of the functional feasibility for the gNB2 CU to:

- consider the received future CCO state from gNB1 CU,

- conclude on a gNB2 CU CCO modification, using all available inference based on new prediction and local info

- send the new predicted gNB2 CU CCO via F1 interface to gNB2 DU for further actions, without sending the prior gNB1 CU received future CCO.

Proposal 1: The gNB2 CU considers the received gNB1 CU future CCO state, predicts with the help of local inference proactively a new CCO state based on received future CCO and any other local info, and finally sends to the gNB2-DU the new predicted CCO issue to generate and execute a new future CCO to match the gNB1 CCO issue, thus optimizing as well the MRO functionality.

Figure 1: Example of two coverage holes in a cluster of neighbor gNB-DU cells

Referring back to figure 1, the existence of one or more CCO coverage holes between more DUs would imply a major problem and further complications in case of multiple neighbor relations in a group (cluster) of neighbor cells via X2 interface. Continue our previous example, for the detected coverage hole gNB1 CU would forward the future CCO state, as concluded from gNB1 DU, to the gNB2 CU. In the same time and in a similar way between gNB2 and gNB3 there is another coverage hole detected which proactively needs some rectifications on the SSB beams. Hence gNB3 CU will predict the CCO issue, inform via F1 interface the gNB3 DU about the condition, thus forcing the gNB3 DU to generate the future CCO state to resolve the problem. As a consequence gNB3 DU will send the future CCO state to the gNB3 CU which, on its turn, will forward via Xn interface the predicted and future CCO issue to the gNB2 CU. Hence gNB2 CU receives two future CCO issue proposals via Xn interface, from gNB1 CU and gNB3 CU due to multiple DUs.

Now if we allow the gNB2 CU to send those two simultaneous future CCO issues to gNB2 DU it might cause severe problems in case of contradicting DU actions to solve simultaneously the coverage CCO holes from both proposals.

Observation 2: When more than one future CCO issues are received in one gNB CU, it is not recommended to send them to gNB DU since they might be totally contradicting and the gNB DU does not have the intelligence and the necessary AI/ML algorithms to handle.

Moreover it is not obvious following 3GPP TS 38.300 or 3GPP TR 38.743 how gNB2 DU would be able or capable to understand the simultaneous needs from all the reporting neighbor gNB CU future CCOs, or to jointly interpret them, or prioritize by any means and algorithms the multiple future forwarded CCO issues since it hold no intelligence or AI/ML algorithms for inference. In this case it makes no sense for the gNB2 CU to forward the two received future CCOs via F1 interface to the gNB2 DU.

Observation 3: If any number of received future CCO issues via Xn interface from a number of neighbour cells (i.e. gNB1 CU and gNB3 CU in figure 1) would be forwarded from gNB2 CU to gNB2 DU, it would eventually increase complexity leading into a non-optimal solution for the gNB2 DU CCO problem resolution.

Since the legacy model inference is located in the gNB2-CU, it is highly proposed to use the model inference intelligence in the CU functionality to review both received CCO issues from gNB1 and gNB3 CUs and decide on:

- either a joint optimum solution to the problem by predicting the most preferable CCO issue and sending it via F1 interface to the gNB2 DU.

- Or prioritize with some priority condition the CCO issues and apply actions sequentially

Observation 4: It is more efficient and beneficial for the network performance and F1 signalling overhead to use the gNB CU intelligence and inference capabilities to handle and analyse multiple received via Xn future CCOs and not to send them as individual future CCOs to gNB DU.

Proposal 2: Any received future CCO issue (including as well the single or multiple CCO issues from a cluster of multiple neighbour cells) should be kept on the receiving gNB CU entity and never forwarded to the gNB DU.

Proposal 3: For more than one future CCO issues the gNB-CU either combines to single interpretation or prioritizes sequentially.

What Jio proposes is, according to figure 1 and  gNB2 CU would:

- consider both the received future CCO state from gNB1 and gNB3 CUs,

- conclude on a jointly optimum gNB2 CU CCO modification, using all available inference based on new prediction and local info

- send the new predicted gNB2 CU CCO via F1 interface to gNB2 DU for further actions, without sending the prior gNB1 CU received future CCO.

We then propose and support the scenario of using the gNB CU AI/ML inference to (jointly in case of multiple) interpret the received future CCOs.

Proposal 4: The gNB CU should keep locally the received future CCO for AI/ML intelligent interpretation.

Proposal 5: The gNB CU, receiving neighbour future CCOs, should conclude on a new predicted CCO to forward it via F1 interface to the gNB DU.

# 3. Conclusions

Based on the discussion in this paper, we derived the following observations and proposals:

Observation 1: Since the legacy model inference resides in the gNB2 CU functionality, any received future CCO issue via Xn interface from a neighbour gNB1 CU is recommended or better to be kept in the gNB2 CU and not sent to the gNB2 DU, in order to avoid complications in the functionality and make the performance, processing load and F1 signalling overhead optimum

Proposal 1: The gNB2 CU considers the received gNB1 CU future CCO state, predicts with the help of local inference proactively a new CCO state based on received future CCO and any other local info, and finally sends to the gNB2-DU the new predicted CCO issue to generate and execute a new future CCO to match the gNB1 CCO issue, thus optimizing as well the MRO functionality.

Observation 2: When more than one future CCO issues are received in one gNB CU, it is not recommended to send them to gNB DU since they might be totally contradicting and the gNB DU does not have the intelligence and the necessary AI/ML algorithms to handle.

Observation 3: If any number of received future CCO issues via Xn interface from a number of neighbour cells (i.e. gNB1 CU and gNB3 CU in figure 1) would be forwarded from gNB2 CU to gNB2 DU, it would eventually increase complexity leading into a non-optimal solution for the gNB2 DU CCO problem resolution.

Observation 4: It is more efficient and beneficial for the network performance and F1 signalling overhead to use the gNB CU intelligence and inference capabilities to handle and analyse multiple received via Xn future CCOs and not to send them as individual future CCOs to gNB DU.

Proposal 2: Any received future CCO issue (including as well the single or multiple CCO issues from a cluster of multiple neighbour cells) should be kept on the receiving gNB CU entity and never forwarded to the gNB DU.

Proposal 3: For more than one future CCO issues the gNB-CU either combines to single interpretation or prioritizes sequentially.

Proposal 4: The gNB CU should keep locally the received future CCO for AI/ML intelligent interpretation.

Proposal 5: The gNB CU, receiving neighbour future CCOs, should conclude on a new predicted CCO to forward it via F1 interface to the gNB DU.

# References

[1] 3GPP TR 38.743 V2.0.0 (2024-08)