3GPP TSG-RAN WG3 Meeting #126                                                                   R3-247273

Orlando, USA, Nov 18th – 22nd, 2024

Source:	CATT

Title:	On support on-demand SIB1 for idle UE

Agenda Item:	17.3

Document for:	Discussion and Decision

1 Introduction

In RAN#105 meeting, RAN plenary agreed to turn the study of on-demand SIB1 for idle/inactive UEs to a normative phase from October. The on-demand SIB1 will focus on Case 2 only. And the new WID[1], in which the objectives for on-demand SIB1 include following aspects, has been agreed for Rel-19 NES enhancement.

- Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3] - Specify procedures and signaling method(s) for Case 2 [RAN1/2] - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell - Triggering method by UL WUS using PRACH - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3] - Note 1: No modification of SSB will be discussed under this objective - Note 2: - UL WUS: Uplink wake-up signal - Cell A: A cell that is periodically transmitting at least its own SIB1 - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE - Note 3: - RAN1 strives to minimize impact to legacy UE - RAN1 specification impact to support this feature should be minimized

During the former RAN3 meetings, following agreements were achieved for on-demand SIB1:

RAN3#123bis[2]: Scenario1: UE obtains the UL WUS configuration from Cell A, and transmits UL WUS to NES Cell. - UL WUS configuration from NES Cell to Cell A via Xn, e.g., before entering on-demand SIB1 mode in the NES Cell or when the UL WUS configuration is changed RAN3#124[3]: Focus on case2 in RAN3 first. Which node decides the WUS configuration and On-demand SIB1 mode? DU or CU or OAM? NES-cell gNB decides on-demand SIB1 operation activation and deactivation? RAN3#125[4]: UL WUS configuration is decided by NES Cell DU and transfers to NES Cell CU. UL WUS configuration transmission from NES Cell DU to NES Cell CU and to CellA. Whether On-Demand SIB1 activation/deactivation is needed？ Case 2 can be supported with the below details: - UL WUS configuration is decided by NES Cell DU. - UL WUS configuration transmission from NES Cell DU to NES Cell CU. NES gNB-CU sends UL WUS configuration to Cell A gNB-CU over Xn Interface. - Upon the reception of the UL WUS configuration, Cell A can provide feedback to NES Cell on whether the WUS configuration Transmission for NES Cell is accepted in Cell A (e.g., confirm, reject). FFS if Cell A broadcasts it right away or not. RAN3#125bis: The UL WUS configuration will be transferred over Xn via a new class1 defined procedure. Introduce the start/stop mechanism from NES cell to Cell A for UL WUS configuration broadcast. Further check on first activation of UL WUS configuration broadcast in Cell A. Cell A gNB can decide and signal to NES Cell gNB that it stops the UL WUS configuration broadcast in its SIB.

In this contribution, we will continue to discuss the on-demand SIB1 topic.

2 Discussion

2.1 On-demand SIB1 activation

In the previous meetings, it was agreed by RAN3 that WUS configuration is decided by NES gNB-DU and the WUS configuration is delivered to NES gNB-CU over F1 and then to Cell A gNB over Xn. However, it’s not clear how to start on-demand SIB1 operation.

2.1.1 F1AP signalling

Since SIB1 delivery is performed by gNB-DU, the energy used for SIB1 transmission is consumed by gNB-DU. The gNB-DU (instead of the gNB-CU) can be aware of energy consumed for SIB1 transmission. In addition, most of the energy used for base band processing are consumed by gNB-DU, so the energy efficiency of a cell can be obtained by gNB-DU. If the energy efficiency of the NES cell is low due to common signal transmission, e.g., when the user data rate is low or few UEs are connected, and the cell supports OD-SIB1 operations, gNB-DU can initiate operation of OD-SIB1 to the NES cell. When deciding to initiate the operation of OD-SIB1, the gNB-DU can send the WUS configuration for the NES cell to the NES gNB-CU, so that the NES gNB-CU delivers the WUS configuration to the Cell A gNB-CU.

However, since the cell operating OD-SIB1 cannot access legacy UEs, the normal cell should be deployed covering the area of the NES cell for legacy UEs to access. It is the gNB-CU (rather than gNB-DU) being aware of the network deployment. So, the NES gNB-DU should request for OD-SIB1 activation to NES gNB-CU and then NES gNB-CU confirms whether the request is permitted or refused. For example, the request with the WUS configuration can be delivered by NES gNB-DU using gNB-DU Configuration Update message. New IEs should be introduced in gNB-DU Configuration Update message for containing the request of the gNB-DU.

Further, according to the network deployment, a NES cell may need one or multiple Cell A(s) to broadcast its WUS configuration, and the multiple Cell As would be served by the same gNB of the NES cell or by different gNBs. It was agreed by RAN3 that Cell A gNB can provide feedback to NES gNB for whether the WUS configuration broadcast for NES cell is accepted or not. The NES gNB-CU can decide whether the request of OD-SIB1 activation can be allowed, e.g, based on the feedbacks from Cell A(s) on acceptance to the request of WUS configuration broadcasting or provision. Then the NES gNB-CU is supposed to indicate the decision to the NES gNB-DU. If there is any cell allowed to activate OD-SIB1 (i.e., enter OD-SIB1 mode), the allowed list of OD-SIB1 can be sent from the NES gNB-CU to the gNB-DU via gNB-CU Configuration Update message. The NES gNB-DU can activate the OD-SIB1 operation to the cells which are indicated by the allowed list of OD-SIB1.

Proposal 1: NES gNB-DU sends the request for on-demand SIB1 activation to NES gNB-CU with the WUS configuration by gNB-DU Configuration Update message.

Proposal 2: NES gNB-CU decide whether the request of on-demand SIB1 activation can be allowed .

Proposal 3: NES gNB-CU indicates whether on-demand SIB1 activation is allowed to NES gNB-DU using the allowed list of OD-SIB1 in gNB-CU Configuration Update message.

Proposal 4: NES gNB-DU can activate OD-SIB1 for the cells which are indicated by the allowed list of OD-SIB1.

On the Cell A side, for the Cell A which accepts broadcasting the WUS configuration for NES cell, the Cell A gNB-CU can deliver the WUS configuration to the Cell A gNB-DU. As RAN2 has decided that the WUS configuration is broadcasted via a new SIB (SIBx) of Cell A, the existing gNB-CU System Information IE in the gNB-CU Configuration Update message is used to convey the WUS configuration over F1 of Cell A gNB.

Proposal 5: For Cell A which accepts broadcasting the WUS configuration, the WUS configuration can be transmitted from Cell A gNB-CU to Cell A gNB-DU by existing gNB-CU Configuration Update message.

2.1.2 XnAP signalling

Per agreements achieved in RAN3#125 meeting, the NES gNB-CU sends WUS configuration, which is received from the NES gNB-DU, to Cell A gNB-CU over Xn. The Cell A gNB can provide feedback to NES gNB whether the WUS configuration can be broadcasted in Cell A.

UL WUS configuration transmission from NES Cell DU to NES Cell CU. NES gNB-CU sends UL WUS configuration to Cell A gNB-CU over Xn Interface. Upon the reception of the UL WUS configuration, Cell A can provide feedback to NES Cell on whether the WUS configuration Transmission for NES Cell is accepted in Cell A (e.g., confirm, reject). FFS if Cell A broadcasts it right away or not.

And in the last RAN3 meeting, it was agreed the WUS configuration will be provided by a new Class-1 procedure.During the offline discussion in the last RAN3 meeting, how Cell A responds the request of WUS configuration broadcast was discussed. Correspondingly, there was an issue whether NES gNB decides to activate OD-SIB1 based on the feedback from Cell A gNB(s) on acceptance of broadcasting WUS configuration, or NES gNB needs an indication from Cell A gNB(s) on starting (or restarting) to broadcast WUS configuration provision. There are two options:

Option A: Cell A responds confirmation to NES gNB at reception of request of WUS configuration broadcast. NES gNB decides to activate OD-SIB1 based on response (accept or reject) from Cell A gNB(s).

WUS configuration provision and request of WUS configuration broadcasting are combined to the same Xn message. For example, Cell A1 and Cell A2 are requested for WUS configuration broadcasting. Cell A1 decides to transmit WUS configuration right away or in the near future, so, it responds acceptance to NES gNB. Whereas, Cell A2 cannot make sure to transmit WUS configuration, then it responds rejecting WUS configuration broadcast. NES gNB receiving the response from both Cell A1 and Cell A2 may decide to enter OD-SIB1 mode (i.e., activate OD-SIB1 mode) based on the acceptance of Cell A1, or decide not to enter OD-SIB1 mode due to Cell A2’s reject.

In case NES gNB decides not to enter OD-SIB1, it may (or may not) indicate Cell A1 that broadcasting WUS configuration is not needed. After a period based on implementation, the NES gNB can initiate a next round of request for WUS configuration broadcasting to Cell A1 and Cell A2. And the NES gNB can decide to enter OD-SIB1 mode if Cell A2 responds acceptance.

Observation: If Cell A does not response acceptance of WUS configuration broadcast, NES gNB would need to request again before entering OD-SIB1 mode.

Option B: Cell A does not response confirmation to NES gNB until it starts broadcasting WUS configuration. NES gNB decides to activate OD-SIB1 based on indication from Cell A gNB(s) on starting the broadcas.

In Option B, Cell A delays response to NES gNB until it starting broadcasting. Take the similar example as Option A, Cell A1 and Cell A2 are provided the WUS configuration. Cell A1 decides to transmit WUS configuration right away or in the near future and it responds acceptance to NES gNB. Cell A2 cannot make sure to transmit WUS configuration, but it can accept the WUS configuration provision, then it also accepts the WUS configuration provision from NES gNB. Additionally, Cell A1 sends an indication to NES gNB when it has started broadcasting the WUS configuration right away or after a short period. NES gNB receiving the confirmation from Cell A1 may decide to enter OD-SIB1 mode.

However, due to the NES gNB does not know how long time it can receive Cell A2’s confirmation, it is not be able to decide the exact time to entering OD-SIB1 mode. On the other hand, as the NES gNB does not receive the explicit reject from Cell A2 and keeps waiting for the confirmation from Cell A2, it has no idea whether Cell A1 broadcasting WUS configuration is still needed. Then it would have no idea whether or when to request Cell A1 to stop broadcasting WUS configuration.

Observation: As NES gNB does not know how long it’s going to receive the confirmation of starting broadcast from Cell A, it would be uncertain about when to enter OD-SIB1, and lose the control of stopping WUS configuration broadcasting in other Cell As which has broadcasted the WUS configuration.

Comparing with two options, Option A is more straightforward, and it seems the procedure is more controllable from the NES gNB point of view. For Option A, although NES gNB needs to send the request again if Cell A did not respond acceptance of WUS configuration broadcast, the NES gNB can avoid asking Cell A too frequently, i.e., the interval of sending the request is up to NES gNB’s implementation.

Proposal 6: Cell A responds confirmation of WUS configuration broadcasting to NES gNB at reception of the request of WUS configuration broadcast. NES gNB decides whether to activate OD-SIB1 based on response (i.e. accept or reject) from Cell A gNB(s).

Nevertheless, WUS configuration provision and request of starting WUS configuration broadcast from NES gNB can be combined in one Xn message, for either Option A or B. That means the Xn message for requesting WUS configuration broadcast can optionally carry the WUS configuration if the WUS configuration of NES cell is unchanged from the last time.

Proposal 7: WUS configuration provision and request of starting WUS configuration broadcast are combined in a single Xn procedure.

From the RAN3#125 meeting, there is an FFS for whether Cell A broadcast WUS configuration right away after feedback the acceptance of broadcasting WUS configuration. So, the issue is whether another request by the NES gNB (e.g., OD-SIB1 activation indication) should be delivered to the Cell A gNB(s) which have accepted broadcasting WUS configuration for the Cell A(s) to start broadcasting. Because the NES gNB may not actually activate OD-SIB1 operation even if one or more Cell A accepted to broadcast WUS configuration, the Cell A gNB(s) which accepted broadcasting the WUS configuration would need an explicit indication on OD-SIB1 activation from the NES gNB, i.e., they will not broadcast the WUS configuration until receiving the activation indication. However, that will complicate the behaviour of Cell A gNB, as Cell A always has to judge whether broadcasting WUS configuration can be performed according to the OD-SIB1 activation indication. Secondly, this issue can be addressed by NES gNB sending a request message to inform the Cell A gNB(s) which feedback acceptance of broadcasting the WUS configuration that the broadcasting is not necessary, if OD-SIB1 operation is not actually activated by NES gNB.

Therefore, we assume there is no need to have another Xn message for requesting Cell A to broadcast WUS configuration, i.e., an additional indication for OD-SIB1 activation over Xn is not required. Cell A can start broadcasting the WUS configuration right after it responding the acceptance of broadcasting WUS configuration over Xn.

Proposal 8: Cell A gNB can start to broadcast the WUS configuration right after it responds confirmation of broadcasting WUS configuration.

Figure 1 Successful OD-SIB1 activation procedure

Figure 2 Unsuccessful OD-SIB1 activation

2.2 On-demand SIB1 deactivation

It was agreed by RAN3 that WUS configuration is decided by NES gNB-DU and the WUS configuration is delivered to NES gNB-CU over F1 and then to Cell A gNB over Xn. However, the procedure for deactivation the on-demand SIB1 operation has not been discussed.

2.2.1 F1AP signalling

After OD-SIB1 operation is activated, it’s possible that load of the NES cell increases due to many idle/inactive mode Rel-19 UEs begin to trigger OD-SIB1 or many connected mode UEs are handed over to the NES cell, that the energy saving for the NES cell would not benefit from OD-SIB1 operation any more. So, it’s necessary to deactivate the OD-SIB1 operation in that case, i.e., to turn the SIB1 transmission from on-demand mode to legacy mode, that broadcasting WUS configuration in Cell A is not needed. The NES gNB-DU can decide whether the OD-SIB1 deactivation is needed. When the NES gNB-DU decides to deactivate OD-SIB1, it can send the request to the NES gNB-CU over F1 interface, so that the NES gNB-CU can inform the Cell A broadcasting the WUS configuration for the NES cell is not required. The gNB-DU Configuration Update message can be used for requesting OD-SIB1 deactivation.

Proposal 9: NES gNB-DU sends the request for OD-SIB1 deactivation to NES gNB-CU by gNB-DU Configuration Update message.

Upon receiving the OD-SIB1 deactivation request, the NES gNB-CU would indicate the Cell A gNB(s) which accepted the WUS configuration broadcasting that WUS configuration broadcasting for the NES cell in the Cell A is not needed. Given RAN2 has decided that the WUS configuration is broadcasted via a new SIB (SIBx) of Cell A, in order to stop broadcasting the WUS configuration for the NES cell in Cell A which has already started the broadcasting, . the Cell A gNB-CU can remove/update the SIBx of Cell A using existing gNB-CU Configuration Update message.

Proposal 10: To make Cell A gNB-DU stop broadcasting the WUS configuration, Cell A gNB-CU can remove/update the SIB1x of Cell A via the existing gNB-DU Configuration Update message.

2.2.2 XnAP signalling

As Section 2.2.1, the NES gNB would inform the Cell A gNB(s) which accepted the WUS configuration broadcasting that it is not required to do the WUS configuration broadcasting, if the OD-SIB1 operation is deactivated. On the other hand, the NES gNB may decide to change the Cell A for the NES cell based on, e.g, the change of network deployment or operator’s strategy. In that case, some neighbour cells may be selected as the new Cell A for the NES cell, or some neighbour cell may not need to act as Cell A anymore. If the NES gNB selects any neighbour cell(s) as the new Cell A, it can request the neighbour cell(s) to broadcast WUS configuration for the NES cell as discussed in Section 2.1. On the contrary, if there is neighbour cell(s) not acting as Cell A anymore, the NES gNB should be able to inform the relevant cell A that broadcasting the WUS configuration for the NES cell is not required, so as to avoid wasting resource.

The information on not requiring WUS configuration broadcasting may be delivered by the Xn message for WUS configuration broadcast request in the procedure of WUS configuration broadcasting. The Cell A gNB receiving the indication from the NES gNB can cancel the WUS configuration broadcasting for the NES cell if it has not started the broadcasting, or the Cell A gNB can stop the broadcasting if it has started the broadcasting..

Proposal 11: The NES gNB can inform the Cell A gNB which has accepted broadcasting WUS configuration that it is not required to broadcast WUS configuration.

Proposal 12: The Cell A gNB receiving the information of not requiring the WUS configuration broadcast can cancel or stop the WUS configuration broadcasting.

Figure 3 OD-SIB1 deactivation procedure

2.3 Cell A gNB stops WUS configuration broadcasting

When the Cell A gNB has broadcasted the UL WUS configuration for the NES cell for a period of time, it may decide to stop broadcasting.

2.3.1 XnAP signalling

Following agreement was made in the last RAN3 meeting:

Cell A gNB can decide and signal to NES Cell gNB that it stops the UL WUS configuration broadcast in its SIB.

According to the agreement, the Cell A gNB can send an indication to the NES gNB on stopping WUS configuration transmission for the NES cell if it decides to stop broadcast. A new Class-2 Xn procedure can be introduced for this indication.

Proposal 13: A new Class-2 Xn procedure is introduced for Cell gNB to indicate NES gNB on stopping WUS configuration broadcast.

The NES gNB receives the indication should judge whether the NES cell can continue to operate OD-SIB1, as the NES cell may not be permitted to operate OD-SIB1 if one or more Cell A stop broadcasting the WUS configuration. If the NES gNB decides the OD-SIB1 operation cannot be continued in the NES cell, the NES cell should deactivate OD-SIB1.

Proposal 14: NES gNB receiving the indication on stopping WUS configuration broadcast can determine whether to deactivate on-demand SIB1 for the NES cell.

2.3.2 F1AP signalling

If the NES gNB-CU receives the indication over the Xn interface saying that the Cell A has stopped broadcasting the WUS configuration for NES cell, and it judges the NES cell is not permitted to continue operating OD-SIB1, it should indicate NES gNB-DU that the OD-SIB1 operation is not allowed in the NES cell so that it can deactivate OD-SIB1 operation in the NES cell, i.e., turn to legacy SIB1 transmission mode. The the allowed list of OD-SIB1 in gNB-CU configuration Update message can be used for the NES gNB-CU to send that information to NES gNB-DU, for example, NES gNB-CU can remove the NES cell from the allowed list of OD-SIB1.

Proposal 15: NES gNB-CU can indicate NES gNB-DU OD-SIB1 operation is not allowed in the NES cell for it to deactivate OD-SIB1 in the NES cell.

Figure 4 Procedure for stopping WUS configuration broadcast

3 Conclusion

In the previous sections we made the following observations and proposals:

On-demand SIB1 activation

Proposal 1: NES gNB-DU sends the request for on-demand SIB1 activation to NES gNB-CU with the WUS configuration by gNB-DU Configuration Update message.

Proposal 2: NES gNB-CU decide whether the request of on-demand SIB1 activation can be allowed .

Proposal 3: NES gNB-CU indicates whether on-demand SIB1 activation is allowed to NES gNB-DU using the allowed list of OD-SIB1 in gNB-CU Configuration Update message.

Proposal 4: NES gNB-DU can activate OD-SIB1 for the cells which are indicated by the allowed list of OD-SIB1.

Proposal 5: For Cell A which accepts broadcasting the WUS configuration, the WUS configuration can be transmitted from Cell A gNB-CU to Cell A gNB-DU by existing gNB-CU Configuration Update message.

Proposal 6: Cell A responds confirmation of WUS configuration broadcasting to NES gNB at reception of the request of WUS configuration broadcast. NES gNB decides whether to activate OD-SIB1 based on response (i.e. accept or reject) from Cell A gNB(s).

Proposal 7: WUS configuration provision and request of starting WUS configuration broadcast are combined in a single Xn procedure.

Proposal 8: Cell A gNB can start to broadcast the WUS configuration right after it responds confirmation of broadcasting WUS configuration.

On-demand SIB1 deactivation

Proposal 9: NES gNB-DU sends the request for OD-SIB1 deactivation to NES gNB-CU by gNB-DU Configuration Update message.

Proposal 10: To make Cell A gNB-DU stop broadcasting the WUS configuration, Cell A gNB-CU can remove/update the SIB1x of Cell A via the existing gNB-DU Configuration Update message.

Proposal 11: The NES gNB can inform the Cell A gNB which has accepted broadcasting WUS configuration that it is not required to broadcast WUS configuration.

Proposal 12: The Cell A gNB receiving the information of not requiring the WUS configuration broadcast can cancel or stop the WUS configuration broadcasting.

Cell A gNB stops WUS configuration broadcasting

Proposal 13: A new Class-2 Xn procedure is introduced for Cell gNB to indicate NES gNB on stopping WUS configuration broadcast.

Proposal 14: NES gNB receiving the indication on stopping WUS configuration broadcast can determine whether to deactivate on-demand SIB1 for the NES cell.

Proposal 15: NES gNB-CU can indicate NES gNB-DU OD-SIB1 operation is not allowed in the NES cell for it to deactivate OD-SIB1 in the NES cell.

4 Reference

- RP-242354, Revised WID: Enhancements of network energy savings for NR, RAN#105, Ericsson

- Draft Report of 3GPP TSG RAN3 meeting #123-bis, MCC

- Draft Report of 3GPP TSG RAN3 meeting #124, MCC

- Draft Report of 3GPP TSG RAN3 meeting #125, MCC

- Draft Report of 3GPP TSG RAN3 meeting #125bis, MCC