3GPP TSG-RAN WG3 #126                                          R3-247286

Orlando, US, 18th – 22nd Nov, 2024

Source:	CATT

Title:	(TP to TR 38.879) A-IoT reader selection

Agenda Item:	16.3

Document for:	Decision

Introduction

The following agreements and FFS on UE reader selection were discussed in last RAN3 meeting:

In T1, an A-IoT RAN node may serve one or multiple readers.

For reader selection:

in T1, reader selection may need the coordination between AIoT RAN and AIoT CN;

in T2, all options are still open, different alternatives are possible as per call flows.

Discuss the reader selection for Topology1 and for Topology2:

Which node decides the area?

In this contribution, we will further analyze whether A-IoT CN or A-IoT RAN to perform the reader selection for topology1 and for topology2 and provide our observations and proposals accordingly.

Discussion

In last RAN3 meeting, which node should take the responsibility to perform reader selection was discussed for topology 1 and for topology 2. Here we continue discussing this issue.

  - Topology 1

For topology 1, the following agreements related to reader selection were achieved in last meeting:

In T1, an A-IoT RAN node may serve one or multiple readers.

For reader selection:

in T1, reader selection may need the coordination between AIoT RAN and AIoT CN;

When the A-IoT CN expects to trigger inventory or command is triggered to certain area, A-IoT CN could select the proper reader(s) within this area, in case the reader information are available for the A-IoT RAN. In another word, A-IoT CN should take the responsibility for reader selection, in case the reader information is available.

Observation 1: If reader information is available in A-IoT CN, the A-IoT CN could take the responsibility for reader selection.

In case A-IoT CN does not know reader information, the reader list will not be provided to A-IoT RAN. Receiving inventory/command request message without reader list, A-IoT RAN may perform reader selection base on implementation, or it just perform the inventory/command towards the device(s) in all of the readers it holds.

Observation 2: If reader information is not available in A-IoT CN, A-IoT CN could not perform reader selection.

As we have discussed and agreed that one A-IoT RAN may hold one or multiple readers, and it’s assumed that the readers are stationary. It’s not difficult to let A-IoT CN know about the reader information, including identification of the reader, location of the reader, potential coverage of the reader, etc.

As for the coordination between A-IoT RAN and A-IoT CN, either signalling procedure or OAM is feasible to exchange reader information. As we assume the reader(s) in Topology 1 are stationary, OAM based solution is slightly preferred.

Proposal 1: Either signalling procedure or OAM is feasible could be used to exchange reader information between A-IoT RAN and A-IoT CN, slightly prefer the OAM based solution.

As above, it’s possible to include reader list optionally in inventory/command request message. With that, A-IoT RAN could either perform the Inventory/command in the selected reader list (if included), or select the readers by implementation and trigger the Inventory/command towards the A-IoT device(s).

Proposal 2: It is proposed to include selected reader list optionally in inventory/command request message.

  - Topology 2

For topology 2, the following agreements related to reader selection were achieved in last meeting:

For reader selection:

in T2, all options are still open, different alternatives are possible as per call flows.

For the topology 2, it’s assumed that one UE reader only holds one “reader”, how to select the UE reader(s) for AIoT inventory and Command to be further considered.

For NAS based solution and UP based solution, A-IoT CN directly send A-IoT inventory/command request to UE reader via NAS or user plane. Thus, A-IoT CN should select UE reader(s) to perform inventory or command procedures.

Proposal 3: For NAS based solution and UP based solution, A-IoT CN should perform UE reader selection, and send the inventory or command request towards the UE readers directly.

For RRC based solution, which node to perform the reader selection depends on whether A-IoT CN knows about the locations of the UE readers, and their connected gNBs.

For RRC-CONNECTED and RRC-INACTIVE UE reader, A-IoT CN could obtain location of the UE reader based on ULI in the UL NGAP signalling, or base on legacy LCS function. While for RRC-IDLE UE reader, A-IoT CN could first trigger paging procedure to move UE to RRC connected state and then acquires UE reader location via ULI or LCS. With UE reader location, A-IoT CN could performs UE reader selection and send the selected UE reader list to A-IoT-enabled gNB in the XXAP Inventory/Command Request message.

Proposal 4: For RRC based solution, A-IoT CN could perform UE reader selection base on acquired location of the readers, and provide UE reader list in the XXAP Inventory/Command Request message.

But there may be another case: if TA is pre-allocated to certain area, TAI list or cell list could be used as area scope.

For example, each warehouse corresponds to a TA and the area scope of inventory/command is certain warehouses. In this case, TA list is enough to indicate the area scope. A-IoT CN could know UE TA location via legacy TAU procedure. So, A-IoT CN may send inventory/command request to A-IoT-enabled gNB with TAI list to indicate area scope and A-IoT-enabled gNB trigger inventory/command procedure within TAI list scope as legacy.

Proposal 5: For RRC based solution, A-IoT CN may provide TAI list or cell list to A-IoT-enabled gNB as area scope, then gNB select proper UE readers within the area scope to perform the requested inventory or command procedures towards the AIoT device.

Conclusions

In this contribution, we mainly discussed the issue on UE reader selection. Based on the discussion above, the following proposals are provided:

Topology 1:

Observation 1: If reader information is available in A-IoT CN, the A-IoT CN could take the responsibility for reader selection.

Observation 2: If reader information is not available in A-IoT CN, A-IoT CN could not perform reader selection.

Proposal 1: Either signalling procedure or OAM is feasible could be used to exchange reader information between A-IoT RAN and A-IoT CN, slightly prefer the OAM based solution.

Proposal 2: It is proposed to include selected reader list optionally in inventory/command request message.

Topology 2:

Proposal 3: For NAS based solution and UP based solution, A-IoT CN should perform UE reader selection, and send the inventory or command request towards the UE readers directly.

Proposal 4: For RRC based solution, A-IoT CN could perform UE reader selection base on acquired location of the readers, and provide UE reader list in the XXAP Inventory/Command Request message.

Proposal 5: For RRC based solution, A-IoT CN may provide TAI list or cell list to A-IoT-enabled gNB as area scope, then gNB select proper UE readers within the area scope to perform the requested inventory or command procedures towards the AIoT device.

Reference

[1] RAN3#125bis Chair’s Notes

TP for TR 38.769

<<<<<<<<<<<<<<<<<< Begin of the changes >>>>>>>>>>>>>>>>>>>>>>>

- 6.4.1	Support of Topology 1

Figure 6.4.1-1 depicts a logical system architecture for topology 1, where the Common reader function and A-IoT RAN node function are deployed within an A-IoT RAN.

Figure 6.4.1-1 Logical system architecture for topology 1

For Toplogy 1, architecture and protocol aspects of split RAN architecture are not studied.

In Topology 1, the XX interface could be based on NG or a new interface carried over NG or a new interface.

Figure 6.4.1-2 shows the Protocol stack for Topology 1, assuming a SCTP-based transport:

Figure 6.4.1-2. Protocol Stack for Topology 1

NOTE:	The protocol stack in Figure 6.4.1-2 does not illustrate how A-IoT upper layer information, if any, is transported over XXAP. Details are subject to SA2 agreements.

For topology 1, the XXAP is terminated at an A-IoT RAN node.

The signalling transport for XXAP at the A-IoT RAN node is SCTP/IP. Other options of signalling transport for XXAP at the A-IoT RAN node (e.g., HTTP/2/TLS/TCP) were discussed, but will not be pursued.

Editor’s Note 3: The A-IoT CN may include AMF and A-IoT related functions which is up to SA2 decision.

In Topology 1, an A-IoT RAN node may serve one or more readers.

The A-IoT RAN node should enable the coordination of the usage of the A-IoT radio resources among readers.

Reader selection may need coordination between A-IoT RAN node and A-IoT CN.

If A-IoT CN select the readers and provide a reader list in the inventory/command request, A-IoT RAN should perform the Inventory/Command procedure by the selected readers. Or else, A-IoT RAN could select the readers by implementation, e.g. select all the readers it holds, to perform the Inventory/Command procedure.

- 6.4.2	Support of Topology 2

Figure 6.4.2-1 depicts a logical system architecture for topology 2, where the Common reader function is located at an A-IoT-enabled UE, and the A-IoT RAN node function is located at an A-IoT-enabled gNB.

The following definitions apply:

A-IoT-enabled gNB:	A gNB supporting A-IoT RAN node function in topology 2, which is able to communicate with the A-IoT-enabled UE via NR Uu interface.

A-IoT-enabled UE:	A UE supporting Common reader function, which is able to communicate with the A-IoTdevice via the A-IoT radio interface.

Figure 6.4.2-1 Logical system architecture for topology 2

Editor’s Note 1:	Figure 6.4.2-1 doesn’t illustrate the protocol between A-IoT enabled UE and A-IoT CN, if needed, the figure needs to be revised in case such is defined by SA2.

Editor’s Note 2:	In Topology 2, the XX interface could be based on NG or a new interface carried over NG or a new interface. XX signaling could be transported via XX-C or XX-U, which is FFS.

Editor’s Note 3:	The A-IoT CN could include AMF and A-IoT related functions, which is up to SA2 decision.

Editor’s Note 4:	The A-IoT enabled gNB performs radio resource management for A-IoT related radio resources, details are pending on RAN1 and RAN2 mechanisms.

Editor’s Note 5:	In Topology 2, it is FFS on reader selection, may need coordination between A-IoT RAN node and A-IoT CN.

In Topology 2, the RAN architecture should enable the coordination of the usage of the A-IoT radio resources among readers.

An A-IoT-enabled gNB could support both topology 1 and topology 2, this is an implementation matter.

- 6.4.2.1	Solutions for Topology 2

6.4.2.1.0	General To support Topology 2, the following solutions are to be studied for conveying A-IoT upper layer information:

-	RRC based solution. With this solution, A-IoT CN applies A-IoT upper layer information explicitly over XXAP signaling. A-IoT upper layer information is then relayed explicitly to/from the A-IoT-enabled UE via NR Uu RRC.

-	NAS based solution. With this solution, there is no explicit termination of A-IoT upper layer information at A-IoT-enabled gNB. A-IoT upper layer information is transmitted over A-IoT enabled UE's NAS.

-	UP based solution. With this solution, there is no explicit termination of A-IoT upper layer information at A-IoT-enabled gNB. A-IoT upper layer information is transmitted as A-IoT-enabled UE's user plane data.

NOTE:	The protocol stack for each solution option does not illustrate A-IoT CN internal architecture and how A-IoT upper layer information is transported, if any. Details are subject to SA2 agreements.

For RRC based solution, the way of reader selection is similar to the Topology 1.

For NAS and UP based solution, A-IoT CN could perform reader selection, and use the UE specific signalling (NAS) or user data (UP) to communicate with the selected readers directly.

- 6.4.2.1.1	Solution1: RRC based solution

Upon receiving XXAP: A-IoT related message from A-IoT CN, the A-IoT-enabled gNB transmits the related information towards the A-IoT-enabled UE via NR Uu RRC, and vice versa.

Figure 6.4.2.1.1-1: RRC based solution of Topology 2

6.5	Impacts on CN-RAN interface

Editor’s Note: Corresponds to the first RAN3 objective in the SID, to identify necessary impacts on signaling and procedures for CN-RAN interface.

- 6.5.1	Information exchanged between A-IoT CN and A-IoT RAN

- 6.5.1.1	Inventory

Inventory can be sent by the A-IoT CN for a single device, or a group of devices, or all devices.

The Inventory Request from the A-IoT CN to the A-IoT RAN, may include the following:

(1)	A-IoT Device Identification (to find a single device, a group of devices, or all devices)

Note 1:	The definition of this identification is out of RAN3 scope.

Editor’s Note 1: It is FFS whether A-IoT RAN needs to interpret/store/process it.

(2) Read list

(3) TAI list for topology 2

Multiple individual A-IoT Device IDs (one ID per device) can be provided to the A-IoT CN via a single Inventory Report.

Editor’s Note 2: It is up to SA2 whether device ID is sent transparent or not.

- 6.5.1.2	Command

Command can be sent by the A-IoT CN for a single device.

(1) Read list

(2) TAI list for topology 2

Editor’s Note 1: it is FFS for command on a group of devices, or all devices.

Editor’s Note 2: it is FFS whether A-IoT RAN can remain agnostic of the type of request from the A-IoT CN (need to differentiate command and inventory).

- 6.5.1.3	A-IoT radio resource allocation in case of NAS/UP based solutions

In NAS/UP based solutions, A-IoT radio resources can be requested in advance to the NAS/UP based communication with the A-IoT device or can be requested along with the NAS/UP based communication. There are different ways to trigger A-IoT radio resource allocation, e.g., upon CN request or or upon UE request, etc. Some aspects of A-IoT radio allocation may be preconfigured by OAM.

----------------End of the Changes----------------