3GPP TSG-RAN WG3 Meeting #126                               R3-247294

Orlando, US, 18th – 22nd, Nov, 2024

Source:	CATT

Title:	Discussion on AI/ML assisted positioning

Agenda Item:	20.2

Document for:	Discussion and Decision

Introduction

In the WID for Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface [1], an objective to support AI/ML assisted positioning for positioning accuracy enhancements, the potential RAN3 impact sub-bullets are highlighted below:

In the last RAN3 meeting, we initially discussed the potential RAN3 impact to support AI/ML assisted Positioning, i.e. the Case 3a. According to the discussion [1], some agreements and open issues as below:

Case 3a agreements: - LMF provides ground truth label and related data (i.e., part B information as called by RAN1) to NG-RAN. FFS on the content of the ground truth label and related data pending on RAN1 progress. FFS on the signalling design. - How the LMF obtains the ground truth label and related data information is outside of RAN3 scope. - NG-RAN obtains measurements from TRP (i.e., part A information as called by RAN1). The details of the measurements are still pending and subject to further discussion in RAN1. - WA: RAN3 will proceed with Option A: NG-RAN node performs monitoring metric calculation for its own model.

In this contribution, we will discuss the potential RAN3 impact to support AI/ML assisted positioning, taking the RAN1 progress/agreements into account. Base on the discussion, we will provide corresponding observations and proposals.

Discussion

Training Data collection

For the Data collection, corresponding RAN1 agreements or working assumptions could be found below:

RAN1#116 Agreement For AI/ML assisted positioning Case 3a, at least LOS/NLOS indicator and/or timing information are supported for reporting. - If LOS/NLOS indicator is reported, the indicator can be reported as soft indicator or hard indicator as defined in 38.214. - If timing information is reported, the timing information at least can be reported via UL RTOA or gNB Rx-Tx time difference as defined in 38.215. - Note: details of the report are pending further discussion.

RAN1#116 bis Agreement For training data generation of AI/ML based positioning Case 3a and 3b, the measurement and its related data (e.g., timestamp) are generated by TRP/gNB. Agreement For training data collection of AI/ML based positioning, the collected data sample can include the following components: Part A: - channel measurement - quality indicator of channel measurement - time stamp of channel measurement Part B: - ground truth label (or its approximation) - quality indicator of label - time stamp of label Note: “Part A” and “Part B” terminologies are only for RAN1 discussion purpose, and may not be used in specification. Note: contents in Part A and Part B may or may not be generated by different entities. Note: Part A and/or Part B, and their contents may or may not apply for each case FFS: detailed definition of channel measurement Agreement For training data generation of AI/ML based positioning Case 3a, the label and its related data (e.g., time stamp) can be generated by at least: - LMF Note: transfer of label and its related data is out of RAN1 scope. Note: whether other network entities can generate label for Case 3a is out of RAN1 scope.

According to RAN1 agreements, for case 3a/3b data collection, Part A and Part B may be generated by different entities. We understand the Part A is generated by gNB, while the Part B maybe generated by the LMF or UE. RAN3 can further discuss the interaction between different entities to obtain a paired Part A and Part B.

In Case 3a, channel measurement is done at gNB, while RAN1 has agreed that the label and its related data (e.g., time stamp) can be generated by at least LMF. In the last RAN3 meeting, it’s discussed and agreed that:

- LMF provides ground truth label and related data (i.e., part B information as called by RAN1) to NG-RAN.

- How the LMF obtains the ground truth label and related data information is outside of RAN3 scope.

FFS on the content of the ground truth label and related data pending on RAN1 progress. FFS on the signalling design.

RAN3 should discuss the above options on how to provide and link the Part A and Part B for data collection. If the Part A of the data collection is reported from gNB to LMF, LMF could associate them with the Part B, and sent it back to the gNB. Or else, LMF just need to provide the Part B to the gNB, and the gNB should associate the Part A and Part B, how they are linked to be further discussed.

- Option 1: For the collected data sample, LMF associate Part A and Part B and send them to the gNB.

- Option 2: LMF provides Part B to the gNB, the gNB associates it with the corresponding Part A.

Proposal 1: RAN3 should discuss how to link the Part A and Part B of the collected data sample.

- Option 1: LMF associated the Part A and Part B and send them to the gNB.

- Option 2: LMF only provides the Part B, the gNB associates it with the Part A. How to associate them should be further considered.

For Case 3a, since the AI/ML model is deployed at gNB side and the channel measurement entity is also gNB, it’s reasonable to allow gNB to request for training data collection Info towards the LMF, e.g. the location of the UE labels, the corresponding measurement results and time info of the measurement, e.g. UL-RTOA, Multi-RTT.

Proposal 2: for Case 3a, gNB should be allowed to request for training data info for data collection from LMF, which include at least the label ID and requested type of information, e.g. location of the label, measurement result of the label.

Moreover, since the AI/ML model is deployed at gNB side, gNB may have its preferred UL SRS configuration for training data collection. However, currently, only LMF has the capability of requesting preferred SRS transmission characteristics, e.g., the density and periodicity of SRS resources. During the request process, LMF does not have prior knowledge about gNB-side models, i.e., LMF does not know the SRS configuration most suitable for gNB-side models, no matter from consistency or measurement perspective. To tackle this issue, gNB can provide its preferred SRS configuration to LMF in advance. In this way, LMF can better negotiate with different TRPs to determine the SRS configuration most suitable for gNB side models. Specifically, the report can be performed in explicit (request specific SRS transmission characteristics) or implicit manner (via associated ID).

Proposal 3: For data collection in Case 3a, it’s beneficial for gNB to provide the preferred UL SRS characteristics/configuration for training data collection to LMF.

Inference

For Model Input/Report for inference, the corresponding agreements in RAN1 could be found below:

RAN1#116 Agreement In Rel-19 AI/ML based positioning, regarding the time domain channel measurements, RAN1 investigate the following alternatives: - Alternative (a). Sample-based measurements, where the timing information is an integer multiple of sampling periods. - Alternative (b). Path-based measurements, where the timing information is according to the detected path timing and may not be an integer multiple of sampling periods. The issues to be studied include, but not limited to, the following: - Tradeoff of positioning accuracy and signaling overhead - Impact and necessary details of gNB/UE implementation to obtain the channel measurement values. - Whether the same Alternative(s) applies to all cases or not - Applicability and necessity of specifying the Alternative(s) to different cases - Note: different sub-cases may have different issues. Note: In addition to timing information, the components for the channel measurement for model input may also include power and potentially phase. To provide the type of the channel measurement in their investigation. Agreement For AI/ML based positioning for all use cases, RAN1 investigate the necessity and feasibility of using phase information (in addition to timing information and power information) for determining model input. The issues to study include: - Tradeoff of positioning accuracy and signaling overhead - The impact of transmitter and receiver implementation - Specification impact - Other aspects are not precluded Note: the phase information may be used in different ways, e.g., one phase value for the first path or first sample only; triplet of {timing information, power information, phase information} for CIR, etc.

RAN1#117 Agreement Sample-based measurement is defined as: - The measurement is composed of Nt' samples of the estimated channel response in time domain. The timing information for the Nt' samples are reported with a timing granularity T, where T=2kxTc. k represents the timing reporting granularity factor. Tc is the basic time unit for NR. - The corresponding measurement (e.g., power if reported) corresponds to the measurement for the reported Nt' samples. - Nt' and k can be signalled - FFS: the value range of Nt'; the value range of integer k for the timing granularity T. - The timing information is defined relative to a reference time Further discussion is expected on the determination of Nt' and k (including signaling) , and a rule to be introduced for selecting Nt' samples. Note: It doesn’t imply the definition of Sample-based measurement will be captured into the spec. Agreement Path-based measurement refers to the measurement in the existing specifications (up to Rel-18) including measurement reporting, with potential enhancements on the number of reported paths (if needed).

RAN1#118 Agreement For the definition of sample-based measurement, select Nt’ samples out of a list of Nt consecutive samples - The Nt samples have timing granularity T. - FFS: the starting time of the list of Nt samples - FFS: the value range of Nt For the sample-based measurement (if accepted in Rel-19), - For measurement by TRP/gNB, the Nt’ selected samples are expected to be those with the highest power. - Note: Choice of the maximum value of Nt, Nt’ should take into account the need to preserve proprietary implementation. Conclusion From RAN1 perspective, for Case 3a measurements, - The existing procedures can be reused in terms of SRS configuration. - Note: parameter values for SRS configuration can be further discussed - These measurements can be used for multiple aspects related to case 3a, e.g. training data collection, monitoring, or inference procedures. - Note: Purpose, such as the training data collection, monitoring, or inference procedures mentioned above, will not necessarily be specified in RAN 1 specifications

RAN1#118bis Agreement From RAN1 perspective, when timing information is reported for Rel-19 AI/ML positioning Case 3a, at least the following are mandatorily or optionally supported in a measurement report from gNB to LMF: - (Mandatory) timing information; - (Optional) Quality of the timing information; - Existing IE “Timing Measurement Quality” can be reused. - (Mandatory) Time stamp. FFS: LOS/NLOS indicator. Note: The final decision of “mandatory” or “optional” presence of each field is up to RAN3.

In RAN1#116 meeting, it’s agreed that in Rel-19 AI/ML based positioning, regarding the time domain channel measurements, RAN1 investigate the following alternatives:

      - Alternative (a).  Sample-based measurements, where the timing information is an integer multiple of sampling periods.

      - Alternative (b).  Path-based measurements, where the timing information is according to the detected path timing and may not be an integer multiple of sampling periods.

Which alternative will be taken is pending to RAN1, RAN3 can wait for the conclusion of RAN1 for our interface design, e.g. including phase information in the channel measurement report.

Proposal 4: In Case 3a, Sample-based or Path-based measurements should be taken for the time domain channel measurements are pending to RAN1, RAN3 could wait a little bit.

According to the RAN1 agreements/conclusion, for Case 3a, the measurements can be used for multiple aspects related to case 3a, e.g. training data collection, monitoring, or inference procedures.

In the last RAN1 meeting, it’s agreed that for Case 3a, at least the following are mandatorily or optionally supported in a measurement report from gNB to LMF:

- (Mandatory) timing information;

- (Optional) Quality of the timing information;

  - Existing IE “Timing Measurement Quality” can be reused.

- (Mandatory) Time stamp.

FFS: LOS/NLOS indicator.

In the existing NRPPa, above information could be provided to LMF in the TRP measurement Result, as shown below:

9.2.37	TRP Measurement Result

This information element contains the measurement result.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Measured Result Item |  | 1 .. <maxnoPosMeas> |  |  | - |  |
| >CHOICE Measured Results Value | M |  |  |  | - |  |
| >>UL Angle of Arrival |  |  | 9.2.38 |  |  |  |
| >>UL SRS-RSRP |  |  | INTEGER (0..126) |  |  |  |
| >>UL RTOA Measurement |  |  | 9.2.39 |  |  |  |
| >>gNB Rx-Tx Time Difference |  |  | 9.2.40 |  |  |  |
| >>Z-AoA |  |  | 9.2.67 |  | YES | reject |
| >>Multiple UL-AoA |  |  | 9.2.71 |  | YES | reject |
| >>UL SRS-RSRPP |  |  | 9.2.72 |  | YES | reject |
| >>UL RSCP |  |  | 9.2.92 |  | YES | reject |
| >Time Stamp | M |  | 9.2.42 |  | - |  |
| >Measurement Quality | O |  | 9.2.43 |  | - |  |
| >Measurement Beam Information | O |  | 9.2.57 |  | - |  |
| >SRS Resource type | O |  | 9.2.73 |  | YES | ignore |
| >ARP ID | O |  | 9.2.75 |  | YES | ignore |
| >LoS/NLoS Information | O |  | 9.2.77 |  | YES | ignore |
| … |  |  |  |  |  |  |

We assume NRPPa Measurement procedure could be reused to support the purpose of training data collection, inference and monitoring for Case 3a. Any other information need to be reported from gNB to LMF is pending to RAN1.

Proposal 5: In Case 3a, NRPPa measurement procedure could be reused for training data collection, inference and monitoring for Case 3a. Any other information need to be reported from gNB to LMF is pending to RAN1.

Performance Monitoring

For performance monitoring, corresponding RAN1 agreements are shown below:

RAN1#118 Agreement For AI/ML positioning Case 3a, for performance monitoring metric calculation in label-based monitoring, from RAN1 perspective, Option A and Option B are feasible, - Option A. NG-RAN node performs monitoring metric calculation for its own model. - Option B. LMF performs monitoring metric calculation for the model located at the NG-RAN node. Note: Final selection of Option A and Option B is out of RAN1 scope. Potential support of Option A and/or Option B is pending RAN3 confirmation. Note: Exact method to perform monitoring metric calculation is up to implementation. Note: For Option A, RAN1 assumes that user data privacy needs to be preserved.

In the last RAN3 meeting, we discussed and made the WA:

WA: RAN3 will proceed with Option A: NG-RAN node performs monitoring metric calculation for its own model.

We understand it’s straightforward to turn the WA to the agreement to go for Option A, NG-RAN node performs monitoring metric calculation for its own model.

Proposal 6: In Case 3a, turn the WA to the agreement:

- For Case 3a, NG-RAN node performs monitoring metric calculation for its own model.

In RAN1 agreements, it’s also noted that:

- Note: Exact method to perform monitoring metric calculation is up to implementation.

- Note: For Option A, RAN1 assumes that user data privacy needs to be preserved.

How to perform performance monitoring, if any other information need to be provided to NG-RAN to be further discussed, taking RAN1 progress into account.

Proposal 7: How to perform performance monitoring in NG-RAN to be further discussed, taking RAN1 progress into account.

Conclusion

In this contribution, discussed the RAN3 impact to support AI/ML assisted positioning for AI assisted Positioning Accuracy enhancements. Based on the discussion above, the following proposals are provided:

Proposal 1: RAN3 should discuss how to link the Part A and Part B of the collected data sample.

- Option 1: LMF associated the Part A and Part B and send them to the gNB.

- Option 2: LMF only provides the Part B, the gNB associates it with the Part A. How to associate them should be further considered.

Proposal 2: for Case 3a, gNB should be allowed to request for training data info for data collection from LMF, which include at least the label ID and requested type of information, e.g. location of the label, measurement result of the label.

Proposal 3: For data collection in Case 3a, it’s beneficial for gNB to provide the preferred UL SRS characteristics/configuration for training data collection to LMF.

Proposal 4: In Case 3a, Sample-based or Path-based measurements should be taken for the time domain channel measurements are pending to RAN1, RAN3 could wait a little bit.

Proposal 5: In Case 3a, NRPPa measurement procedure could be reused for training data collection, inference and monitoring for Case 3a. Any other information need to be reported from gNB to LMF is pending to RAN1.

Proposal 6: In Case 3a, turn the WA to the agreement:

- For Case 3a, NG-RAN node performs monitoring metric calculation for its own model.

Proposal 7: How to perform performance monitoring in NG-RAN to be further discussed, taking RAN1 progress into account.

Reference

- RP-240774 Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- RAN1#116/#116bis/#117/#118/118bis Chair’s Notes