3GPP TSG-RAN WG3 Meeting #126	R3-247313

Orlando, US, 18th – 22nd November 2024

Agenda item:	16.3

Source:	Xiaomi

Title:	AIoT interface impacts between RAN and CN

Document for:	Discussion

# 1	Introduction

In this contribution, we’d like to discuss the interface impacts on the following aspects:

- reader selection

- inventory/command information

# 2	Reader Selection

## Reader selection in T1

For reader selection:

in T1, reader selection may need the coordination between AIoT RAN and AIoT CN;

For T1, RAN3 agreed that there may need coordination between AIoT RAN and AIoT CN. In our view, the reader selection procedures can be as follows:

Figure 1 reader selection for T1

The AIoTF requests at least one AIoT RAN node for the reader information in step 1, as one based station may have multiple readers deployed in different places and have different AIoT capabilities and resources, the AIoT RAN node may response a list of readers and corresponding reader capabilities in step 2. In step 3, if there is a service request from AF, AIoTF select the RAN readers based on the service request and reader information received in step 2.

Observation 1, it’s useful to introduce a reader information procedure for effective reader selection.

Proposal 1, introduce a new class 1 message (e.g., Reader Information Exchange) between AIoT RAN and AIoTF for AIoT reader selection.

## Reader information

The following information can be considered as reader information provided to the AIoTF for reader selection.

| Reader information | Analysis |
| --- | --- |
| Reader ID | The reader ID can be RAN node ID and TRP ID |
| Location information of the reader | The location information of a reader can be utilized for reader selection in AIoTF, taking into account the geographical data associated with a specific AIoT service request. |
| AIoT resource information of the reader | The frequency, timing, and power information of the reader can provide valuable support for AIoT operations when selecting readers in AIoTF based on QoS and device types for specific AIoT service |

Proposal 2, The following information can be provided as reader selection criteria from AIoT RAN to AIoTF, taking into account the AIoT service request:

- Reader ID

- Location information of the reader

- AIoT resource information of the reader

## Reader selection in T2

There may be different cases and options for UE reader selection.

Option 1, AF indicate the UE ID directly by application layer implementation.

Option 2, AF provides the area information for AIoT service request, AIoTF performs the selection by obtaining the candidate UE(s) from AMF, shown in figure 2.

Figure 2 UE reader selection in AIoTF in T2

Option 3, AF provides the area info for AIoT service request, AIoTF send the request directly to the AIoT RAN in the area, and the RAN node perform the UE reader selection, as shown in figure 3.

Figure 3 UE reader selection in AIoT RAN

We think above options can be used for difference scenarios. In some scenario, the AF knows the UE reader, it can indicate the AIoTF directly. In some scenario, AIoTF only receives the inventory/command area information from AF, then AIoTF can either ask AMF for UE information or ask AIoT RAN node in the area to perform the UE reader selection.

Proposal 3, RAN3 agree to capture the following options for UE reader selection

Option 1, the UE reader is indicated directly by AF.

Option 2, AIoTF performs UE reader selection by enquiring AMF with the area information.

Option 3, AIoT RAN performs UE reader selection by receiving the AIoT operation request.

# 3	Inventory/command information

## Inventory request

In RAN2#127 meeting, RAN2 achieved the following agreements for A-IOT functions.

Agreements

-	Send an LS to RAN1 regarding TBS size. Ask SA2 on the max and typical upper layer packet size.

-	RAN2 will study the need and use of energy status report to be delivered from the device to the reader in case the device does not have energy for the follow up messages.   FFS details on signaling

-	RAN2 will study the need and use of a simple message “size” reporting to the reader

-	At least the following information are considered useful to be visible to the reader from CN

-	The service type of A-IoT (e.g. inventory, command) . FFS if more information on command type (e.g. read/write/disable) is useful

-	targeted for one or more than one devices;

-	approximate number of target devices (if available).

FFS on mandatory/optional

-	RAN2 assumes that commands (e.g., read/write/disable) and/or inventory are carried over the AIOT interface as upper layer data.

The following information is considered useful to be visible to the AIoT RAN from CN from RAN2 perspective.

•	The service type of A-IoT (e.g., inventory, command)

•	Targeted for one or more than one device.

•	Approximate number of target devices (if available)

And RAN3 agreed that the AIoT Device Identification and Scope of inventory request can be included in inventory request.

The following are some analyses for information in the inventory request message:

| Information in inventory request message | Analysis |
| --- | --- |
| AIoT Device Identification (to find a single device, a group of devices, or all devices) | This identification can be MASK info, which will be further transfer to device, so that device can decide whether to response the inventory request. |
| Scope of inventory request (e.g., a certain area in which the inventory is to be triggered) | We think this area information can be used for gNB to decide whether a reader needs to perform this inventory request. This area information can be list of TRPs, a list of cells, etc. |
| service type of A-IoT (e.g. inventory, command) | Agreed by RAN2, could be valuable for a reader in initiating CFRA or CBRA to allocate dedicated/shared resources for subsequent D2R transmission. In addition, this information can also be useful for the AIoT RAN to decide whether to keep the device context for subsequent AIoT operations. |
| Targeted for one or more than one device | Agreed by RAN2, can be utilized by readers to determine the type of RA and schedule the resource. |
| Estimated Number of devices (Approximate number of target devices) | Agreed by RAN2. This may help reader to allocate radio resources, to perform re-triggering or to set Q value, but the above can also be done by reader’s implementation. E.g. the reader can know from the identification info whether a single or multiple device are expected, the reader can set Q and re-trigger inventory by knowing the Collision over AIoT radio. This info may help to reduce the processing complexity of the reader. |
| Session/Task ID | gNB may receive multiple inventory requests from AIoTF(s), similar to positioning, there’s transaction ID to identify a specific procedure. |
| Periodicity | It is possible that the inventory process is periodic |
| The type of the ID included in the A-IOT paging message (e.g., group ID/no ID or independent device ID) | RAN2 agrees to study the following cases for AIoT paging message: • a message containing an ID of a single A-IoT device. • a message containing a group ID that maps to multiple A-IoT devices. • a message that does not contain an ID, i.e., addressed for all devices that can receive the AIoT message. • a message containing multiple IDs of A-IoT devices. the information of one or more than one device is used by the reader to determine the type of random access, e.g., CFRA or CBRA. However, this information alone is not sufficient if more than one device is targeted. This is because these three cases are all targeted at multiple devices, however if the group ID is contained or no ID is contained, reader can only trigger CBRA while if multiple independent IDs are contained, reader can either trigger CFRA or CBRA. |
| QoS requirement (e.g., E2E latency) | In TR 38.848, it indicates the latency requirement for A-IOT, which is E2E latency between the reader and the device. the reader should also be able to derive the latency request from the service request from CN so that the reader can execute the related AS procedure according to the latency requirement |

Proposal 4, the AIoT Device Identification in inventory request message can be MASK info that will finally be sent to device.

Proposal 5, Scope of inventory request in the inventory request message can be list of TRPs.

Proposal 6, the following information can be introduced in inventory request from AIoTF to AIoT RAN

- Service type of A-IoT (e.g. inventory, command)

- Targeted for one or more than one device

- Estimated Number of devices (Approximate number of target devices)

- Session/Task ID

- Periodicity

- Type of the ID included in the A-IOT paging message (e.g., group ID/no ID or independent device ID)

- QoS requirement (e.g., E2E latency)

## Inventory response/report

As there may be more than one AIoT operations for a AIoT device, e.g., inventory and command operation, in this case, it’s beneficial to introduce a RAN specific Device ID for the device when the AIoT RAN receives the response from a reader. the RAN specific device ID (similar to NGAP UE ID) is used to identify a specific device within its own when receiving responses from reader.

Proposal 7, the following information can be introduced in inventory response/report from AIoT RAN to AIoTF

- Session/Task ID

- RAN specific device ID

- AIoT device information (can be a container)

## Command request

For command operation, the following information is useful for command request:

| Information in command request message | Analysis |
| --- | --- |
| Session/Task ID | gNB may receive multiple command requests from AIoTF(s), similar to positioning, there’s transaction ID to identify a specific procedure. |
| RAN specific device ID | This ID can be used for the reader to know which device is targeted to, which means the command procedure can be device-associated. |
| command type (e.g., read/write/disable) | this information is useful since the response for different command is of different size, with this information, the reader is able to schedule the following D2R resource accordingly e.g., relatively larger resource for response of read command than write command. In addition, this is aligned with RFID where the command type is visible to reader. |

Proposal 8, the following information can be introduced in command request from AIoTF to AIoT RAN:

- Session/Task ID

- RAN specific device ID

- Command type (e.g., read/write/disable)

## Command response

In command response, the session ID, RAN specific device ID, and an optional command result information from AIoT device can be included.

Proposal 9, the following information can be introduced in command response from AIoT RAN to AIoTF

- Session/Task ID

- RAN specific device ID

- command result (optional)

# 4	Conclusion

In this contribution, we discussed the RAN-CN interface impacts to support AIoT in topology 1 and topology 2, and have the following observations and proposals.

## Reader selection in T1

Observation 1, it’s useful to introduce a reader information procedure for effective reader selection.

Proposal 1, introduce a new class 1 message (e.g., Reader Information Exchange) between AIoT RAN and AIoTF for AIoT reader selection.

Proposal 2, The following information can be provided as reader selection criteria from AIoT RAN to AIoTF, taking into account the AIoT service request:

- Reader ID

- Location information of the reader

- AIoT resource information of the reader

## Reader selection in T2

Proposal 3, RAN3 agree to capture the following options for UE reader selection

Option 1, the UE reader is indicated directly by AF.

Option 2, AIoTF performs UE reader selection by enquiring AMF with the area information.

Option 3, AIoT RAN performs UE reader selection by receiving the AIoT operation request.

## Inventory request and response/report

Proposal 4, the AIoT Device Identification in inventory request message can be MASK info that will finally be sent to device.

Proposal 5, Scope of inventory request in the inventory request message can be list of TRPs.

Proposal 6, the following information can be introduced in inventory request from AIoTF to AIoT RAN

- Service type of A-IoT (e.g., inventory, command)

- Targeted for one or more than one device

- Estimated Number of devices (Approximate number of target devices)

- Session/Task ID

- Periodicity

- Type of the ID included in the A-IOT paging message (e.g., group ID/no ID or independent device ID)

- QoS requirement (e.g., E2E latency)

Proposal 7, the following information can be introduced in inventory response/report from AIoT RAN to AIoTF

- Session/Task ID

- RAN specific device ID

- AIoT device information (can be a container)

## Command request/response

Proposal 8, the following information can be introduced in command request from AIoTF to AIoT RAN:

- Session/Task ID

- RAN specific device ID

- Command type (e.g., read/write/disable)

Proposal 9, the following information can be introduced in command response from AIoT RAN to AIoTF

- Session/Task ID

- RAN specific device ID

- command result (optional)