3GPP TSG-RAN3 Meeting #126	R3-247331

Orlando, US, Nov 18-22, 2024

Agenda Item:	15.2

Source: 	Qualcomm Incorporated

Title: 	Discussion on Support for IoT NTN for Regenerative Payload

Document for:	Discussion/Decision

# Background

The IoT NTN WID objectives are RAN3 enhancements required to support Regenerative payload for IoT NTN with full eNB on board and to support Store and forward mechanisms in IoT NTN. However in the last meeting there was no clear consensus on the scope on the WI.

Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore: - Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2, RAN4] -At least specify necessary enhancements e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3] Note: Strive to minimise UE impact. Note: Coordination with SA2 (Rel-19 SA2 led Sat-Arch ph3 SI) is needed on the detail requirements (e.g. traffic type, or QoS parameters for S&F), network architecture (e.g. whether consider (partial) core network on satellite) etc.; further coordination with CT1 might be required. RAN3#125 RAN3 decides to work on Split MME architecture and Full CN on board architecture. Whether eNB only on board architecture is feasible or not can be discussed later.

In this paper we discuss our views scope of the WI and the changes needed in RAN3 signalling.

# Discussion

## SA2 LS on IoT NTN WI Scope Clarification

In the last meeting there was no clear understanding among the RAN3 companies on the store and forward scope and whether RAN3 needs to work on supporting regenerative payload for IoT NTN without S&F mode.

SA2 has sent an LS R3-247016 clarifying that their study conclusion for FS_5GSAT_Ph3 for “KI#1 and KI#2 is for both NR NTN and IoT NTN and not just NR NTN. This means that when IoT NTN eNB is not in S&F mode, then the eNB acts as a regenerative payload with MME on ground and RAN3 needs to support signalling procedures to achieve the same.

However, in the RAN WID RP-242397, only S&F is mentioned as RAN3 scope. Hence we propose that RAN3 agrees to work on Regenerative payload for IoT NTN with and without S&F mode.

SA2 LS R3-247016 clarifies that conclusion for FS_5GSAT_Ph3 for “KI#1 and KI#2 is applicable for both NR NTN and IoT NTN. Whereas in RAN WID RP-242397 only S&F is mentioned as RAN3 scope.

- RAN3 agrees that Regenerative Payload for IoT NTN is applicable for both with and without Store and Forward mode and agree to the SA2 LS response in R3-24xxxx

## Store and Forward support

The architecture options agreed in SA2 for regenerative payload in S2-2407186 has whole CN on board in the satellite and proxies of CN deployed in the ground. The proxies deployed in the ground are out of 3GPP scope. Another option is split MME with MME onboard and MME on ground. The MME on ground and the interface between the MMEs is out of 3GPP scope as agreed in S2-2407191.

S2-2407186 The whole CN including eNB, MME, SGW, PGW, HSS, E-SMLC, SMSC etc are on board each satellite. Proxies are deployed on the satellite and the ground for application traffic, including support of MT traffic, MO traffic, SMS, etc. - The implementation of the proxies and the interface between them is out of 3GPP scope. S2-2407191 1) In the Split MME architecture, HSS is on the ground. 2) MME functionality is split into two parts: MME-onboard - the MME part which is onboard the satellite and MME-ground – the MME part which is on the ground network with an interface out of scope of 3GPP.

In both the architecture options agreed in SA2, the whole eNB is on board and the S1 interface between eNB and MME is not impacted.. In the last meeting companies proposed a third architecture with eNB only on board which is not in the SA2 concluded architecture options for Store and Forward mode. Hence RAN3 should only analyse full CN on board and split MME architecture options for RAN3 impact for S&F.

The architecture options in SA2 for S&F for Regenerative payload has whole CN on board as one option and split MME on board in another option. eNB only on board is not proposed in SA2 concluded architecture option for S&F.

In both full CN on board and split MME architecture option, the whole eNB is on board and the S1 interface is also on board the satellite. In architecture option where the whole CN is on board, we see no RAN3 impact.

RAN3 to analyze only full CN on board and split MME architecture options agreed in SA2 for RAN3 impact and not eNB only on board architecture option.

## Regenerative Payload

### S1 Connection Management

The agreement in NR NTN Regenerative Payload WI for NG Connection management can be replicated for S1 as well. The NG Removal procedure agreed in NR NTN can be mirrored for IoT NTN to support SI connection management for Regenerative payload

Agreement in NR NTN Regenerative payload to introduce NG Removal procedure for NG connection management can be mirrored for S1 as well to support S1 connection management for IoT NTN for Regenerative Payload.

### Feeder Link Switch over

Feeder link switch over scenarios can be discussed in NR NTN. The agreements achieved in NR NTN can be replicated in IoT NTN as well. In S1, there is no support for multiple TNLs. In the last RAN3 meeting it was agreed for NR NTN that “The current mechanisms including, e.g. multiple TNLA association, are sufficient for TNL management during feeder link switch.” Similar agreement can be replicated for IoT NTN and can be discussed further on how to address the lack of multiple TNL support.

Feeder link switch over scenarios can be discussed in NR NTN and the agreements in NR NTN can be replicated in IoT NTN.

### Mapped Cell ID

The usage of Mapped Cell ID for Regenerative Architecture can be similar to that of Transparent Payload. NR NTN agreements on Mapped Cell ID can be replicated for IoT NTN as well.

The usage of Mapped Cell ID in Regenerative Payload can be similar to that of transparent payload.

# Summary

Based on the discussion above, we have following observations:

Observation 1.	SA2 LS R3-247016 clarifies that conclusion for FS_5GSAT_Ph3 for “KI#1 and KI#2 is applicable for both NR NTN and IoT NTN. Whereas in RAN WID RP-242397 only S&F is mentioned as RAN3 scope.

Observation 2.	The architecture options in SA2 for S&F for Regenerative payload has whole CN on board as one option and split MME on board in another option. eNB only on board is not proposed in SA2 concluded architecture option for S&F.

Observation 3.	In both full CN on board and split MME architecture option, the whole eNB is on board and the S1 interface is also on board the satellite. In architecture option where the whole CN is on board, we see no RAN3 impact.

Based on these observations and discussion above, we have the following proposals:

Proposal 1:	RAN3 agrees that Regenerative Payload for IoT NTN is applicable for both with and without Store and Forward mode and agree to the SA2 LS response in R3-24xxxx

Proposal 2:	RAN3 to analyze only full CN on board and split MME architecture options agreed in SA2 for RAN3 impact and not eNB only on board architecture option.

Proposal 3:	Agreement in NR NTN Regenerative payload to introduce NG Removal procedure for NG connection management can be mirrored for S1 as well to support S1 connection management for IoT NTN for Regenerative Payload.

Proposal 4:	Feeder link switch over scenarios can be discussed in NR NTN and the agreements in NR NTN can be replicated in IoT NTN.

Proposal 5:	The usage of Mapped Cell ID in Regenerative Payload can be similar to that of transparent payload.