3GPP TSG RAN3 126	R3-247347

# Orlando, US, 18 - 22 November, 2024

Agenda item:	19.2

Source: 	Samsung

Title: 	Discussion on information exchange among gNBs for CLI handling

Document for:	Discussion and decision

# Introduction

This paper provides further discussion on the information exchange between gNBs for CLI handling, based on the conclusion of RAN3#125bis meeting:

RAN3 agrees to Exchange the following information over Xn interface and F1 interface:

- Semi-static cell-specific SBFD time and frequency location configuration with cell granularity (FFS on container or explicit IEs over XnAP and F1AP)

- Measurement resource configuration

- Strongest DL beam information (FFS on the details)

- CLI-mitigation request (FFS on the details)

# Discussion

## 2.1 Semi-static cell-specific SBFD time/frequency location configuration

Before RAN3 starts the discussion on SBFD, RAN1 has converged to the support of cell-specific configuration on time and frequency location of SBFD sub-bands within a TDD carrier. RAN2 have also discussed the semi-static cell specific SBFD time and frequency location and achieved several agreements on the basic principles and parameters when configuring SBFD. At RAN2#126 meeting in Netherland, the following agreement was captured:

Cell-specific SBFD time/frequency configuration is provided by SIB1 (or via dedicated signalling to covey cell specific configuration). FFS on UE specific dedicated RRC configuration if needed, pending on RAN1 progress.

In our understanding, the cell-specific SBFD time and frequency configuration in the SIB1 shall be configured along with the TDD DL/UL configuration, which can be sent from a gNB to other gNBs via the XnAP Setup Request and/or NG-RAN node configuration update message. with the intention to reduce the cross-link interference between gNBs, it is useful to send the SBFD time and frequency configuration to the other gNBs for information, such that the neighbouring gNBs can avoid transmitting via the DL resources which could cause CLI.

For split architecture, similarly, the cell specific SBFD configuration can be sent from to DU along with the TDD DL/UL configuration via F1 Setup Request message and gNB-DU Configuration Update message.

Proposal 1: The semi-static cell specific SBFD time and frequency configuration can be exchanged between gNBs via Xn Setup Request message and NG-RAN node Configuration Update message.

Proposal 2: The semi-static cell specific SBFD time and frequency configuration can be sent from the gNB-DU to the gNB-CU via F1 Setup Request message and gNB-DU Configuration Update message.

when it comes to the specific parameters within the configuration, RAN1 has the following agreements or working assumptions:

time domain: For configuration of SBFD symbols within a TDD-UL-DL pattern period, the following parameters are supported - A starting slot indexes - A starting symbol index within the starting slot - An ending slot index - An ending symbol index within the ending slot frequency domain: working assumption For cell-specific configuration of frequency locations of SBFD UL sub-band, for each SCS configuration in SCS-SpecificCarrierList for UL, starting RB and bandwidth of SBFD UL subband are indicated by a RIV-based indication as defined in 38.214 setting NBWPsize=275. For cell-specific configuration of frequency locations of SBFD DL subband(s), for each SCS configuration in SCS-SpecificCarrierList for DL, starting RB and bandwidth of each SBFD DL subband are indicated by a RIV-based indication as defined in 38.214 setting NBWPsize=275. - One or two SBFD DL subbands can be configured

The RAN1 agreements on specific parameters of the SBFD time and frequency configuration would be taken into account when RAN2 specify the SBFD configurations in SIB1, it is suggested that RAN3 wait for the stable specification of RAN2 before RAN3 define the detailed IEs over network interfaces.

Proposal 3: For the detailed IEs of cell-specific time and frequency location configuration, RAN3 wait RAN2 normative work for corresponding specification in SIB1.

## 2.2 Measurement resource configuration

For the measurement resource configuration, according to the information from RAN1 and TS 38.331, the resource information such as SSB and/or periodic NZP CSI-RS are already defined in the CSI-ResourceConfig IE of 38.331, where the SSB and NZP CSI-RS resource for channel measurement are configured.

In RAN1#118bis meeting, the following agreement is achieved:

Extend CSI-ResourceConfig to include two CLI measurement resource set lists for SRS-RSRP and CLI-RSSI measurements based on Rel-16 SRS-ResourceConfigCLI and rssi-ResourceConfigCLI defined in MeasObjectCLI for L3 based SRS-RSRP and CLI-RSSI measurement. - The resourceType for the two new CLI measurement resource set lists can be set to periodic, semi-persistent or aperiodic - The number of periodic/semi-persistent CLI measurement resource sets is restricted to 1 in a CSI-ResourceConfig as in current specifications. - Note: No new usage needs to be defined for SRS resource sets

Although RAN2 has not started the work on specifications about the measurement configuration, we believe the corresponding standardization work in RAN2 would be based on existing and any further RAN1 agreements. When it comes to the signaling enchantment in RAN3, the CSI-ResourceConfig can be simply referred as a container, which would be exchanged between gNBs as the measurement resource configuration for UEs.

Proposal 4: The CSI-ResourceConfig IE in 38.331 can be referred as a container for the exchange of measurement resource configuration between gNBs.

With Regard to the procedure used to transfer the measurement resource allocation, according to the contributions submitted last meeting, there are basically three options over XnAP:

- Option 1: To reuse interface management procedures, i.e., the XnAP Setup procedure and the NG-RAN node configuration Update procedure;

- Option 2: To reuse the Resource Status Reporting Initiation procedure for measurement configuration and Resource Status Reporting procedure for measurement results;

- Option 3: To introduce new procedures for the exchange of measurement resource configuration.

Firstly, we find the logic of Option 2 is not correct because it is not aligned with what RAN1 has decided for CLI handling. In RAN1, beam nulling and beam pairing mechanisms were discussed as the spatial solutions for CLI handling. With respect to beam nulling, it is the victim gNB to provide the measurement resource configuration to the aggressor gNB, while the aggressor gNB can perform beam nulling by its implementation, based on the corresponding measurement results. Hence, a mechanism similar to resource status reporting in Option 2 is not needed. On the other hand, for beam pairing, the aggressor gNB can initiate the measurement reporting from the victim gNB, but in our mind, it shall always be the victim gNB to trigger the mechanism of CLI handling, so, in this case, what option 2 proposes does not stand either. In a word, we believe that a mechanism similar to resource status reporting does not fit for the exchange of measurement resource configuration and results.

Then, between Option 1 and Option 3, we have a slight preference for Option 1. The existing XnAP Setup and NG-RAN Node Configuration Update procedures can be well reused for transmitting the measurement resource configurations, since most of the other information in this message is optional, which means the message can be dedicatedly used for the exchange of measurement resource configuration, without much burden on the interface for unnecessary information elements. With this understanding, for the sake of minimized impact on the specifications, there is no bother to interduce new procedures merely for the intention to exchange measurement resource configuration between gNBs.

Proposal 5: The measurement resource configuration can be transferred in the XnAP Setup Request message and NG-RAN node configuration Update message over XnAP.

As for split architecture, similarly, as mentioned above, the F1 Setup Request and gNB-DU Configuration Update message can be used to send the measurement resource allocation configuration from the gNB-DU to the gNB-CU.

Proposal 6: The measurement resource configuration can be transferred in the F1 Setup Request and gNB-DU Configuration Update message over F1AP.

## 2.3 Strongest DL beam information

The downlink strongest DL beam information is the measurement result which is collected based on the exchanged measurement resource configuration in section 2.2. It should be sent from the victim gNB to the aggressor gNB, so that the aggressor gNB can perform beam pairing based on the channel measurement result from the victim gNB.

As RAN1 said in the incoming LS of last meeting, the strongest DL beam information can be referred as a CRI in the context of NZP-CSI-RS and an SSB-index in the context of SSBs..

Strongest DL beam information in the context of NZP-CSI-RS is defined as a CSI-RS Resource Indicator (CRI) value. CRI is a relative index in the range [1 .. N], where N is the number of CSI-RS resources within a set of resources for which the configuration is provided to a gNB. Strongest DL beam information in the context of SSBs is defined as an SSB index. SSB index is an absolute index among the SSB(s) for which the configuration is provided to a gNB.

How to transfer the strongest DL beam information shall be discussed. Following similar logic as in section 2.2, we think that the NG-RAN node configuration update procedure is sufficient for the victim gNB to transfer the strongest DL beam information to the aggressor. There is no need to introduce new procedures or to reuse the Resource Status Reporting procedure. Some concern was raised in some contributions that the reporting of strongest DL beam information might be quite frequent, which may bring much burden on the Xn interface if the NG-RAN node configuration update procedure is reused. This argument makes some sense to us, but from our point of view, the strongest DL beam information would not change too frequently in a good gNB implementation.

Proposal 7: The strongest DL beam information can be transferred in the NG-RAN Node Configuration Update message over XnAP.

For split architecture, since the CSI measurement are lower layer measurement results transferred via PUCCH and PUSCH, only DU is aware of this information, to support the exchange between gNBs, the gNB-DU shall send the strongest DL beam information to the gNB-CU, after which the gNB-CU can send this information to the other gNB-CU(s). Hence, the gNB-DU Configuration Update procedure could be used for the gNB-DU to report the strongest DL beam information to the gNB-CU.

Proposal 8: The strongest DL beam information can be transferred in the gNB-DU Configuration Update massage from the gNB-DU to the gNB-CU over F1AP.

## 2.4 CLI-mitigation request

It is noted that RAN1 did not provide any specific information or opinion about the CLI-mitigation request in the incoming LS, and there is no further discussion in RAN1 either. To some extent, it should depend on our RAN3 discussion and evaluation about whether and how to introduce such kind of CLI-mitigation request. Generally, the issue on CLI mitigation request can also be discussed with respect to the mechanisms of beam nulling and beam pairing.

For beam nulling, it is the victim gNB to provide the measurement resource configuration to the aggressor gNB, and what the aggressor gNB should do is to adjust the beam scheduling based on the its measurement results. In such scenario, there seems no need for the victim gNB to send any other CLI-mitigation request to the aggressor gNB, since the measurement resource configuration can be taken as an implicit CLI-mitigation request. Note that the behaviour of the aggressor gNB who receives the measurement resource configuration is to change its own scheduling for mitigating the interface it caused to the victim gNB, which is exactly the original intention of a CLI mitigation request.

Observation 1: The measurement resource configuration can be taken as an implicit CLI-mitigation request in beam nulling.

However, for beam pairing, an implicit request doesn’t work well in our mind. In the mechanism of beam pairing, the victim gNB shall measure the channel information based on the measurement resource configuration provided by the aggressor gNB. If the victim gNB is not aware of the measurement resource configuration, it can not trigger the CLI-mitigation. So, in the beginning, the victim gNB shall be allowed to send an CLI-mitigation request to the aggressor gNB, after which the aggressor gNB could provide the measurement resource configuration to the victim gNB if it has not yet sent it. When the victim measures the channel information based on the aggressor’s measurement resource configuration, it can provide the measurement results to the aggressor. Based on the measurement results from the victim gNB, the aggressor gNB can perform beam pairing based on its implementation.

Observation 2: An explicit CLI-mitigation request is needed for the victim gNB to acquire the measurement resource configuration from the aggressor gNB for beam pairing.

With the above analysis and observations, it can be concluded that an explicit CLI-mitigation request is needed over Xn, to support the mechanism of CLI-handling between gNBs. From our point of view, there is no need to indicate the beam management type (e.g., beam pairing or beam nulling) in the indication, because the management of beam scheduling is based on the implementation of the aggressor, not indicated by the victim gNB.

Proposal 9: An explicit CLI-mitigation request over Xn is needed, to support the CLI handling between gNBs.

There were also papers of last meeting which discussed the procedure to be used for transmitting the CLI-mitigation request. Two typical solutions were proposed, one option is to reuse the NG-RAN node configuration update procedure, the other one is to define a new class-2 procedure for the CLI-mitigation request. The main difference is that the new procedure of CLI-mitigation is a class-2 procedure and could save some signalling burden, compared to the class-1 NG-RAN node configuration update procedure. But we are open to both of the opens and can have further discussion during the meeting.

Proposal 10: Discuss how to transfer the CLI mitigation request between gNBs.

With all the above discussion, TPs to the BLCRs of TS38.423 and TS38.473 is provided in [1].

Since most of the information exchanged between gNBs is dependent on the other WGs, especially RAN2 for RRC signaling design, we think that an LS to RAN2 would be better to trigger the corresponding work in RAN2. Otherwise RAN2 might postpone some discussion and maybe even wait RAN3 progress for something. An LS would be the best method to inform the other WGs of our side and trigger their discussion.

Proposal 11: Send an LS to RAN2 to ask about the RRC standardization progress on time/frequency location configuration of sub-band full duplex and measurement resource configuration.

A draft LS is provided in the Annex.

# Conclusion

In this contribution, the RAN3 impact of CLI-handling were analysed. The following proposals are provided in this paper:

Proposal 1: The semi-static cell specific SBFD time and frequency configuration can be exchanged between gNBs via Xn Setup Request message and NG-RAN node Configuration Update message.

Proposal 2: The semi-static cell specific SBFD time and frequency configuration can be sent from the gNB-DU to the gNB-CU via F1 Setup Request message and gNB-DU Configuration Update message.

Proposal 3: For the detailed IEs of cell-specific time and frequency location configuration, RAN3 wait RAN2 normative work for corresponding specification in SIB1.

Proposal 4: The CSI-ResourceConfig IE in 38.331 can be referred as a container for the exchange of measurement resource configuration between gNBs.

Proposal 5: The measurement resource configuration can be transferred in the XnAP Setup Request message and NG-RAN node configuration Update message over XnAP

Proposal 6: The measurement resource configuration can be transferred in the F1 Setup Request and gNB-DU Configuration Update message over F1AP.

Proposal 7: The strongest DL beam information can be transferred in the NG-RAN Node Configuration Update message over XnAP.

Proposal 8: The strongest DL beam information can be transferred in the gNB-DU Configuration Update massage from the gNB-DU to the gNB-CU over F1AP.

Observation 1: The measurement resource configuration can be taken as an implicit CLI-mitigation request in beam nulling.

Observation 2: An explicit CLI-mitigation request is needed for the victim gNB to acquire the measurement resource configuration from the aggressor gNB for beam pairing.

Proposal 9: An explicit CLI-mitigation request over Xn is needed, to support the CLI handling between gNBs.

Proposal 10: Discuss how to transfer the CLI mitigation request between gNBs.

Proposal 11: Send an LS to RAN2 to ask about the RRC standardization progress on time and frequency location of sub-band full duplex and measurement resource configuration.

# References

[1] R3-247348 TPs to BLCRs of TS38.423 and TS38.473 on CLI handling

# Annex: draft LS to RAN2

Title:	[draft] LS to RAN2 on sub-band full duplex configurations

Release:	Release 19

Work Item:	NR_duplex_evo

Source:	Samsung [To be RAN3]

To:	TSG RAN WG2

Cc:	TSG RAN WG1

Contact person:	Man Zhang

man1.zhang@samsung.com

# 1	Overall description

RAN3 is discussing the information exchanges between gNBs for CLI handling, based on the agreements of RAN1.

It has been agreed in RAN3 that the semi-static cell specific SBFD time and frequency configuration to be exchanged between gNBs can be defined as a container referring to the corresponding defined in the SIB1 message by RAN2.

Similarly, RAN3 also agrees that the measurement resource configuration to be exchanged in RAN3 interfaces can take the CSI-ResourceConfig IE in 38.331 as a reference.

With the above understanding, RAN3 would like to kindly ask RAN2 about the progress of corresponding normative work on the definition of SBFD time/frequency location configuration and measurement resource configuration, so that RAN3 can continue to work on the procedure and IE design for network interfaces regarding CLI handling.

# 2	Actions

To TSG RAN2

ACTION: 	RAN3 respectfully asks RAN2 to take the above information into account and provide any feedback on the definition of SBFD time/frequency location configuration and measurement resource configuration.

# 3	Dates of next TSG RAN WG1 meetings

RAN3#126	18th – 22nd November 2024	 	Orlando, US

RAN3#127             17th – 21st February 2025         Athens, GR