3GPP TSG-RAN WG3 Meeting #126	R3-247400

Orlando, USA, 18th – 22th November, 2024

Agenda Item:	14.3

Source:	Huawei

Title:	(TP for TS 38.300) Support of regenerative payload

Document for:	discussion

# 1	Introduction

In the last meeting, RAN3 achieved the following agreements for the regenerative payload [1]:

The supported TAIs of an on-board gNB could be provided to AMF by OAM/pre-configuration. The current mechanisms including, e.g. multiple TNLA association, are sufficient for TNL management during feeder link switch.

In this paper, we continue to discuss the NG impact and the inactive support in the regenerative payload.

# 2	Discussion

## 2.1 NG impact

Since it was already agreed to introduce the NG removal procedure, there is no strong need for introducing NG suspend/resume procedures. The usage to implement NG suspend/resume is ambiguous.

The only advantage for the NG suspend/resume procedure may be a little signalling overhead reduction when establishing the NG connection with the same AMF again and not re-establishing the interface context (NG configurations). But it is doubtful whether the same NG configurations can be actually reused when the satellite gNB accesses the same AMF again. For example, when the gNB resumes NG connection from different location, the supported TA information should be different, so the old stored supported TA is useless; and, if the gNB resumes NG connection via a different GW, the current TNL address is different from the old TNL address, so the old NG-U tunnel configuration is also outdated.

In the last meeting it was discussed and noted by chair notes that:

RAN CONFIGURATION UPDATE procedure could be reused to support NG suspend/Resume, by introducing suspend/resume indicators to avoid frequent TNL disconnection and re-connection?

This idea was first proposed by the paper [2] for the case of hard feeder link switchover. The feeder link switch is generally caused by the changing of satellite coverage, and as analyzed above, if the coverage area changes and GW changes, the NG configurations cannot be reused directly.

Observation 1: The old NG configurations cannot be reused when the gNB accesses the AMF at different location or via different GW.

Then, the NG suspend/resume operation requires the gNB stores those NG configurations related to multiple AMFs. Considering the earth rotation, it may take a long time for the gNB connects to the same AMF again, which causes much storage consumption on the gNB for retaining multiple AMFs’ NG configurations.

Observation 2: NG suspend/resume operation requires the gNB to store NG configurations relevant to multiple AMFs, which leads to much storage consumption on the regenerative payload/gNB.

Based on the observations above, we propose:

- No need to introduce the NG suspend/resume towards the AMF.

## 2.2	INACTIVE supporting

The paging and UE context relocation for INACTIVE UEs are relying on the Xn interface. Due to the satellite mobility when there is no Xn connection between the UE’s last serving gNB and the UE’s current camped gNB, the RAN paging would fail. Companies discussed candidate solutions in the previous meetings, and the solutions can be classified into three directions:

- Proactive context pushing

In this solution, the last serving gNB can proactively push the UE context to the next satellite (we call it candidate gNB in our following analysis) [3]. But as we proposed in our papers for the previous meetings, there exists the following problem for pushing the UE context:

When the INACTIVE UE resuming connection, the UE’s verification is executed by the candidate gNB via comparing the shortMAC-I reported by the UE’s RRCResumeRequest and included in the RETRIEVE UE CONTEXT REQUEST, and the shortMAC-I computed by itself. Specifically, the candidate gNB computes the shortMAC-I using the Krrcint as the key, and the source C-RNTI, source PCI, target cell ID as the input parameters. The key Krrcint should be the key of the gNB releasing the UE to INACTIVE, i.e., the last serving gNB, which is different from the candidate gNB. But normally the key of a gNB cannot be transferred to another gNB. Therefore, to make the context pushing solution feasible, the last serving gNB should infer all the possible target cells where the UE may resume the connection, and generate the possible shortMAC-I values using the Krrcint of itself and the possible target cell IDs, and send these together with the UE context to the candidate gNB, which is hard to be implemented and with heavy signalling overhead.

- Stage-2 enhancements

The INACTIVE UE’s last serving gNB can simply initialize the RAN paging for the UE, when the Xn interface is to be disconnected between the last serving gNB and a member in the gNBs relevant to the UE's RNA. In this way, the UE context can be guaranteed to be relocated to its current camped gNB, which is the UE’s current serving gNB after the paging, and the key is also refreshed.

This enhancement causes RAN paging signalling before the UE context “may” be lost. It is essentially a solution to sacrifice some signalling overhead to ensure the reliability as well as Inactive benefits. But generally, the Xn can be maintained either via the ISL or via the feeder link. Hence the RAN paging can happen less than the expected, and such signalling overhead is at the tolerable degree.

- UE contexts stored on ground

There were some proposals to store the INACTIVE UE context on the ground gNB or the ground AMF, or the UE’s current gNB fetches the context from the anchor gNB with the help of ground AMF.

First, the context storage at the ground gNB is unworkable, considering the TN-NTN Xn interface and TN-NTN connected mode mobility is out of scope for this release.

Second, for the context storage at ground AMF, this is a totally new function from AMF perspective. Also, it may happen that the UE’s current gNB may be inaccessible to the ground AMF storing the UE context.

At last, for the “context fetching from the anchor gNB via ground AMF”, even if the route path is possible, it comes to the question why the logical Xn connection cannot be maintained while the NG connection is available. It seems easier that the Xn can be maintained via the feeder link.

Based on the analysis above, direction 2 seems to be simple and with little cost and spec impact.

- RAN3 to agree the stage-2 descriptions that the UE's last serving gNB may send paging messages to candidate gNB(s) included in the UE's RNA, when the Xn interface is about to be disconnected between the last serving gNB and a candidate gNB. (the TP is provided in the annex)

# 3	Conclusion

This contribution discusses the regenerative payload for the NR NTN. It is proposed:

Observation 1: The old NG configurations cannot be reused when the gNB accesses the AMF at different location or via different GW.

Observation 2: NG suspend/resume operation requires the gNB to store NG configurations relevant to multiple AMFs, which leads to much storage consumption on the regenerative payload/gNB.

- No need to introduce the NG suspend/resume towards the AMF.

- RAN3 to agree the stage-2 descriptions that the UE's last serving gNB may send paging messages to candidate gNB(s) included in the UE's RNA, when the Xn interface is about to be disconnected between the last serving gNB and a candidate gNB. (the TP is provided in the annex)

# 4	References

- RAN3#125b Chair notes.

- R3-245434, (TP for TS 38.300) Discussion on RAN Signaling impacts for NR NTN Regenerative Payload, Qualcomm Incorporated, RAN3 125b.

- R3-243355, (TP for TS 38.300 and TS 38.413) Discussion on the support of Regenerative payload, Nokia, Nokia Shanghai Bell, RAN3 124.

- R3-243949 Support for Regenerative Payload in NR NTN, Ericsson, Thales, Deutsche Telekom, Nokia, ESA, CATT, ZTE, Sateliot, Huawei, Dish Networks, Echostar, Eutelsat Group, Xiaomi, Samsung, CMCC, LG Electronics

# 5	Annex: TP for BLCR for TS 38.300

<<<<<<<<<<<<<<<<<<<< Changes begin>>>>>>>>>>>>>>>>>>>>

### 16.14.6	AMF (Re-)Selection

The gNB implements the NAS Node Selection Function specified in TS 38.410 [16].

For an RRC_CONNECTED UE, when the gNB is configured to ensure that the UE connects to an AMF that serves the country in which the UE is located, if the gNB detects that the UE is in a different country to that served by the serving AMF, then it should perform an NG handover to change to an appropriate AMF, or initiate an UE Context Release Request procedure towards the serving AMF (in which case the AMF may decide to de-register the UE).

For the purpose of selecting an appropriate AMF, the 5GC may verify the UE location according to TS 23.501 [3] and TS 38.305 [42] after the UE has attached to the network.

NOTE:	UE location verification for AMF selection should not be necessary if NTN cell(s) do not extend across countries.

NEXT CHANGE

### 16.14.X	INACTIVE support in regenerative payload

In case that the Xn disconnection between the two on-board gNBs is foreseeable, the UE's last serving gNB may send RAN paging messages to candidate gNB(s) included in the UE's RNA, and transfer the UE context to the UE’s current serving gNB when receiving the RETRIEVE UE CONTEXT REQUEST message.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>