3GPP TSG-RAN WG3 Meeting #126	R3-247424

Orlando, USA, Nov.18–22, 2024

Agenda item:	 10.2

Source:		Lenovo

Title:		(TP for SON BLCR for 38.401) Discussion on MRO for LTM

Document for:		Discussion and Decision

1	Introduction

RAN3#123bis meeting agreed that:

Work on scenarios of near failure LTM

Work on scenarios for the differentiation of too early LTM, too late LTM and LTM to wrong cell

RAN3#124 meeting agreed that:

RAN3 to prioritize MCG LTM over SCG LTM.

RAN3 takes RAN2#125-bis agreement on MRO for LTM scenarios as baseline for further study.

For failures due to wrong selection of candidate LTM cell, CU is in charge of root cause analysis and performs optimization.

For failures due to inappropriate cell switch triggering (e.g. wrong cell selection at cell switch, wrong cell switch timing, …) source DU performs optimization.

RAN3 to discuss enhancements related to the information of RACH-less access and RACH-based access from UE related to MRO for LTM.

RAN3#125 meeting agreed that:

Reuse the existing connection failure definition of too late handover, too early handover, handover to wrong cell for LTM failure case. Add a sentence for LTM handover.

Take the existing detection mechanism descriptions as baseline.

For failures due to inappropriate cell switch triggering, CU initiates analysis and may forwards the RLF report to the DU responsible for the failure.

CU needs to forward the RLF related information to DU using existing Access and Mobility Indication procedure.

RAN3#125bis meeting agreed that:

For TP to BL CR to TS 38.300:

- Remove editor’s note 1 and 2. No need to capture LTM recovery in the failure definitions.

- For detection mechanism, use same approach as for CHO i.e. reuse existing sub-bullets to add LTM.

- Change “successful handover" definition, as proposed in R3-245230.

For TP to BL CR to TS 38.401:

- Capture wrong LTM prepared cell selection.

Add failure type to F1AP ACCESS AND MOBILITY INDICATION message.

CU does not forward the RLF Report to DU if failure is due to wrong selection of prepared LTM cell.

In this paper, we would further discuss the details of MRO for LTM.

2	Discussion

  - MRO for connection failure in LTM

Based on agreements achieved so far, we can sumarize the failure cause analysis and optimisation as well as RLF report forwarding in F1 as below:

- For failures due to wrong selection of candidate LTM cell, CU is in charge of root cause analysis and performs optimization, CU does not forward the RLF Report to DU;

- For failures due to inappropriate cell switch triggering (e.g. wrong cell selection at cell switch, wrong cell switch timing, …), CU initiates analysis and may forward the RLF report to the DU responsible for the failure, the DU responsible for the failure performs optimization.

In current TP for TS38.401, only operation in case of failure due to inappropriate cell switch triggering is added, however, how to handle the failure case due to wrong selection of candidate LTM cell is missing. However, RAN3#125bis meeting agreed to capture wrong LTM prepared cell selection in TS 38.401, so we should capture the corresponding mechanism to make the spec clearer. The TP is given in Annex.

Proposal 1: Agree the TP for TS38.401 in Annex.

Regarding the issue about whether fallback from RACH-less to RACH based LTM is supported:

- UE performs RACH-less LTM when TA value received in the LTM cell switch command is valid or when UE based TA measurement is configured and UE successfully calculates the TA value. When RACH-less LTM fails (i.e. T304 expires), the UE would perform RRC re-establishment or RACH-based LTM recovery procedure.

- If the TA value in LTM cell switch command is invalid (e.g. TA validity timer expires at network side) or UE does not successfully calculate the TA value, the UE performs RACH-based LTM directly, there is no fallback mechanism since RACH-less LTM is not executed at all.

Observation 1: Fallback from RACH-less to RACH based LTM is not supported.

RAN2#127 meeting agreed to include the specific access type in the RLF report, i.e. whether it is RA-based or RA-less cell switch, and FFS details e.g. if explicit or implicitly signalled. RAN3 can wait for RAN2 progress, and have no further discussion on additional information regarding RACH-less or RACH based LTM in RLF report.

Observation 2: Wait for RAN2 progress, and RAN3 have no further discussion on additional information regarding RACH-less or RACH based LTM in RLF report.

Additionally, RAN2#127bis meeting agreed that unless RAN3 defines a NW-based solution, the UE logs and reports whether and how the UE got the TA value used for a failed LTM switch (gNB indicated or UE determined). Since it is network that configures the UE to perform UE based TA measurement or PDCCH ordered early TA acquisition, network can identify how the UE got the TA value based on the stored UE context, it is not needed for the UE to log and report whether and how the UE got the TA value used for a failed LTM cell switch.

Proposal 2: It is not needed for the UE to log and report whether and how the UE got the TA value used for a failed LTM cell switch in RLF report.

  - MRO for near failure in LTM

In legacy, for analysis of sub-optimal successful handover, the UE may collect SHR based on the triggers, and makes the SHR available to the network. For legacy SHR, T310/T312/T304 related trigger threshold is configured by network to trigger SHR storing or generating at UE side. Since T310/T312/T304 are reused for LTM, T310/T312/T304 related triggering conditions can be reused for LTM SHR.

RAN2#127 meeting agreed that RA-based access or HO interruption time will not be a new triggering condition for the SHR report. Regarding other new trigger condition, e.g. a threshold related to number of RLC retransmissions of the first UL transmission, we can wait for RAN2 progress.

Observation 3: T310/T312/T304 related triggering conditions can be reused for LTM SHR.

3	Conclusion

In this paper, MRO for LTM are further discussed. We have the following observations and proposals:

Observation 1: Fallback from RACH-less to RACH based LTM is not supported.

Observation 2: Wait for RAN2 progress, and RAN3 have no further discussion on additional information regarding RACH-less or RACH based LTM in RLF report.

Observation 3: T310/T312/T304 related triggering conditions can be reused for LTM SHR.

Proposal 1: Agree the TP for TS38.401 in Annex.

Proposal 2: It is not needed for the UE to log and report whether and how the UE got the TA value used for a failed LTM cell switch in RLF report.

Annex: TP for TS38.401

## 7.X	MRO support for LTM

The gNB-CU receives a RLF report associated to an LTM mobility event from the UE or via the Failure indication message over Xn. The gNB-CU performs initial analysis and in case of failure due to inappropriate cell switch triggering may forward the RLF report to the gNB-DU responsible for the failure. The gNB-CU does not forward the RLF Report to DU in case of failure due to wrong selection of LTM candidate cell.