3GPP TSG-RAN WG3 Meeting #126	R3-247431

Orlando, USA, 18th – 22th November, 2024

Agenda Item:	12.3

Source: 	Lenovo

Title: 	TP to 38.300 for NR Femto

Document for:	Discussion and decision

# Introduction

In this paper, text is proposed to capture the aspects related to NR Femto based on conclusions in another paper [1] and TR 38.799 [2].

# Reference

- R3-24xxx3. Architecture and access control for NR Femto. Lenovo.

- TR 38.799. Study on additional topological enhancements for NR (Release 19).

# TP to 38.300

## 4.X	Support of NR Femtos

### 4.X.1	Architecture

Figure 4.X.1-1 shows a logical architecture for the NR Femto that has a set of NG interfaces to connect the NR Femto to the 5GC.

Figure 4.X.1-1: NR Femto Logical Architecture

NOTE: the SeGW is out of RAN scope.

The NR Femto may directly connect to the 5GC. The NG-RAN architecture may also deploy an NR Femto Gateway (NR Femto GW) to allow the concentration of the NG-C interface between the NR Femto and the 5GC. Based on implementation, the transport of NG-U between the NR Femto and the 5GC may be optionally concentrated in the NR Femto GW.

For NR Femto, the NG-C interface is defined as the interface:

-	Between the NR Femto GW and the Core Network;

-	Between the NR Femto and the NR Femto GW;

-	Between the NR Femto and the Core Network;

The NR Femto GW appears to the AMF as a gNB. The NR Femto GW appears to the NR Femto as an AMF. The NG interface between the NR Femto and the 5GC is the same regardless whether the NR Femto is connected to the 5GC via an NR Femto GW or not.

The functions supported by the NR Femto shall be the same as those supported by a gNB (with possible exceptions e.g. NNSF when the NR Femto connects via the NR Femto GW) and the procedures run between an NR Femto and the 5GC shall be the same as those between a gNB and the 5GC.

Xn-connectivity is supported between NR Femtos and between NR Femtos and gNBs, independent of whether any of the involved NR Femtos is connected to an NR Femto GW.

An NR Femto shall only connect to a single NR Femto GW at one time and will not simultaneously connect to another NR Femto GW or AMF. In addition, An NR Femto may have more than one cell, and the global gNB ID can be reused to identify the NR Femto.

For the NR Femto GW, Xn interface is not supported between NR Femto node and other nodes.

NEXT CHANGE

### 4.x.y	Access Control

- With the existing CAG mechanism, the open, hybrid and closed access mode, can be supported as follows:

-	To support the open access mode: The NR Femto node activates a PLMN cell, which can be accessed by legacy UE without access control of CAG.

-	To support the hybrid access mode: The NR Femto activates a cell shared by both PLMN and CAG, through broadcasting both the plmn-IdentityInfoList and the npn-IdentityInfoList-r16 in the SIB1, but without the cellReservedForOtherUse. Then, this cell is accessible to UEs which have the allowed CAG list including a CAG-ID broadcasted by the cell. For the legacy UE not supporting CAG, this cell is viewed as a normal PLMN cell.

-	To support the closed access mode: The NR Femto node activates an NPN-only cell by broadcasting the cellReservedForOtherUse IE with value “true”, then this cell can only be accessed by the UEs whose allowed CAG list includes a CAG-ID broadcasted by the cell.

Xn-based HO involving NR Femto node is allowed as shown in Table 4.x.3-1. For the UE handover of NR Femto, the handover cannot be supported if the target cell is a non-CAG cell.

Table 4.x.y-1: Xn-based HO support

| Source | Target | Notes |
| --- | --- | --- |
| gNB or any NR Femto node | open access NR Femto node |  |
| gNB or any NR Femto node | hybrid access NR Femto node |  |
| Hybrid access NR Femto node or closed access NR Femto node | closed access NR Femto node | Only applies for same CAG ID and PLMN, and if the UE is a member of the CAG cell. |
| Any NR Femto node | gNB |  |