3GPP TSG-RAN WG3 Meeting #126	R3-247436

Orlando, USA, Nov. 18th – 22nd, 2024                          Revision of R3-245451

Agenda item:		14.3

Source:		Lenovo

Title:		Interface management for regenerative payload in NTN

Document for:		Discussion

- Introduction

The latest RAN3 meetings made the following agreements regarding regenerative payload in NTN:

RAN3#125: NG Interface management Issue: Option 2 NG Removal/Setup can be used for Regenerative Payload NG interface management. Inactive UE support: RAN3 assumes that current mechanisms to support UEs performing RRC connection resume and RRC connection re-establishment may be reused in regenerative payload architecture, subject to implementation and deployment. Feeder link switch issue: RAN3 assumes that during the feeder link switch, AMF is not changed for the UEs. Whether there has any standard impact needs to be further checked. RAN3#125bis: The supported TAIs of an on-board gNB could be provided to AMF by OAM/pre-configuration. The current mechanisms including, e.g. multiple TNLA association, are sufficient for TNL management during feeder link switch.

In this contribution we discuss some potential issues and solutions in interface management of supporting regenerative payload.

- Discussion

In Rel-17 and Rel-18 NTN, transparent payload has always been assumed to focus the study on more essential aspects. For the transparent payload, the gNBs are on the ground and thus the information exchange between gNBs and AMF is similar to that in TN. As a result, no impact was identified for the signaling exchange between gNB and AMF via NG interface. However, in Rel-19 NTN with regenerative payload, the accessibility between gNB on-board satellite and AMF is impacted by feeder link switch between satellite and NTN gateway on earth as in Figure 1.

Figure 1. Impact on NG interface from feeder link switch in NTN regenerative deployment

Observation 1: The accessibility between gNB on board satellite and AMF can be impacted by feeder link switch between satellite and NTN gateway on earth.

Thanks to the ephemeris information from OAM, the exact feeder link switch time, or the remaining validity duration of a feeder link can be derived or predicted. To minimize the impact of feeder link switch on NG interface, such information can be exchanged between gNB and AMF over NG (e.g., in NG setup procedure and in NG removal procedure if supported).

Proposal 1: Information of NG interface change due to feeder link switch (e.g., feeder link switch time or feeder link validity duration) is included in NG Setup message.

Proposal 2: The reason of NG interface change (feeder link switch) is included in NG Removal message.

Similarly, in Rel-19 NTN with regenerative payload, the accessibility between source and target gNBs on-board two satellites can be impacted by ISL existence between satellites and feeder link switch (if there is no ISL) as in Figure 2.

Figure 2. Impact on Xn interface from ISL change and feeder link switch in NTN regenerative deployment

Observation 2: The accessibility between two gNBs on board different Satellites can be impacted by ISL existence between satellites and feeder link switch between NTN payload and AMF entity on earth.

If the connection between two gNBs cannot be maintained due to ISL change or feeder link switch, the corresponding mobility procedures could be interrupted. For example, a UE may perform handover to a target gNB that may lose connection to the source gNB, and as a result the target gNB may not be able to complete the handover procedures or to retrieve UE context from the source gNB. To minimize the impact of feeder link switch or ISL change on Xn interface, the corresponding information can also be exchanged between gNBs over Xn (e.g., in Xn setup procedures).

Proposal 3: Information of Xn interface change due to feeder link switch (e.g., feeder link switch time or feeder link validity duration) is included in Xn Setup message.

Proposal 4: Information of Xn interface change due to ISL change (e.g., ISL change time or ISL validity duration) is included in Xn Setup message.

Moreover, the UE context transfer procedure for UE in RRC_INACTIVE may be impacted by the accessibility change between two gNBs on board different satellites. For the UE is in RRC_INACTIVE, if the last serving gNB receives DL data from the UPF or DL UE-associated signalling from the AMF (except the UE Context Release Command message), it pages in the cells corresponding to the RNA. If the UE accesses a gNB other than the last serving gNB, the receiving gNB triggers the XnAP Retrieve UE Context procedure to get the UE context from the last serving gNB. This fixed design combined with the moving satellite also brings in new issues related to UE AS context retrieval.

Observation 3: Due to the accessibility change between two gNBs on board different satellites, the XnAP Retrieve UE Context procedure for RRC_INACTIVE may fail.

Proposal 5: RAN3 to discuss inter-gNB UE context transfer for UE in RRC_INACTIVE before accessibility change between gNBs on board different satellites due to ISL change or feeder link switch.

- Conclusion

In this contribution we discuss the potential issues of supporting regenerative payload focusing on the accessibility change between two gNBs on board different Satellites. The following observations are given:

Observation 1: The accessibility between gNB on board satellite and AMF can be impacted by feeder link switch between satellite and NTN gateway on earth.

Observation 2: The accessibility between two gNBs on board different Satellites can be impacted by ISL existence between satellites and feeder link switch between NTN payload and AMF entity on earth.

Observation 3: Due to the accessibility change between two gNBs on board different satellites, the XnAP Retrieve UE Context procedure for RRC_INACTIVE may fail.

And it is proposed that:

Proposal 1: Information of NG interface change due to feeder link switch (e.g., feeder link switch time or feeder link validity duration) is included in NG Setup message.

Proposal 2: The reason of NG interface change (feeder link switch) is included in NG Removal message.

Proposal 3: Information of Xn interface change due to feeder link switch (e.g., feeder link switch time or feeder link validity duration) is included in Xn Setup message.

Proposal 4: Information of Xn interface change due to ISL change (e.g., ISL change time or ISL validity duration) is included in Xn Setup message.

Proposal 5: RAN3 to discuss inter-gNB UE context transfer for UE in RRC_INACTIVE before accessibility change between gNBs on board different satellites due to ISL change or feeder link switch.