3GPP TSG-RAN WG3 Meeting #126	R3-247437

Orlando, USA, 18 - 22 Nov, 2024

Agenda item:	16.2

Source: 	Lenovo

Title: 	Considerations on bistatic mode of operation

Document for:	Discussion and Approval

- Introduction

Besides the monostatic mode, the bistatic mode has been defined by RAN1. This paper tries to discuss whether bistatic mode in both topology 1 and topology 2 is supported or not.

- Discussion

The current discussions in RAN3 assumes the monostatic mode of operation, i.e. the communication of AIoT devices with only a single reader. That means, the reader transmits the CW for D2R backscattering and receives the D2R backscattering. However, this will result in self-interference in the reader when e.g. the CW is transmitted in UL spectrum. Therefore, in order to avoid the self-interference in the reader, the bistatic mode of operation may need to be considered.

Table 1 below shows some scenarios with regards to bistatic mode of operation which have been defined in TR 38.769 [1]. The scenarios in Table 1 consider only AIoT devices of type 1 and 2a.

Table 1: Scenarios for bistatic mode of operation considering AIoT devices of type 1 and 2a

| Scenario | CW inside/outside topology | Diagram | Comment |
| --- | --- | --- | --- |
| D1T1-A1 | CW inside topology |  | Reader1 (R1) and Reader2 (R2) are TRPs operating in bistatic mode to transmit R2D and receive D2R resp. |
| D2T2-A1 | CW inside topology |  | Reader1 (R1) and Reader2 (R2) are intermediate UEs operating in bistatic mode to transmit R2D and receive D2R resp. |

In case of scenario D1T1-A1, reader1 (R1) and reader2 (R2) are AIOT enabled RAN nodes operating in bistatic mode where the reader1 transmits the R2D message and reader2 receives the D2R message from the AIoT device. To support this scenario there needs to be coordination between these BS/TRPs to exchange the information related to when to expect the D2R response from the device side and the corresponding frequency resource. Such information is necessary so that the reader2 is in the receive-only mode at the right frequency and does not transmit anything at that time. For this scenario, RAN3 needs to study whether inter-BS signaling interface can be used for the purpose of coordination in bistatic mode of operation.

Proposal 1: RAN3 study signaling exchange to support bistatic mode of operation for D1T1-A1 scenario.

In case of scenario D2T2-A1, reader1 and reader2 are intermediate UEs operating in bistatic mode. In this scenario both intermediate UEs are under the control of BS by using the signaling interface between intermediate UE and BS. For this scenario RAN3 can study e.g. on how to signal the expected D2R response and the corresponding frequency resource to an intermediate UE for monitoring the D2R response.

Proposal 2: RAN3 study signaling exchange to support bistatic mode of operation for D1T2-A1 scenario.

3. Conclusion

Besides the monostatic mode, the bistatic mode has been defined by RAN1. This paper tries to discuss whether bistatic mode in both topology 1 and topology 2 is supported or not. And we propose:

Proposal 1: RAN3 study signaling exchange to support bistatic mode of operation for D1T1-A1 scenario.

Proposal 2: RAN3 study signaling exchange to support bistatic mode of operation for D1T2-A1 scenario.