3GPP TSG-RAN WG3 Meeting #126	R3-247478

Orlando, USA, 18 – 22 November 2024

Agenda item:	9.2

Source:	Nokia, Huawei, Qualcomm Incorporated, CMCC, CBN, China Broadnet

Title:	Correction of MBS PDCP packets discard

Document for:	Discussion and Decision

# 1	Discussion

At the last RAN3#125 the unresolved issue of PDCP discard of broadcast packets related to MBS broadcast resource sharing release 18 feature was discussed again without concluding.

The issue arises for both MOCN case and multiple cell ID case when there is a switch of MBS shared tunnel.

At RAN3#125, the CR in [1] proposed solutions which were not agreed.

The proposed solutions were:

- For MOCN case: when the gNB-CU switches the NG-U tunnel from PLMN1 to PLMN2, the gNB-CU ensures PDCP SN continuity i.e. it sets the PDCP SN of the first packet received over the new NG-U 2 and to be delivered over the F1-U tunnel with the PDCP SN of the next packet to be delivered to the UE (PDCP SN continuity), so that the UE will not discard the packets but instead re-initialize its PDCP SN with the received value.

NOTE 1: It is expected that the gNB CU-UP supports PDCP SN continuity after establishment of shared NG-U resources with 5GC2.

- For multiple-cell ID case, it was proposed that the DU may use different MRB configuration in order to trigger a release/add MRB action for the UEs which would re-establish the PDCP.

NOTE 2: The gNB-DU1/2 may use different MRB configuration after the shared NG-U and F1-U path change, to enable the UEs to release and add related MRB to avoid unexpected packets discard.

It was commented that the note 2 does not work because it relies on UE implementation choice and therefore the CR in [1] was noted altogether without discussion on note 1.

Actually, the proposal in note 1 is very different than note 2. Because the MBS broadcast does not require the same synchronization over the 5GC as multicast, the PDCP SN set by the gNB-CU is not controlled by the QoS flow SN of the MB-UPF and the gNB-CU can freely change the PDCP SN at the path switch into the SN value of the next packet to be delivered.

Therefore, the proposal on Note 1 can work.

Moreover, similar solution could be applied to multiple cell ID case i.e. if the gNB-CU of PLMN2 is made aware of the SN of next packet to be delivered by the gNB-DU there is no reason why the gNB-CU of PLMN2 could not initialize or re-initialize as well the PDCP SN to this value over the new F1-U tunnel of PLMN2.

Proposal 1: agree the PDCP SN continuity to solve the PDCP discard issue for both MOCN and Multiple Cell ID case and the corresponding CRs in [2] and [3].

# 2	Conclusion and Proposals

This paper has continued the discussion on PDCP discard issue initiated by tdoc [1] and makes the following proposal to solve it:

Proposal 1: agree the PDCP SN continuity to solve the PDCP discard issue for both MOCN and Multiple Cell ID case and the corresponding CRs in [2], [3].

# 3	References

- R3-244097, Correction on PDCP packets discard in case of MBS broadcast path update

- R3-247479, Correction on PDCP packets discard in case of MBS broadcast path update, TS 38.401

- R3-247481, Correction on PDCP packets discard in case of MBS broadcast path update, TS 38.473