3GPP TSG-RAN WG3 Meeting #126	R3-247535

Orlando, USA, 18th–22nd November, 2024

Title:	AI/ML support for NR-DC

Source:	Ericsson

Agenda item:	11.4

Document Type:	Discussion and agreement

# Introduction

At the RAN3#125-bis meeting, normative work for AI/ML assisted mobility optimization for NR-DC was started based on the conclusions captured in the TR 38.743. The following agreement and “to-be-continued” were captured in the meeting minutes:

As a starting point, SN Addition and MN-initiated SN Change procedures are considered to be enhanced to trigger the collection of UE performance from SN to MN. This assumes that the involved gNBs have an existing Data Collection Reporting Initiation procedure in place configuring the collection of UE performance.

Whether to use the Data Collection Reporting Initiation procedure to configure the collection of UE performance at SN side or whether to reuse existing DC procedure to configure the collection of UE performance at SN side and a new procedure for the reporting of UE performance needs further discussion.

In this contribution, we discuss the consequences of the above agreement and other aspects related to the enhancements needed in NG-RAN interfaces to support this use case.

# Discussion

## Procedures to support mobility optimization for NR-DC

According to the agreement captured in the RAN3#125-bis meeting minute, the baseline for normative work for AI/ML assisted mobility optimization for NR-DC consists of enhancing SN Addition and MN-initiated SN Change procedures to trigger collection of measured UE performance. It was furthermore agreed that there is an assumption that both MN and SN have an existing Data Collection Reporting context in place.

This assumption points to a tacit agreement of using the Data Collection Reporting Initiation procedure for configuring the collection of measured UE performance and the Data Collection Reporting procedure for reporting the measured UE performance. Therefore, given the above agreement during the last meeting, the following FFS is already resolved:

Whether to use the Data Collection Reporting Initiation procedure to configure the collection of UE performance at SN side or whether to reuse existing DC procedure to configure the collection of UE performance at SN side and a new procedure for the reporting of UE performance needs further discussion.

Proposal 1:  RAN3 confirms that the Data Collection Reporting Initiation procedure and the Data Collection Reporting procedure are used to configure and report UE performance for AI/ML assisted mobility optimization for NR-DC.

## Enhancements to support mobility optimization for NR-DC

Given the conclusions captured in the TR 38.743, we would like to focus the discussion on two scenarios:

- a UE is in single connectivity, and the serving gNB infers the decision to initiate SN Addition

- a UE is in NR-DC, and the MN infers the decision to initiate SN Change

The following types of enhancements are needed to support the scenarios above:

- Enhancements to assist inference at the MN (future MN or current MN, depending on the scenario)

- Enhancements to provide feedback to the MN

- Additional data collection for NR-DC

To assist inference at the MN (both if it is a “future MN” or a “current MN”), it is possible to reuse to a large extent the Rel-18 signaling design established to support AI/ML assisted use cases. As already agreed in the RAN3#125-bis meeting, we may reuse the Data Collection Reporting Initiation procedure and Data Collection Reporting procedure to allow an MN (future or current) to request and obtain from a neighbor gNB (acting as SN) the reporting of UE performance.

In Rel-18, the collection of UE performance is triggered by the inclusion of the Data Collection ID IE in the XnAP HANDOVER REQUEST message. As described in the semantics description for this IE, when a Data Collection Reporting Initiation is executed, a Data Collection Reporting context is established. The definition of the Data Collection ID IE is shown below:

| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Together with NG-RAN node2 Measurement ID, identifies a Data Collection Reporting context. |
| --- | --- | --- | --- | --- |
| NG-RAN node2 Measurement ID | M |  | INTEGER (1..4095,...) | Together with NG-RAN node1 Measurement ID, identifies a Data Collection Reporting context. |

If the Data Collection ID IE is included in the HANDOVER REQUEST message, the target gNB understands that it should start collecting UE performances as configured via the previous Data Collection Reporting Initiation procedure (which corresponds to the Data Collection ID). For NR-DC, a similar approach can be used, both for SN Addition and MN-initiated SN Change procedures, as described below.

- The (future or current) MN requests UE performance to a neighbour gNB, which is a potential (new or target) SN. The Data Collection Reporting context is set up, and both the (future or current) MN and the potential (new or target) SN are aware of the Data Collection ID;

- The (future or current) MN infers the decision to select the neighbour gNB as (new or target) SN;

- The (future or current) MN includes the Data Collection ID IE in the S-NODE ADDITION REQUEST message, and the (new or target) SN understands that it should start collecting the UE performance previously requested by the MN.

Proposal 2:  For AI/ML assisted mobility optimization for NR-DC, add the Data Collection ID IE in the S-NODE ADDITION REQUEST message to signal the SN to start collecting UE performance at the SN as requested by the MN.

Another part of the Rel-18 solution concerns the conditions that the target gNB needs to monitor to stop the collection of UE performance requested by the source gNB. For NR-DC, similar conditions can be defined, so that the SN takes them into account to terminate the collection of UE performance requested by the MN. In our view, at least the following conditions should be considered:

- Expiration of a timer (e.g. reporting time interval), after successful SN Addition;

- the SN is removed from NR-DC (MN initiated or SN initiated).

Proposal 3:  For AI/ML assisted mobility optimization for NR-DC, define the following conditions for the SN to stop collection of UE performance requested by the MN: 1) exliration of a timer, after successful SN Addition; 2) SN removal (MN initiated or SN initiated).

Furthermore, in the Rel-18 solution, a condition is specified according to which the target gNB should stop collecting UE performance when the “UE is handed over to another cell”. UE performance is currently reported from target gNB to source gNB without an explicit indication of the cell they refer to. This is good enough for the Handover case, since the target cell is explicitly signalled in the HANDOVER REQUEST and the target gNB knows exactly for which cell it should report the UE performance. Also, since the collection should stop when UE is handed over to another cell, there is only one cell the UE performance can refer to. With some enhancements, the existing signaling design can be reused for NR-DC as we discuss below.

First, the reported UE performance should be extended to include the PSCell used by the UE, so that it is clear to the MN as to which cell the UE performance refers. The MN can use this information, e.g., to understand whether its assumptions to select the SN were optimal or not.

Proposal 4:  For AI/ML assisted mobility optimization for NR-DC, extend the UE Performance IE to include the PSCell serving the UE in the SN.

Second, to extend to NR-DC the logic used for mobility in single connectivity, we propose to specify that SN stops collection of UE performances in the cell used as PSCell when the PSCell is changed.

Proposal 5:  For AI/ML assisted mobility optimization for NR-DC, the SN stops collection of data requested by the MN at PSCell change.

It is reasonable to think that the MN has inferred the action of SN Addition (or SN Change) assuming that adding (or changing) the SN would bring some benefits to the UE. We think that it is good if the MN informs the SN about some of the assumptions made, so that the SN can try to provide the best possible service to the UE.

For example, when an MN infers the action to add or change SN, it can assume that a specific PSCell will be selected for the UE. This is obvious because the UE performance feedback received in the past for each UE is specific to a PSCell (described in Proposal 4). Hence, if UEs were served well by an SN, on one or more PSCells, the MN is more likely to predict that the SN and one or more specific PSCells, are the best options for a DC configuration at the UE.

Another example is that when the MN carries out an AI/ML generated SN Addition, it may request to the SN the establishment of SN-terminated bearers. However, the inferred decision can be based on the assumption that specific QoS flows will be served by the SN (as SCG). This is not in line with what could happen today, where an SN may decide that all the QoS flows for which the MN requests SN-terminated bearers, shall be served by MCGs. Therefore, when the MN derives an AI/ML based request to the SN for some resources for SN terminated bearers, it should tell the SN that this request was made assuming that SN could serve some or all of the QoS flows as SCGs. After receiving this notification, the SN is left free to decide whether to serve the QoS flows as MCG or SCG, but it can use the information and try to accommodate the MN assumptions, if possible.

Therefore, we propose that, at SN Addition, the MN could tell the SN that the MN assumes that a specific cell, e.g. CellA, is selected as PSCell. The SN is free to decide independently, but it can try to select CellA as PSCell if possible. It is also proposed that the MN indicates to the SN which are the assumptions in terms of the QoS Flows the SN will have to serve as SCG.

Proposal 6:  Enhance dual connectivity procedures (e.g., SN addition, MN-initiated SN change) to indicate MN assumptions to be considered by the SN (e.g., which cell the MN assumed as PSCell, which QoS flows the MN assumed to be served as SCGs).

## Additional data collection for NR-DC

To better support inference at the (future or current) MN, the Data Collection Reporting Initiation and Data Collection Reporting procedures could also be extended, e.g., to collect and report additional non-UE Associated metrics that can be used as inputs to infer the next SN addition or SN change.

An example of non-UE associated metric that can be advantageous to collect is the propagation delay over the F1 interface that affects an SN/PSCell. Such latency is often the largest delay component in the traffic propagation delay over an NR-DC leg, and it is mainly dependent on the transport network performance. It should also be pointed out that this performance is variable, namely F1 delays vary dynamically depending on transport network conditions.

It is known that NR-DC configurations do not deliver a good performance if the traffic propagation delay between the two legs of the NR-DC configuration is unbalanced (i.e., very different delays between the MCG and SCG branches and/or very high delays on both branches). It is therefore very useful for an MN to understand what is the F1 propagation delay of the SN/PSCell to select in the future those target SN/PSCell that are affected by more favourable propagation delays.

Proposal 7:  For AI/ML assisted mobility optimization for NR-DC, add support for requesting and reporting of non-UE associated metrics, e.g., the F1 propagation delay for a SN/PSCell.

# Conclusions

Based on the discussion in this paper, we derived the following proposals:

Proposal 1:  RAN3 confirms that the Data Collection Reporting Initiation procedure and the Data Collection Reporting procedure are used to configure and report UE performance for AI/ML assisted mobility optimization for NR-DC.

Proposal 2:  For AI/ML assisted mobility optimization for NR-DC, add the Data Collection ID IE in the S-NODE ADDITION REQUEST message to signal the SN to start collecting UE performance at the SN as requested by the MN.

Proposal 3:  For AI/ML assisted mobility optimization for NR-DC, define the following conditions for the SN to stop collection of UE performance requested by the MN: 1) exliration of a timera time interval, after successful SN Addition; 2) SN removal (MN initiated or SN initiated).

Proposal 4:  For AI/ML assisted mobility optimization for NR-DC, extend the UE Performance IE to include the PSCell serving the UE in the SN.

Proposal 5:  For AI/ML assisted mobility optimization for NR-DC, the SN stops collection of data requested by the MN at PSCell change.

Proposal 6:  Enhance dual connectivity procedures (e.g., SN addition, MN-initiated SN change) to indicate MN assumptions to be considered by the SN (e.g., which cell the MN assumed as PSCell, which QoS flows the MN assumed to be served as SCGs).

Proposal 7:  For AI/ML assisted mobility optimization for NR-DC, add support for requesting and reporting of non-UE associated metrics, e.g., the F1 propagation delay for a SN/PSCell.

A TP mirroring the proposals above is presented in R3-247536