3GPP TSG-RAN WG3 Meeting #126	R3-247542

Orlando, USA, 18th - 22th November, 2024

Agenda Item:	9.2

Source:	ZTE Corporation, CMCC, China Telecom

Title:	Discussion on Location Reporting Control for UPF Relocation

Document for:	Discussion

1  Introduction

In last RAN3 meeting [1], one issue was raised that the location reporting control mechanism over NG should be enhanced due to the UPF relocation, i.e., adding a new event type for the existing Location Reporting Request Type IE.

However, there was no consensus on this issue in last meeting, and some companies wanted to check the feasibility of the scenario and suggested involving other working group to get the whole picture of the potential solution.

# 25_LocationReport - Check the issue based on the scenario and check whether any coordination with SA2 is needed? (moderator - ZTE) Summary of offline disc R3-244713 noted For NTN case, there could be multiple UPFs to serve the UEs within the NTN cell. With its own movement, the UE may move out of the coverage of the original UPF. Then, the UE needs to re-select the UPF. To be continued...

In this contribution, we provide more clarification on the scenario and the potential solution to this issue. In addition, the corresponding CRs and the draft LS to SA2 are also given.

2  Discussion

2.1  Clarification on the scenario

In the real deployment, the coverage of NTN cell is quite large, and there are multiple TACs within one NTN cell. Meanwhile, there could be multiple UPFs serving the UEs within one NTN cell.

In this case, the movement of the UE may lead to the change of the TAC, if the AMF want to re-select the UPF, it should be aware of the specific TA where the UE is currently located. While, in current Location Reporting Control procedure, there is no TAC-level event type in the Location Reporting Request Type IE, which means that the gNB cannot reporting the ULI including the UE Location Derived TAC in NR NTN IE to the AMF when the TA where the UE is located is changed.

------------------------------------------------TS 38.413-----------------------------------------------------

9.3.1.65	Location Reporting Request Type

This IE indicates the type of location request to be handled by the NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Event Type | M |  | ENUMERATED (direct, change of serving cell, UE presence in the area of interest, stop change of serving cell, stop UE presence in the area of interest, cancel location reporting for the UE, …) |  | - |  |
| Report Area | M |  | ENUMERATED (cell, …) |  | - |  |
| Area of Interest List |  | 0..1 |  |  | - |  |
| >Area of Interest Item |  | 1..<maxnoofAoI> |  |  | - |  |
| >>Area of Interest | M |  | 9.3.1.66 |  | - |  |
| >>Location Reporting Reference ID | M |  | 9.3.1.76 |  | - |  |
| Location Reporting Reference ID to be Cancelled | C- ifEventTypeisStopUEPresinAoI |  | Location Reporting Reference ID 9.3.1.76 |  | - |  |
| Additional Location Information | O |  | ENUMERATED (Include PSCell, ...) |  | YES | ignore |
| Additional Cancelled Location Reporting Reference ID List |  | 0..1 |  |  | YES | reject |
| >Additional Cancelled Location Reporting Reference ID Item |  | 1..<maxnoofAoIMinusOne> |  |  | - |  |
| >>Location Reporting Reference ID to be Cancelled | M |  | Location Reporting Reference ID 9.3.1.76 |  | - |  |

------------------------------------------------TS 38.413-----------------------------------------------------

Observation: When the TA UE located is changed, the gNB should send the updated ULI to the AMF to re-select the UPF.

2.2  Potential solution

After checking the existing NGAP specification, there are two options to solve above issue.

For Option 1, A new event type could be added in the Location Reporting Request Type. In this case, if the gNB detects the UE’s current TA has been changed, it can send the updated ULI (with the UE Location Derived TAC in NR NTN IE to the AMF) via the LOCATION REPORT message , and the AMF is able to re-select the appropriate UPF for the UE.

For Option 2, When the gNB detects the UE’s current TA has been changed, it can send the updated ULI via other subsequent UL NGAP message, and the AMF is able to select the appropriate UPF for the UE.

Both the options may solve the ULI reporting issue, while the Option 2 seems to be a kind of implementation, and the gNB cannot report the updated ULI in time. If there is no subsequent UL NGAP message from the gNB, the updated ULI cannot even be reported. In this case, the UPF cannot be re-selected timely, and the UE’s service could be interrupted.Therefore, introducing a new event type is more appropriate solution to address the ULI reporting issue due to the change of serving TAC.

Proposal 1: A new event type should be introduced in the Location Reporting Control procedure to support the ULI reporting triggered by the change of serving TAC.

2.3  Contact with SA2

In last meeting, some companies commented that this issue should be discussed by SA2 since it is related to the UPF relocation. While, as the scenario of this issue has been clarified in RAN3 and there could be standard impact over NG, RAN3 should send a LS to SA2 to confirm the feasibility of UPF relocation issue for NR NTN.

Proposal 2: RAN3 should send a LS to SA2 to confirm the feasibility of the UPF relocation issue for NR NTN.

3  Conclusion

Observation: When the TA UE located is changed, the gNB should send the updated ULI to the AMF to re-select the UPF.

Proposal 1: A new event type should be introduced in the Location Reporting Control procedure to support the ULI reporting triggered by the change of serving TAC.

Proposal 2: RAN3 should send a LS to SA2 to confirm the feasibility of the UPF relocation issue for NR NTN.

The draft LS to SA2 in given in the Annex.

The corresponding NGAP CRs for Option 1 is given in [2] and [3].

References

- RAN3#125 Chair Notes

- R3-247543, Correction on Location Reporting Control procedure for NR NTN_R17, ZTE Corporation, CMCC, China Telecom

- R3-247544, Correction on Location Reporting Control procedure for NR NTN_R18, ZTE Corporation, CMCC, China Telecom

Annex

3GPP TSG-RAN WG3 Meeting #126	R3-24xxxx

Orlando, USA, 14th – 18th Oct 2024

Title:	[Draft] LS on feasibility of UPF relocation issue for NR NTN

Response to:	N/A

Release:	Rel-17

Work Item:	NR_NTN_solutions

Source:	ZTE (to be RAN3)

To:	SA2

Cc:	RAN2

Contact Person:

      - Name:	Jiren Han

      - E-mail Address:	han.jiren@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

- Overall Description:

In the real deployment, the coverage of NTN cell is quite large, and there are multiple TACs within one NTN cell. Meanwhile, there could be multiple UPFs serving the UEs within one NTN cell.

In this case, the movement of the UE may lead to the change of the TAC, if the AMF want to re-select the UPF, it should be aware of the specific TA where the UE is currently located. While, in current Location Reporting Control procedure, there is no TAC-level event type in the Location Reporting Request Type IE, which means that the gNB cannot reporting the ULI including the UE Location Derived TAC in NR NTN IE to the AMF in time when the TA where the UE is located is changed. Consequently, the AMF cannot obtain the updated TAC and re-select the appropriate UPF for the UE.

To address the above issue, RAN3 provides two options as below.

Option 1: A new event type could be added in the Location Reporting Request Type. In this case, if the gNB detects the UE’s current TA has been changed, it can send the updated ULI (with the UE Location Derived TAC in NR NTN IE to the AMF) via the LOCATION REPORT message , and the AMF is able to re-select the appropriate UPF for the UE.

Option 2, When the gNB detects the UE’s current TA has been changed, it can send the updated ULI via other subsequent UL NGAP message, and the AMF is able to select the appropriate UPF for the UE.

However, the Option 2 seems to be a kind of implementation, and the gNB cannot report the updated ULI in time. If there is no subsequent UL NGAP message from the gNB, the updated ULI cannot even be reported. In this case, the UPF cannot be re-selected timely, and the UE’s service could be interrupted.Therefore, introducing a new event type is more appropriate solution to address the ULI reporting issue due to the change of serving TAC.

2. Actions:

To SA2 group.

ACTION: 	RAN3 kindly asks SA2 to check the feasibility of UPF relocation issue due to change of serving TAC and evaluate the potential solutions provided by RAN3.

3. Date of Next RAN3 Meetings:

TSG RAN WG3 Meeting #127     17th – 21th, Feb. 2025 		 Athens, Greece