3GPP TSG-RAN WG3 Meeting #126	R3-247547

Orlando, USA, 18th - 22th Nov 2024

Agenda Item:	14.3

Source:	ZTE Corporation

Title:	Further discussion on support of regenerative payload

Document for:	Discussion

1  Introduction

In previous meetings, the discussion on support of regenerative payload was discussed, and the following agreements were reached[1].

NG Interface management Issue: Option 2 NG Removal/Setup can be used for Regenerative Payload NG interface management. RAN CONFIGURATION UPDATE procedure could be reused to support NG suspend/Resume, by introducing suspend/resume indicators to avoid frequent TNL disconnection and re-connection? Inactive UE support: RAN3 assumes that current mechanisms to support UEs performing RRC connection resume and RRC connection re-establishment may be reused in regenerative payload architecture, subject to implementation and deployment. Feeder link switch issue: RAN3 assumes that during the feeder link switch, AMF is not changed for the UEs. Whether there has any standard impact needs to be further checked. The current mechanisms including, e.g. multiple TNLA association, are sufficient for TNL management during feeder link switch. Supported TAI list of the on-board gNB: The supported TAIs of an on-board gNB could be provided to AMF by OAM/pre-configuration.

In this contribution, we provide our further views on the support of regenerative payload for NTN in Rel-19.

2  Discussion

2.1  NG connection management

In RAN3#125 meeting, it has been agreed that the NG Removal/Setup mechanism can be used for NG interface management, and another option, NG Suspend/Resume mechanism can be further discussed.

For the NG Suspend/Resume mechanism, when the gNB decides to leave the service area of AMF based on the ephemeris information, it shall trigger the existing NGAP procedure (e.g. RAN Configuration Update procedure) to the AMF to suspend the NG interface instead of removing it. In this case, the gNB and the AMF shall store the UE context and the configuration information related to the NG interface. When the gNB re-enters the service area of AMF, the stored UE context and configuration information could be reused, and the NG interface shall be resumed.

Considering the NGAP signalling, two NGAP procedures need to be handled for NG Setup/Removal mechanism, i.e., the NG Setup procedure and NG Removal procedure, and the NG Removal procedure is a totally new NGAP procedure. While, for NG Suspend/Resume procedure, only one NGAP procedure need to be handled, e.g, the existing NGAP RAN Configuration Update procedure. Therefore, from the prospective of standard impact on NG interface, no new procedure needs to be introduced for the NG Suspend/Resume procedure.

Considering the energy consumption, for the NG Suspend/Resume procedure, since the gNB and AMF shall store the UE context and configuration information, it shall consume some energy for storage. While, with the development of storage technology, after 3 to 5 years, when the Regenerative payload is officially deployed, the storage energy cost shall be much less, and the storage is not a big issue possibly.

In addition, with the NG Suspend/Resume procedure, the NG interface can be timely resumed and the UE can attach the network quickly, which can bring the better user’s experience. In some specific circumstances, for the satellite whose moving orbit is almost vertical to equator, the satellite will fly back to the AMF easily, then the storage time will be not much.

Proposal 1: In addition to the NG Setup/Removal mechanism, the NG Suspend/Resume mechanism should be applied for the management of NG connection.

2.2  LS from SA2

In [2], SA2 identified that the LS from RAN3[3] referred only to “5GS” while in the last paragraph also referred to “AMF/MME” and noticed that the RAN WI “Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3” (RP-242397) did not contain objectives for Regenerative-based Satellite Access for E-UTRAN/EPC. In addition, they inform RAN3 that the conclusions for FS_5GSAT_Ph3 for “KI#1: Support of Regenerative-based satellite access” without Store and Forward in clause 8.1 of TR 23.700-29 apply to both E-UTRAN/EPS and NG-RAN/5GS. And the conclusions for FS_5GSAT_Ph3 for “KI#2: Support of Store and Forward Satellite operation” in clause 8.2 of TR 23.700-29 apply only to E-UTRAN/EPS.

Based on the LS form SA2, there is some ambiguity on the understanding of RAN3’s previous reply LS. And the reason behind is that SA2 has studied the Rel-19 Regenerative-based satellite access in one SI, including both NR NTN and IoT NTN. While, in RAN3, NR NTN and IoT NTN are discussed separately in different Rel-19 WIs.

The previous reply LS from RAN3[3] was sent in RAN3#124 meeting, and the IoT NTN WI was not started at that time. Since some questions shall be discussed in IoT NTN WI, RAN3 provided some feedback on SA2’s question, especially from NR NTN’s prospective. While, for the mapped cell ID handling, it could be a common understanding of RAN3 that it could be handled as in previous releases for both NR NTN and IoT NTN WI, which could be solved by operator’s implementation.

With respect to NG connection management: RAN3 has just begun to discuss the options, and there is no conclusion yet. RAN3 will continue discussing this issue in Rel-19 and inform SA2 on the conclusion. With respect to IP address changes due to soft feeder link switch: RAN3 confirms that the gNB IP address change due to soft feeder link switch is supported by the existing NGAP procedures. With respect to Mapped Cell ID handling: RAN3 also thinks that the AMF/MME can treat the Mapped Cell ID as in previous releases.

Considering the information from SA2, “the conclusions for FS_5GSAT_Ph3 for “KI#1: Support of Regenerative-based satellite access” without Store and Forward in clause 8.1 of TR 23.700-29 apply to both E-UTRAN/EPS and NG-RAN/5GS”, it is related to the working scope of IoT NTN WI in RAN, i.e., whether RAN3 to support the eNB as regenerative payload on board without Store and Forward satellite operation in Rel-19 IoT NTN WI. And the details should be discussed in the IoT NTN WI.

Proposal 2: The issue raised by SA2 in the LS is related to the working scope of Rel-19 IoT NTN WI, which should be discussed in the IoT NTN WI.

3  Conclusion

Proposal 1: In addition to the NG Setup/Removal mechanism, the NG Suspend/Resume mechanism should be applied for the management of NG connection.

Proposal 2: The issue raised by SA2 in the LS is related to the working scope of Rel-19 IoT NTN WI, which should be discussed in the IoT NTN WI.

The CR to support the NG Suspend/Resume mechanism in given in the Annex.

References

- RAN3#125bis Chairman Notes

- R3-247016, Reply LS on Support of Regenerative-based Satellite Access, SA2

- R3-243954, Reply LS on Support of Regenerative-based Satellite Access, RAN3

Annex

TP to BL CR for TS38.413 on NG Suspend/Resume mechanism for regenerative payload

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

8.7.2	RAN Configuration Update

8.7.2.1	General

The purpose of the RAN Configuration Update procedure is to update application level configuration data needed for the NG-RAN node and the AMF to interoperate correctly on the NG-C interface. This procedure does not affect existing UE-related contexts, if any. The procedure uses non UE-associated signalling.

8.7.2.2	Successful Operation

Figure 8.7.2.2-1: RAN configuration update: successful operation

The NG-RAN node initiates the procedure by sending a RAN CONFIGURATION UPDATE message to the AMF including an appropriate set of updated configuration data that it has just taken into operational use. The AMF responds with a RAN CONFIGURATION UPDATE ACKNOWLEDGE message to acknowledge that it successfully updated the configuration data. If an information element is not included in the RAN CONFIGURATION UPDATE message, the AMF shall interpret that the corresponding configuration data is not changed and shall continue to operate the NG-C interface with the existing related configuration data.

If the Supported TA List IE is included in the RAN CONFIGURATION UPDATE message, the AMF shall overwrite the whole list of supported TAs and the corresponding list of supported slices for each TA, and use them for subsequent registration area management of the UE.

If the Configured TAC Indication IE set to "true” is included for a Tracking Area contained in the Supported TA List IE in the RAN CONFIGURATION UPDATE message, the AMF may take it into account to optimise NG-C signalling towards this NG-RAN node.

If the Global RAN Node ID IE is included in the RAN CONFIGURATION UPDATE message, the AMF shall associate the TNLA to the NG-C interface instance using the Global RAN Node ID.

If the RAN CONFIGURATION UPDATE message includes the NG-RAN TNL Association to Remove List IE, the AMF shall, if supported, initiate removal of the TNL association(s) indicated by NG-RAN TNL endpoint(s) and AMF TNL endpoint(s) if the TNL Association Transport Layer Address at AMF IE is present, or the TNL association(s) indicated by NG-RAN TNL endpoint(s) if the TNL Association Transport Layer Address at AMF IE is absent:

- 	if the received TNL Association Transport Layer Address IE includes the Port Number IE, the NG-RAN TNL endpoint is identified by the Endpoint IP Address IE and the Port Number IE. Otherwise, the NG-RAN TNL endpoints correspond to all NG-RAN TNL endpoints identified by the Endpoint IP Address IE and any port number(s).

-    if the received TNL Association Transport Layer Address at AMF IE includes the Port Number IE, the AMF TNL endpoint is identified by the Endpoint IP Address IE and the Port Number IE. Otherwise, the AMF TNL endpoints correspond to all AMF TNL endpoints identified by the Endpoint IP Address IE and any port number(s).

If the RAN CONFIGURATION UPDATE message includes the RAN Node Name IE, the AMF may store it or update this IE value if already stored, and use it as a human readable name of the NG-RAN node. If the RAN CONFIGURATION UPDATE message includes the Extended RAN Node Name IE, the AMF may store it or update this IE value if already stored, and use it as a human readable name of the NG-RAN node and shall ignore the RAN Node Name IE if also included.

If the NB-IoT Default Paging DRX IE is included in the RAN CONFIGURATION UPDATE message, the AMF shall overwrite any previously stored NB-IoT default paging DRX value for the NG-RAN node.

If the RAT Information IE is included in the RAN CONFIGURATION UPDATE message, the AMF shall handle this information as specified in TS 23.502 [10].

If the NID IE within the NPN Support IE is included within a Broadcast PLMN Item IE in the RAN CONFIGURATION UPDATE message, the AMF shall consider that the NG-RAN node supports the indicated S-NSSAI(s) for the corresponding tracking area code for the SNPN identified by the PLMN Identity IE and the NID IE.

If the TAI NSAG Support List IE is included in the Broadcast PLMN Item IE in the RAN CONFIGURATION UPDATE message, the AMF shall, if supported, use this information as specified in TS 23.501 [9].

If the NTN Suspend Indication IE is included in the RAN CONFIGURATION UPDATE message, the AMF shall, if supported, suspend the NG interface.

If the NTN Resume Indication IE is included in the RAN CONFIGURATION UPDATE message, the AMF shall, if supported, resume the NG interface.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.2.6.4	RAN CONFIGURATION UPDATE

This message is sent by the NG-RAN node to transfer updated application layer information for an NG-C interface instance.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| RAN Node Name | O |  | PrintableString (SIZE(1..150, …)) |  | YES | ignore |
| Supported TA List |  | 0..1 |  | Supported TAs in the NG-RAN node. | YES | reject |
| >Supported TA Item |  | 1..<maxnoofTACs> |  |  | - |  |
| >>TAC | M |  | 9.3.3.10 | Broadcast TAC | - |  |
| >>Broadcast PLMN List |  | 1 |  |  | - |  |
| >>>Broadcast PLMN Item |  | 1..<maxnoofBPLMNs> |  |  | - |  |
| >>>>PLMN Identity | M |  | 9.3.3.5 | Broadcast PLMN | - |  |
| >>>>TAI Slice Support List | M |  | Slice Support List 9.3.1.17 | Supported S-NSSAIs per TAC, per PLMN or per SNPN. | - |  |
| >>>>NPN Support | O |  | 9.3.3.44 | If the NID IE is included, it identifies a SNPN together with the PLMN Identity IE. | YES | reject |
| >>>>Extended TAI Slice Support List | O |  | Extended Slice Support List 9.3.1.191 | Additional Supported S-NSSAIs per TAC, per PLMN or per SNPN. | YES | reject |
| >>>>TAI NSAG Support List | O |  | 9.3.1.238 | NSAG information associated with the slices per TAC, per PLMN or per SNPN. | YES | ignore |
| >>Configured TAC Indication | O |  | 9.3.3.50 |  | YES | ignore |
| >>RAT Information | O |  | 9.3.1.125 | RAT information associated with the TAC of the indicated PLMN(s). | YES | reject |
| Default Paging DRX | O |  | Paging DRX 9.3.1.90 |  | YES | ignore |
| Global RAN Node ID | O |  | 9.3.1.5 |  | YES | ignore |
| NG-RAN TNL Association to Remove List |  | 0..1 |  |  | YES | reject |
| >NG-RAN TNL Association to Remove Item |  | 1..<maxnoofTNLAssociations> |  |  | - |  |
| >>TNL Association Transport Layer Address | M |  | CP Transport Layer Information 9.3.2.6 | Transport layer address of the NG-RAN node. | - |  |
| >>TNL Association Transport Layer Address at AMF | O |  | CP Transport Layer Information 9.3.2.6 | Transport layer address of the AMF. | - |  |
| NB-IoT Default Paging DRX | O |  | 9.3.1.137 |  | YES | ignore |
| Extended RAN Node Name | O |  | 9.3.1.193 |  | YES | ignore |
| NTN Suspend Indication | O |  | ENUMERATED (true, ...) | Indicates the NG interface is suspended. This IE only applies to NTN. | YES | ignore |
| NTN Resume Indication | O |  | ENUMERATED (true, ...) | Indicates the NG interface is resumed. This IE only applies to NTN. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofTACs | Maximum no. of TACs. Value is 256. |
| maxnoofBPLMNs | Maximum no. of Broadcast PLMNs. Value is 12. |
| maxnoofTNLAssociations | Maximum no. of TNL Associations between the NG-RAN node and the AMF. Value is 32. |

<<<<<<<<<<<<<<<<<<<<End of Changes >>>>>>>>>>>>>>>>>>>>

ASN.1 part will be added later