3GPP TSG-RAN WG3 Meeting #126	R3-247568

Orlando, USA,  18 – 22 November, 2024

Agenda item:	8.1

Source:	Nokia

Title:	Discussion on LS from SA5 on renewable energy based LBO

Document for:	Discussion and Decision

# 1	Introduction

We here analyse LS from SA5 received in [1].

# 2	Discussion

We assume in this discussion that Distributed SON LBO (Load Balancing Optimization) feature mentioned in [1] refers to the MLB (Mobility Load Balancing) feature described in clause 15.5.1 of TS 38.300, and in particular clause 15.5.1.4 (statement “All automatic changes on the HO and/or reselection parameters must be within the range allowed by OAM”) from which the OAM requirements are derived. For connected mode UEs, this clause describes the feature enabled via the XnAP Mobility Settings Change procedure.

MLB is in current specification used to balance load, and important parameters for MLB used by the RAN today are available capacity or load in source cell vs. candidate target cells. The Mobility Settings Change (MSC) procedure is primarily intended for use in intra-frequency scenario to create an imbalance in terms of radio conditions (handover triggers) for the purpose of balancing load. In the proposal from SA5, MSC could be used for offloading towards a cell or gNB that is fully powered by renewable energy. From an XnAP signalling perspective, in a simple scenario, a gNB that via configuration becomes aware that it is fully powered by renewable energy could today trigger the XnAP Mobility Settings Change procedure and negotiate with its neighbours extension of its coverage within the limits provided by the OAM. However more complex scenarios where e.g. two neighbour nodes are both fully powered by renewable energy, or one node is using MSC for legacy load balancing while the neighbour node will use the MSC procedure in order to trigger offloading for energy saving is not supported in current standard.

Offloading for the purpose of energy saving is in current specification supported e.g. for the case of cell switch-off scenario, where handovers are triggered with the following cause (TS 38.423):

| Switch Off Ongoing | The reason for the action is an ongoing switch off i.e. the concerned cell will be switched off after offloading and not be available. It aides the receiving NG-RAN node in taking subsequent actions, e.g. selecting the target cell for subsequent handovers. |
| --- | --- |

However, AI/ML-based energy saving introduced in Rel-18 also currently provides support for offloading actions using an Energy Cost metric signalled over Xn. Work on Energy Cost for split architecture is also part of Rel-19.

The standardized Energy Cost metric is as per SA5 and RAN3 specification today normalized with regards to minimum and maximum energy consumption within an area. Proposal to generalize this normalization based on cost, and not limit normalization to the consumption aspect only, was not agreed in Rel-18. Still, such generalization could be considered in future work and in this way result in metric(s) reflecting more explicitly e.g. carbon emission, carbon emission efficiency and renewable energy as mentioned by SA5.

The current (Rel-18, Rel-19) Energy Cost metric definition is therefore based on an assumption that energy consumption levels in different nodes has comparable cost. This may in practice mean that all nodes are connected to the energy grid, but that doesn’t seem to be SA5’s assumption in the LS [1] nor in TR 28.880 v.1.1.0 clause 5.7. Furthermore, connectivity to the energy grid could also mean that the gNB A (drawing in the LS), which is both an energy producer and an energy consumer, would be able to inject produced excess energy into the grid where it would be optimally consumed. In that case, RAN operation could continue focusing on minimizing the overall energy consumption in the network as well as in the UEs, which has been the paradigm for work on energy saving and energy efficiency in earlier releases and is still the current paradigm used for the ongoing Rel-19 work, as e.g. illustrated by the following common understanding captured during Rel-18 work:

It’s the common understanding that AI/ML based energy saving aims to optimize the overall energy efficiency of the coverage of a gNB and its neighbours.

RAN3 would therefore need SA5 to clarify the network topologies where some NG-RAN nodes are connected to the energy grid and some are not, and additionally confirm whether it is a valid scenario for work in RAN3 that an energy-producing NG-RAN node is connected to the energy grid but can’t inject excess produced energy into this grid.

Additional information on the energy saving scenarios are useful to RAN3 to determine their validity and impact on the RAN.

In the reply LS to SA5, RAN3 should also inform SA5 that RAN3’s work plan for Rel-19 does not include work in relation with the energy saving use case described by SA5.

Proposal: Send a reply LS to SA5 requesting clarifications on the intended use case, and indicating that RAN3’s work plan for Rel-19 does not include work in relation with the energy saving use case described by SA5.

# 3	Conclusion

Proposal: Send a reply LS to SA5 requesting clarifications on the intended use case, and indicating that RAN3’s work plan for Rel-19 does not include work in relation with the energy saving use case described by SA5.

A corresponding reply LS has been submitted to this meeting in [2].

# References

[1]		S5-246028 – R3-247114, LS to RAN3 on renewable energy based LBO, SA5

[2]		R3-247663, [Draft] Reply LS on renewable energy based LBO (to SA5), Nokia