3GPP TSG RAN WG3 Meeting ##126	R3-247571

Orlando, U.S.A., 18 - 22 November, 2024

Agenda Item:	8.1

Source:	Ericsson

Title :	Discussion on SA2 LS regarding PDU Set Information Marking Support without QoS parameters

Document for:	Discussions & Approval

# Introduction

In this contribution, we discuss the LS received from SA2 [1], in which SA2 informs RAN3 of its PDU set concept enhancement and requests feedback from RAN3 on the signalling approach to be adopted from RAN to CN, when RAN receives the new indication that PDU set marking is supported in CN without PDU Set QoS Parameters (PSQP). The LS text is copied below.

1. Overall Description: SA2 would like to update RAN 2 and RAN3 on the latest progress of Rel-19 FS_XRM_Ph2: At the CN side, the PDU Set information marking on the GTP-U header of the downlink packets can be supported, even when the PDU Set QoS parameters are absent. Related to this, SA2 has agreed the following: - If the PDU Set QoS parameters are absent, but the PDU Set Information marking is supported, the SMF sends NG-RAN a new PDU Set Information Marking Support Indicator to indicate that DL PDU Set Information Marking is supported. - Based on the received PDU Set Information Marking Support Indicator from the CN, the NG-RAN sends information to the SMF to activate the PDU Set Information Marking for the PDU Set based handling (e.g., PSI based discarding due to congestion). Based on the NG-RAN provided information, the SMF instructs the PSA UPF to perform PDU Set information marking in the GTP-U header. Q1:SA2 would like RAN2 to provide feedback (e.g. the usefulness) on the PDU Set based handling without PDU set QoS parameters. SA2 would like to understand what information is sent by the NG-RAN to the SMF. Some companies think the NG-RAN can respond with a PDU Set Based Handling Support Indication, to indicate that PDU Set based handling is supported and to activate the PDU Set information marking. Some other companies think the NG-RAN could dynamically request the SMF to activate or deactivate the PDU Set Information marking at any time with an NG-RAN initiated procedure. Q2:SA2 would like RAN3 feedback which approach to support in Rel-19.

# Background and Discussion

During the Rel-18 XR work, RAN3 specified that if the NG-RAN supports PDU set handling based on receiving the PSQP from the CN, the NG-RAN signals a PDU Set handling support indicator as defined in NGAP and copied below:

| TS 38.413: 8.2.1.2 […] If the PDU Set QoS Parameters IE is included in the PDU SESSION RESOURCE SETUP REQUEST message, the NG-RAN node shall, if supported, report in the PDU SESSION RESOURCE SETUP RESPONSE message the PDU Set based Handling Indicator IE in the PDU Session Resource Setup Response Transfer IE. If the PDU Set based Handling Indicator IE is included in the PDU Session Resource Setup Response Transfer IE in the PDU SESSION RESOURCE SETUP RESPONSE message, the SMF shall, if supported, handle this information as specified in TS 23.501 [9]. |  |
| --- | --- |
| 9.3.1.268 PDU Set based Handling Indicator This IE indicates whether PDU Set based Handling is supported for the NG-RAN node. IE/Group Name; Presence; Range; IE type and reference; Semantics description. PDU Set based Handling Indicator; M; ENUMERATED (supported, …). |

However, now in SA2’s Rel-19 work on PDU Set enhancement, there is an option that no PSQPs are provided by the AF, thus the SMF does not include any PDU Set QoS information (PSDB, PSER and PSIHI) in the QoS parameters within the NGAP PDU Session message to RAN. SA2 has agreed instead that a new PDU Set Information Marking Support Indicator is sent to RAN to indicate that CN supports PDU Set marking.

Observation 1: In Rel-18, the NG-RAN indicates its support of handling PDU sets by sending the PDU Set based Handling Indicator IE when receiving the PSQP from CN. In Rel-19 SA2 work, no PSQP are sent from CN, but a new DL PDU Set Information Marking Support Indicator is sent to indicate CN’s support of PDU Set marking.

SA2 is then asking RAN3 in [1] how can the NG-RAN feedback to CN its support of PDU Set handling in this case.

A (straightforward) solution as mentioned by SA2 in their LS is to re-use the existing PDU Set Based Handling Support Indication IE, and to extend its definition when the new PDU Set Information Marking Support Indicator is received by RAN. In our view, this can be done as follows, by highlighting the new changes to TS 38.413:

TS 38.413 - 8.2.1.2 If the PDU Set QoS Parameters IE and/or the DL PDU Set Information Marking Support Indication IE is included in the PDU SESSION RESOURCE SETUP REQUEST message, the NG-RAN node shall, if supported, report in the PDU SESSION RESOURCE SETUP RESPONSE message the PDU Set based Handling Indicator IE in the PDU Session Resource Setup Response Transfer IE. If the PDU Set based Handling Indicator IE is included in the PDU Session Resource Setup Response Transfer IE in the PDU SESSION RESOURCE SETUP RESPONSE message, the SMF shall, if supported, handle this information as specified in TS 23.501 [9]. Note1: the above can also be described in dedicated procedural text

#### 9.3.1.12 QoS Flow Level QoS Parameters This IE defines the QoS parameters to be applied to a QoS flow. IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. CHOICE QoS Characteristics; M; -. >Non-dynamic 5QI. >>Non Dynamic 5QI Descriptor; M; 9.3.1.28; -. >Dynamic 5QI. >>Dynamic 5QI Descriptor; M; 9.3.1.18; -. Allocation and Retention Priority; M; 9.3.1.19; -. GBR QoS Flow Information; O; 9.3.1.10; This IE shall be present for GBR QoS flows and is ignored otherwise.; -. Reflective QoS Attribute; O; ENUMERATED (subject to, …); Details in TS 23.501 [9]. This IE may be present in case of Non-GBR QoS flows and is ignored otherwise.; -. Additional QoS Flow Information; O; ENUMERATED (more likely, …); This IE indicates that traffic for this QoS flow is likely to appear more often than traffic for other flows established for the PDU session. This IE may be present in case of Non-GBR QoS flows and is ignored otherwise.; -. QoS Monitoring Request; O; ENUMERATED (UL, DL, Both, …, stop); Indicates to measure UL, or DL, or both UL/DL delays for the associated QoS flow or stop the corresponding QoS monitoring.; YES; ignore. QoS Monitoring Reporting Frequency; O; INTEGER (1.. 1800, …); Indicates the reporting frequency for RAN part delay for QoS monitoring. Units: second; YES; ignore. PDU Set QoS Parameters; 0..1; YES; ignore. >UL PDU Set QoS Information; O; PDU Set QoS Information 9.3.1.264; -. >DL PDU Set QoS Information; O; PDU Set QoS Information 9.3.1.264; -. DL PDU Set Information Marking Support Indication; O; ENUMERATED (true, …); Indicates of PDU Set marking support; YES; ignore.

Observation 2: The new DL PDU Set Information Marking Support Indication IE can be introduced in the QoS parameters per QoS flow level, in the message from CN to indicate support of DL PDU set marking

Observation 3: The existing PDU Set Based Handling Support Indication IE can be re-used when the NG-RAN receives the new DL PDU Set Information Marking Support Indication IE from the CN to indicate its support of PDU Set handling with minimum NGAP procedural text change.

We note that other specifications: XnAP, F1AP may also be affected due to the presence of this IE in other procedures.

Regarding the other option mentioned by some companies in the SA2 LS, that NG-RAN dynamically indicates a request the SMF to activate or deactivate the PDU Set Information, this option cannot work or would require some substantial protocol impacts, as the NGAP PDU Session Resource Management procedures based on CP signalling are not designed for the purpose of dynamically indicating RAN capability status update. It is also not clear what benefits this dynamic indication will have, not mentioned in SA2 LS, considering the RAN3 impacts and changes in protocols functionality. Besides, it is unclear how NG-RAN node determines when to enable or disable the PDU set information, but it is clear that such requirement would further increase the complexity in NG-RAN node .

Observation 4: the alternative option mentioned by some companies in the SA2 LS that NG-RAN dynamically indicates request the SMF to activate or deactivate the PDU Set Information is incoherent wrt NGAP PDU Session Resource Management procedures design.

Observation 5: It is unclear how NG-RAN node determines when to enable or disable the PDU set information, but it is clear that such requirement would further increase the complexity in NG-RAN node.

Therefore, we propose to proceed with the basic solution mentioned by SA2, i.e, to re-use the PDU Set Based Handling Support Indication IE defined in the RAN3 specs (other specs will need to be affected and this can be discussed during normative phase after December 2024 plenary), and add adequate procedural text that the NG-RAN can provide back this IE to CN when it receives the new DL PDU Set Information Marking Support Indication.

Proposal 1: RAN3 to agree that CN to send the “DL PDU Set Information Marking Support Indication” IE to indicate to NG-RAN node that PDU Set marking is required.

Proposal 2: RAN3 to agree that NG-RAN node to re-use the existing “PDU Set Based Handling Support Indication” IE to indicate to CN that PDU Set handling can be performed.

Proposal 3: Reply to SA2 the above agreements.

A reply LS is available in [2].

# Conclusion

In this contribution we discuss the questions to RAN3 from the SA2 LS [1].

Observations and proposals are listed below:

Observation 1: In Rel-18, the NG-RAN indicates its support of handling PDU sets by sending the PDU Set based Handling Indicator IE when receiving the PSQP from CN. In Rel-19 SA2 work, no PSQP are sent from CN, but a new PDU Set Information Marking Support Indicator is sent to indicate CN’s support of PDU Set marking.

Observation 2: The new DL PDU Set Information Marking Support Indication IE can be introduced in the QoS parameters per QoS flow level, in the message from CN to indicate support of DL PDU set marking

Observation 3: The existing PDU Set Based Handling Support Indication IE can be re-used when the NG-RAN receives the new DL PDU Set Information Marking Support Indication IE from the CN to indicate its support of PDU Set handling with minimum NGAP procedural text change.

Observation 4: the alternative option mentioned by some companies in the SA2 LS that NG-RAN dynamically indicates request the SMF to activate or deactivate the PDU Set Information is incoherent wrt NGAP PDU Session Resource Management procedures design.

Observation 5: It is unclear how NG-RAN node determines when to enable or disable the PDU set information, but it is clear that such requirement would further increase the complexity in NG-RAN node.

Proposal 1: RAN3 to agree that CN to send the “DL PDU Set Information Marking Support Indication” IE to indicate to NG-RAN node that PDU Set marking is required.

Proposal 2: RAN3 to agree that NG-RAN node to re-use the existing “PDU Set Based Handling Support Indication” IE to indicate to CN that PDU Set handling can be performed.

Proposal 3: Reply to SA2 the above agreements.

# References

[1] R3-247113, LS for PDU Set Information Marking Support without QoS parameters, SA2(Vivo)

[2] R3-247572, Reply LS to SA2 regarding PDU Set Information Marking Support without QoS parameters