3GPP TSG-RAN WG3 Meeting #126	R3-247578

Orlando, US, 18 - 22 Nov, 2024

Agenda Item:	20.2

Source:	Ericsson

Title:	Discussion on NRPPa signalling design for AI/ML for PHY

Document for:	Discussion, Approval

# 1	Introduction

## WID

In RAN#105 meeting, the updated WID [1] on AI/ML for PHY targeting Positioning accuracy improvements was approved with the below objectives involving RAN3:

- Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]: - Direct AI/ML positioning: - (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning - (2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning - (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning - AI/ML assisted positioning - (2nd priority) Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning - (1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning - Specify necessary measurements, signalling/mechanism(s) to facilitate LCM operations specific to the Positioning accuracy enhancements use cases, if any - Investigate and specify the necessary signalling of necessary measurement enhancements (if any) - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE for relevant positioning sub use cases - Note: RAN1 will not discuss any proposals targeting 2nd priority issues in Q4 of 2024.

## RAN1 agreements

For the sake of tracking and clarity, we list below the full up-to-date list of RAN1 agreements, up to RAN1#118bis, related to Cases 3a and 3b which are of relevance to RAN3. This might result in a long list of agreements, but it is important for RAN3 to track what RAN1 has decided on this feature, as RAN1 is the main driver for this WI.

RAN1 agreements: - Measurements for model input - General Agreement For Rel-19 AI/ML based positioning, the measurements for determining model input are based on the DL PRS and UL SRS defined in TS38.211. - Note: The use of SRS for MIMO resource is transparent to UE. - Measurement types - Sample vs path Agreement In Rel-19 AI/ML based positioning, regarding the time domain channel measurements, RAN1 investigate the following alternatives: - Alternative (a). Sample-based measurements, where the timing information is an integer multiple of sampling periods. - Alternative (b). Path-based measurements, where the timing information is according to the detected path timing and may not be an integer multiple of sampling periods. The issues to be studied include, but not limited to, the following: - Tradeoff of positioning accuracy and signaling overhead - Impact and necessary details of gNB/UE implementation to obtain the channel measurement values. - Whether the same Alternative(s) applies to all cases or not - Applicability and necessity of specifying the Alternative(s) to different cases - Note: different sub-cases may have different issues. Note: In addition to timing information, the components for the channel measurement for model input may also include power and potentially phase. To provide the type of the channel measurement in their investigation. Agreement Sample-based measurement is defined as: - The measurement is composed of Nt' samples of the estimated channel response in time domain. The timing information for the Nt' samples are reported with a timing granularity T, where T=2kxTc. k represents the timing reporting granularity factor. Tc is the basic time unit for NR. - The corresponding measurement (e.g., power if reported) corresponds to the measurement for the reported Nt' samples. - Nt' and k can be signalled - FFS: the value range of Nt'; the value range of integer k for the timing granularity T. - The timing information is defined relative to a reference time Further discussion is expected on the determination of Nt' and k (including signaling) , and a rule to be introduced for selecting Nt' samples. Note: It doesn’t imply the definition of Sample-based measurement will be captured into the spec. Agreement Path-based measurement refers to the measurement in the existing specifications (up to Rel-18) including measurement reporting, with potential enhancements on the number of reported paths (if needed). Agreement For the definition of sample-based measurement, select Nt’ samples out of a list of Nt consecutive samples - The Nt samples have timing granularity T. - FFS: the starting time of the list of Nt samples - FFS: the value range of Nt For the sample-based measurement (if accepted in Rel-19), - For measurement by TRP/gNB, the Nt’ selected samples are expected to be those with the highest power. - Note: Choice of the maximum value of Nt, Nt’ should take into account the need to preserve proprietary implementation. Agreement For Rel-19 AI/ML based positioning Case 3b, regarding sample-based measurement (if supported), from RAN1 perspective, - LMF can signal parameter values of Nt, Nt', k to gNB via NRPPa. - Time domain info Agreement - For AI/ML based positioning case 3b, at least the following types of time domain channel measurements are supported for reporting: - timing information; - paired timing information and power information. - Phase information Agreement For AI/ML based positioning for all use cases, RAN1 investigate the necessity and feasibility of using phase information (in addition to timing information and power information) for determining model input. The issues to study include: - Tradeoff of positioning accuracy and signaling overhead - The impact of transmitter and receiver implementation - Specification impact - Other aspects are not precluded Note: the phase information may be used in different ways, e.g., one phase value for the first path or first sample only; triplet of {timing information, power information, phase information} for CIR, etc. - Measurement reporting - Case 3a Agreement For AI/ML assisted positioning Case 3a, at least LOS/NLOS indicator and/or timing information are supported for reporting. - If LOS/NLOS indicator is reported, the indicator can be reported as soft indicator or hard indicator as defined in 38.214. - If timing information is reported, the timing information at least can be reported via UL RTOA or gNB Rx-Tx time difference as defined in 38.215. - Note: details of the report are pending further discussion. Agreement From RAN1 perspective, when timing information is reported for Rel-19 AI/ML positioning Case 3a, at least the following are mandatorily or optionally supported in a measurement report from gNB to LMF: - (Mandatory) timing information; - (Optional) Quality of the timing information; - Existing IE “Timing Measurement Quality” can be reused. - (Mandatory) Time stamp. FFS: LOS/NLOS indicator. Note: The final decision of “mandatory” or “optional” presence of each field is up to RAN3. - Case 2b/3b Agreement For AI/ML based positioning Case 3b, for gNB channel measurements reported to LMF, the timing information is represented relative to a reference time. - FFS: Whether any specification impact of the reference time used to represent the timing information. Details of the reference time Agreement For AI/ML based positioning Case 3b, for gNB channel measurements reported to LMF, the timing information is represented relative to the existing UL RTOA reference time T0+tSRS as defined in TS 38.215. FFS: whether it is applicable when Case 3b is used to support multi-RTT Agreement For AI/ML positioning Case 2b and 3b, regarding the power information for determining the model input, - For downlink power measurement, use DL PRS-RSRPP defined in TS 38.215 as a starting point. - For measurement report of DL PRS-RSRPP, use the existing measurement report mapping table for PRS-RSRPP in 38.133 as a starting point. - For uplink power measurement, use UL SRS-RSRPP defined in TS 38.215 as a starting point. - For measurement report of UL SRS-RSRPP, use the existing measurement report mapping table for SRS-RSRPP in 38.133 as a starting point. Agreement From RAN1 perspective, for model inference of AI/ML positioning Case 3b, at least the following are mandatorily or optionally supported in a measurement report from gNB to LMF: - (Mandatory) Channel measurement; - (Optional) Quality of the channel measurement; - FFS: details of the quality - (Mandatory) Time stamp of the channel measurement. - Performance monitoring Agreement For LMF-side model, RAN1 studies whether/what assistance information and/or measurement report may be sent from UE/PRU, and/or gNB to LMF to assist at least for the performance monitoring. - RAN1 understands that it is out of RAN1 scope to define monitoring metric calculation and related model management decisions for LMF-side model. Agreement For AI/ML positioning Case 3a, for model performance monitoring metric calculation in label-based model monitoring, study the feasibility of the following options. To provide information on how to generate information on ground truth label for each option. - Option A. NG-RAN node performs monitoring metric calculation for its own model. - Option B. LMF performs monitoring metric calculation for the model located at the NG-RAN node. Note: Final selection of Option A and Option B is out of RAN1 scope, but RAN1 can make recommendation about the option(s), and potential support of Option A and/or Option B is pending RAN3 confirmation. Note: Exact method to perform the monitoring metric calculation is up to implementation Agreement For AI/ML positioning Case 3a, for performance monitoring metric calculation in label-based monitoring, from RAN1 perspective, Option A and Option B are feasible, - Option A. NG-RAN node performs monitoring metric calculation for its own model. - Option B. LMF performs monitoring metric calculation for the model located at the NG-RAN node. Note: Final selection of Option A and Option B is out of RAN1 scope. Potential support of Option A and/or Option B is pending RAN3 confirmation. Note: Exact method to perform monitoring metric calculation is up to implementation. Note: For Option A, RAN1 assumes that user data privacy needs to be preserved. - Training and Data collection - General Agreement For training data collection of AI/ML based positioning, if a training data sample contains both Part A and Part B, RAN1 assumes that Part A and Part B in one training data sample are: - for a same UE (PRU or Non-PRU UE), and - for a same location associated with Part B. Note: the association can be discussed Conclusion - It is out of RAN1 scope to decide whether/how synthetic data (i.e., not direct physical data) and related entities are used in AI/ML based positioning. In RAN1 discussion, data (e.g., measurement data, label data) refer to physical data, not synthetic data. Conclusion From RAN1 perspective, for Case 3a measurements, - The existing procedures can be reused in terms of SRS configuration. - Note: parameter values for SRS configuration can be further discussed - These measurements can be used for multiple aspects related to case 3a, e.g. training data collection, monitoring, or inference procedures. - Note: Purpose, such as the training data collection, monitoring, or inference procedures mentioned above, will not necessarily be specified in RAN 1 specifications Agreement For training data collection of AI/ML based positioning, the quality indicator of timing information in Part A when reported is: - When applicable, the existing IE for timing quality, i.e., NR-TimingQuality in 37.355 and IE “Timing Measurement Quality” in 38.455; - FFS: details on how to associate quality indicator to timing information - Training data collection for case 3a and 3b Agreement For training data generation of AI/ML based positioning Case 3a and 3b, the measurement and its related data (e.g., timestamp) are generated by TRP/gNB. Working Assumption For training data generation of AI/ML based positioning Case 3b, the label and its related data (e.g., time stamp) can be generated by: - PRU - FFS: Non-PRU UE with estimated location - LMF Note: transfer of label and its related data is out of RAN1 scope. Working Assumption (replaces the one above) For training data generation of AI/ML based positioning Case 3b, the label and its related data (e.g., time stamp) can be generated by: - PRU - Non-PRU UE with estimated location - LMF Note: transfer of label and its related data is out of RAN1 scope. Note: It is assumed that user data privacy of non-PRU UE is preserved. Note: Previous related working assumption made in RAN1#116bis for training data generation of AI/ML based positioning Case 3b will not need to be confirmed. Agreement For training data generation of AI/ML based positioning Case 3a, the label and its related data (e.g., time stamp) can be generated by at least: - LMF Note: transfer of label and its related data is out of RAN1 scope. Note: whether other network entities can generate label for Case 3a is out of RAN1 scope. Agreement For training data collection of AI/ML based positioning case 3b, for time stamp of channel measurement, - For channel measurement generated by TRP/gNB, existing IE “Time Stamp” in TS 38.455 can be reused from RAN1 perspective - Note: Purpose, such as above “training data collection", will not necessarily be specified in RAN 1 specifications

## RAN3 agreements

Similarly, for the sake of tracking our internal RAN3 progress, we list the agreements and FFS taken in RAN3#125 meeting:

- The existing LCS framework is re-used for AI/ML positioning. - Case 3a agreements: - The AI/ML model (inference and training functions) is at the gNB. - Consider aggregated gNB case first, then split architecture. - LMF provides ground truth label and related data (i.e., part B information as called by RAN1) to NG-RAN. - FFS on the content of the ground truth label and related data pending on RAN1 progress. FFS on the signalling design. - How the LMF obtains the ground truth label and related data information is outside of RAN3 scope. - NG-RAN obtains measurements from TRP (i.e., part A information as called by RAN1). - The details of the measurements are still pending and subject to further discussion in RAN1. - WA: RAN3 will proceed with Option A: NG-RAN node performs monitoring metric calculation for its own model. - Case 3b agreements: - Re-using the Measurement procedure defined in NRPPa to support requesting and reporting the related measurements to LMF for Case 3b. The details of the related measurements are still pending and subject to further discussion in RAN1. - NG-RAN gets measurements (i.e., part A information as called by RAN1) from TRP and provides them to LMF. - The details of the measurements are still pending and subject to further discussion in RAN1.

In this contribution, we discuss the corresponding RAN3 impacts, namely NRPPa impacts, to support data collection for Case 3a and Case 3b.

# 2	Discussion

## 2.1	Case 3b (LMF-sided model) discussion

At the last RAN3 meeting, RAN3 confirmed that current NRPPa signalling can be reused to request and report sample data measurements from the TRPs as model input to the LMF for model inference purposes. Such measurements, according to RAN1 agreements, can be timing measurements (e.g., UL-RTOA) and timing measurement paired with Power measurement (e.g., UL-SRS-RSRPP). Lately, RAN1 is investigating the addition of phase measurement (e.g., UL-RSCP) as part of model input.

Currently, the Measurement procedures specified in NRPPa (and F1AP) support requesting all these types of measurements, as well as for path-based measurement (see TS 38.455 §9.2.39). The procedures can hence be easily extended if RAN1 defines new sample data measurements (e.g., new codepoint in the Measurement Request message and the associated new IE in the TRP Measurement report in the Measurement Response message). Therefore, little impact is expected for case 3b (or rather, business as usual in RAN3). A small impact could be if new additional paths for measurements are added by RAN1. As of Rel-18, there are up to 6 reported paths per measurement (see TS 38.455 §9.2.74). See also extract from RAN1 agreement below regarding path-based measurements:

- Path-based measurement refers to the measurement in the existing specifications (up to Rel-18) including measurement reporting, with potential enhancements on the number of reported paths (if needed).

Observation1: NRPPa currently supports requesting all types of measurements (time, power, phase), as well as for path-based measurements. The question of whether new sample measurements are introduced and whether the number of reported paths is extended (from 6) are currently being examined by RAN1.

Another potential impact to NRPPa for Case 3b can be the addition of the sampling parameters in the Measurement request message. This addition stems from this RAN1 agreement:

- LMF can signal parameter values of Nt, Nt', k to gNB via NRPPa. - […] - Further discussion is expected on the determination of Nt' and k (including signaling) , and a rule to be introduced for selecting Nt' samples.

But as seen from the above agreement’s extract, the discussion is still on-going in RAN1. We can expect that once RAN1 concludes the discussion on the sampling measurements and associated parameters, RAN3 will receive an LS from RAN1 on the new impacts. Since the NRPPa signalling design has already been discussed and agreed last meeting to re-use the Measurement procedures, there is not much RAN3 can discuss for the moment on case 3b. RAN3 can simply wait for RAN1 progress.

Proposal 1: For Case 3b, RAN3 can simply wait for additional RAN1 progress on measurements and sampling parameters definitions.

## 2.2	Case 3a: (NG-RAN-sided model) discussion

Case 3a, on the other hand, requires further discussion in RAN3, as there are several aspects to consider:

- how to support model training

- how to support model inference

- location of the model

### 2.2.1 Model Training for Case 3a:

Data Collection can in practice happen before model training, where the NG-RAN node receives ground truth labels (i.e., part B information from RAN1 agreements) from the LMF, and input data measurements (i.e., part A information from the RAN1 agreements) from the TRPs. Part B information is signalled over NRPPa, while part A information, if the model is residing at the gNB-CU, is signalled over F1AP.

Since the discussion on AI/ML model placement in split NG-RAN has been postponed, RAN3 should discuss the NRPPa signalling design for providing part B information from the LMF. A new functionality is needed so that the NG-RAN can receive training data from LMF. From last RAN3 meeting, two options ([2] and [3]) have been proposed for this purpose.

#### 2.2.1.1 Solution 1: gNB-triggered Data Collection Request

In the solution proposed by [2], the NG-RAN node sends a request for label delivery to an LMF, via the AMF, as illustrated in Figure 1 below:

Figure 1: General data collection procedure for gNB-side model training in Case 3a [2]

In [2], it is described that “the serving gNB sends a request to an LMF via the serving AMF for the ground truth label. This could be a new NGAP message, followed by a Namf service operation between AMF and LMF, or a new NRPPa message”. In our understanding, this procedure is UE-associated and is depending on whether there is an on-going positioning procedure (e.g., UL SRS transmission by the UE) at the time of the data collection request. This signalling option provides flexibility to gNB to decide when it should initiate the request for ground-truth labels. However, there are different implications depending on whether the gNB triggers the data collection request via NGAP or NRPPa.

For the sake of clarity, Solution 1 is split into two sub solutions:

- Solution 1a: gNB-triggered Data Collection Request via a new NGAP message

- Solution 1b: gNB-triggered Data Collection Request via a new NRPPa message

In Solution 1a, if the NG-RAN node triggers the request via a new NGAP message to the AMF, there would be the need for the AMF receiving the message to:

- Determine which LMF has the capabilities to support such data request (in fact, it could be that the LMF positioning the UE, does not have such capability, and AMF must then select a supporting LMF).

- Determine which (supporting) LMF has the training data requested by the NG-RAN.

- Determine whether user consent is in place for data collection relative to the user of the UE in question (Note: the need for user consent was agreed by SA2 for training data collection in case 3b: it seems obvious that the same will be required for case 3a).

All of the above has a rather high impact on the AMF. Moreover, the above implies that procedures involving more than one CN function will have to be triggered every time a data request from the RAN is initiated. The latter will need the involvement of SA2 and CT4. 
Further, with this approach AI/ML assisted positioning depends on whether the AMF supports this feature. Namely, we would create one extra (strong) dependency, where without AMF’s support, the feature would not work.

On the other side, the advantage of this option would be to preserve the principle that an NRPPa transaction can only be initiated by the server, as specified in [4]. As it is understood that new procedures will have to be defined over NGAP for this purpose.

Observation 2: Summary of Solution 1a: gNB-triggered Data Collection Request via a new NGAP procedure

| Concept | Advantages | Drawbacks |
| --- | --- | --- |
| The serving gNB sends a request to an LMF via the serving AMF for the ground truth label via a new NGAP message, followed by a Namf service operation between AMF and LMF | - The gNB has flexibility to decide when to request for training data. - Preserve the principle that an NRPPa transaction can only be initiated by the server - Ensures that only an LMF with training data, that has UE consent, and is Rel-19- upgraded can be involved in the data collection procedure. | - High AMF and CN impacts, with extra (strong) dependency on AMF supporting this feature. - Since the procedure is UE associated, the AMF needs to undergo a UE consent check involving other CN nodes. - The LMF may not have training data for that UE, which would result in a procedure failure. - Trial & errors leading to high frequency of request failures. |

In other Solution 1b, the NG-RAN node triggers the request via NRPPa message, encapsulated in an NGAP message, which could be e.g., the UPLINK UE ASSOCIATED NRPPA TRANSPORT message defined in TS 38.413. We notice that there are the same issues as above regarding: UE consent check that needs to be done involving different CN nodes; whether the LMF indicated by the Routing ID has the requested content of training data or whether it is even supporting such functionality; this option can also lead to a higher level of trial and error in signalling between the RAN and the LMF compared to solution 1a, since the NG-RAN does not know whether the LMF is supporting or not. Moreover, this option is a radical change in NRPPa protocol functionality, since, as stated already, TS 38.305 specifies that an NRPPa transaction can only be initiated by the server [4].

Observation 3: Summary of Solution 1b: gNB-triggered Data Collection Request via a new NRPPa message

| Concept | Advantages | Drawbacks |
| --- | --- | --- |
| The serving gNB sends a request to an LMF for the ground truth label via a new NRPPa message directly to LMF | - The gNB has flexibility to decide when to request for training data. | - Contradicts NRPPa protocol principle that transaction is always initiated by server. - Since the procedure is UE associated, the LMF needs to undergo a UE consent check involving other CN nodes. - The LMF may not have training data for that UE, or not be Rel-19 upgraded, which would result in a procedure failure. - Higher trial & errors leading to high frequency of request failures. |

#### 2.2.1.2 Solution 2: “Xn-like Data Collection” over NRPPa

The other solution proposed by [3] is to integrate similar procedures as specified in XnAP for Data Collection into NRPPa, where the LMF initiates an NRPPa transaction (e.g., via a DATA COLLECTION INFORMATION NOTIFICATION message) indicating to the NG-RAN node that ground truth labels are available for a list of TRPs and list of UEs (for which the consent has been granted).

Compared to solution 1, this is a class 2 non-UE associated procedure that is associated to an area indicated by the TRPs. Note that, in this option, the LMF is aware of the UEs for which data is available because a positioning process is underway for these UEs. This is an essential condition for the LMF to be able to provide ground truth to the NG-RAN. Next, the NG-RAN node can trigger a new class 1 procedure with a DATA COLLECTION INFORMATION REQUEST message, including a “start” indication, and indicating the training data it needs and the periodicity at which it wishes to receive the training data reporting. The LMF replies with a DATA COLLECTION INFORMATION RESPONSE message to acknowledge the request.

Afterwards, a second class 2 procedure, via a DATA COLLECTION INFORMATION UPDATE message, is triggered by the LMF where it provides the training data to the NG-RAN node at the requested periodicity, and which includes the requested (and previously indicated available) ground truths, such as UEs’ location (if consent for this information was and is still given). Similarly to the Xn procedures, the NG-RAN node can send a DATA COLLECTION INFORMATION REQUEST message with a “stop” flag to indicate the LMF to stop the reporting.

In case the LMF during a reporting of positioning information can no longer provide it, for example due to a UE’s consent being revoked or that training data in a TRP is no longer available, the LMF can send a class 2 DATA COLLECTION INFORMATION FAILURE INDICATION message and terminate the procedure, hence no more DATA COLLECTION INFORMATION UPDATE messages will be issued.

Figure 2 summarizes the procedures mentioned above. (Note that if the model is hosted at the gNB-CU, we gain a very considerable signalling reduction over the F1.)

Fig.2 Signalling to support Training Data collection for Case 3a

The advantage of this solution is that it is aligned both with the design of the NRPPa protocol (a transaction is always initiated by the server) and with XnAP's established procedures for data collection transfer. This solution can in fact be seen as a matter of porting the XnAP data collection messages to NRPPa with the necessary adjustments, as illustrated in figure 2. The solution also ensures that only an upgraded Rel-19 LMF, that has the training data, with UE consent granted, can provide this data to the gNB and for many TRPs and UEs, and thus avoids the trial-and-error problem of solution 1.

It was pointed out during the last RAN3 meeting that a potential issue with Solution 2 is that the LMF may have to keep an NRPPa transaction open after step 2 and not receive a request message from the NG-RAN node (i.e., step 3). However, this would not be an issue because keeping an NRPPa transaction open without any signalling being transmitted would not generate load at the LMF. The LMF must also continue to update the information periodically as the training data changes over time, triggering new notification messages. This could be a waste of NRPPa signalling, as the NG-RAN may not need to collect data. However, this problem is also inherent in the design of XnAP data collection signalling.

Another advantage is that, with this solution, it could be left to the gNB’s implementation when to request training data once it has received the LMF notification message, which would address the above issues.

Observation 4: Summary of Solution 2: “Xn-like Data Collection” over NRPPa [3]

| Concept | Advantages | Drawbacks |
| --- | --- | --- |
| Introduce similar procedures specified in XnAP for Data Collection transfer in NRPPa, where the LMF first notifies the NG-RAN if training data is available for a list of TRPs/UEs. The gNB replies back indicating the desired training data and the requested periodicity. If the procedure is successful, the LMF provides training data periodically. | - In-line with NRPPa protocol design. - Re-uses XnAP’s established framework for data collection signalling. - Ensures that only an LMF with training data, that has UE consent, and is Rel-19- upgraded can be involved in the data collection procedure. - Avoids trials and errors of solutions 1a/1b. | - The LMF opens the NRPPa transaction but does not know when the NG-RAN will ask for training data. - The LMF needs to continue updating the NG-RAN if the training data has changed. |

Based on the above comparison analysis, it is proposed that RAN3 discusses the presented Data Collection signalling solutions. Considering the advantages and Drawbacks of each, solution 2 is preferred.

Proposal 2: RAN3 to discuss Data Collection signalling solutions. Considering the advantages and Drawbacks of each , solution 2 based on “Xn-like Data Collection” over NRPPa is preferred.

A TP to NRPPa BL CR capturing solution 2 is provided in section 6 as annex of this paper.

### 2.2.2 Model Inference for Case 3a:

Another signalling aspect for RAN3 to discuss is the inference output from NG-RAN to LMF. In last meeting, several companies have discussed offline to reuse existing NRPPa measurement reporting procedures for this purpose. However different point of views on how to encode the model output have been raised:

- Option 1: no difference in NRPPa between legacy measurements and AI/ML measurements

In this option, there is no difference at LMF’s level between a Rel-19 AI/ML positioning measurement and a legacy positioning measurement. Meaning, if the LMF requests for, e.g., UL-RTOA, but the gNB hosting the model decides to switch to AI/ML estimated UL-RTOA (due to e.g., NLOS), this won’t be distinguishable in the NRPPa TRP Measurement report to the LMF. This option is muddled with inefficiencies, as without such distinction there is no interoperability because a positioning function at LMF may very likely act differently depending on whether the received measurements are AI/ML generated or not. For example, if the measurements are inferred, the LMF may ask measurements from a higher number of gNBs. Also, if the location is derived based on inferred AI/ML measurements, the LMF may need to indicate such to the LCS client. However, without knowing if a measurement is AI/ML inferred or not, such differentiations and different behaviours would not be possible.

Proposal 3: The option of having no differentiation between legacy NRPPa positioning measurements and new NRPPa AI/ML inferred positioning measurements is excluded by RAN3.

- Option 2: Specific codepoints in the NRPPa MEASUREMENT REQUEST message, and new IEs in the TRP Measurement Report IE

In this option, AI/ML positioning measurements are considered as new measurements and the LMF explicitly requests them in the MEASUREMENT REQUEST message (as new codepoints). They are also provided explicitly in the TRP Measurement Result IE in the NRPPa response message as new IEs. Example as below in highlight:

#### 9.1.4.1 MEASUREMENT REQUEST This message is sent by the LMF to request the NG-RAN node to configure a positioning measurement. Direction: LMF ® NG-RAN node. IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. Message Type; M; 9.2.3; YES; reject. NRPPa Transaction ID; M; 9.2.4; -. LMF Measurement ID; M; INTEGER (1..65536, …); YES; reject. TRP Measurement Request List; 1; YES; reject. >TRP Measurement Request Item; 1..<maxnoofMeasTRPs>; -. >>TRP ID; M; 9.2.24; -. >>Search Window Information; O; 9.2.26; -. >>Cell ID; O; NR CGI 9.2.9; The Cell ID of the TRP identified by the TRP ID IE.; YES; ignore. >>AoA Search Window Information; O; UL-AoA Assistance Information 9.2.66; YES; ignore. >>Number of TRP Rx TEGs; O; ENUMERATED (2, 3, 4, 6, 8, …); YES; ignore. >>Number of TRP RxTx TEGs; O; ENUMERATED (2, 3, 4, 6, 8, …); YES; ignore. Report Characteristics; M; ENUMERATED (OnDemand, Periodic, ...); YES; reject. Measurement Periodicity; C-ifReportCharacteristicsPeriodic; ENUMERATED (120ms, 240ms, 480ms, 640ms, 1024ms, 2048ms, 5120ms, 10240ms, 1min, 6min, 12min, 30min, 60min,…, 20480ms, 40960ms, extended); The codepoint 120ms, 240ms, 480ms, 1024ms, 2048ms, 1min, 6min, 12min, 30min, and 60min are not applicable; YES; reject. TRP Measurement Quantities; 1; YES; reject. >TRP Measurement Quantities Item; 1 .. <maxnoPosMeas>; -. >>TRP Measurement Type; M; ENUMERATED (gNB-RxTxTimeDiff, UL-SRS-RSRP, UL-AoA, UL-RTOA,…, Multiple UL-AoA, UL SRS-RSRPP, UL-RSCP, AI/ML UL-RTOA, AI/ML gNB-RxTxTimeDiff); The UL-RSCP measurement is applicable only when the UL-RTOA and/or gNB-RxTxTimeDiff measurement(s) is also requested.; -.

### 9.2.37 TRP Measurement Result This information element contains the measurement result. IE/Group Name; Presence; Range; IE Type and Reference; Semantics Description; Criticality; Assigned Criticality. Measured Result Item; 1 .. <maxnoPosMeas>; -. >CHOICE Measured Results Value; M; -. >>UL Angle of Arrival; 9.2.38. >>UL SRS-RSRP; INTEGER (0..126). >>UL RTOA Measurement; 9.2.39. >>gNB Rx-Tx Time Difference; 9.2.40. >>Z-AoA; 9.2.67; YES; reject. >>Multiple UL-AoA; 9.2.71; YES; reject. >>UL SRS-RSRPP; 9.2.72; YES; reject. >>UL RSCP; 9.2.92; YES; reject. >>AI/ML UL RTOA Measurement; 9.2.39; YES; reject. >>AI/ML gNB Rx-Tx Time Difference; 9.2.40; YES; reject. >Time Stamp; M; 9.2.42; -. >Measurement Quality; O; 9.2.43; -. >Measurement Beam Information; O; 9.2.57; -. >SRS Resource type; O; 9.2.73; YES; ignore. >ARP ID; O; 9.2.75; YES; ignore. >LoS/NLoS Information; O; 9.2.77; YES; ignore.

The LMF can ask in the same procedure inferred and non-inferred measurements. This could be useful for aggregation of different types of measurements, comparing measurements, etc. However this option has additional LMF impacts.

- Option 3: A new AI/ML “flag” is added in the TRP Measurement Report IE

In this option a “flag” is added in the TRP measurement report to indicate that the requested measurement has been generated based on based on Rel-19 AI/ML positioning.

### 9.2.37 TRP Measurement Result This information element contains the measurement result. IE/Group Name; Presence; Range; IE Type and Reference; Semantics Description; Criticality; Assigned Criticality. Measured Result Item; 1 .. <maxnoPosMeas>; -. >CHOICE Measured Results Value; M; -. >>UL Angle of Arrival; 9.2.38. >>UL SRS-RSRP; INTEGER (0..126). >>UL RTOA Measurement; 9.2.39. >>gNB Rx-Tx Time Difference; 9.2.40. >>Z-AoA; 9.2.67; YES; reject. >>Multiple UL-AoA; 9.2.71; YES; reject. >>UL SRS-RSRPP; 9.2.72; YES; reject. >>UL RSCP; 9.2.92; YES; reject. >Time Stamp; M; 9.2.42; -. >Measurement Quality; O; 9.2.43; -. >Measurement Beam Information; O; 9.2.57; -. >SRS Resource type; O; 9.2.73; YES; ignore. >ARP ID; O; 9.2.75; YES; ignore. >LoS/NLoS Information; O; 9.2.77; YES; ignore. >AI/ML Positioning measurement; O; ENUMERATED (true,…); This IE is only valid for UL-RTOA and gNB Rx-Tx Time Difference measurements; YES; ignore.

This option avoids introducing new IEs in both the request and response messages, which can be especially useful if later other positioning measurements (e.g., AoA) are introduced as part of model output. This also has small spec impacts. For Rel-19, this IE should be limited to UL-RTOA and gNB Rx-Tx Time Difference measurements. This can be done via semantics.

Observation 4: The AI/ML positioning measurements should be distinguishable from the legacy measurements in the TRP Measurement report.

Proposal 4: RAN3 to discuss how to encode the model output in Case 3a: whether to introduce new IEs in the NRPPa Request and Response messages for AI/ML UL-RTOA and gNB Rx-Tx TD measurements; or to add a flag in the TRP report that the associated measurement is AI/ML generated; or other option.

On another note, we remark the following agreement made recently by RAN1:

Agreement From RAN1 perspective, when timing information is reported for Rel-19 AI/ML positioning Case 3a, at least the following are mandatorily or optionally supported in a measurement report from gNB to LMF: - (Mandatory) timing information; - (Optional) Quality of the timing information; - Existing IE “Timing Measurement Quality” can be reused. - (Mandatory) Time stamp. FFS: LOS/NLOS indicator. Note: The final decision of “mandatory” or “optional” presence of each field is up to RAN3.

From a positioning perspective, it beneficial to clarify the meaning of the above agreement and IEs to avoid confusion with AI/ML terminology and principles:

- The timing information (Time Stamp IE in NRPPa) has the same meaning as legacy, i.e., it corresponds to the time of reception of a specific SRS by the TRP, and not the time of when the measurement was inferred. Therefore, the Time Stamp IE has no change with respect to legacy meaning.

- Quality of the timing information (Measurement Quality IE in NRPPa) is not necessarily related to the quality of the AI/ML model output. In fact, in positioning, it can be related to, e.g., the input data quality, quality of SRS, etc. It is up to gNB (DU) implementation to derive it and is never requested by LMF. 
Therefore, the Measurement Quality IE has no change with respect to legacy meaning.

- LOS/NLOS indicator (LoS/NLoS Information IE in NRPPa) should be interpreted as the legacy information on the channel status, i.e., info on the received channel monitored by the TRP. Therefore, the LoS/NLoS Information IE has no change with respect to from legacy meaning.

Based on the above, signalling of measurements output for case 3a can re-use the existing procedures for positioning measurement specified over NRPPa. In fact, they should be re-used as we agreed last meeting to re-use the positioning framework.

Proposal 5: The Time Stamp and LoS/NLoS Information are not related to AI/ML model output. They have the same meaning as legacy and refer to SRS time detection, and channel status, respectively.

Proposal 6: The Measurement Quality is not necessarily related to AI/ML model output. It has the same meaning as legacy and it represents an indication of confidence of the measurement that could also refer to the input measurements quality.

Proposal 7: signalling of measurements output for case 3a can re-use the existing procedures for positioning measurement specified over NRPPa.

## 2.3	Other aspects

Regarding split architecture, since Rel-16, Positioning in RAN3 has always considered NRPPa and F1AP extensions to be specified both at the same time, due to: NRPPa is terminated at gNB-CU and that TRPs are residing in gNB-DU. This principle should be maintained for AI/ML Phy work. In fact, it should be considered business as usual.

RAN3 should revisit the statement captured at the last meeting and discuss the placement of the Al/ML model. In last meeting, several companies have in fact expressed a preference for gNB-CU hosting the model. There are many advantages to this, such as increased capacity and a higher number of input data when aggregated from different sites. We also note that we save on F1 signalling for gNB training data collection request, as we do not need to have the request originating from the TRP, nor sending the ground truth labels over F1AP to the TRP. Hence, we do not see the need to postpone the discussion on the split architecture, as this can be considered in Rel-19 with minimum spec impacts.

Proposal 8: The Al/ML model is placed at gNB-CU. RAN3 to consider split gNB work during Rel-19

Finally, another aspect is to confirm the WA taken last meeting:

- WA: RAN3 will proceed with Option A: NG-RAN node performs monitoring metric calculation for its own model.

Option B cannot work due to inter-operability issues, therefore there is no need to pursue it.

Proposal 9: RAN3 to confirm WA on Option A. Only option A can be pursued.

# 3	Conclusion

In this contribution, we have made the following observations and propose the following:

Observation1: NRPPa currently supports requesting all types of measurements (time, power, phase), as well as for path-based measurements. The question of whether new sample measurements are introduced and whether the number of reported paths is extended (from 6) are currently being examined by RAN1.

Proposal 1: For Case 3b, RAN3 can simply wait for additional RAN1 progress on measurements and sampling parameters definitions.

Proposal 2: RAN3 to discuss Data Collection signalling solutions. Considering the advantages and Drawbacks of each , solution 2 based on “Xn-like Data Collection” over NRPPa is preferred.

Proposal 3: The option of having no differentiation between legacy NRPPa measurements and new NRPPa AI/ML inferred measurements is excluded by RAN3.

Observation 4: The AI/ML positioning measurements should be distinguishable from the legacy measurements in the TRP Measurement report.

Proposal 4: RAN3 to discuss how to encode the model output in Case 3a: whether to introduce new IEs in the NRPPa Request and Response messages for AI/ML UL-RTOA and gNB Rx-Tx TD measurements; or to add a flag in the TRP report that the associated measurement is AI/ML generated; or other option.

Proposal 5: The Time Stamp and LoS/NLoS Information are not related to AI/ML model output. They have the same meaning as legacy and refer to SRS time detection, and channel status, respectively.

Proposal 6: The Measurement Quality is not necessarily related to AI/ML model output. It has the same meaning as legacy and it represents an indication of confidence of the measurement that could also refer to the input measurements quality.

Proposal 7: signalling of measurements output for case 3a can re-use the existing procedures for positioning measurement specified over NRPPa.

Proposal 8: The Al/ML model is placed at gNB-CU. RAN3 to consider split gNB work during Rel-19

Proposal 9: RAN3 to confirm WA on Option A. Only option A can be pursued.

# 4	References

- RP-242399, Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- R3-245461, AI/ML based positioning accuracy enhancements, QC inc., RAN3#125-bis

- R3-245473 Discussion on RAN3 impacts to support AI/ML Positioning accuracy enhancements, Ericsson, RAN3#125-bis

- TS 38.305 v18.3.0, section 7.1

# 5	Annex – LS to RAN1 (TBD)

3GPP TSG-RAN WG3 Meeting #126	<TDoc#>

Orlando, US, 18 - 22 Nov, 2024

# Title:	LS reply on RAN3 agreements on AI/ML positioning

# Response to:

# Release:	Release 19

# Work Item:	NR_AIML_air-Core

Source:	RAN3

To:	RAN1, RAN2

Cc:

Contact Person:

#### Name:	Yazid Lyazidi

#### Tel. Number:

#### E-mail Address:	Yazid.lyazidi@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:

1. Overall Description:

RAN3 has started the discussions on support of Case3a and Case 3b for AI/ML positioning.

RAN3 has made the following agreements:

Case 3b, direct AI/ML positioning, LMF-sided model: - Re-using the Measurement procedure defined in NRPPa to support requesting and reporting the related measurements to LMF for Case 3b. The details of the related measurements are still pending and subject to further discussion in RAN1. - NG-RAN gets measurements (i.e., part A information as called by RAN1) from TRP and provides them to LMF. - The details of the measurements are still pending and subject to further discussion in RAN1. Case 3a, assisted AI/ML positioning, NG-RAN hosted model: - LMF provides ground truth label and related data (i.e., part B information as called by RAN1) to NG-RAN. - FFS on the content of the ground truth label and related data pending on RAN1 progress. FFS on the signalling design. - How the LMF obtains the ground truth label and related data information is outside of RAN3 scope. - NG-RAN obtains measurements from TRP (i.e., part A information as called by RAN1). - The details of the measurements are still pending and subject to further discussion in RAN1. - WA: RAN3 will proceed with Option A: NG-RAN node performs monitoring metric calculation for its own model.

RAN3 kindly asks RAN1 for updates on Part A information (input data from TRPs) for use cases 3a and 3b, and for Part B (ground truth labels from LMF) for case 3a.

2. Actions:

To RAN1, RAN2.

ACTION: 	RAN3 asks RAN1 and RAN2 to take the above into account and for RAN1 to provide feedback on the content of Part A and Part B information mentioned above.

3. Date of Next TSG RAN WG3 Meetings:

TSG-RAN3 Meeting #127        17th Feb – 21st Feb, 2025			Athens, Greece

TSG-RAN3 Meeting #127bis	7 - 11 April 2025				China, CN

# 5	Annex – TP to NRPPa BL CR

3GPP TSG-RAN WG3 Meeting #126	<TDoc#>

Orlando, US, 18 - 22 Nov, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.455 | CR | 0170 | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of AI/ML Positioning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_AIML_air-Core |  |  |  |  |  | Date: |  |  | 2024-11-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Support of AI/ML for positioning. Introduce support for Case 3a NG-RAN-side model and Case 3b LMF-side model |  |  |  |  |  |  |  |  |
| Summary of change: |  | For Case 3b: - Introduce of sample parameters (Nt, Nt', k) in the MEASUREMENT REQUEST message For Case 3a: - Introduce new procedures for Data Collection Information Transfer: - DATA COLLECTION INFORMATION NOTIFICATION, from LMF. - DATA COLLECTION INFORMATION REQUEST, NG-RAN initiated, with RESPONSE and FAILURE messages. - DATA COLLECTION INFORMATION UPDATE, LMF-initiated. - DATA COLLECTION INFORMATION FAILURE INDICATION, LMF-initiated. - Introduce new AI/ML measurement flag in the TRP Measurement Report IE FFS on structure and names on all messages and IEs |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support of AI/ML positioning |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR ... CR TBD |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: resubmission to RAN3#126 |  |  |  |  |  |  |  |  |

START OF CHANGES

# 7	Functions of NRPPa

The NRPPa protocol provides the following functions:

-	E-CID Location Information Transfer. This function allows the NG-RAN node to exchange location information with LMF for the purpose of E-CID positioning and NR E-CID positioning.

-	OTDOA Information Transfer. This function allows the NG-RAN node to exchange information with the LMF for the purpose of OTDOA positioning.

-	Reporting of General Error Situations. This function allows reporting of general error situations, for which function specific error messages have not been defined.

-	Assistance Information Transfer. This function allows the LMF to exchange information with the NG-RAN node for the purpose of assistance information broadcasting.

-	Positioning Information Transfer. This function allows the NG-RAN node to exchange positioning information with the LMF for the purpose of positioning.

-	Measurement Information Transfer. This function allows the LMF to exchange measurement information with the NG-RAN node for the purpose of positioning.

-	TRP Information Transfer. This function allows an LMF to obtain TRP related information from an NG-RAN node.

-	PRS Information Transfer. This function allows the LMF to exchange PRS related information with the NG-RAN node.

-	Measurement Preconfiguration Information Transfer. This function allows the LMF to request the NG-RAN node to preconfigure and activate measurement gap and/or PRS processing window.

-	Area-specific SRS Information Transfer. This function allows the LMF to notify the NG-RAN node about area-specific SRS configuration information.

-	Data Collection Information Transfer: This function allows the LMF to exchange positioning information with the NG-RAN node for the purpose of Data Collection.

The mapping between the above functions and NRPPa EPs is shown in the table below.

Table 7-1: Mapping between NRPPa functions and NRPPa EPs

| Function | Elementary Procedure(s) |
| --- | --- |
| E-CID Location Information Transfer | a) E-CID Measurement Initiation b) E-CID Measurement Failure Indication c) E-CID Measurement Report d) E-CID Measurement Termination |
| OTDOA Information Transfer | OTDOA Information Exchange |
| Assistance Information Transfer | a) Assistance Information Control b) Assistance Information Feedback |
| Reporting of General Error Situations | Error Indication |
| Positioning Information Transfer | a) Positioning Information Exchange b) Positioning Information Update c) Positioning Activation d) Positioning Deactivation |
| TRP Information Transfer | TRP Information Exchange |
| Measurement Information Transfer | a) Measurement b) Measurement Update c) Measurement Report d) Measurement Abort e) Measurement Failure Indication |
| PRS Information Transfer | PRS Configuration Exchange |
| Measurement Preconfiguration Information Transfer | Measurement Preconfiguration Measurement Activation |
| Area-specific SRS Information Transfer | SRS Information Reservation Notification |
| Data Collection Information Transfer | a) Data Collection Information Notification b) Data Collection Information Initiation b) Data Collection Information Update c) Data Collection Information Failure Indication |

# 8	NRPPa procedures

## 8.1	Elementary procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs.

Table 8.1-1: Class 1 Elementary Procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |
| --- | --- | --- | --- |
|  |  | Response message | Response message |
| E-CID Measurement Initiation | E-CID MEASUREMENT INITIATION REQUEST | E-CID MEASUREMENT INITIATION RESPONSE | E-CID MEASUREMENT INITIATION FAILURE |
| OTDOA Information Exchange | OTDOA INFORMATION REQUEST | OTDOA INFORMATION RESPONSE | OTDOA INFORMATION FAILURE |
| Positioning Information Exchange | POSITIONING INFORMATION REQUEST | POSITIONING INFORMATION RESPONSE | POSITIONING INFORMATION FAILURE |
| TRP Information Exchange | TRP INFORMATION REQUEST | TRP INFORMATION RESPONSE | TRP INFORMATION FAILURE |
| Measurement | MEASUREMENT REQUEST | MEASUREMENT RESPONSE | MEASUREMENT FAILURE |
| Positioning Activation | POSITIONING ACTIVATION REQUEST | POSITIONING ACTIVATION RESPONSE | POSITIONING ACTIVATION FAILURE |
| PRS Configuration Exchange | PRS CONFIGURATION REQUEST | PRS CONFIGURATION RESPONSE | PRS CONFIGURATION FAILURE |
| Measurement Preconfiguration | MEASUREMENT PRECONFIGURATION REQUIRED | MEASUREMENT PRECONFIGURATION CONFIRM | MEASUREMENT PRECONFIGURATION REFUSE |
| Data Collection Information Initiation | DATA COLLECTION INFORMATION REQUEST | DATA COLLECTION INFORMATION RESPONSE | DATA COLLECTION INFORMATION FAILURE |

Table 8.1-2: Class 2 Elementary Procedures

| Elementary Procedure | Initiating Message |
| --- | --- |
| E-CID Measurement Failure Indication | E-CID MEASUREMENT FAILURE INDICATION |
| E-CID Measurement Report | E-CID MEASUREMENT REPORT |
| E-CID Measurement Termination | E-CID MEASUREMENT TERMINATION COMMAND |
| Error Indication | ERROR INDICATION |
| Assistance Information Control | ASSISTANCE INFORMATION CONTROL |
| Assistance Information Feedback | ASSISTANCE INFORMATION FEEDBACK |
| Positioning Information Update | POSITIONING INFORMATION UPDATE |
| Measurement Report | MEASUREMENT REPORT |
| Measurement Update | MEASUREMENT UPDATE |
| Measurement Abort | MEASUREMENT ABORT |
| Measurement Failure Indication | MEASUREMENT FAILURE INDICATION |
| Positioning Deactivation | POSITIONING DEACTIVATION |
| Measurement Activation | MEASUREMENT ACTIVATION |
| SRS Information Reservation Notification | SRS INFORMATION RESERVATION NOTIFICATION |
| Data Collection Information Notification | DATA COLLECTION INFORMATION NOTIFICATION |
| Data Collection Information Update | DATA COLLECTION INFORMATION UPDATE |
| Data Collection Information Failure Indication | DATA COLLECTION INFORMATION FAILURE INDICATION |

Next change

## 8.X	Data Collection Information Transfer

### 8.X.1	Data Collection Information Notification

#### 8.X.1.1	General

The Data Collection Information Notification procedure allows the LMF to indicate the NG-RAN node that positioning information for a list of UEs is available for data collection in a specific area.

#### 8.X.1.2	Successful Operation

Figure 8.5.1.2.1: Data Collection Information Notification procedure. Successful operation.

The LMF initiates the procedure by sending a DATA COLLECTION INFORMATION NOTIFICATION message to the NG-RAN node. The Data Collection Information List IE in the DATA COLLECTION INFORMATION NOTIFICATION message contains the positioning information for a list of UEs that can be requested for data collection.

#### 8.X.1.3	Unsuccessful Operation

Not applicable.

#### 8.X.1.4	Abnormal Conditions

Void.

### 8.X.2	Data Collection Information Initiation

#### 8.X.2.1	General

The Data Collection Information Initiation procedure allows the NG-RAN to request positioning information from the LMF for data collection.

#### 8.X.2.2	Successful Operation

Figure 8.X.2.2.1: Data Collection Information Exchange procedure. Successful operation.

The NG-RAN initiates the procedure by sending a DATA COLLECTION INFORMATION REQUEST message to the LMF. If the Registration Request for Data Collection Information IE is set to "start", the LMF shall reply with a DATA COLLECTION INFORMATION RESPONSE message and shall initiate the Data Collection Information Update procedure including the list of requested positioning information indicated in the Requested Positioning Information List IE and in the area of interest indicated in the TRP Area list IE.

If the Periodicity IE is included in the Requested Positioning Information Item IE, the LMF shall, if supported, consider is as the periodicity of reporting the requested Positioning information.

#### 8.X.2.3	Unsuccessful Operation

Figure 8.X.2.3.1: Data Collection Information Exchange procedure. Unsuccessful operation.

If the LMF cannot provide any of the requested positioning information in the Requested Positioning Information List IE indicated in the DATA COLLECTION INFORMATION REQUEST message, it shall respond with a DATA COLLECTION INFORMATION FAILURE message with an appropriate cause value.

#### 8.x.2.4	Abnormal Conditions

Void.

### 8.X.3	Data Collection Information Update

#### 8.X.3.1	General

The Data Collection Information Update procedure allows the LMF to provide positioning information to the NG-RAN for data collection in a specific area.

#### 8.X.3.2	Successful Operation

Figure 8.X.3.2.1: Data Collection Information Update procedure. Successful operation.

The LMF initiates the procedure by sending a DATA COLLECTION INFORMATION UPDATE message to the NG-RAN node. The DATA COLLECTION INFORMATION UPDATE message contains the requested positioning information for data collection at NG-RAN in the area indicated by the TRP Area List IE.

#### 8.X.3.3	Unsuccessful Operation

Not applicable.

#### 8.X.3.4	Abnormal Conditions

Void.

### 8.X.4	Data Collection Information Failure Indication

#### 8.X.4.1	General

The Data Collection Information Failure Indication procedure allows the LMF to indicate that it cannot provide any data collection information to the NG-RAN node.

#### 8.X.4.2	Successful Operation

Figure 8.X.4.2.1: Data Collection Information Failure Indication procedure. Successful operation.

The LMF initiates the procedure by sending a DATA COLLECTION INFORMATION FAILURE INDICATION message to the NG-RAN node.

#### 8.X.4.3	Unsuccessful Operation

Not applicable.

#### 8.X.4.4	Abnormal Conditions

Void.

Next change

### 8.5.1	Measurement

#### 8.5.1.1	General

The Measurement procedure allows the LMF to request one or more TRPs in the NG-RAN node to perform and report positioning measurements. This procedure applies only if the NG-RAN node is a gNB.

#### 8.5.1.2	Successful Operation

Figure 8.5.1.2.1: Measurement procedure. Successful operation.

The LMF initiates the procedure by sending a MEASUREMENT REQUEST message to the NG-RAN node, indicating in the TRP Measurement Request List IE the TRP(s) from which measurements are requested. The NG-RAN node shall use the included information to configure positioning measurements by the indicated TRP(s). If at least one of the requested measurements has been successful for at least one of the TRPs, the NG-RAN node shall reply with a MEASUREMENT RESPONSE message including the TRP Measurement Response List IE.

If the Report Characteristics IE is set to "OnDemand", the NG-RAN node shall return the corresponding measurement results in the MEASUREMENT RESPONSE message, and the LMF shall consider that this reporting has been terminated by the NG-RAN node. If the Report Characteristics IE is set to "Periodic", the NG-RAN node shall initiate the corresponding measurements, and it shall reply with the MEASUREMENT RESPONSE message without including any measurement results in the message. The NG-RAN node shall then periodically initiate the Measurement Report procedure for the corresponding measurements, with the requested reporting periodicity.

If the Measurement Beam Information Request IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall include the Measurement Beam Information IE in the TRP Measurement Result IE of the MEASUREMENT RESPONSE message.

If the Measurement Quality IE is included in the TRP Measurement Result IE in the MEASUREMENT RESPONSE message, the LMF may take it into account as the TRP estimate of the measurement quality. If the Measurement Quality IE includes the Zenith Quality IE, the LMF may take it into account within the angle measurement quality.

If the Timing Reporting Granularity Factor IE is included in the TRP Measurement Quantities IE in the MEASUREMENT REQUEST message, the NG-RAN node may take it into account when configuring measurements including UL RTOA and gNB Rx-Tx Time Difference.

If the System Frame Number IE and/or the Slot Number IE are included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, consider that the respective information indicates the activation time of SRS transmission.

If the Report Characteristics IE is set to "OnDemand" and the Response Time IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, return the corresponding measurement results in the MEASUREMENT RESPONSE message within the indicated time.

If the Measurement Characteristics Request Indicator IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, take the requested measurement characteristics into account when configuring measurements, and include the requested information, if available, in the MEASUREMENT RESPONSE message.

If the Number of TRP Rx TEGs IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, use it to measure the same SRS resource with different TRP Rx TEGs for the indicated TRP, and report the corresponding UL-RTOA and/or gNB Rx-Tx time difference measurements.

If the Number of TRP RxTx TEGs IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, use it to measure the same SRS resource with different TRP RxTx TEGs with the same TRP Tx TEG for the indicated TRP, and report the corresponding gNB Rx-Tx time difference measurements.

If the Measurement Time Occasion IE is included in the MEASUREMENT REQUEST message, the NG-RAN node may take it into account as the number of SRS measurement time occasions for a measurement instance.

If the Time Window Information Measurement List IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, measure the UL SRS resources from the UE within the indicated time window(s).

If the Data Sampling Parameters IE is included in the TRP Measurement Quantities IE in the MEASUREMENT REQUEST message, the NG-RAN node may take it into account when configuring sample-based measurement. [FFS]

If the AI/ML measurement IE is included in the TRP Measurement Result IE in the MEASUREMENT RESPONSE message, the LMF shall, if supported, consider that the associated measurement has been derived using AI/ML inference.

Next change

### 9.1.4	Messages for Measurement Information Transfer Procedures

#### 9.1.4.1	MEASUREMENT REQUEST

This message is sent by the LMF to request the NG-RAN node to configure a positioning measurement.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| TRP Measurement Request List |  | 1 |  |  | YES | reject |
| >TRP Measurement Request Item |  | 1..<maxnoofMeasTRPs> |  |  | - |  |
| >>TRP ID | M |  | 9.2.24 |  | - |  |
| >>Search Window Information | O |  | 9.2.26 |  | - |  |
| >>Cell ID | O |  | NR CGI 9.2.9 | The Cell ID of the TRP identified by the TRP ID IE. | YES | ignore |
| >>AoA Search Window Information | O |  | UL-AoA Assistance Information 9.2.66 |  | YES | ignore |
| >>Number of TRP Rx TEGs | O |  | ENUMERATED (2, 3, 4, 6, 8, …) |  | YES | ignore |
| >>Number of TRP RxTx TEGs | O |  | ENUMERATED (2, 3, 4, 6, 8, …) |  | YES | ignore |
| Report Characteristics | M |  | ENUMERATED (OnDemand, Periodic, ...) |  | YES | reject |
| Measurement Periodicity | C-ifReportCharacteristicsPeriodic |  | ENUMERATED (120ms, 240ms, 480ms, 640ms, 1024ms, 2048ms, 5120ms, 10240ms, 1min, 6min, 12min, 30min, 60min,…, 20480ms, 40960ms, extended) | The codepoint 120ms, 240ms, 480ms, 1024ms, 2048ms, 1min, 6min, 12min, 30min, and 60min are not applicable | YES | reject |
| TRP Measurement Quantities |  | 1 |  |  | YES | reject |
| >TRP Measurement Quantities Item |  | 1 .. <maxnoPosMeas> |  |  | - |  |
| >>TRP Measurement Type | M |  | ENUMERATED (gNB-RxTxTimeDiff, UL-SRS-RSRP, UL-AoA, UL-RTOA,…, Multiple UL-AoA, UL SRS-RSRPP, UL-RSCP) | The UL-RSCP measurement is applicable only when the UL-RTOA and/or gNB-RxTxTimeDiff measurement(s) is also requested. | - |  |
| >>Timing Reporting Granularity Factor | O |  | INTEGER (0..5) | Value (0..5) corresponds to (k0..k5) TS 38.133 [16]. This IE is ignored when the Timing Reporting Granularity Factor Extended IE is included. | - |  |
| >>Timing Reporting Granularity Factor Extended | O |  | INTEGER (-6..-1, …) | Value -6 corresponds to kminus6, value -5 corresponds to kminus5 and so on, see TS 38.133 [16] | - |  |
| >>Data Sampling Parameters (FFS) | O |  |  |  | - |  |
| >>>Nt |  |  |  | Size of the time window over which measurements are collected. FFS on the granularity of the time information |  |  |
| >>>Nt’ |  |  |  | Number of measurement samples to be collected |  |  |
| >>>k |  |  |  | Time between different measurement samples. FFS on the granularity of the time information |  |  |
| SFN initialisation Time | O |  | Relative Time 1900 9.2.36 | If this IE is not present, the TRP may assume that the value is same as its own SFN initialisation time. | YES | ignore |
| SRS Configuration | O |  | 9.2.28 |  | YES | ignore |
| Measurement Beam Information Request | O |  | ENUMERATED (true,...) | This IE is ignored when the Measurement Characteristics Request Indicator IE is included. | YES | ignore |
| System Frame Number | O |  | INTEGER(0..1023) |  | YES | ignore |
| Slot Number | O |  | INTEGER(0..79) |  | YES | ignore |
| Measurement Periodicity Extended | C-ifMeasPerExt |  | ENUMERATED (160ms, 320ms, 1280ms, 2560ms, 61440ms, 81920ms, 368640ms, 737280ms, 1843200ms, …) |  | YES | reject |
| Response Time | O |  | 9.2.68 | This IE is ignored when the Report Characteristics IE is set to “periodic”. | YES | ignore |
| Measurement Characteristics Request Indicator | O |  | 9.2.81 |  | YES | ignore |
| Measurement Time Occasion | O |  | ENUMERATED (o1, o4, …) |  | YES | ignore |
| Measurement Amount | O |  | ENUMERATED (0, 1, 2, 4, 8, 16, 32, 64) | This IE is ignored if the Report Characteristics IE is set to ‘OnDemand’. Value 0 represents an infinite number of periodic reporting. | YES | ignore |
| Time Window Information Measurement List | O |  | 9.2.91 |  | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifReportCharacteristicsPeriodic | This IE shall be present if the Report Characteristics IE is set to the value "Periodic". |
| ifMeasPerExt | This IE shall be present if the Measurement Periodicity IE is set to the value "extended". |

| Range bound | Explanation |
| --- | --- |
| maxnoPosMeas | Maximum no. of measured quantities that can be configured and reported with one positioning measurement message. Value is 16384. |
| maxnoofMeasTRPs | Maximum no. of TRPs that can be included within one message. Value is 64. |

Next change

### 9.2.37	TRP Measurement Result

This information element contains the measurement result.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Measured Result Item |  | 1 .. <maxnoPosMeas> |  |  | - |  |
| >CHOICE Measured Results Value | M |  |  |  | - |  |
| >>UL Angle of Arrival |  |  | 9.2.38 |  |  |  |
| >>UL SRS-RSRP |  |  | INTEGER (0..126) |  |  |  |
| >>UL RTOA Measurement |  |  | 9.2.39 |  |  |  |
| >>gNB Rx-Tx Time Difference |  |  | 9.2.40 |  |  |  |
| >>Z-AoA |  |  | 9.2.67 |  | YES | reject |
| >>Multiple UL-AoA |  |  | 9.2.71 |  | YES | reject |
| >>UL SRS-RSRPP |  |  | 9.2.72 |  | YES | reject |
| >>UL RSCP |  |  | 9.2.92 |  | YES | reject |
| >Time Stamp | M |  | 9.2.42 |  | - |  |
| >Measurement Quality | O |  | 9.2.43 |  | - |  |
| >Measurement Beam Information | O |  | 9.2.57 |  | - |  |
| >SRS Resource type | O |  | 9.2.73 |  | YES | ignore |
| >ARP ID | O |  | 9.2.75 |  | YES | ignore |
| >LoS/NLoS Information | O |  | 9.2.77 |  | YES | ignore |
| >Mobile TRP Location Information | O |  | 9.2.88 |  | YES | ignore |
| >Measured Frequency Hops | O |  | ENUMERATED (singleHop, multiHop, …) |  | YES | ignore |
| >Aggregated Positioning SRS Resource ID List |  | 0..1 |  | Indicates the used Positioning SRS resources across aggregated carriers. | YES | ignore |
| >>Aggregated Positioning SRS Resource ID Item |  | 2..< maxnoaggregatedPosSRS-Resources > |  |  | - |  |
| >>>Positioning SRS Resource ID | M |  | INTEGER (0..63) |  |  |  |
| >>>Point A | M |  | INTEGER (0..3279165) |  | YES | ignore |
| >>>SCS Specific Carrier |  | 1 |  |  | YES | ignore |
| >>>>Offset To Carrier | M |  | INTEGER (0..2199, …) |  |  |  |
| >>>>Subcarrier Spacing | M |  | ENUMERATED(kHz15, kHz30, kHz60, kHz120, …, kHz480, kHz960) |  |  |  |
| >>>>Carrier Bandwidth | M |  | INTERGER (1..275, …) |  |  |  |
| >>>PCI | O |  | INTEGER (0..1007) |  | YES | ignore |
| >Measurement Based On Aggregated Resources | O |  | ENUMERATED(true, …) |  | YES | ignore |
| >AI/ML measurement | O |  | ENUMERATED(true, …) | This IE is only valid for UL-RTOA and gNB Rx-Tx Time Difference [FFS] | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoPosMeas | Maximum no. of measured quantities that can be configured and reported with one positioning measurement message. Value is 16384. |
| maxnoaggregatedPosSRS-Resources | Maximum no of aggregated Positioning SRS resources per UL BWP. Value is 3. |

Next change

### 9.1.X	Messages for Data Collection Information Transfer Procedures

#### 9.1.X.1	DATA COLLECTION INFORMATION NOTIFICATION

This message is used to indicate the NG-RAN node that positioning information is available for data collection.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | ignore |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| TRP Area List |  | 1 |  | Indicates the available area for data collection | YES | ignore |
| >TRP Area Item |  | 1..<maxnoofMeasTRPs> |  |  | EACH | ignore |
| >>TRP ID | M |  | 9.2.24 |  | - |  |
| Data Collection Information List |  | 1 |  |  | YES | reject |
| >Data Collection Information Item |  | 1 .. <maxnoDataCCollectionTypes> |  |  | EACH | reject |
| >>UE ID | M |  | INTEGER(1..N) |  | - |  |
| >>CHOICE Information Type | M |  |  |  |  |  |
| >>>UE location | M |  | ENUMERATED(true,…) |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoDataCollectionTypes | Maximum no. Data Collection information types that can be requested and reported from the LMF. Value is 64. |
| maxnoofMeasTRPs | Maximum no. of TRPs that can be included within one message. Value is 64. |

#### 9.1.X.2	DATA COLLECTION INFORMATION REQUEST

This message is initiated to request positioning information for data collection.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| Registration Request for Data Collection Information | M |  | ENUMERATED(start, stop, …) | Type of request for which the information is required. | YES | reject |
| TRP Area List |  | 1 |  | Indicates the area of interest | YES | ignore |
| >TRP Area Item |  | 1..<maxnoofMeasTRPs> |  |  | EACH | ignore |
| >>TRP ID | M |  | 9.2.24 |  | - |  |
| Requested Positioning Information List |  | 1 |  |  | YES | reject |
| >Requested Positioning Information Item |  | 1 .. <maxnoDataCCollectionTypes> |  |  | EACH | reject |
| >>UE ID | O |  | INTEGER(1..N) |  | - |  |
| >>CHOICE Information Type | M |  |  |  |  |  |
| >>>UE location | M |  | ENUMERATED(true,…) |  | - |  |
| >>Periodicity | O |  | ENUMERATED(160ms, 320ms, 1280ms, 2560ms, 61440ms, 81920ms, 368640ms, 737280ms, …) |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoDataCollectionTypes | Maximum no. Data Collection information types that can be requested and reported from the LMF. Value is 64. |
| maxnoofMeasTRPs | Maximum no. of TRPs that can be included within one message. Value is 64. |

#### 9.1.X.3	DATA COLLECTION INFORMATION RESPONSE

This message is sent to confirm start reporting of the requested positioning information for data collection.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| Criticality Diagnostics | O |  | 9.2.2 |  | YES | ignore |

#### 9.1.X.3	DATA COLLECTION INFORMATION FAILURE

This message is sent to report failure of initiating data collection of positioning information.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| Cause | M |  | 9.2.1 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.2 |  | YES | ignore |

#### 9.1.X.3	DATA COLLECTION INFORMATION UPDATE

This message is sent to report positioning information for data collection.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| TRP Area List |  | 1 |  | Indicates the available area | YES | ignore |
| >TRP Area Item |  | 1..<maxnoofMeasTRPs> |  |  | EACH | ignore |
| >>TRP ID | M |  | 9.2.24 |  | - |  |
| Requested Positioning Information List |  | 1 |  |  | YES | reject |
| >Requested Positioning Information Item |  | 1 .. <maxnoDataCCollectionTypes> |  |  | EACH | reject |
| >>UE ID | O |  | INTEGER(1..N [FFS]) |  | - |  |
| >>CHOICE Data Collection Information Type | M |  |  |  | - |  |
| >>>UE location | M |  |  |  |  |  |
| >>>>Location Information | M |  | OCTET STRING | Location of the mobile TRP, Includes the locationEstimate IE as defined in TS 37.355 [14]. | - |  |
| >>>>Time Stamp | O |  | 9.2.42 | Location time stamp. | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoDataCollectionTypes | Maximum no. Data Collection information types that can be requested and reported from the LMF. Value is 64. |
| maxnoofMeasTRPs | Maximum no. of TRPs that can be included within one message. Value is 64. |

#### 9.1.X.4	DATA COLLECTION INFORMATION FAILURE INDICATION

This message is sent to indicate that the previously requested positioning information can no longer be reported. for data collection.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| Cause | M |  | 9.2.1 |  | YES | ignore |

Next change

ASN.1 to be added later