3GPP TSG-RAN WG3 Meeting #126	R3-247588

Orlando, Florida, U.S., 18th – 22nd November 2024

Agenda Item:	16.4

Source:	Ericsson

Title:	[TP for TR 38.769] On locating AIoT devices

Document for:	Discussions & Approval

# 1	Introduction

This document provides a text proposal for TR 38.769 which contains reasoning for concluding that “reader ID” is not sufficient for actual use cases.

# 2	Text Proposal for TR 38.769 v1.1.0 based on R3-247036/R3-245849

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

## 6.9	Locating ambient IoT devices

### 6.9.1	General

Editor’s note: Corresponds to the third RAN3 objective in the SID, to identify potential solutions for locating an Ambient IoT device with no specification impact, e.g., reusing existing user location report, or minimal specification impact to convey location information to core network.

A-IoT device location information may be used for the following purposes:

(a) improving the A-IoT operation itself, e.g., by A-IoT CN sending a Command to one or more readers (e.g., the last reader(s)) associated to the device rather than sending it blindly.

(b) providing location information to the consumer of the A-IoT service.

Locating an Ambient IoT device at “reader ID granularity” is useful for both purposes, but is likely not sufficient for purpose (b).

For topology 1, A-IoT RAN node ID can be considered as a location of A-IoT device. For topology 2, UE ID of the A-IoT-enabled UE can be considered as a location of A-IoT device.

Editor’s Note 1:	How to know the “reader” location is FFS. Whether to use more than one “readers” within one A-IoT RAN for location purposes is FFS.

Editor’s Note 2:	Signalling details on how to provide the location information of the A-IoT device to the A-IoT CN needs further study.

### 6.9.2	Locating AIoT devices with finer accuracy

It can be assumed that the Common reader function’s location is available or can be made available to the AIoT enabled 5GS.

Further, it can be expected that for the scenarios in the ongoing study, an AIoT device’s location in “Reader ID granularity” is known with an accuracy in the range of the projected communication range of AIoT devices, i.e. within several tens of meters.

Locating AIoT devices should also take into account requirements to cope with a related AIoT device density of ~1,5 devices per square meters (see TS 22.369 [10]) as well as the fact that some of the indoor deployment scenarios may need to cope with readers deployed in a 3D setup.

It can be concluded that providing an AIoT device location with “Reader ID granularity” may only give a rough approximation of the AIoT device’s location, basically only sufficient for purposes summarized under item (a) in section 6.9.1, but not suitable for avoiding (human) labour intensive and time-consuming operations and not suitable for performing subsequent automated operations, e.g. retrieval of goods for further transportation by automatic means.

For purpose (b), i.e. providing location information to the consumer of the A-IoT service, further improvement of the locating accuracy may be achieved by involving multiple readers for e.g. measuring the power of the backscattered signal. With regards to necessary RAN functions, in topology 1, this approach can be realized by means of AIoT RAN node internal functions, in topology 2, this approach would require at least specification work on RRC.

How to integrate functions to locate AIoT devices into the 5GS positioning architecture may be part of future studies and work.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>