3GPP TSG-RAN WG3 Meeting #126	R3-247598

Orlando, USA, 18 – 22 November 2024

Agenda Item:		19.2

Source:		Charter Communications

Title:	SBFD related CLI measurement configuration, CLI mitigation and CLI testing

Document for:		Discussion and agreement

# Introduction

According to the WID RP-241614 [1] the following SBFD related issues are to be handled by RAN3:

- Specify enhancement for CLI handling [RAN1, RAN2, RAN3]

  - Information exchange among gNBs, including [RAN3]

    - Semi-static cell-specific SBFD time and frequency location configuration

    - Measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS

    - Strongest DL beam information

    - CLI-mitigation request

RAN3 has recently received liaison R3-2450006 / R1-2407533 from RAN1, which indicates:

- From RAN1 perspective, exchange among gNBs of the configuration info for a set of one or more periodic NZP CSI RS resources (relevant IE in 38.331 are NZP-CSI-RS-Resource, NZP-CSI-RS-ResourceSet) is supported.

- Strongest DL beam information in the context of NZP-CSI-RS is defined as a CSI-RS Resource Indicator (CRI) value. CRI is a relative index in the range [1..N], where N is the number of CSI-RS resources within a set of resources for which the configuration is provided to a gNB.

- Strongest DL beam information in the context of SSBs is defined as an SSB index.  SSB index is an absolute index among the SSB(s) for which the configuration is provided to a gNB.

- Note: RAN1 will not discuss information exchange among gNBs for gNB-gNB CLI handling unless there is an LS from RAN3.

At RAN3 #125-bis meeting in Hefei, China, it was decided that:

RAN3 agrees to exchange the following information over Xn interface and F1 interface:

- Semi-static cell-specific SBFD time and frequency location configuration with cell granularity (FFS on container or explicit IEs over XnAP and F1AP)

- Measurement resource configuration

- Strongest DL beam information (FFS on the details)

This contribution addresses CLI measurement resource configuration, CLI mitigation request, and CLI testing.

# Discussion

We believe that CLI mitigation and testing procedures, must be flexible enough to allow the action to be started / stopped at any particular point in time, to provide whatever testing parameters are necessary, and to provide the result of testing to the requesting entity. The existing Xn messages RESOURCE STATUS REQUEST, RESOURCE STATUS UPDATE and the corresponding F1 RESOURCE STATUS REQUES, RESOURCE STATUS UPDATE, with appropriate modifications specific to the CLI mitigation / testing would support that functionality.

The existing RESOURCE STATUS REQUEST message already contains the infrastructure to:

- start / stop a particular request;

- which cell should report the results,

- which SSB should report the results;

- a bitmap to identify one or more types of requests.

- a field to specify the periodity of reporting, if so desired

Proposal 1: The existing Xn RESOURCE STATUS REQUEST and RESOURCE STATUS UPDATE, appropriately modified as proposed in Annex A, shall be used to:

- configure CLI testing measurement configuration (such as particular NZP-CSI RS, SSB, etc)

- request CLI mitigation,

- request CLI testing

- report the CLI test results (such as strongest downlink beam information or a list of downlink beams and their respective measured interference power, etc) back to the requesting gNB.

Proposal 2: In case of a disaggregated gNB architecture, the existing F1 RESOURCE STATUS REQUEST and RESOURCE STATUS UPDATE messages, appropriately modified as suggested in Annex B, shall be used to request CLI mitigation, request CLI testing and provide the appropriate measurement resource configuration (such as particular CSI-RS, SSB, SRS, etc), and to report the results (such as strongest downlink beam information, or a list of downlink beams and their respective measured interference power, etc) back to the requesting CU.

  - CLI measurement configuration

Proposal 3: A gNB can send the configuration of its CLI testing signals to a neighbor gNB by sending a Xn RESOURCE STATUS REQUEST with:

- the existing “Registration Request” IE set to the new value of “configure”,

- the existing “ReportCharacteristics” IE set to the new value of “CLI testing”

- not including any “Cell To Report List” IE

- not including the “Reporting Periodicity” IE

- including the new “CLI measurement configuration” IE

After a gNB1 sends its “CLI measurement configuration” (i.e. the configuration of its NZP-CSI RS per served cell and/or SSB signals per served beam, that will be used for any future CLI test) to a gNB2, then gNB2 can measure on its own (i.e., without being requested by gNB1 to perform any CLI testing), the level of CLI that any of its cells and/or beams is experiencing from specific cells and/or beams of gNB1.

  - CLI-mitigation request

Proposal 4: A victim gNB can request an aggressor gNB to start CLI mitigation action by sending a RESOURCE STATUS REQUEST with:

- the existing “Registration Request” IE set to “start”,

- the existing “ReportCharacteristics” IE set to the new value to indicate “Mitigation Request”,

- the existing “Cell to Report List” IE including the list of cells of the neighboring gNB that is causing the unwanted CLI that the victim gNB wants mitigated,

- the existing “SSB to Report List” IE including the list of SSBs of the neighboring gNB that is causing the unwanted CLI that the victim gNB wants mitigated,

- the new “CLI Mitigation Information” IE which includes

  - in the case of cell-level testing, the affected cell ID, the CLI power measured by the victim gNB cell, and the CRI of the NZP-CSI RS Configuration information.

  - In the case of SSB-level testing, the cell ID which contains the SSB, the affected SSB index, and the corresponding CLI Power.

Proposal 5: A gNB receiving a RESOURCE STATUS REQUEST that requests the start of “CLI Mitigation” shall respond with either a RESOURCE STATUS RESPONSE, if it in fact starts mitigation actions, or a RESOURCE STATUS FAILURE, if it does not start any mitigation action.

Proposal 6: A gNB that has previously requested start of CLI mitigation, and received acknowledgment that CLI mitigation has been started, but which still detects unacceptable level of CLI can send a RESOURCE STATUS REQUEST with

- the existing “Registration Request” IE set to the new value of “continue”,

- the existing “Report Characteristics” IE set to the new value to indicate “Mitigation Request”

- the existing “Cell to Report List” and/or existing “SSB to Report List” IE, including the appropriate cells/SSBs for which it wants the next level of mitigation to be applied.

Proposal 7: A gNB that has received a RESOURCE STATUS REQUEST requesting that CLI mitigation be continued, shall respond with a RESOURCE STATUS RESPONSE, if it can or is willing to apply the next level of CLI mitigation, or with a RESOURCE STATUS FAILURE otherwise.

Proposal 8: A victim gNB that has requested CLI mitigation, and that eventually is satisfied with the level of CLI mitigation applied by the aggressor, shall send a RESOURCE STATUS REQUEST indicating that the CLI mitigation is to be stopped.

Proposal 9: A gNB receiving a RESOURCE STATUS REQUEST that requests the stop of “CLI Mitigation” shall stop the CLI mitigation actions and respond with a RESOURCE STATUS RESPONSE.

  - CLI testing

Proposal 10: If a gNB wants to be proactive and want to start a CLI testing in order to find out how a particular configuration of its power level, antenna setting, etc might impact neighboring gNBs can start a CLI testing by sending a RESOURCE STATUS REQUEST with

- the existing “Registration Request” IE set to “start”,

- the existing “Report Characteristics” IE set to the new value of “CLI Testing”,

- the existing “Cell to Report List” IE including the list of cells of the neighbor gNB that it is requesting to perform the tests,

- the existing “SSB to Report List” IE including the list of SSBs, if any, that it is requesting to perform the tests.

- If configuration of the CLI testing signals had not been previously done, it can be implicitly done at the same time that CLI testing is being requested by including the new “CLI measurement configuration” IE

Proposal 11: If a gNB receives a RESOURCE STATUS REQUEST message requesting a CLI test, it shall respond with a RESOURCE STATUS RESPONSE, if it can in fact start the test, or with a RESOURCE STATUS FAILURE, if it cannot start the testing.

Proposal 12: When a gNB completes a CLI testing which was requested by a neighboring gNB, it shall reply to the neighboring gNB with a RESOURCE STATUS UPDATE message that includes the new IE “Cell CLI Resource Status”, which contains the test results.

Proposal 13: The gNB that requested the starting of CLI testing can request that the testing be stopped, if it so desires.

  - Handling disaggregated architecture

Proposal 14: In the case of a disaggregated gNB, a CU that has received a Xn RESOURCE STATUS REQUEST requesting the start or stop of an action shall send to the proper DU the corresponding F1 RESOURCE STATUS REQUEST.

Proposal 15: In the case of a disaggregated gNB, a DU that has received a F1 RESOURCE STATUS REQUEST message requesting the start of an action shall respond to the sending CU with either a F1 RESOURCE STATUS RESPONSE, if it in fact starts the action, or a F1 RESOURCE STATUS FAILURE, if it does not start any action.

Proposal 16: In the case of a disaggregated gNB, a DU receiving a F1 RESOURCE STATUS REQUEST that requests the stop of an action shall stop the action and respond with a F1 RESOURCE STATUS RESPONSE.

Proposal 17: In the case of a disaggregated gNB, a CU that has received a F1 RESOURCE STATUS RESPONSE, or F1 RESOURCE STATUS FAILURE, or F1 RESOURCE STATUS UPDATE message from a DU, should send to the neighboring gNB, the corresponding Xn RESOURCE STATUS RESPONSE or Xn RESOURCE STATUS FAILURE or Xn RESOURCE STATUS UPDATE message, respectively.

# Annex A – Changes to XnAP (TS38.423)

#### 9.1.3.18	RESOURCE STATUS REQUEST

This message is sent by NG-RAN node1 to NG-RAN node2 to initiate the requested measurement according to the parameters given in the message.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | C-ifRegistrationRequestStoporAdd |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | ignore |
| Registration Request | M |  | ENUMERATED(start, stop, add, continue, configure,…) | Type of request for which the resource status is required. | YES | reject |
| Report Characteristics | C-ifRegistrationRequestStart |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates measurement object the NG-RAN node2 is requested to report. First Bit = PRB Periodic, Second Bit = TNL Capacity Ind Periodic, Third Bit = Composite Available Capacity Periodic, Fourth Bit =Number of Active UEs Periodic, Fifth Bit =RRC connections Periodic, Sixth Bit = NR-U Channel List Periodic. Seventh Bit = CLI Testing Eighth Bit = CLI Mitigation Other bits shall be ignored by the NG-RAN node2. | YES | reject |
| Cell To Report List |  | 0..1 |  | Cell ID list to which the request applies. | YES | ignore |
| >Cell To Report Item |  | 1 .. <maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | – |  |
| >>CLI Mitigation Information | C-ifMitigationRequestStartCont |  | 9.2.2.x |  | -- |  |
| >>SSB To Report List |  | 0..1 |  | SSB list to which the request applies. | – |  |
| >>>SSB To Report Item |  | 1 .. < maxnoofSSBAreas> |  |  | – |  |
| >>>>SSB-Index | M |  | INTEGER (0..,63..) |  | – |  |
| >>>>CLI Mitigation Information | C-ifMitigationRequestStartCont |  | 9.2.2.x |  | -- |  |
| >>Slice To Report List |  | 0..1 |  | S-NSSAI list to which the request applies. | – |  |
| >>>Slice To Report Item |  | 1 .. < maxnoofBPLMNs > |  |  | – |  |
| >>>>PLMN Identity | M |  | 9.2.2.4 | Broadcast PLMN | – |  |
| >>>>S-NSSAI List |  | 1 |  |  | – |  |
| >>>>>S-NSSAI Item |  | 1 .. < maxnoofSliceItems> |  |  | – |  |
| >>>>>>S-NSSAI | M |  | 9.2.3.21 |  | – |  |
| Reporting Periodicity | O |  | ENUMERATED(500ms, 1000ms, 2000ms, 5000ms, 10000ms, …) | Periodicity that can be used for reporting of indicated measurements. Also used as the averaging window length for all measurement object if supported. This IE is ignored if the Registration Request IE is set to "add". | YES | Ignore |
| CLI Measurement Configuration | C-ifConfigStartCLITesting |  | 9.2.2.y |  | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifRegistrationRequestStoporAdd | This IE shall be present if the Registration Request IE is set to the value "stop" or "add". |
| ifRegistrationRequestStart | This IE shall be present if the Registration Request IE is set to the value "start". |
| ifConfigStartCLITesting | This IE shall be present if the RegistrationRequest has the values “Configure”, or “Start”, and the ReportCharacteristics IE is set to the value “CLI Testing” |
| ifMitigationRequestStartCont | This IE shall be present if the Report Characteristics IE is set to “CLI Mitigation” AND the Registration Request IE is set to either “start” or “continue”. |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a NG-RAN node cell. Value is 64. |
| maxnoofSliceItems | Maximum no. of signalled slice support items. Value is 1024. |

--------------------------------Next Change--------------------------------

#### 9.1.3.21	RESOURCE STATUS UPDATE

This message is sent by NG-RAN node2 to NG-RAN node1 to report the results of the requested measurements.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Cell Measurement Result |  | 1 |  |  | YES | ignore |
| >Cell Measurement Result Item |  | 1 .. < maxnoofCellsinNG-RANnode > |  |  | YES | ignore |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | – |  |
| >> CLI Resource Status | O |  | 9.2.2.z |  |  |  |
| >>Radio Resource Status | O |  | 9.2.2.50 |  | – |  |
| >>TNL Capacity Indicator | O |  | 9.2.2.49 |  | – |  |
| >>Composite Available Capacity Group | O |  | 9.2.2.51 |  | – |  |
| >>Slice Available Capacity | O |  | 9.2.2.55 |  | – |  |
| >>Number of Active UEs | O |  | 9.2.2.62 |  | –- |  |
| >>RRC Connections | O |  | 9.2.2.56 |  | – |  |
| >>NR-U Channel List |  | 0..1 |  |  | YES | ignore |
| >>>NR-U Channel Item |  | 1..<maxnoofNR-UChannelIDs> |  |  | – |  |
| >>>>NR-U Channel ID | M |  | INTEGER (1.. maxnoofNR-UchannelIDs, …) | The NR-U channel utilised in the last reporting period | – |  |
| >>>>Channel occupancy time percentage DL | M |  | INTEGER (0..100) | The percentage of time for which the channel resources have been utilised for DL traffic served by the corresponding NR-U Channel of the serving cell. Value 100 indicates that the channel resources have been utilized for DL traffic served by the corresponding NR-U Channel of the serving cell for the whole duration between consecutive reporting. | – |  |
| >>>>Energy Detection Threshold DL | M |  | INTEGER (-100..-50,…) | Average ED Threshold used for DL channel sensing at the gNB. Value is in dBm. | – |  |
| >>>>Channel Occupancy Time Percentage UL | O |  | INTEGER (0..100) | The percentage of time for which the channel resources have been utilised for UL traffic served by the corresponding NR-U Channel of the serving cell for UEs that transmit to the serving cell. Value 100 indicates that the channel resources have been utilized for UL traffic served by the corresponding NR-U Channel of the serving cell for the whole duration between consecutive reporting. | YES | ignore |
| >>>>Energy Detection Threshold UL | O |  | INTEGER (-100..-50,…) | Indicates the average of the maximum ED Threshold configured by the gNB for UL channel sensing. Value is in dBm. | YES | ignore |
| >>>>Radio Resource Status NR-U | O |  | 9.2.2.104 | Indicates the radio resource status per NR-U channel. | YES | Ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| maxnoofNR-UchannelIDs | Maximum no. NR-U channel IDs in a cell. Value is 16. |

--------------------------------Next Change--------------------------------

#### 9.2.2.x	CLI Mitigation Information

This IE contains information that may be used to mitigate CLI across gNBs.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CLI Information |  |  |  |  |
| >CLI Power detected | M | [0..X] | INTEGER (0..X) | Number of dBm of CLI power |
| >CRI | O |  | INTEGER (1..64) | Includes the CSI-RS Resource Indicator (CRI) value. CRI is an index indicating a CSI-RS resource within a set of resources |
| >SSB Index | O |  | INTEGER (0..63) | Identifier of the SSB beam. |

--------------------------------Next Change--------------------------------

9.2.2.y CLI Measurement Configuration

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CLITestType | M | ENUMERATED(gNB-to-gNB, …) |  |  |
| ServedCellList | 1 .. <maxnoofCellsinNG-RANnode> |  |  |  |
| >ServedCellItem | M |  |  |  |
| >>NZP-CSI-RS Configuration | M |  | 9.2.2.w |  |
| >>SSBCLIList | 0..1 |  |  |  |
| >>SSBCLIItem | 1.. <maxnoofSSBAreas> |  |  |  |
| >>SSB-Specification | M |  |  | Coding to be agreed |

| Range bound | Explanation |
| --- | --- |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a gNB node cell. Value is 64. |

--------------------------------Next Change--------------------------------

9.2.2.z	CLI Resource Status

This IE contains information about the CLI test result

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Report Level |  |  |  |  |
| >cell_based |  |  |  |  |
| >>CLI Power | M | [0..X] | INTEGER (0..X) | Number of dBm of CLI power |
| >>CRI | M |  | INTEGER(1..64) | The CSI-RS Resource Indicator (CRI) value, i.e the index of a CSI-RS resource within a set of resources of the NZP-CSI-RS configured by the aggressor. |
| >SSB_based |  |  |  |  |
| >> SSB Area CLI List | 0..1 |  |  |  |
| >>> SSB Area CLI Item | 1 .. <maxnoofSSBAreas> |  |  |  |
| >>>>SSB Index | M |  | INTEGER(0..63) |  |
| >>>>CLI Power | M | [0..X] | INTEGER (0..X) | Number of dBm of CLI power |

| Range bound | Explanation |
| --- | --- |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a gNB node cell. Value is 64. |

--------------------------------Next Change--------------------------------

#### 9.2.2.w	NZP-CSI-RS Configuration

This IE contains information describing the NZP-CSI-RS configuration for a cell.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| NZP-CSI-RS-ResourceSet | M |  | OCTET STRING | Includes the NZP-CSI-RS-ResourceSet IE, as defined in TS 38.331 [10]. |
| NZP-CSI-RS-Resource List |  | 1 |  | List of CLI mitigation assistance information per cell |
| >NZP-CSI-RS-Resource Item |  | 1 .. <maxnoofNZP-CSI-RS-RESOURCE> |  |  |
| >>NZP-CSI-RS-Resource | M |  | OCTET STRING | Includes the NZP-CSI-RS-Resource IE, as defined in TS 38.331 [10]. |

| Range bound | Explanation |
| --- | --- |
| maxnoofNZP-CSI-RS-RESOURCE | Maximum no. of NZP-CSI-RS-Resoures. Value is 64. |

# Annex B – Changes to F1 AP (TS38.473)

#### 9.2.1.20	RESOURCE STATUS REQUEST

This message is sent by gNB-CU to gNB-DU to initiate the requested measurement according to the parameters given in the message.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence |  | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  |  | 9.3.1.23 |  | YES | reject |
| gNB-CU Measurement ID | M |  |  | INTEGER (0..4095,...) | Allocated by gNB-CU | YES | reject |
| gNB-DU Measurement ID | C-ifRegistrationRequestStoporAdd |  |  | INTEGER (0..4095,...) | Allocated by gNB-DU | YES | ignore |
| Registration Request | M |  |  | ENUMERATED(start, stop,add, continue, configure, …) | Type of request for which the resource status is required. | YES | ignore |
| Report Characteristics | C-ifRegistrationRequestStart |  |  | BIT STRING (SIZE(32)) | Each position in the bitmap indicates measurement object the gNB-DU is requested to report. First Bit = PRB Periodic, Second Bit = TNL Capacity Ind Periodic, Third Bit = Composite Available Capacity Periodic, Fourth Bit = HW LoadInd Periodic, Fifth Bit = Number of Active UEs Periodic, Sixth Bit = NR-U Channel List Periodic. Seventh Bit = CLI Testing Eight Bit = CLI Mitigation Other bits shall be ignored by the gNB-DU. | YES | ignore |
| Cell To Report List |  |  | 0..1 |  | Cell ID list to which the request applies. | YES | ignore |
| >Cell To Report Item |  |  | 1 .. <maxCellingNBDU> |  |  | - |  |
| >>Cell ID | M |  |  | NR CGI 9.3.1.12 |  | - |  |
| >>CLI Mitigation Information |  |  | C-ifMitigationRequestStartCont |  |  |  | 9.2.2.x |  |  |  | -- |  |  |  |
| >>SSB To Report List |  |  | 0..1 |  | SSB list to which the request applies. | - |  |
| >>>SSB To Report Item |  |  | 1 .. < maxnoofSSBAreas> |  |  | - |  |
| >>>>SSB index | M |  |  | INTEGER (0..63) |  | - |  |
| >>>>CLI Mitigation Information |  | C-ifMitigationRequestStartCont |  | 9.2.2.x |  | -- |  |
| >>Slice To Report List |  |  | 0..1 |  | S-NSSAI list to which the request applies. | - |  |
| >>>Slice To Report Item |  |  | 1..< maxnoofBPLMNsNR> |  |  | - |  |
| >>>>PLMN Identity | M |  |  | 9.3.1.14 | Broadcast PLMN | - |  |
| >>>>S-NSSAI List |  |  | 1 |  |  | - |  |
| >>>>>S-NSSAI Item |  |  | 1 .. < maxnoofSliceItems> |  |  | - |  |
| >>>>>>S-NSSAI | M |  |  | 9.3.1.38 |  | - |  |
| Reporting Periodicity | O |  |  | ENUMERATED(500ms, 1000ms, 2000ms, 5000ms,10000ms, …) | Periodicity that can be used for reporting of indicated measurements. Also used as the averaging window length for all measurement object if supported. This IE is ignored if the Registration Request IE is set to “add”. | YES | ignore |
| CLI Measurement Configuration | C-ifCLITesting |  |  | 9.3.1.y |  | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifRegistrationRequestStoporAdd | This IE shall be present if the Registration Request IE is set to the value "stop" or "add". |
| ifRegistrationRequestStart | This IE shall be present if the Registration Request IE is set to the value "start". |
| ifConfigStartCLITesting | This IE shall be present if the RegistrationRequest has the values “Configure”, or “Start”, and the ReportCharacteristics IE is set to the value “CLI Testing” |
| ifMitigationRequestStartCont | This IE shall be present if the Report Characteristics IE is set to “CLI Mitigation” AND the Registration Request IE is set to either “start” or “continue”. |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum no. cells that can be served by a gNB-DU. Value is 512. |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a gNB node cell. Value is 64. |
| maxnoofSliceItems | Maximum no. of signalled slice support items. Value is 1024. |
| maxnoofBPLMNsNR | Maximum no. of PLMN Ids.broadcast in a cell. Value is 12. |

--------------------------------Next Change--------------------------------

#### 9.2.1.23	RESOURCE STATUS UPDATE

This message is sent by gNB-DU to gNB-CU to report the results of the requested measurements.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| gNB-CU Measurement ID | M |  | INTEGER (0..4095,...) | Allocated by gNB-CU | YES | reject |
| gNB-DU Measurement ID | M |  | INTEGER (0..4095,...) | Allocated by gNB-DU | YES | ignore |
| Hardware Load Indicator | O |  | 9.3.1.136 |  | YES | ignore |
| TNL Capacity Indicator | O |  | 9.3.1.128 |  | YES | ignore |
| Cell Measurement Result |  | 0..1 |  |  | YES | ignore |
| >Cell Measurement Result Item |  | 1 .. <maxCellingNBDU > |  |  | - |  |
| >>Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >> CLI Resource Status | O |  | 9.3.1.z |  | -- |  |
| >>Radio Resource Status | O |  | 9.3.1.129 |  | - |  |
| >>Composite Available Capacity Group | O |  | 9.3.1.130 |  | - |  |
| >>Slice Available Capacity | O |  | 9.3.1.134 |  | - |  |
| >>Number of Active UEs | O |  | 9.3.1.135 |  | - |  |
| >>NR-U Channel List |  | 0..1 |  |  | YES | ignore |
| >>>NR-U Channel Item |  | 1..<maxnoofNR-UChannelIDs> |  |  | - |  |
| >>>>NR-U Channel ID | M |  | INTEGER (1.. maxnoofNR-UChannelIDs) | Identifies a portion of the NR-U Channel Bandwidth on which channel access procedure in shared spectrum has been performed in the last reporting period. | - |  |
| >>>>Channel Occupancy Time Percentage DL | M |  | INTEGER (0..100) | The percentage of time for which the channel resources have been utilised for DL traffic served by the corresponding NR-U Channel of the serving cell. Value 100 indicates that the channel resources have been utilized for DL traffic served by the corresponding NR-U Channel of the serving cell for the whole duration between consecutive reporting. | - |  |
| >>>>Energy Detection Threshold DL | M |  | INTEGER (-100..-50,…) | Average ED Threshold used for DL channel sensing at the gNB. Value is in dBm. | - |  |
| >>>>Channel Occupancy Time Percentage UL | O |  | INTEGER (0..100) | The percentage of time for which the channel resources have been utilised for UL traffic served by the corresponding NR-U Channel of the serving cell for UEs that transmit to the serving cell. Value 100 indicates that the channel resources have been utilized for UL traffic served by the corresponding NR-U Channel of the serving cell for the whole duration between consecutive reporting. | YES | ignore |
| >>>>Radio Resource Status NR-U | O |  | 9.3.1.295 | Indicates the radio resource status per NR-U channel. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum no. cells that can be served by a gNB-DU. Value is 512. |
| maxnoofNR-UChannelIDs | Maximum no. NR-U Channel IDs in a cell. Value is 16. |

--------------------------------Next Change--------------------------------

#### 9.3.1.x	CLI Mitigation Information

This IE contains information that may be used to mitigate CLI across gNBs.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CLI Information |  |  |  |  |
| >CLI Power detected | M | [0..X] | INTEGER (0..X) | Number of dBm of CLI power |
| >CRI | O |  | INTEGER (1..64) | Includes the CSI-RS Resource Indicator (CRI) value. CRI is an index indicating a CSI-RS resource within a set of resources |
| >SSB Index | O |  | INTEGER (0..63) | Identifier of the SSB beam. |

--------------------------------Next Change--------------------------------

9.3.1.y CLI Test Parameters

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CLITestType | M | ENUMERATED(gNB-to-gNB, …) |  |  |
| ServedCellCLIList | 1 .. <maxnoofCellsinNG-RANnode> |  |  |  |
| >ServedCellCLIItem | M |  |  |  |
| >>NZP-CSI-RS Configuration | M |  | 9.3.1.w |  |
| >>SSBCLIList | 0..1 |  |  |  |
| >>SSBCLIItem | 1..<maxnoofSSBAreas> |  |  |  |
| >>SSB-Configuration | M |  |  | Coding to be agreed |

| Range bound | Explanation |
| --- | --- |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a gNB node cell. Value is 64. |

--------------------------------Next Change--------------------------------

9.3.1.z	CLI Resource Status

This IE contains information about the CLI test result

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Report Level |  |  |  |  |
| >cell_based |  |  |  |  |
| >>CLI Power | M | [0..X] | INTEGER (0..X) | Number of dBm of CLI power |
| >>CRI | M |  | INTEGER(1..64) | The CSI-RS Resource Indicator (CRI) value, i.e the index of a CSI-RS resource within a set of resources of the NZP-CSI-RS configured by the aggressor. |
| >SSB_based |  |  |  |  |
| >> SSB Area CLI List | 0..1 |  |  |  |
| >>> SSB Area CLI Item | 1 .. <maxnoofSSBAreas> |  |  |  |
| >>>>SSB Index | M |  | INTEGER(0..63) |  |
| >>>>CLI Power | M | [0..X] | INTEGER (0..X) | Number of dBm of CLI power |

| Range bound | Explanation |
| --- | --- |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a gNB node cell. Value is 64. |

--------------------------------Next Change--------------------------------

#### 9.3.1.w	NZP-CSI-RS Configuration

This IE contains information describing the NZP-CSI-RS configuration for a cell.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| NZP-CSI-RS-ResourceSet | M |  | OCTET STRING | Includes the NZP-CSI-RS-ResourceSet IE, as defined in TS 38.331 [10]. |
| NZP-CSI-RS-Resource List |  | 1 |  | List of CLI mitigation assistance information per cell |
| >NZP-CSI-RS-Resource Item |  | 1 .. <maxnoofNZP-CSI-RS-RESOURCE> |  |  |
| >>NZP-CSI-RS-Resource | M |  | OCTET STRING | Includes the NZP-CSI-RS-Resource IE, as defined in TS 38.331 [10]. |

| Range bound | Explanation |
| --- | --- |
| maxnoofNZP-CSI-RS-RESOURCE | Maximum no. of NZP-CSI-RS-Resoures. Value is 64. |