3GPP TSG RAN WG3 Meeting #126	R3-247600

Orlando, U.S.A., 18 - 22 November, 2024

Agenda Item:	14.3

Source:	ETRI

Title:	Discussion on NG interface management through ISL

Document for:	Discussion

- Introduction

The Rel-19 NR NTN WID[1] includes the following objectives to support regenerative payload with full gNB on board:

Support of regenerative payload [RAN3, RAN2, RAN4] - Specify the support of gNB on board in TS 38.300 - Specify, if needed, any necessary enhancements related to the intra and inter-gNB mobility, especially for Xn interface over feeder link or over ISL. [RAN3] - Note: if any additional necessary stage-3 specifications impact for e.g. NGAP is identified, RAN3 will handle it.

In the previous meetings, RAN3 got the following progresses on NR NTN regenerative payload:

RAN3#125: NG Interface management Issue: Option 2 NG Removal/Setup can be used for Regenerative Payload NG interface management. Inactive UE support: RAN3 assumes that current mechanisms to support UEs performing RRC connection resume and RRC connection re-establishment may be reused in regenerative payload architecture, subject to implementation and deployment. Feeder link switch issue: RAN3 assumes that during the feeder link switch, AMF is not changed for the UEs. Whether there has any standard impact needs to be further checked. RAN3#125bis: The supported TAIs of an on-board gNB could be provided to AMF by OAM/pre-configuration. The current mechanisms including, e.g. multiple TNLA association, are sufficient for TNL management during feeder link switch. Legacy NG HO procedure can be reused for inter AMF HO for NTN regenerative payload. RAN CONFIGURATION UPDATE procedure could be reused to support NG suspend/Resume, by introducing suspend/resume indicators to avoid frequent TNL disconnection and re-connection? Continue the discussion on above open issues…

In the last meeting, we proposed to discuss an open issue on the NG interface management through ISL[2]. This contribution is the revised version of the previous contribution.

- Discussion

  - NG interface management

3GPP Rel-19 NTN normative work includes the support of regenerative payload. The satellite with regenerative payload provides the full gNB functionality and also provides Inter-Satellite Link(ISL) between the satellites. Therefore, the UE served by an on-board gNB could access the 5G Core on the ground via feeder link or ISL[3].

Observation 1: The UE served by an on-board gNB could access the 5G Core on the ground via feeder link or ISL.

In the RAN3 #125 meeting, regarding the NG interface management, RAN3 has reached an agreement such that Option 2 NG Removal/Setup can be used for Regenerative Payload NG interface management.

Observation 2: NG Removal/Setup can be used for Regenerative Payload NG interface management.

  - NG Setup via feeder link or ISL

Due to the movement of NGSO satellites with regenerative payload, the on-board gNB can visit the serving area of the AMFs. As shown in Figure 1, if the on-board gNB of SAT-1(on-board SAT-1 gNB) visits the serving area of AMF-1 and it has a feeder link with NTN GW-1, the gNB needs to set up TNL association with a new TNL address anchored in the NTN GW-1. Thus, after changing its TNL address, the gNB sets up TNL association and NG with AMF-1 via the feeder link.

Figure 1: NG Setup via feeder link

However, as shown in Figure 2, when visiting the serving area of AMF-1, the on-board SAT-1 gNB may have no feeder link but an ISL with SAT-2 which has a feeder link with NTN GW-1. In this case, the on-board SAT-1 gNB needs to get a new TNL address to be anchored in NTN GW-1. After changing its TNL address, the on-board SAT-1 gNB sets up TNL association and NG with AMF-1 through ISL with SAT-2. SAT-2 should relay all packets for TNL association and NG setup procedure between on-board SAT-1 gNB and AMF-1 through ISL and over the feeder link connected with NTN GW-1.

Figure 2: NG Setup through ISL

Observation 3: The on-board gNB may have only ISL with neighbouring satellites. When visiting the serving area of a new AMF, the on-board gNB having only ISL also needs to get a new TNL address to be anchored in a new NTN GW.

Observation 4: With the new TNL address, the on-board gNB having only ISL sets up TNL association and NG with a AMF through ISL, and the neighbouring satellite(or satellites in case of multi-hop ISL) should relay all packets for TNL association and NG setup procedure between the gNB and the AMF through ISL and over the feeder link connected with the NTN GW.

Regarding Observation 3 and 4, there are 2 issues from the perspective of on-board SAT-1 gNB:

- how to get a new TNL address to be anchored in NTN GW-1

- how to recognize that SAT-2 can relay packets for TNL association and NG setup procedure between on-board SAT-1 gNB and AMF-1.

As shown in Figure 3, the on-board SAT-2 gNB already has a TNL association and NG connection with AMF-1 through NTN GW-1 when the on-board SAT-1 gNB visits the serving area of AMF-1.

Figure 3: Obtaining the information for NG Setup through ISL

Therefore, we can consider 2 possible options:

(Option 1) OAM-based solution: The on-board SAT-1 gNB is pre-configured with new TNL address and relaying satellite information by OAM

(Option 2) Signalling-based solution: The on-board SAT-1 gNB gets TNL address of new NTN-GW and relaying satellite information from neighbouring satellites via Xn interface

Observation 5: We need to consider the solution for the on-board gNB having only ISL to obtain the information about new TNL address anchored in a new NTN-GW and relaying satellites for NG Setup through ISL.

Proposal 1: RAN3 to discuss how to set up TNL association and NG with a AMF through ISL.

  - NG Removal via feeder link or ISL

Due to the movement of NGSO satellites with regenerative payload, the on-board gNB can leave the serving area of the AMFs. As shown in Figure 4, if the on-board SAT-1 gNB is leaving the serving area of AMF-1 and it has a feeder link with NTN GW-1, the gNB removes TNL association and NG with AMF-1 over the feeder link.

However, as shown in Figure 5, when leaving the serving area of AMF-1, the on-board SAT-1 gNB may have no feeder link but an ISL with SAT-2 which has a feeder link with NTN GW-1. In this case, the on-board SAT-1 gNB needs to remove TNL association and NG with AMF-1 through ISL with SAT-2. SAT-2 should relay all packets for TNL association and NG removal procedure between on-board SAT-1 gNB and AMF-1 through ISL and over the feeder link with NTN GW-1.

Observation 6: If the on-board gNB having only ISL leaves the serving area of a AMF, the gNB needs to remove TNL association and NG with a AMF through ISL, and the neighbouring satellite(or satellites in case of multi-hop ISL) should relay all packets for TNL association and NG removal procedure between the gNB and the AMF through ISL and over the feeder link with the NTN GW.

Proposal 2: RAN3 to discuss how to remove TNL association and NG with a AMF through ISL.

Figure 4: NG Removal via feeder link

Figure 5: NG Removal through ISL

- Conclusion

Based on the discussion in section 2, we have the following observations and proposals:

Observation 1: The UE served by an on-board gNB could access the 5G Core on the ground via feeder link or ISL.

Observation 2: NG Removal/Setup can be used for Regenerative Payload NG interface management.

Observation 3: The on-board gNB may have only ISL with neighbouring satellites. When visiting the serving area of a new AMF, the on-board gNB having only ISL also needs to get a new TNL address to be anchored in a new NTN GW.

Observation 4: With the new TNL address, the on-board gNB having only ISL sets up TNL association and NG with a AMF through ISL, and the neighbouring satellite(or satellites in case of multi-hop ISL) should relay all packets for TNL association and NG setup procedure between the gNB and the AMF through ISL and over the feeder link connected with the NTN GW.

Observation 5: We need to consider the solution for the on-board gNB having only ISL to obtain the information about new TNL address anchored in a new NTN-GW and relaying satellites for NG Setup through ISL.

Proposal 1: RAN3 to discuss how to set up TNL association and NG with a AMF through ISL.

Observation 6: If the on-board gNB having only ISL leaves the serving area of a AMF, the gNB needs to remove TNL association and NG with a AMF through ISL, and the neighbouring satellite(or satellites in case of multi-hop ISL) should relay all packets for TNL association and NG removal procedure between the gNB and the AMF through ISL and over the feeder link with the NTN GW.

Proposal 2: RAN3 to discuss how to remove TNL association and NG with a AMF through ISL.

- Reference

RP-240775	Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3

R3-245588 Discussion on NG interface management through ISL (ETRI)

TR 38.821 Solutions for NR to support NTN