3GPP TSG-RAN WG3 #126	R3-247603

Orlando, US, 18 – 22 Nov. 2024

Agenda Item:	11.4. R18 leftovers

Source:	NTT DOCOMO INC.

Title:	Discussion on AIML for split architecture

Document for:	Discussion, agreement

Introduction

At the RAN3#125 meeting [1], we captured TR 38.743 and had following description for split architecture use case.

5.2	Split architecture support for Rel-18 use cases

5.2.1	Use case description

The split architecture should be enhanced to support the Rel-18 use cases, e.g, Load Balancing, Energy Saving, and Mobility Optimization.

In case of CU-DU architecture, the following solutions are possible:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU(-CP);

-	AI/ML Model Training and Model Inference are both located in the gNB-CU(-CP).

5.2.2	Potential Standard impacts

The following standard impacts are listed for subsequent Rel-19 normative work compared with what was specified during Rel-18:

-	The details of signaling measured UE performance metrics from gNB-DU to gNB-CU-CP and/or from gNB-CU-UP to gNB-CU-CP need further discussion during normative phase.

-	Measured Energy Cost from gNB-DU to gNB-CU.

Also, RAN3 achieved following agreement in RAN3#125bis[2].

Introducing Data Collection Reporting Initiation and Data Collection Reporting procedures over F1 interface to transfer measured EC from DU to CU.

In this contribution, we have further discussion regarding energy saving use case in split architecture.

Discussion

At the previous meeting, we agreed to introduce the Data Collection Reporting Initiation procedure and the Data Collection Reporting procedure in F1 same as in Xn. This enables the collection of Energy Cost per DU on F1, with CU as requesting nodes and DU as requested nodes.

Whether per-DU energy cost should be sent to other gNB should also be discussed. Even if we only support per-DU energy cost transfer, it can be used for energy saving within the gNB and this means there is no improvement on energy saving functionality on Xn but just support of disaggregated gNBs. However, per-DU energy cost transferred over Xn would improve energy saving between gNBs because per-DU energy cost reporting can isolate the changes of energy cost of the target DU from undesired effect by other DUs in the target gNB. For this reason, we also propose to exchange per-DU energy cost over Xn. In case of per-DU EC can be reported, whether the report should include both per-DU and per-gNB EC or per-gNB EC can be optional would be further discussed.

Proposal 1:	RAN3 to support signalings carrying measured energy cost per-DU over Xn.

Lastly, we think we should clarify how to define and configure the energy cost mapping rule for per-DU energy cost to the DU. In our standing, optimal mapping rule for per-DU may be different from one for per-gNB, so operator should have responsibility to define energy cost mapping rule suitable for all DUs across gNBs within a specific area. This point also should be discussed.

Proposal 2:	RAN3 to discuss how to configure energy cost mapping rule for per-DU granularity.

Proposal 3:	Energy cost mapping rule for per-DU granularity is also provided from OAM to DUs and it is determined by operator same as one for per-gNB granularity. FFS on whether per DU EC mapping rule is same as per-gNB EC or separate one.

Conclusions and proposals

Our proposals are summarized below.

Proposal 1:	RAN3 to support signalings carrying measured energy cost per-DU over Xn.

Proposal 2:	RAN3 to discuss how to configure energy cost mapping rule for per-DU granularity.

Proposal 3:	Energy cost mapping rule for per-DU granularity is also provided from OAM to DUs and it is determined by operator same as one for per-gNB granularity. FFS on whether per DU EC mapping rule is same as per-gNB EC or separate one.

References

- RAN3#125 Chair Notes

- RAN3#125bis Chair Notes