3GPP TSG-RAN WG3 #126	R3-247629

Orlando, US, 18 – 22 Nov. 2024

Agenda Item:	13.2. Support for inter-CU LTM

Source:	NTT DOCOMO INC.

Title:	Discussion on inter-CU LTM non-DC case

Document for:	Discussion, agreement

Introduction

At the previous meeting, RAN3 reached the following agreements on inter-CU LTM [1].

Current SSB information in Xn Setup and Configuration Update procedures can be reused for LTM preparation phase.

WA: For inter-CU LTM mobility, a separate LTM request message (i.e. HANDOVER REQUEST message) is used for each candidate cell.

The error handling of multiple UE associations need to be considered.

WA: The source gNB-CU sends the CSI resource configuration of candidate cells to other candidate gNB-CUs via Handover Request message, and the candidate gNB-CU sends the CSI report configuration to the source gNB-CU via Handover Request ACK message.

The LTM Configuration IDs are allocated by the source CU.

WA: Reuse the existing XnAP UE CONTEXT RELEASE message at the source gNB if no LTM candidate cell(s) exist in the source gNB.

Follow F1AP, the source gNB-CU sends the CSI resource configuration of candidate cells to candidate gNB-CUs via Handover Request message for subsequent LTM, and the candidate gNB-CU sends the CSI report configuration to the source gNB-CU via Handover Request ACK message.

How the source gNB-CU sends the reference configuration to all candidate gNBs is pending on RAN2 progress.

Confirm the name of the new procedure as “LTM Configuration Update”.

Introduce a new procedure for candidate gNB-initiated LTM cancellation.

Allow UE association in-between candidate CUs (in case the Xn connectivity existed) for subsequent LTM. When and how to establish the UE association is FFS.

And also, potential issues [2] on inter-gNB LTM related to PDCP anchor solution proposed by Qualcomm was discussed in RAN3, but no agreement was reached.

Based on the above, this paper discusses the open points of Inter-CU LTM and without PDCP anchor change.

Discussion

Inter-CU LTM without PDCP anchor change

Qualcomm is proposing solution for LTM that avoids security key change and PDCP reestablishment in inter-CU LTM by switching the UE to the DU under the target gNB without changing the PDCP anchor in the C/U-plane. This method could maximize the benefit of low interruption of LTM with minimum signaling overhead in the following scenarios, for example.

- When a UE moves in an area near a single gNB and repeatedly performs intra-CU LTM, and sometimes ping-pong occurs between inter-gNB cells when crossing the boundary with neighbor gNB.

- When a single gNB covers a large area and a UE that moves out of the area to an neighbor gNB is expected to return to the original gNB within a certain period of time.

- When a UE is expected to move between the coverage boundaries of two gNBs

On the other hand, if intra-CU cell switch occurs frequently after moving to a neighboring gNB, Xn signaling may increase compared to normal inter-CU LTM. Therefore, both method of inter-CU LTM with and without PDCP anchor change should be supported in order to allow gNB to select better one based on situations.

Besides, SA3 is discussing security mechanism for inter-CU LTM and they sent the LS [3] to RAN2/3 saying that currently SA3 thinks security key refresh should be done by protected RRC signaling and continues further analysis on MAC CE based solution and RRC pre-config based solution. This does not eliminate the possibility to avoid RRC signaling for security key change between cell switches, however, we cannot ensure that the RRC signaling can be avoided and we can support subsequent inter-CU LTM at this moment. If SA3 would conclude to support only security key change with L3 signaling at every cell switch, this heavily degrades the benefit of LTM, and PDCP anchor solution would be good way to solve it. However, we may not be able to receive final conclusion from SA3 soon. So we think we should not ruled out that solution.

In a past meeting, a solution was proposed to perform security key change avoiding RRC signaling for each cell switch by using only horizontal key derivation [4]. This is possible if the AMF selects horizontal key derivation, but our understanding is that the AMF basically does not care about the RAN behavior when choosing the security refresh mechanism, and this choice may involve operator policy. If the AMF always chooses vertical key derivation based on the implementation or operators policy, then this mechanism does not work. It is not a good way for the AMF to choose a security refresh mechanism considering RAN behavior. It is also undesirable that whether it is a sequent LTM or a one-time LTM is determined based on AMF's decision even though the RAN has decided to configure sequent LTM. For the above reasons, the subsequent inter-CU LTM should consider a solution that does not involve CN impact regarding security key refresh.

Observation 1:	Inter-CU LTM without PDCP anchor change has significant advantages in terms of low interruption in certain scenarios.

Observation 2:	Security mechanism for inter-CU LTM is under discussion in SA3, and subsequent inter-CU LTM could not be supported if SA3 would not confirm the solutions RAN2 raised or propose new solution.

Proposal 1:	As well as inter-CU LTM with PDCP anchor change, inter-CU LTM without PDCP anchor change should also be discussed.

LS from RAN1 [5]

RAN3 received a LS from RAN1 in which they ask us the feasibility of the NW coordination signaling for semi-persistent CSI-RS. This mechanism was proposed in RAN1 to support semi-persistent transmission of CSI-RS in order to save radio resources.

The NW coordination signaling would involve F1 in inter-DU LTM and Xn in inter-CU LTM. This is because this coordination would be done between source DU and candidate DU since target CSI-RS related configuration is managed and generated at the target DU. In our understanding, UE is pre-configured with semi-persistent CSI-RS configuration in LTM preparation phase, and then the source DU sends activation command of CSI-RS to the UE. After that, UE starts using the CSI-RS e.g. for L1 measurement.

Based on above analysis, RAN3 should consider following points:

- Which node to trigger CSI-RS activation?

SP CSI-RS should be activated at the UE side by the command from the NW, and the node which sends the command is source DU because the activation is sent via MAC CE in the serving cell in order to let the UE perform L1 measurement, so that the source DU triggers the coordination for the activation.

Proposal 2:	RAN3 to assume that the coordination for SP CSI-RS transmission activation/deactivation would be triggered by source DU.

- When to trigger CSI-RS activation?

In our understanding, SP CSI-RS should be activated (/deactivated) on real time demand, so that it should be involve the coordination between source DU and candidate DU(s) after LTM preparation. Since source DU triggers the activation, this could be happen at any time after preparation phase upon source DU decisions (e.g. L1 measurement). We think RAN3 can discuss other perspectives based on above mentioned understanding.

Proposal 3:	RAN3 to assume that the coordination for SP CSI-RS transmission activation/deactivation could happen at any time after LTM preparation based on source DU decisions.

- What information should be exchanged during the coordination?

In our understanding, most of information which the UE needs to use the CSI-RS is configured to the UE in LTM preparation phase via RRC, so that the information needs to be exchanged in the coordination is limited (i.e. information to be sent in CSI-RS activation command to the UE via MAC CE). Based on this analysis, we think processing time, signaling burden or normative effort would not be a big problem. Anyway, the details of the information should be decided by RAN1.

Proposal 4: RAN3 to assume that the coordination for SP CSI-RS transmission activation/deactivation doesn’t have big problem in terms of processing time, signaling burden or normative effort. Signaling details are pending on RAN1 discussion.

- Can the latency of the coordination be a problem?

Since source DU is responsible for triggering of the activation, the DU needs to inform it of both the UE and the target DU. In this case, the latency at the NW side would be problem, because if the source DU triggers these two commands almost at the same time, the UE may activate the CSI-RS and start measurement for the CSI-RS even though the target DU doesn’t start transmission of it. On the other hand, this issue might be solved based on DU implementation with proper timing adjustments considering network signaling latency. For another option, if the coordination between source and target DU is performed via class 1 procedure, source DU can send the command the UE after receiving the acknowledge by target DU ensuring the transmission of CSI-RS is already started. This point should be further discussed in RAN3.

Proposal 5:	RAN3 to discuss how to synchronize the timing of activation of CSI-RS between UE and target DU.

RAN3 already supported F1 signaling to carry notification from one DU to another DU via CU, i.e. CU-DU/DU-CU Cell Switch Notification procedure and CU-DU/DU-CU TA Information Transfer procedure. Also, we already agreed to introduce Cell Switch Notification in Xn as well, so that for our view, support of a signaling for the coordination for semi-persistent CSI-RS is not so new mechanism. Also, based on above analyses, processing time, signaling burden or normative effort would not be problems. For these reasons, we think specifying the signaling for coordination between serving cell and candidate cell(s) on the transmission of semi-persistent CSI-RS(s) is feasible from RAN3 perspective.

Proposal 6:	From RAN3 perspective, specifying the signaling for coordination between serving cell and candidate cell(s) on the transmission of semi-persistent CSI-RS(s) is feasible.

Proposal 7:	Agree the reply LS [6]

LTM preparation

In order to support the Xn signaling for the preparation phase similar as Rel-18 intra-CU LTM F1AP, the following information needs to be sent from the source CU to the target CU in the preparation phase.

- Candidate gNB-DU ID List

Similar to Rel-18 intra-CU LTM, when the candidate DUs allocate RACH resources, the CU should send a Candidate gNB-DU ID List in order to allocate the same RACH resource to the same source DU to save resources. Since the uniqueness of gNB-DU ID is guaranteed only within one gNB, candidate CU which receives a list of candidate gNB-DU IDs may not be able to distinguish candidate DUs from different gNBs. So that this list should also include Global NG-RAN Node ID which the DU is corresponding to.

Proposal 8:	Candidate gNB-DU List should be sent to the all candidate CU(s) via Handover Request message.

Proposal 9:	Candidate gNB-DU List should include gNB-DU ID associated with Global NG-RAN Node ID.

LTM configuration transfer for subsequent inter-CU LTM

In the offline discussion in the last meeting, following three options were raised for inter-gNB coordination for subsequent LTM but not treated.

- Option 1: Send the collected LTM information to the candidate gNB(s) after the initial handover preparation.

- Option 2: Send the collected LTM information to the target gNB when the source gNB send the LTM Cell Switch Command to the UE (e.g., via LTM Cell Switch Notification message).

- Option 3: Send the collected LTM information to the target gNB when the source gNB receives the Handover Success message from the target gNB.

This mechanism is needed because once cell switch is triggered and serving DU/gNB is changed, new serving DU should make decision for next cell switch and thus necessary information should be delivered to all candidate CUs/DUs. If this transfer is not done during preparation phase and done in execution phase, it may read latency on triggering of cell switch, and we think it should be avoided.

Regarding the message carrying the RRC configuration, we think LTM modification message which would be used as 2nd round trip in preparation phase can be used. This configuration could also be sent via Cell Switch Notification or Handover Success after every cell switch, but this make these notification message bigger, so that we think it should be done prior to execution phase, i.e. preparation phase.

Proposal 10:	LTM configuration sent to the UE is shared among source CU and all candidate CU(s) in preparation phase (i.e. 1st or 2nd round trip btw source and candidate CU(s).)

UE association management among gNBs

In the last meeting, it was discussed how to establish UE association and sync current serving cell information between candidate gNBs for LTM cancellation or modification in subsequent LTM, and the following options were raised:

- Option 1: During the LTM preparation phase, the candidate gNB allocates UE XnAP ID and send to other candidate gNBs.

- Option 2: After each cell switch, the new serving gNB signals XnAP ID to other candidate gNBs, and then the candidate gNB reply with the Target node UE XnAP ID.

- Option 3: After each cell switch, the old serving gNB transfers the UE XnAP ID(s) used by the candidate gNBs to the new serving gNB.

It seems that either option will work, but we believe that Option 2 is best based on the following reasons:

- This mechanism differs from LTM configuration transfer for subsequent LTM in that the synchronization of information about the latest seving gNB between candidate gNBs is not always necessary for a triggering of subsequent cell switch. It is necessary to keep track of the latest serving gNB for LTM candidate cancellation/modification, etc.

- XnAP UE ID association between all candidate gNBs need not be established in advance.

- Since not all candidate gNBs will necessarily be source gNBs in a series of cell switches based on a single LTM configuration. Therefore, it is preferable to minimize the signaling to establish this XnAP UE ID association if possible.

- If this is done in the LTM preparation phase, it may further increase the signaling overhead/delay in the preparation phase.

Additionally, this should be done after completion of cell switch, not at the time of cell switch notification. Handover Success message could be used to carry this signaling, but the purpose of this message is notification to the source node, so that which message is used for this signaling should be further discussed.

Proposal 11:	After each cell switch, the new serving gNB signals XnAP ID to other candidate gNBs, and then the candidate gNB reply with the Target node UE XnAP ID.

Conclusions and proposals

Our proposals are summarized below.

Observation 1:	Inter-CU LTM without PDCP anchor change has significant advantages in terms of low interruption in certain scenarios.

Observation 2:	Security mechanism for inter-CU LTM is under discussion in SA3, and subsequent inter-CU LTM could not be supported if SA3 would not confirm the solutions RAN2 raised or propose new solution.

Proposal 1:	As well as inter-CU LTM with PDCP anchor change, inter-CU LTM without PDCP anchor change should also be discussed.

Proposal 2:	RAN3 to assume that the coordination for SP CSI-RS transmission activation/deactivation would be triggered by source DU.

Proposal 3:	RAN3 to assume that the coordination for SP CSI-RS transmission activation/deactivation could happen at any time after LTM preparation based on source DU decisions.

Proposal 4:	RAN3 to assume that the coordination for SP CSI-RS transmission activation/deactivation doesn’t have big problem in terms of processing time, signaling burden or normative effort. Signaling details are pending on RAN1 discussion.

Proposal 5:	RAN3 to discuss how to synchronize the timing of activation of CSI-RS between UE and target DU.

Proposal 6:	From RAN3 perspective, specifying the signaling for coordination between serving cell and candidate cell(s) on the transmission of semi-persistent CSI-RS(s) is feasible.

Proposal 7:	Agree the reply LS [6]

Proposal 8:	Candidate gNB-DU List should be sent to the all candidate CU(s) via Handover Request message.

Proposal 9:	Candidate gNB-DU List should include gNB-DU ID associated with Global NG-RAN Node ID.

Proposal 10:	LTM configuration sent to the UE is shared among source CU and all candidate CU(s) in preparation phase (i.e. 1st or 2nd round trip btw source and candidate CU(s).)

Proposal 11:	After each cell switch, the new serving gNB signals XnAP ID to other candidate gNBs, and then the candidate gNB reply with the Target node UE XnAP ID.

References

- RAN3#125bis Chair Notes

- R3-245315, “Impact analysis of coupling LTM cell switch and CU anchor changes on subsequent inter-gNB LTM”, Qualcomm

- S3-244316, “Reply LS on security handling for inter-CU LTM in non-DC cases”, SA3

- R3-242098, “High-level principles for Inter-CU LTM security master key handling”, LGE

- R3-247006, “LS on the support of semi-persistent CSI-RS resource for LTM candidate cells”, RAN1

- R3-247630, “[Draft] Reply LS on the support of semi-persistent CSI-RS resource for LTM candidate cells”, NTT DOCOMO