3GPP TSG-RAN WG3 Meeting #126	                                   R3-247664

Orlando, USA, Nov 18 - Nov 22, 2024

Agenda item:	19.2

Source:	CMCC

Title:	Discussion on supporting CLI handling in SBFD

Document for:	Discussion

Introduction

In RAN3#125bis meeting, the following agreements are made on the enhancements for CLI handling in SBFD, shown as:

RAN3 agrees to Exchange the following information over Xn interface and F1 interface: - Semi-static cell-specific SBFD time and frequency location configuration with cell granularity (FFS on container or explicit IEs over XnAP and F1AP) - Measurement resource configuration - Strongest DL beam information (FFS on the details) - CLI-mitigation request (FFS on the details)

In this contribution, we would like to provide our views on the information exchange among gNBs for CLI handling in SBFD.

Discussion

Exchange of semi-static cell-specific SBFD time and frequency location configuration

In RAN1#118 meeting [1], the following agreements and working assumption are made:

Agreement For configuration of SBFD symbols within a TDD-UL-DL pattern period, the following parameters are supported - A starting slot index - A starting symbol index within the starting slot - An ending slot index - An ending symbol index within the ending slot Working Assumption For cell-specific configuration of frequency locations of SBFD UL sub-band, for each SCS configuration in SCS-SpecificCarrierList for UL, starting RB and bandwidth of SBFD UL sub-band are indicated by a RIV-based indication as defined in 38.214 setting =275. For cell-specific configuration of frequency locations of SBFD DL sub-band(s), for each SCS configuration in SCS-SpecificCarrierList for DL, starting RB and bandwidth of each SBFD DL sub-band are indicated by a RIV-based indication as defined in 38.214 setting =275. - One or two SBFD DL sub-bands can be configured

For the exchange of semi-static cell-specific SBFD time and frequency location configuration among gNBs, it should be conducted when Xn-C interface established or when the cell-specific SBFD time and frequency location configuration changes. To this end, the semi-static cell-specific SBFD time and frequency location configuration can be contained in Xn Setup procedure and NG-RAN node Configuration Update procedure.

Observation 1: For the exchange of semi-static cell-specific SBFD time and frequency location configuration among gNBs, it should be conducted when Xn-C interface established or when the cell-specific SBFD time and frequency location configuration changes.

Proposal 1: The semi-static cell-specific SBFD time and frequency location configuration can be contained in Xn Setup procedure and NG-RAN node Configuration Update procedure.

For specific IE design, the time configuration of TDD cell is contained in TDD UL-DL Configuration Common NR IE in TDD Info of Served Cell Information NR currently. For TDD UL-DL Configuration Common NR IE, it is directly referred to tdd-UL-DL-ConfigurationCommon IE in TS 38.331. To this end, the design of semi-static cell-specific SBFD time and frequency location configuration in RAN3 is closely related to RAN2’s progress on the configuration methods which have not been made. To be specific, if RAN2 decides to extend the tdd-UL-DL-ConfigurationCommon IE in TS 38.331, then current XnAP specification can support the exchange of the SBFD time configuration accordingly. Otherwise, new IE or field need to be discussed to be introduced in RAN3 within TDD Info of Served Cell Information NR currently.

For the frequency configuration, the frequency configuration of TDD cell is contained in Frequency Info and Transmission Bandwidth or Carrier List IE in TDD Info of Served Cell Information NR currently. Based on the agreement that the starting RB and bandwidth of SBFD UL sub-band are indicated by a RIV-based indication per SCS configuration, the relative cell-specific SBFD frequency configuration including starting RB and bandwidth of SBFD UL sub-band may need to be discussed by RAN3 on where and how to capture them. From RAN3 perspective, a new IE named SBFD configuration NR which contains SBFD Time Info and SBFD Frequency Info can be introduced. While, whether SBFD Time Info and SBFD Frequency Info can refer to parameter(s) in TS 38.331 can wait for the progress in RAN2.

Observation 2: The time configuration of TDD cell is contained in TDD UL-DL Configuration Common NR IE in TDD Info of Served Cell Information NR currently.

Observation 3: The frequency configuration of TDD cell is contained in Frequency Info and Transmission Bandwidth or Carrier List IE in TDD Info of Served Cell Information NR currently.

Observation 4: For the semi-static cell-specific SBFD time and frequency location configuration design, it is closely related to RAN2’s progress on the configuration methods which have not been made.

Proposal 2: From RAN3 perspective, a new IE named SBFD configuration NR which contains SBFD Time Info and SBFD Frequency Info can be introduced in Served Cell Information NR IE. FFS the details of SBFD Time Info and SBFD Frequency Info

In terms of split architecture, the exchange of semi-static cell-specific SBFD time and frequency location configuration should be supported in F1 interface. In the current F1AP specification, the TDD UL-DL Configuration Common NR IE and Transmission Bandwidth or Carrier List IE are provided in Served Cell Information IE from gNB-DU to gNB-CU which is simlar to that in XnAP. Therefore, for the enhancement of F1 interface, the enhancement of Xn interface can be referred.

Proposal 3: The enhancement on F1 interface can be referred to Xn interface.

Exchange of measurement resource configuration and strongest DL beam information

For a clearer expression, this section will discuss the measurement configuration and the reporting of measurement results, i.e., strongest DL beam information together. In the LS form RAN1, the following agreements are informed:

Agreement From RAN1 perspective, exchange among gNBs of the configuration info for a set of one or more periodic NZP CSI-RS resources (relevant IE in 38.331 are NZP-CSI-RS-Resource, NZP-CSI-RS-ResourceSet) is supported. Strongest DL beam information in the context of NZP-CSI-RS is defined as a CSI-RS Resource Indicator (CRI) value. CRI is a relative index in the range [1 .. N], where N is the number of CSI-RS resources within a set of resources for which the configuration is provided to a gNB. Strongest DL beam information in the context of SSBs is defined as an SSB index. SSB index is an absolute index among the SSB(s) for which the configuration is provided to a gNB.

To be specific, three options are provided for gNBs performing the exchange of measurement resource configuration and strongest DL beam information mentioned above:

- Option 1: Xn Setup and/or NG-RAN node Configuration Update procedure

- Option 2: Resource Status Reporting Initiation and Resource Status Reporting procedure

- Option 3: Newly introduced procedure

For Option 1, the measurement resource configuration and/or strongest DL beam information may be captured in XN SETUP REQUEST/RESPONSE message and NG-RAN NODE CONFIGURATION UPDATE/ UPDATE ACKNOWLEDGE message. Note that for SSB configuration, it can be already contained in the Measurement Timing Configuration IE signalled as part of the Served Cell Information NR IE. For NZP-CSI-RS configuration, it is already specified in TS 38.331 as part of the NZP-CSI-RS-Resource IE and NZP-CSI-RS-ResourceSet IE. To this end, new IE called NZP-CSI-RS Configuration can be introduced in Served Cell Information NR IE. For the exchange of strongest DL beam information, a new IE CLI Measurement Report can be introduced in NG-RAN NODE CONFIGURATION UPDATE and NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message

For Option 2, the RESOURCE STATUS REQUEST message and RESOURCE STATUS UPDATE message can be enhanced to support the exchange measurement resource configuration and strongest DL beam information. With the consideration that currently RESOURCE STATUS REQUEST message and RESOURCE STATUS UPDATE message is used to initiate the requested measurement and report the results of the requested measurements respectively, it can be taken into consideration to support the exchange of measurement resource configuration and strongest DL beam information by enhancing them. Apart from this, there are also some similar IEs/fields in RESOURCE STATUS REQUEST message and RESOURCE STATUS UPDATE message that can be referenced when designing new IEs, such as SSB To Report List IE(s).

For Option 3, a new procedure can be designed to exchange of measurement resource configuration and strongest DL beam information in SBFD. Regarding Option 3, the benefits of introducing a new procedure to exchange of measurement resource configuration and strongest DL beam information should be considered.

Observation 5: For SSB configuration, it is already contained in the Measurement Timing Configuration IE signalled as part of the Served Cell Information NR IE. For NZP CSI-RS configuration, it is already specified in TS 38.331 as part of the NZP-CSI-RS-Resource IE and NZP-CSI-RS-ResourceSet IE.

Observation 6: Regarding Option 3, the benefits of introducing a new procedure to exchange of measurement resource configuration and strongest DL beam information should be considered.

Proposal 4: RAN3 should discuss three options for gNBs conducting the exchange of measurement resource configuration and strongest DL beam information as following:

- Option 1: Xn Setup and/or NG-RAN node Configuration Update procedure

- Option 2: Resource Status Reporting Initiation and Resource Status Reporting procedure

- Option 3: Newly introduced procedure

Exchange of CLI-mitigation request

In the revised WID [2], it has been stated that:

- Notes: Following are assumed for information exchange among gNBs - No normative behavior is implied for the gNB receiving the CLI-mitigation request. No need to further send a stop message for CLI mitigation. Detailed signalling can be discussed in RAN3

Therefore, RAN3 only need to discuss how to exchange the CLI-mitigation request, not the behavior of the gNB after receiving it. Similar to the analysis given in other sections, there are also two possible options for exchanging of CLI-mitigation request:

- Option 1: Existing procedure, e.g. NG-RAN node Configuration Update procedure, etc.

- Option 2: Newly introduced procedure

Whatever Option 1 or Option 2, a new IE/field which indicates CLI-mitigation is requested should be designed. With the consideration that no normative behavior is implied for the gNB receiving the CLI-mitigation request, we suggest to exchange the CLI-mitigation request using existing procedure, e.g. NG-RAN node Configuration Update procedure, etc.

Observation 7: RAN3 only needs to discuss how to exchange the CLI-mitigation request, not the behavior of the gNB after receiving it.

Proposal 5: RAN3 should discuss two possible options for exchanging of CLI-mitigation request:

- Option 1: Existing procedure, e.g. NG-RAN node Configuration Update procedure, etc.

- Option 2: Newly introduced procedure

Option 1 is preferred.

Conclusions

Based on the discussions mentioned above, in this contribution we provide some discussions on random access in SBFD:

Observation 1: For the exchange of semi-static cell-specific SBFD time and frequency location configuration among gNBs, it should be conducted when Xn-C interface established or when the cell-specific SBFD time and frequency location configuration changes.

Observation 2: The time configuration of TDD cell is contained in TDD UL-DL Configuration Common NR IE in TDD Info of Served Cell Information NR currently.

Observation 3: The frequency configuration of TDD cell is contained in Frequency Info and Transmission Bandwidth or Carrier List IE in TDD Info of Served Cell Information NR currently.

Observation 4: For the semi-static cell-specific SBFD time and frequency location configuration design, it is closely related to RAN2’s progress on the configuration methods which have not been made.

Observation 5: For SSB configuration, it is already contained in the Measurement Timing Configuration IE signalled as part of the Served Cell Information NR IE. For NZP CSI-RS configuration, it is already specified in TS 38.331 as part of the NZP-CSI-RS-Resource IE and NZP-CSI-RS-ResourceSet IE.

Observation 6: Regarding Option 3, the benefits of introducing a new procedure to exchange of measurement resource configuration and strongest DL beam information should be considered.

Observation 7: RAN3 only needs to discuss how to exchange the CLI-mitigation request, not the behavior of the gNB after receiving it.

Proposal 1: The semi-static cell-specific SBFD time and frequency location configuration can be contained in Xn Setup procedure and NG-RAN node Configuration Update procedure.

Proposal 2: From RAN3 perspective, a new IE named SBFD configuration NR which contains SBFD Time Info and SBFD Frequency Info can be introduced in Served Cell Information NR IE. FFS the details of SBFD Time Info and SBFD Frequency Info

Proposal 3: The enhancement on F1 interface can be referred to Xn interface.

Proposal 4: RAN3 should discuss three options for gNBs conducting the exchange of measurement resource configuration and strongest DL beam information as following:

- Option 1: Xn Setup and/or NG-RAN node Configuration Update procedure

- Option 2: Resource Status Reporting Initiation and Resource Status Reporting procedure

- Option 3: Newly introduced procedure

Proposal 5: RAN3 should discuss two possible options for exchanging of CLI-mitigation request:

- Option 1: Existing procedure, e.g. NG-RAN node Configuration Update procedure, etc.

- Option 2: Newly introduced procedure

Option 1 is preferred.

References

- RAN1 Chair’s Notes, RAN1#118, Maastricht, NL.

- RP-241614, “Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, RAN#104,  Huawei.