3GPP TSG-RAN WG3 Meeting #126	R3-247666

Orlando, USA, Nov 18th – 22th, 2024

Agenda Item:	10.3.1

Source:	CMCC

Title:	(TP to 38.300) Discussion on SON/MDT enhancement for NTN

Document for:	Discussion

# 1 Introduction

In RAN3#125bis meeting, there was discussion on SON/MDT enhancement for Intra-NTN mobility. Some agreements were achieved and some issues were captured [1] and need to be continued in next meeting:

For location based trigger condition:

Too late CHO Execution: RLF happens in the source cell before CHO execution. The UE re-establishes in a cell other than the source cell.

Too Early CHO Execution/CHO Execution to Wrong Cell: HOF happens or RLF happens shortly after successful CHO, and the UE re-connects in the source cell or a third cell.

For earth fixed case, there is no need to report the measured distance from the UE.

FFS on the earth moving case.

For joint trigger condition:

Case 1: Too Late CHO Execution: RLF happens in the source cell. The UE re-establishes in the cell other than the source cell.

Case 2: Too Early CHO Execution/CHO Execution to Wrong Cell: HOF happens or RLF happens in the target cell short after successful HO. The UE re-establishes in the source cell or a third cell.

UE reports the fulfilled CHO trigger conditions before RLF occurs in case of “time and measurement based trigger condition” or “location and measurement based trigger condition”, it is up to RAN2 to decide to reuse the existing IE or not.

In this contribution, we would discuss on the issues of MRO for failure scenario for intra-NTN and our initial consideration on MRO for near failure scenario.

# 2 Discussion

2.1 MRO for failure scenarios

Based on the following agreements achieved in RAN3#125 meeting [2], we can see that there are two cases (Too Late CHO Execution, Too Early CHO Execution/CHO Execution to Wrong Cell) in MRO for time-based CHO.

MRO for CHO with a time-based trigger condition:

Case 1 Too Late CHO Execution: RLF happens in the source cell before UE executes CHO (before or after T1 threshold starts), and the UE re-connects the cell other than the source cell.

- Distinguish if the RLF happens before or after T1

Case 2 Too Early CHO Execution/CHO Execution to Wrong Cell: HOF happens or RLF happens shortly after successful CHO, and the UE re-connects in the source cell or a third cell.

Case 1 (Too Late CHO Execution) can be further divided into RLF happens before and after T1 threshold starts. Network could consider to move T1 threshold to early, so that UEs could start the CHO execution earlier for RLF happens before T1 threshold starts in Case 1. For RLF happens after T1 threshold starts in Case 1, it may lead to RLF happens before CHO execution that the value of Duration is set too large. Network could consider to decrease the value of the Duration in this sub-case. For the case of Too Early CHO Execution in Case 2, CHO execution before HOF happens or RLF happens. Network could consider to move T1 threshold to right to avoid UEs start the CHO execution early. For the case of CHO Execution to Wrong Cell, UE successfully executes CHO before RLF happens and the UE re-connects in a third cell. So it seems the candidate cell of CHO needs to be optimized rather than the configuration of T1 threshold and Duration.

Proposal 1: The objective of MRO for time-based CHO is to optimize configuration issues of T1 threshold, Duration or CHO candidate cell.

For location based trigger condition:

Too late CHO Execution: RLF happens in the source cell before CHO execution. The UE re-establishes in a cell other than the source cell.

Too Early CHO Execution/CHO Execution to Wrong Cell: HOF happens or RLF happens shortly after successful CHO, and the UE re-connects in the source cell or a third cell.

For earth fixed case, there is no need to report the measured distance from the UE.

FFS on the earth moving case.

As per the above agreements achieved in last RAN3 meeting, we can see that there also are two cases (Too late CHO Execution and Too Early CHO Execution/CHO Execution to Wrong Cell) in the scenario where a location-based trigger condition configured. For this scenario, we think the information about the distance between UE and the referenceLocation/moving referenceLocation in serving cell and in candidate cell respectively when the failure occurs is helpful to analysis whether the distance thresholds configured in location-based trigger condition have the potential problem.

Observation 1: For location based trigger condition, the distance between UE and the referenceLocation/moving referenceLocation in serving cell and in candidate cell respectively when the failure occurs is helpful to analysis whether the distance thresholds configured in location-based trigger condition have the potential problem.

Proposal 2: The objective of MRO for location-based CHO is to optimize configuration issues of the distance thresholds in trigger condition.

For earth fixed case, Network can get to know the UE location based on the existing IE locationInfo in RLF report and derive the distance between UE and the referenceLocation in serving cell and in candidate cell respectively. Therefore, there is no need to report the measured distance from the UE.

For the earth moving case, the moving reference location based on movingReferenceLocation and the corresponding satellite ephemeris and epoch time that are broadcast in SIB19 for the serving cell. So we think it is hard to derive directly the distance between UE and the moving referenceLocation in serving cell and in candidate cell based on IE locationInfo in RLF report.

Observation 2: It is hard to derive directly the distance between UE and the moving referenceLocation in serving cell and in candidate cell based on IE locationInfo in RLF report.

Proposal 3: For location-based trigger condition in intra-NTN mobility, RLF report could be enhanced to include the distance between UE and the moving referenceLocation in serving cell and in candidate cell when the failure occurs, for the earth moving case.

For multiple trigger conditions configured (e.g., a location-based and Measurement-based combined trigger condition), we think the distance between UE and the moving referenceLocation in serving cell and in candidate cell when the failure occurs is also needed for the earth moving case.

Proposal 4: For location-based and Measurement-based combined trigger conditions in intra-NTN mobility, RLF report could be enhanced to include the distance between UE and the moving referenceLocation in serving cell and in candidate cell when the failure occurs, for the earth moving case.

Based on the agreements about the cases description of MRO for failure scenarios with a time-based trigger condition, location based trigger condition and joint trigger condition in RAN3#125 meeting and RAN3#125bis meeting, the corresponding TP for TS38.300 is attached in the Annex.

2.2 MRO for near failure scenarios

Based on the agreements achieved in RAN3#123bis meeting [3], the work on MRO for near failure scenarios could start after failure scenarios. Considering the progress of normative work on MRO for failure scenarios, we propose to start the work on optimization for near failure scenarios.

MRO mechanisms for intra-NTN should be addressed based on failure scenarios first, work on near failure scenarios later.

For near failure scenarios, Successful Handover Report (SHR) could be enhanced to support successful Conditional handovers in NTN to detect a sub-optimal successful CHO event. Considering the time-based and location-based trigger condition for CHO are supported in NTN, a new time-based SHR trigger and a location-based SHR trigger configured by network could be considered accordingly to trigger the SHR report when UE execute CHO successfully to a candidate cell with high risk of failure. Specifically,

- A new time-based SHR trigger for NTN could be the threshold(s) of time (e.g., in terms of percentage values) used to trigger the SHR report. For example, UE collects SHR if the ratio between the value of elapsed time since t1(t1 is configured by NW in CondEventT1) until UE execute CHO and the value of Duration (Duration is configured by NW in CondEventT1) is greater than the threshold, and makes the SHR available to the network.

- A new location-based SHR trigger for NTN could be the threshold(s) of distance used to trigger the SHR report. For example, in the case that the threshold of distance is in terms of percentage values, UE collects SHR if the ratio between the value of distance between UE and the referenceLocation/moving referenceLocation in serving cell when UE execute CHO, and the threshold configured by NW is greater than the location-based SHR trigger, and makes the SHR available to the network.

Proposal 5: Successful Handover Report (SHR) could be enhanced to support MRO for near failure scenarios in NTN.

Proposal 6: The introduction of a new time-based SHR trigger and a location-based SHR trigger could be considered for optimization of near failure scenarios in NTN.

Based on initial analysis, we think that the following information maybe helpful for optimization of CHO trigger conditions in NTN and could be logged by UE in the enhanced SHR.

- The time point that the first CHO trigger condition is satisfied both for one or multiple CHO trigger conditions are configured by network; this information helps to identify whether too late CHO may occur and optimize T1 threshold or Duration in the time-based trigger condition or distance thresholds/timeToTrigger in location-based trigger condition or the Thresholds in CHO events A3/A4/A5.

- The first CHO trigger condition and the time elapsed between the two triggered conditions, if multiple CHO trigger conditions are configured by network; this information helps to identify whether the second CHO trigger condition satisfied late that may cause too late CHO and optimize Duration in the time-based trigger condition or distance thresholds/timeToTrigger in location-based trigger condition or the Thresholds in CHO events A3/A4/A5.

- The distance between UE and the moving referenceLocation in serving cell and the distance between UE and the moving referenceLocation in candidate cell when timetotrigger configured in CHO trigger condition are fulfilled; the information could be used for location-based trigger condition to identify whether too late CHO or too early CHO may occur and optimize distance thresholds or timeToTrigger in location-based trigger condition, for the earth moving case.

- The cause value that triggers the SHR; this information helps to know which configurations of CHO trigger condition have potential problems.

Proposal 7: RAN3 to consider SHR enhancement to support optimization of near failure scenarios in NTN, e.g.,

- The time point that the first CHO trigger condition is satisfied both for one or multiple CHO trigger conditions are configured by network;

- The first CHO trigger condition and the time elapsed between the two triggered conditions, if multiple CHO trigger conditions are configured by network;

- The distance between UE and the moving referenceLocation in serving cell and the distance between UE and the moving referenceLocation in candidate cell when timetotrigger configured in CHO trigger condition are fulfilled, for the earth moving case;

- The cause value that triggers the SHR, e.g., the threshold(s) of time in time-based SHR trigger and the threshold(s) of distance in location-based SHR trigger.

# 3 Conclusion

In this paper, we discuss on MRO mechanisms for intra-NTN mobility, we have the following observations and proposals:

MRO for failure scenarios

Proposal 1: The objective of MRO for time-based CHO is to optimize configuration issues of T1 threshold, Duration or CHO candidate cell.

Observation 1: For location based trigger condition, the distance between UE and the referenceLocation/moving referenceLocation in serving cell and in candidate cell respectively when the failure occurs is helpful to analysis whether the distance thresholds configured in location-based trigger condition have the potential problem.

Proposal 2: The objective of MRO for location-based CHO is to optimize configuration issues of the distance thresholds in trigger condition.

Observation 2: It is hard to derive directly the distance between UE and the moving referenceLocation in serving cell and in candidate cell based on IE locationInfo in RLF report.

Proposal 3: For location-based trigger condition in intra-NTN mobility, RLF report could be enhanced to include the distance between UE and the moving referenceLocation in serving cell and in candidate cell when the failure occurs, for the earth moving case.

Proposal 4: For location-based and Measurement-based combined trigger conditions in intra-NTN mobility, RLF report could be enhanced to include the distance between UE and the moving referenceLocation in serving cell and in candidate cell when the failure occurs, for the earth moving case.

MRO for near failure scenarios

Proposal 5: Successful Handover Report (SHR) could be enhanced to support MRO for near failure scenarios in NTN.

Proposal 6: The introduction of a new time-based SHR trigger and a location-based SHR trigger could be considered for optimization of near failure scenarios in NTN.

Proposal 7: RAN3 to consider SHR enhancement to support optimization of near failure scenarios in NTN, e.g.,

- The time point that the first CHO trigger condition is satisfied both for one or multiple CHO trigger conditions are configured by network;

- The first CHO trigger condition and the time elapsed between the two triggered conditions, if multiple CHO trigger conditions are configured by network;

- The distance between UE and the moving referenceLocation in serving cell and the distance between UE and the moving referenceLocation in candidate cell when timetotrigger configured in CHO trigger condition are fulfilled, for the earth moving case;

- The cause value that triggers the SHR, e.g., the threshold(s) of time in time-based SHR trigger and the threshold(s) of distance in location-based SHR trigger.

# Reference

- RAN3_125bis_agenda_20241018_1430_EOM, Oct 2024.

- RAN3_125_agenda_20240823EOM, Aug 2024.

- RAN3_123bis_agenda_20240419_EOM, Apr 2024.

# Annex: TP for TS38.300

15.5.2.2.2	Connection failure due to intra-system mobility

One of the functions of Mobility Robustness Optimization is to detect connection failures that occur due to Too Early or Too Late Handovers, or Handover to Wrong Cell. These problems are defined as follows:

-	Intra-system Too Late Handover: an RLF occurs after the UE has stayed for a long period of time in the cell; the UE attempts to re-establish the radio link connection in a different cell.

-	Intra-system Too Early Handover: an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in the source cell.

-	Intra-system Handover to Wrong Cell: an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in a cell other than the source cell and the target cell.

In the definition above, the "successful handover" refers to the UE state, namely the successful completion of the RA procedure.

In case of CHO, the Too Late Handover, Too Early Handover and Handover to Wrong Cell in the definition above means Too Late CHO Execution, Too Early CHO Execution and CHO Execution to Wrong Cell.

<<<<<< Start of CHANGES >>>>>>

In case of CHO in Intra-NTN mobility (i.e., CHO with a time-based trigger condition, a location based trigger condition and joint trigger condition), Too Late CHO Execution means RLF happens in the source cell before UE executes CHO, and the UE re-connects the cell other than the source cell. Too Early CHO Execution/CHO Execution to Wrong Cell means HOF happens or RLF happens shortly after successful CHO, and the UE re-connects in the source cell or a third cell.

<<<<<< End of CHANGES >>>>>>