3GPP TSG-RAN WG3 Meeting #126	R3-247673

Orlando, USA, 18th – 22th Oct 2024

Title: 	(TP to 38.401) OAM Requirements for Energy Saving for split architecture

Source: 	CMCC, Samsung

Agenda item: 	11.4

Document Type:	Other

- Introduction

In RAN3 #124 meeting, agreements as following have been achieved for split architecture:

Transferring Measured UE performance from CU-UP to CU-CP.

Measured Energy Cost can be transferred from DU to CU.

And in RAN3 #125-bis meeting, it has been agreed that:

Introducing Data Collection Reporting Initiation and Data Collection Reporting procedures over F1 interface to transfer measured EC from DU to CU.

2. TP to TS 38.401

7.11	Support of AI/ML for NG-RAN

7.11.1	General

The support of AI/ML for NG-RAN is specified in TS 38.300 [2].

In case of CU-DU split architecture, the following scenarios may be supported:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU.

-	AI/ML Model Training and Model Inference are both located in the gNB-CU.

And the node level Measured Energy Cost shall be calculated on gNB-CU to include all the consumed energy of gNB-DUs within the split gNB.

7.11.2	OAM requirements

OAM configures the following on gNB-CU:

-	the minimum and maximum energy consumption values corresponding to the minimum and maximum EC index values respectively, based on an implementation-specific mapping rule, which is unified within a defined area; and

-	the recommended time interval within which an gNB-CU selects an implementation-specific time window for averaging of the measurements of all the gNB-CU and gNB-DUs’ consumed energy for this NR-RAN node.

3. Reference

- 3GPP RAN3#125bis Chairman’s notes (RAN3_125bis_agenda_20241018_1430_EOM)

- RP-242385 WID Enhancements for Artificial Intelligence (AI) Machine Learning (ML) for NG-RAN (ZTE, NEC), 3GPP RAN#105

- R3-240998, Reply LS on Reply to LS on AI/ML for NG-RAN Energy Saving Energy Cost index (Nokia), 3GPP RAN#123

- R3-245773, OAM Requirements for EC stage2 CR(Nokia), 3GPP RAN#125bis

- TS 38.401, Architecture description