3GPP TSG-RAN WG3 Meeting #126	R3-247726

Orlando, USA, 18 – 22 November 2024

Agenda Item:	16.2

Source: 				LG Electronics Inc.

Title:			Discussion on RAN impact for Ambient IoT

Document for:		Discussion and Decision

1.	Introduction

New Rel-19 SID on solutions for Ambient IoT (Internet of Things) in NR was agreed at RAN #102 [1]. One of the objectives in this SID is to identify necessary impacts on RAN architecture aspects including whether support for split architecture is necessary as follows:

… - RAN3-led: - Identify necessary impacts on signaling and procedures for CN-RAN interface, to enable: - Paging - Device context management - Data transport - Identify RAN architecture aspects, including whether support for split architecture is necessary. - Identify potential solutions for locating an Ambient IoT device with no specification impact, e.g. reusing existing user location report, or minimal specification impact to convey location information to core network. …

In this contribution, we will discuss the remaining issue on RAN architecture aspects including whether support for split architecture is necessary, and provide our view on it.

2.	Discussion

In Clause 4.2.1 of TR 38.769 [2], RAN1 captured the following scenarios for the purpose of potential evaluation.

Table 4.2.1-1: Evaluation scenarios (in TR 38.769)

| Scenario | CW Inside/outside topology | Diagram of the scenario | Description of the scenario | Device 1/2a/2b | CW spectrum | D2R spectrum | R2D spectrum |
| --- | --- | --- | --- | --- | --- | --- | --- |
| D1T1-A1 | CW inside topology |  | CW node inside topology 1 ‘CW’ in CW2D and ‘R2’ in D2R are different ‘CW’ in CW2D and ‘R1’ in R2D are same ‘R1’ in R2D and ‘R2’ in D2R are different | Device 1, 2a | Case 1-1 (inside topology, DL) Case 1-2 (inside topology, UL) | Same as CW |  |
| D1T1-A2 |  |  | CW node inside topology 1 Same ‘CW’ and ‘R’ node for CW2D, D2R and R2D |  | Same as D1T1-A1 | Same as CW |  |
| D1T1-B | CW outside topology |  | CW node outside topology 1 ‘CW’ in CW2D and ‘R’ in D2R are different ‘CW’ in CW2D and ‘R’ in R2D are different ‘R’ in R2D and ‘R’ in D2R are same |  | Case 1-4 (outside topology, UL) | Same as CW |  |
| D1T1-C | No CW |  | No CW Node. | Device 2b | N/A | UL |  |
| D2T2-A1 | CW inside topology |  | CW node inside topology 2 ‘CW’ in CW2D and ‘R2’ in D2R are different ‘CW’ in CW2D and ‘R1’ in R2D are same ‘R1’ in R2D and ‘R2’ in D2R are different BS communicates with R1 and R2 | Device 1, 2a | Case 2-2 (inside topology, UL) | Same as CW |  |
| D2T2-A2 |  |  | CW node inside topology 2 Same ‘CW’ and ‘R’ node for CW2D, D2R and R2D BS communicates with R |  | Same as D2T2-A1 | Same as CW |  |
| D2T2-B | CW outside topology |  | CW node outside topology 2 ‘CW’ in CW2D and ‘R’ in D2R are different ‘CW’ in CW2D and ‘R’ in R2D are different ‘R’ in R2D and ‘R’ in D2R are same BS communicates with R |  | Case 2-3 (outside topology, DL) Case 2-4 (outside topology, UL) | Same as CW |  |
| D2T2-C | No CW |  | No CW Node. BS communicates with R | Device 2b | N/A | FFS |  |
| Note: This table is for the case where D2R is in the same spectrum as CW2D. |  |  |  |  |  |  |  |

One of possible scenarios considered in Topology 1 is that as shown in Figure 1, the reader involved in R2D transmission is different from that in D2R backscattering.

Figure 1: D1T1-A1 scenario [2]

In this scenario (i.e., D1T1-A1), a coordination between both readers may be needed. For example, when the A-IoT random access is triggered by the reader, the A-IoT Device can send the Msg1 including the random ID generated by the A-IoT Device to the reader. For contention resolution, the reader responds with the successfully received random ID in Msg2. Therefore, in D1T1-A1 case, the reader involved in R2D transmission needs to know the random ID included in Msg1. It is also possible that in D2R, the A-IoT Device may send the message size/status indication to the reader according to RAN2 agreement. This indication may be used by the reader in R2D to allocate the resource of subsequent D2R message.

Observation 1: In D1T1-A1 case, the coordination between the reader in R2D and the reader in D2R may be needed.

If non-split A-IoT RAN is considered and both readers are located at same A-IoT RAN, how to exchange the device related information (e.g., random ID generated by AIoT device) between both readers is up to RAN implementation. In addition, if A-IoT RAN only serves a single reader, it implies that in D1T1-A1 scenario, the reader in R2D and the reader in D2R connect to different BSs. Therefore, a new Xn-like signaling may be defined to support the coordination between both readers. Consequently, from RAN3 point of view, further discussion on whether and how to support the D1T1-A1 scenario is needed.

Proposal 1: In Topology 1, it is proposed to discuss whether and how to support the D1T1-A1 scenario (i.e., the reader involved in R2D transmission is different from that in D2R backscattering).

In Topology 2, it is also captured that the reader involved in R2D transmission is different from that in D2R backscattering as follow:

Figure 2: D2T2-A1 scenario [2]

In this scenario (i.e., D2T2-A1), as in Observation 1, the coordination between the reader in R2D and the reader in D2R is also needed. In Topology 2, since the reader resides in the A-IoT enabled UE, D2T2-A1 scenario implies that both readers connect to the different A-IoT enabled UEs. Therefore, in order to support the D2T2-A1 case, the reader in D2R should provide the device related information to the reader in R2D. As shown in Figure 2, these A-IoT enabled UEs are served by same A-IoT enabled gNB. However, there is no direct link between readers in different A-IoT enabled UEs. If the RRC-based solution is used in Topology 2, the device related information needs to be sent to the reader in R2D via the A-IoT enabled gNB. However, if NAS/UP-based solution is used, the reader in D2R may forward the information to the reader in R2D via the A-IoT CN, thus resulting in unnecessary latency. Therefore, we think that it is better for the reader in D2R to forward the device related information to the reader in R2D via the A-IoT enabled gNB.

Proposal 2: In Topology 2, the reader in D2R forwards the device related information to the reader in R2D via the A-IoT enabled gNB.

3.	Conclusion

In this contribution, we focused on the remaining issue on RAN architecture aspects including whether support for split architecture is necessary, and provided our view on it. The following proposals are kindly suggested to RAN3:

Proposal 1: In Topology 1, it is proposed to discuss whether and how to support the D1T1-A1 scenario (i.e., the reader involved in R2D transmission is different from that in D2R backscattering).

Proposal 2: In Topology 2, the reader in D2R forwards the device related information to the reader in R2D via the A-IoT enabled gNB.

4.	Reference

- RP-240826, “Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR,” CMCC, Huawei, T-Mobile USA, March, 2024.

- TR 38.769, “Study on solutions for ambient IoT (Internet of Things) (Release 19),” v1.0.0.